3GPP TSG-RAN WG3 #127	          R3-250224

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 36.412 and TS 36.413) Discussion on the support of regenerative architecture

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received the SA2 LS ([1]) on Support of Regenerative-based Satellite Access. RAN#106 updated the IoT NTN WID. This contribution analyses the impact to RAN3.

# 2	Discussion on SA2 LS and WID update

The SA2 LS ([1]) describes following:

1. Overall Description: SA2 thanks for the LS on Support of Regenerative-based Satellite Access. SA2 identified that the LS from RAN3 referred only to “5GS” while in the last paragraph also referred to “AMF/MME”. SA2 also noticed that the RAN WI “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” (RP-242397) did not contain objectives for Regenerative-based Satellite Access for E-UTRAN/EPC. SA2 would like to inform RAN3 that the conclusions for FS_5GSAT_Ph3 for “KI#1: Support of Regenerative-based satellite access” without Store and Forward in clause 8.1 of TR 23.700-29 apply to both E-UTRAN/EPS and NG-RAN/5GS and therefore the related RAN3 impacts can affect both S1 and NG procedures. On the contrary, conclusions for FS_5GSAT_Ph3 for “KI#2: Support of Store and Forward Satellite operation” in clause 8.2 of TR 23.700-29 apply only to E-UTRAN/EPS. 2. Actions: To RAN3, RAN: ACTION: RAN3 and RAN is requested to take the above information into account.

As described by the SA2 LS, the regenerative payload for E-UTRAN/EPS is NOT for Store and Forward, and it is similar to Rel-19 NR NTN regenerative payload. Similar as legacy release, the NR NTN conclusion can be reused for IoT NTN.

Based on current NR NTN progress, the following conclusion from NR NTN regenerative payload can be applied to IoT NTN regenerative payload.

- Interface management:

NR NTN agreed to introduce NG Removal procedure. For LTE, S1 Removal procedure need to be introduced.

- Knowledge of supported TAIs:

NR NTN agreed the supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. For LTE, the supported TAIs of an on-board eNB could be provided to MME by OAM/pre-configuration. SA2/SA5 need to be informed.

- Feeder Link Switch Over (FLSO)

NR NTN agreed that during the feeder link switch, AMF is not changed for the UEs.

For LTE NTN, the MME is not changed during the feeder link switch over.

- Soft Feeder Link Switch Over

NR NTN agreed current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.  In soft FLSO,

- the gNB/satellite have the “old” TNLA association via the “old” NTN Gateway.

- the gNB/satellite then setup the “new” TNLA association via the “new” NTN Gateway. The gNB informs the AMF for the new TNLA and remove the old TNLA by the RAN CONFIGURATION UPDATE message over the new TNLA Association.

- The gNB/satellite then disconnects from the “old” NTN Gateway.

So the soft FLSO actually only use the multiple TNL endpoints in the gNB (i.e. the old TNL endpoint related to the old NTN Gateway, and the new TNL endpoint related to the new NTN Gateway). The multiple TNL endpoints in MME is not required.

However, multiple TNLA is not supported in S1AP. If RAN3 agreed to support multiple TNLA, S1AP need to be enhanced to support multiple TNL endpoints in the eNB, and specify this is only for IoT NTN.

TP is needed for TS 36.412 and TS 36.413, to align with TS 38.412 and TS 38.413 for multiple TNLA association support in the eNB.

Any other aspects can wait for NR NTN discussion.

Proposal 1: Based on NR NTN conclusion, RAN3 agree following aspects:

- * introduce S1 Removal procedure

- * the supported TAIs of an on-board eNB could be provided to MME by OAM/pre-configuration.

- * the MME is not changed during the feeder link switch over.

- * Introduce multiple TNL endpoints in the eNB for S1AP.

TS 36.412 TP can be found in Annex A – TP for TS 36.412

TS 36.413 TP can be found in Annex B – TP for TS 36.413

# 3	Conclusion

In this contribution, we discussed how to support regenerative payload for E-UTRAN/EPS. Our proposals are:

Proposal 1: Based on NR NTN conclusion, RAN3 agree following aspects:

- * introduce S1 Removal procedure

- * the supported TAIs of an on-board eNB could be provided to MME by OAM/pre-configuration.

- * the MME is not changed during the feeder link switch over.

- * Introduce multiple TNL endpoints in the eNB for S1AP.

TS 36.412 TP can be found in Annex A – TP for TS 36.412

TS 36.413 TP can be found in Annex B – TP for TS 36.413

# References

- R3-247016/S2-2410918 Reply LS on Support of Regenerative-based Satellite Access

# Annex A – TP for TS 36.412

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply.

eNB	E-UTRAN Node B

EPC

DiffServ	Differentiated Service

IP	Internet Protocol

MME	Mobility Management Entity

NTN 		Non-Terrestrial Network

PPP	Point to Point Protocol

SCTP	Stream Control Transmission Protocol

-----------------Next Change-------------------

# 7	Transport layer

SCTP (IETF RFC 4960 [5]) shall be supported as the transport layer of S1-MME signalling bearer. The Payload Protocol Identifier (ppid) to be used by SCTP for the application layer protocol S1AP is assigned by IANA in [6]. The byte order of the ppid shall be big-endian.

SCTP refers to the Stream Control Transmission Protocol developed by the Sigtran working group of the IETF for the purpose of transporting various signalling protocols over IP network.

There shall be only one SCTP association established between one MME and eNB pair.

In NTN with regenerative payload, configurations with multiple SCTP endpoints per eNB should be supported. When the configuration with multiple SCTP endpoints per eNB is supported and the eNB wants to add additional SCTP endpoints, the eNB configuration update procedure shall be the first S1AP procedure triggered on an additional TNLA of an already setup S1-C interface instance after the TNL association has become operational, and the MME shall associate the TNLA to the S1-C interface instance using the included Global eNB ID.

The eNB shall establish the SCTP association. The SCTP Destination Port number value to be used by S1AP is assigned by IANA in [7].

Within the SCTP association established between one MME and eNB pair:

-	a single pair of stream identifiers shall be reserved for the sole use of S1AP elementary procedures that utilize non UE-associated signalling.

-	At least one pair of stream identifiers shall be reserved for the sole use of S1AP elementary procedures that utilize UE-associated signallings. However a few pairs (i.e. more than one) should be reserved.

-	A single UE-associated signalling shall use one SCTP stream and the stream should not be changed during the communication of the UE-associated signalling.

Transport network redundancy may be achieved by SCTP multi-homing between two end-points, of which one or both is assigned with multiple IP addresses. SCTP end-points shall support a multi-homed remote SCTP end-point. For SCTP endpoint redundancy an INIT may be sent from MME or eNB, at any time for an already established SCTP association, which shall be handled as defined in IETF RFC 4960 [5] in subclause 5.2.

The SCTP congestion control may, using an implementation specific mechanism, initiate higher layer protocols to reduce the signalling traffic at the source and prioritise certain messages.

# Annex B – TP for TS 36.413

### 8.7.4	eNB Configuration Update

#### 8.7.4.1	General

The purpose of the eNB Configuration Update procedure is to update application level configuration data needed for the eNB and the MME to interoperate correctly on the S1 interface. This procedure does not affect existing UE-related contexts, if any.

#### 8.7.4.2	Successful Operation

Figure 8.7.4.2-1: ENB Configuration Update procedure: Successful Operation.

The eNB initiates the procedure by sending an ENB CONFIGURATION UPDATE message to the MME including an appropriate set of updated configuration data that it has just taken into operational use. The MME responds with ENB CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If information element(s) is/are not included in the ENB CONFIGURATION UPDATE message, the MME shall interpret that the corresponding configuration data is/are not changed and shall continue to operate the S1 with the existing related configuration data.

If the supported TA(s) is/are to be updated, the whole list of supported TAs, including those that are not to be updated, shall be included in the Supported TAs IE. The MME shall overwrite the whole list of TAs.

If the supported CSG ID(s) is/are to be updated, the whole list of supported CSG IDs, including those that are not to be updated, shall be included in the CSG Id List IE. The MME shall overwrite the whole list of CSG Ids.

If the ENB CONFIGURATION UPDATE message contains the eNB Name IE, the MME may use this IE as a human readable name of the eNB.

If the Default Paging DRX IE is included, the MME shall overwrite any previously stored default paging DRX value for the eNB.

If the NB-IoT Default Paging DRX IE is included in the ENB CONFIGURATION UPDATE message, the MME shall overwrite any previously stored NB-IoT default paging DRX value for the eNB.

If the Connected en-gNB to be Added List IE is included in the ENB CONFIGURATION UPDATE message, the MME shall replace, if applicable, any previously received information for the concerned en-gNBs and take it into account as specified in TS 36.300 [14].

If the Connected en-gNB to be Removed List IE is included in the ENB CONFIGURATION UPDATE message, the MME shall remove any stored information for the concerned en-gNBs.

The updated configuration data shall be stored in both the eNB and the MME and used for the duration of the TNL association or until any further update is triggered by the eNB.

If the Global eNB ID IE is included in the ENB CONFIGURATION UPDATE message, the MME shall associate the TNLA to the S1-C interface instance using the Global eNB ID.

If the ENB CONFIGURATION UPDATE message includes the eNB TNL Association to Remove List IE, the MME shall, if supported, initiate removal of the TNL association(s) indicated by eNB TNL endpoint(s).

The eNB may initiate a further eNB Configuration Update procedure only after a previous eNB Configuration Update procedure has been completed.

-----------------Next Change-------------------

#### 9.1.8.7	ENB CONFIGURATION UPDATE

This message is sent by the eNB to transfer updated information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| eNB Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Supported TAs |  | 0..<maxnoofTACs> |  | Supported TAs in the eNB. | GLOBAL | reject |
| >TAC | M |  | 9.2.3.7 | Broadcast TAC. | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >RAT-Type | O |  | 9.2.1.117 | RAT Type associated with the TAC of the indicated PLMN(s). | YES | reject |
| CSG Id List |  | 0..1 |  |  | GLOBAL | reject |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Default Paging DRX | O |  | 9.2.1.16 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.2.1.114 |  | YES | ignore |
| Connected en-gNB To Be Added List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list of the added en-gNB. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 |  | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| Connected en-gNB To Be Removed List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list of the en-gNB to be removed. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 |  | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| Global eNB ID | O |  | 9.2.1.37 |  | YES | ignore |
| eNB TNL Association to Remove List |  | 0..1 |  |  | YES | ignore |
| >eNB TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>eNB Transport Layer Address | M |  | CP Transport Layer Address 9.2.2.1 | Transport layer address of the eNB | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 6. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |
| maxnoofConnecteden-gNBs | Maximum no. of en-gNBs connected to the eNB. Value is 256. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the eNB and the MME. Value is 32. |

-----------------Next Change-------------------

### 9.3.3	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UEAggregateMaximumBitrate,

BearerType,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

SecurityResult,

LTE-NTN-TAI-Information,

CoarseUELocationRequested,

CoarseUELocation,

ENB-TNLAssociationToRemoveList

FROM S1AP-IEs

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-CoarseUELocationRequested,

id-CoarseUELocation,

id-ENB-TNLAssociationToRemoveList

FROM S1AP-Constants;

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- eNB Configuration Update

--

-- **************************************************************

ENBConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ENBConfigurationUpdateIEs} },

...

}

ENBConfigurationUpdateIEs S1AP-PROTOCOL-IES ::= {

{ ID id-eNBname						CRITICALITY ignore	TYPE ENBname					PRESENCE optional}|

{ ID id-SupportedTAs				CRITICALITY reject	TYPE SupportedTAs				PRESENCE optional}|

{ ID id-CSG-IdList					CRITICALITY reject	TYPE CSG-IdList					PRESENCE optional}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX	PRESENCE optional}|

{ ID id-ConnectedengNBToAddList		CRITICALITY ignore	TYPE ConnectedengNBList			PRESENCE optional}|

{ ID id-ConnectedengNBToRemoveList	CRITICALITY ignore	TYPE ConnectedengNBList			PRESENCE optional}|

{ ID id-Global-ENB-ID				CRITICALITY ignore	TYPE Global-ENB-ID				PRESENCE optional}|

{ ID id-ENB-TNLAssociationToRemoveList 	CRITICALITY ignore	TYPE ENB-TNLAssociationToRemoveList 		PRESENCE optional},

...

}

-----------------Next Change-------------------

### 9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RABInformationListItem,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo,

maxnoofTACsInNTN,

maxnoofSensorName,

maxnoofTNLAssociations

FROM S1AP-Constants

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

ENB-StatusTransfer-TransparentContainer		::= SEQUENCE {

bearers-SubjectToStatusTransferList		Bearers-SubjectToStatusTransferList,

iE-Extensions			ProtocolExtensionContainer { {ENB-StatusTransfer-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

ENB-StatusTransfer-TransparentContainer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-TNLAssociationToRemoveList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF ENB-TNLAssociationToRemoveItem

ENB-TNLAssociationToRemoveItem ::= SEQUENCE {

eNB-TransportLayerAddress 		TransportLayerAddress,

iE-Extensions 					ProtocolExtensionContainer { {ENB-TNLAssociationToRemoveItem-ExtIEs} } OPTIONAL,

...

}

ENB-TNLAssociationToRemoveItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-UE-S1AP-ID				::= INTEGER (0..16777215)

-----------------Next Change-------------------

### 9.3.6	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo		INTEGER ::= 8

maxnoofTACsInNTN								INTEGER ::= 12

maxnoofSensorName								INTEGER ::= 3

maxnoofTNLAssociations							INTEGER ::= 32

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-Bearers-SubjectToDLDiscardingList				ProtocolIE-ID ::= 352

id-CoarseUELocationRequested						ProtocolIE-ID ::= 353

id-CoarseUELocation									ProtocolIE-ID ::= 354

id-ENB-TNLAssociationToRemoveList					ProtocolIE-ID ::= a

END

-- ASN1STOP

-----------------End of Change-------------------