3GPP TSG-RAN WG3 #127	          R3-250225

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	22.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the support of multi-hop Layer-2 UE-to-Network relay

Document for:	Discussion and Decision

# 1	Introduction

RAN WID ([1]) describes following RAN3 related objective:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay… - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching …

This contribution analyses the impact to RAN3.

# 2	multi-hop authorization

SA2 TS 23.304 has agreed following:

- Provision the NG-RAN with indication about the UE authorization status about the following:

…

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay);

-	5G ProSe Layer-2 multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay, as 5G ProSe Layer-2 Intermediate UE-to-Network Relay, as 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay).

To support SA2 decision, it is suggested to update current NGAP/XnAP/F1AP 5G ProSe Authorized IE to include the new authorization indication for 5G ProSe Layer-3 multi-hop UE-to-Network Relay, 5G ProSe Layer-2 multi-hop UE-to-Network Relay, 5G ProSe Layer-2 Intermediate UE-to-Network Relay, and 5G ProSe Layer-2 multi-hop UE-to-Network Remote.

Proposal 1: update current NGAP/XnAP/F1AP 5G ProSe Authorized IE to include the new authorization indication for 5G ProSe Layer-3 multi-hop UE-to-Network Relay, 5G ProSe Layer-2 multi-hop UE-to-Network Relay, 5G ProSe Layer-2 Intermediate UE-to-Network Relay, and 5G ProSe Layer-2 multi-hop UE-to-Network Remote.

TP for TS 38.413 can be found in ([2])

# 3	QoS

Similar to legacy U2N Relay, the gNB-CU determine the QoS split for Uu leg and PC5 leg. The only difference is the potential multiple PC5 legs. So the gNB-CU need to consider the number of hops to the Remote UE when perform QoS split. For example, the gNB-CU derive the maximum packet delay budget that applies to the PC5 link over each adjacent hop from the end-to-end QoS info based on a hop adjustment factor, e.g. 1/5 of the standardized PDB value of the PQI for hop count =5. This may require a small clarification in Stage-2 (e.g. TS 38.470).

Proposal 2: the gNB-CU need to consider the number of hops to the remote UE when performing QoS split.

# 4	Control Plane procedures

RAN2 reached following agreements:

Agreement:

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

All agreements in this WI apply to both these cases unless otherwise specified.

However, RAN3 impact (e.g. Stage-2 spec update) is still unclear. We prefer to wait for RAN2 progress.

Proposal 3: RAN3 wait for RAN2 progress on control plane procedures.

# 5	intra-gNB service continuity

At current stage, RAN3 only need to consider Scenario A and Scenario B.

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

For Scenario A, there is no RAN3 impact.

For Scenario B, in case of inter-gNB-DU path switch, it is similar to Rel-18 inter-gNB-DU indirect to indirect path switch. Existing F1AP interface can be reused, and there may be no RAN3 impact. Considering RAN2 is still studying it, RAN3 can wait for RAN2 progress.

Proposal 4: there may be no RAN3 impact to support Scenario A and Scenario B. Further RAN3 impact can wait for RAN2 progress.

# 6	Conclusion

In this contribution, we discussed how to support multi-hop UE-to-Network Relay. Our proposals are:

Proposal 1: update current NGAP/XnAP/F1AP 5G ProSe Authorized IE to include the new authorization indication for 5G ProSe Layer-3 multi-hop UE-to-Network Relay,5G ProSe Layer-2 multi-hop UE-to-Network Relay, 5G ProSe Layer-2 Intermediate UE-to-Network Relay, and 5G ProSe Layer-2 multi-hop UE-to-Network Remote.

TP for TS 38.413 can be found in ([2])

Proposal 2: the gNB-CU need to consider the number of hops to the remote UE when performing QoS split.

Proposal 3: RAN3 wait for RAN2 progress on control plane procedures.

Proposal 4: there may be no RAN3 impact to support Scenario A and Scenario B. Further RAN3 impact can wait for RAN2 progress.

# References

- RP-242349, Revised WID on NR sidelink multi-hop relay

- R3-250226, (TP for TS 38.413) support for multi-hop UE-to-Network Relay