3GPP TSG-RAN WG3 Meeting #127	R3-250236

Athens, Greece, 17 - 21 February 2025

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for BL CR for TS 38.300) Functional split for NR Femto

Document for:	Discussion

1	Introduction

This paper provides the TP for capturing the functional supported by the NR femto GW.

Annex 1——TP for TS 38.300

Start of Change

4	Overall Architecture and Functional Split

## 4.X	Support of NR Femtos

### 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto to the 5GC.

Figure 4.X.1-1: NR Femto Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW.

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto and the NR Femto GW;

-	Between the NR Femto and the Core Network;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW.

4.X.2	Functional Split

An NR Femto hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE: The NR Femto may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto shall only connect to a single NR Femto GW at one time:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

NOTE: NR Femtos may be deployed without network planning. An NR Femto may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-	The TAC and PLMN ID(s) used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is performed by the NR Femto GW:

-	AMF selection is based on the GUAMI or requested NSSAI, if received from the UE, provided to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message.

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the PATH SWITCH REQUEST message.

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NGAP messages between the AMF serving the UE and the NR Femto, except the UE CONTEXT RELEASE REQUEST message received from the NR Femto with an explicit GW Context Release Indication. In that case, the NR Femto GW terminates the NG UE Context Release Request procedure and releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by an NR Femto attached to it. Otherwise it ignores the message.

-	In case of INITIAL CONTEXT SETUP REQUEST message and NG HANDOVER REQUEST message, informing the NR Femto about any GUAMI corresponding to the serving AMF, the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE. In case of PATH SWITCH REQUEST ACKNOWLEDGE message, informing the NR Femto about the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	Terminating non-UE associated NG application part procedures towards the NR Femto and towards the AMF.

Editor’s Note: Whether the NR Femto GW verifies the identity of NR Femto in case of NG SETUP REQUEST message, PWS RESTART INDICATION message and PWS FAILURE INDICATION message is pending SA3 confirmation.

-	Upon receiving an OVERLOAD START/STOP message, the NR Femto GW should send the OVERLOAD START/STOP message towards the NR Femto(s) including in the message the identities of the affected AMF node. The NR Femto uses this information received from the OVERLOAD START message to identify to which traffic the above defined rejections shall be applied. The NR Femto shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

NOTE:	If a NR Femto GW is deployed, non-UE associated procedures shall be run between NR Femtos and the NR Femto GW and between the NR Femto GW and the AMF.

-	Optionally transport NG-U interface between the NR Femto and the UPF.

-	Supporting TAC and PLMN ID used by the NR Femto.

-	Xn interfaces shall not be established between the NR Femto GW and other nodes.

-	Routing the NG PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto.

End of Change