3GPP TSG-RAN WG3 Meeting #127	R3-250238

# Athens, Greece, 17th – 21st February 2025

Agenda Item:	13.2

Source:	Google

Title:	(TP to BL CR to 38.300 and 38.423) Support for reference configurations for inter-CU LTM

Document for:	Discussions & Approval

# 1	Discussions

In Rel-19 LTM in RAN3#125bis, the following To Be Continued item is captured.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

In RAN3#126 we had some discussions on the reference configuration topic, but only concluded not pursuing the multiple reference configurations (i.e., aligning the RAN2 agreement). As for how the reference configuration comes from, there were arguments that the reference configuration has to be the current UE configuration in use between the UE and the gNB according to the current RAN2 specification (i.e., TS 38.331). However, as excerpted below, when the LTM cell switch is triggered, the UE first releases/clears all current dedicated and common radio configurations and applies either the ltm-CandidateConfig (if the ltm-ConfigComplete is included in the ltm-Candidate) or the ltm-CandidateConfig on top of the ltm-ReferenceConfiguration. This is actually more like the actions of when full configuration is performed. As LTM supports subsequent mobility by default, the serving gNB changes after inter-CU MCG LTM cell switch. It is unlike the immediate mobility among the same vendor gNBs that delta configuration can be kept added after each handover. Therefore, it is not like the immediate mobility or the (one-time) conditional mobility case that the configuration to apply after the mobility is in general delta configuration against the serving cell configuration (unless Full configuration is necessary).

Observation 1: For LTM, when cell switch is triggered, the handling of ltm-CandidateConfig and/or ltm-ReferenceConfiguration is more like Full Configuration at the UE side.

##### 5.3.5.18.6 LTM cell switch execution Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall: 1> if the LTM cell switch is triggered on the MCG: 2> release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following: - the radio bearer configuration (configured via RadioBearerConfig) - the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4]; - the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4]; - the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID; - the ltm-Config; - the MCG C-RNTI; - the AS security configurations associated with the master key; - the logged measurement configuration; 1> else, if the LTM cell switch is triggered on the SCG: 2> release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following: - the radio bearer configuration (configured via RadioBearerConfig IE) - the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4]; - the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4]; - the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID; - the ltm-Config; - the AS security configurations associated with the secondary key; … 1> if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3: 2> consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure; 2> if measConfig is included within ltm-ReferenceConfiguration in ltm-Config; 3> perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig: NOTE 1: When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause. 1> if the LTM cell switch is triggered by an indication from lower layers: 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3; 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3; 1> release the radio bearer(s) and the logical channel(s) that were part of the UE configuration before of this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete). NOTE 2: When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

Assuming a scenario that only the source gNB is made by vendor X while the candidate gNBs are made by vendor Y, it makes more sense that the source gNB requests a reference configuration from the first candidate gNB and provide it to the other candidate gNB(s) in the following preparations. It is likely in this case the candidate gNB(s) provides a candidate configuration on top of the reference configuration instead of a complete one. Nevertheless it is still up to implementation that the reference configuration is created based on the serving gNB configuration so it can be available for the source gNB from the very beginning of the inter-CU MCG LTM preparation. Therefore, we think the related proposals can be revisited in this meeting with the above clarifications.

Proposal 1: The source gNB can either request a reference configuration or, if avalable, provide a reference configuration for LTM in a Handover Request message.

Proposal 2: A candidate gNB, if being requested by the source gNB, provides one reference configuration for LTM in a Handover Request Acknowledge message.

Proposal 3: The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge message.

In addition to the discussion we had for the inter-CU MCG LTM, in the meantime, RAN2 had substaintial progress related to inter-CU SCG LTM for the RAN2-127, RAN2-127bis and RAN2-128 meetings. The part specifically related to the Inter-CU MCG LTM and Inter-CU SCG LTM are excerpted below. RAN2 had made quite some agreements and mainly for the dual connectivity case.

RAN2-127:

Agreements on inter-CU LTM Reference configuration: - Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported. Inter-CU SCG LTM: - Inter-CU SCG LTM preparation can be initiated by source SN. - The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. - For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

RAN2-127bis:

Agreements on inter-CU LTM

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

RAN2-128:

Agreements on inter-CU LTM: - RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it. - RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19. - From RAN2 perspective, the following coexistence cases in NR-DC can be supported: - Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM - Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM - In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations. - RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message. - RAN2 to support intra-CU MCG LTM with SCG configuration. - It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM. - RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In the RAN2 running CR ([POST128][107][MOB] LTM and CLTM (Ericsson)), the following changes were proposed to reflect the above RAN2 agreements - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

#### – CG-Config This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB. It can also be used by a CU to request a DU to perform certain actions, e.g. to request the DU to perform a new lower layer configuration. Direction: Secondary gNB or eNB to master gNB or eNB, alternatively CU to DU. CG-Config message -- ASN1START -- TAG-CG-CONFIG-START CG-Config ::= SEQUENCE { … CG-Config-IEs ::= SEQUENCE { scg-CellGroupConfig OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, scg-RB-Config OCTET STRING (CONTAINING RadioBearerConfig) OPTIONAL, … CG-Config-v1800-IEs ::= SEQUENCE { candidateServingFreqRangeListNR-r18 CandidateServingFreqRangeListNR-r18 OPTIONAL, candidateServingFreqListNR-r16 CandidateServingFreqListNR-r16 OPTIONAL, idc-TDM-AssistanceConfig-r18 ENUMERATED {enabled} OPTIONAL, candidateCellInfoListSubsequentCPC-r18 CandidateCellInfoListCPC-r17 OPTIONAL, scpac-ReferenceConfigurationSCG-r18 ReferenceConfiguration-r18 OPTIONAL, subsequentCPAC-Information-r18 CandidateCellInfoListCPC-r17 OPTIONAL, successPSCell-Config-r18 SuccessPSCell-Config-r18 OPTIONAL, nonCriticalExtension CG-Config-v19xy-IEs OPTIONAL } CG-Config-v19xy-IEs ::= SEQUENCE { ltm-ReferenceConfiguration-r19 ReferenceConfiguration-r18 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } … } -- TAG-CG-CONFIG-STOP -- ASN1STOP #### – CG-ConfigInfo This message is used by master eNB or gNB to request the SgNB or SeNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB or SeNB to set the SCG configuration. It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, or modify an MCG or SCG. Direction: Master eNB or gNB to secondary gNB or eNB, alternatively CU to DU. CG-ConfigInfo message -- ASN1START -- TAG-CG-CONFIG-INFO-START CG-ConfigInfo ::= SEQUENCE { … CG-ConfigInfo-IEs ::= SEQUENCE { ue-CapabilityInfo OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList) OPTIONAL,-- Cond SN-AddMod candidateCellInfoListMN MeasResultList2NR OPTIONAL, candidateCellInfoListSN OCTET STRING (CONTAINING MeasResultList2NR) OPTIONAL, measResultCellListSFTD-NR MeasResultCellListSFTD-NR OPTIONAL, scgFailureInfo SEQUENCE { failureType ENUMERATED { t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailure-SCG, scg-reconfigFailure, srb3-IntegrityFailure}, measResultSCG OCTET STRING (CONTAINING MeasResultSCG-Failure) } OPTIONAL, configRestrictInfo ConfigRestrictInfoSCG OPTIONAL, drx-InfoMCG DRX-Info OPTIONAL, measConfigMN MeasConfigMN OPTIONAL, sourceConfigSCG OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, scg-RB-Config OCTET STRING (CONTAINING RadioBearerConfig) OPTIONAL, mcg-RB-Config OCTET STRING (CONTAINING RadioBearerConfig) OPTIONAL, … CG-ConfigInfo-v1800-IEs ::= SEQUENCE { musim-GapConfigInfo-r18 MUSIM-GapConfig-r17 OPTIONAL, musim-CapRestrictionInfo-r18 SEQUENCE { musim-CapRestriction-r18 MUSIM-CapRestriction-r18 OPTIONAL, musim-CandidateBandList-r18 MUSIM-CandidateBandList-r18 OPTIONAL } OPTIONAL, scpac-ReferenceConfiguration-r18 ReferenceConfiguration-r18 OPTIONAL, subsequentCPAC-Candidates-r18 CandidateCellListCPC-r17 OPTIONAL, nonCriticalExtension CG-ConfigInfo-v19xy-IEs OPTIONAL } CG-ConfigInfo-v19xy-IEs ::= SEQUENCE { ltm-ReferenceConfiguration-r19 ReferenceConfiguration-r18 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } … }

Therefore, for inter-CU SCG LTM, RAN3 should also start some discussion regarding how reference configuration is signaled between the source SN, MN, and the target SN. In general we propose to first confirm in RAN3 the messages for the SN-initiated Inter-CU SCG LTM. We also assume the S-CPAC principles can be followed for the stage 3 unless there is a special concern.

Observation 2: For Inter-CU SCG LTM, the ltm-ReferenceConfiguration has been included in the Inter-Node RRC messages CG-Config and CG-ConfigInfo in the RRC running CR for LTM.

Proposal 4: S-CPAC-similar principles can be applied for inter-CU SCG LTM as baseline.

Proposal 5: The MN can either request a reference configuration or, if has acquired from the source SN in the SN Change Required message, provide a reference configuration for LTM in a SN Addtion Request message to the candidate SN.

Proposal 6: A candidate SN, if being requested by the MN, provides one reference configuration for LTM in a SN Request Acknowledge message.

Proposal 7: The candidate SN indicates whether a LTM candidate configuration is a complete candidate configuration in the SN Request Acknowledge message.

As for the reference configuration part, we propose to follow what RAN2 had implemented (i.e., using the INM messages CG-Config and CG-ConfigInfo to signal the ltm-ReferenceConfiguration for inter-CU SCG LTM). Given that RAN3 had a dedicated XnAP Reference Configuration IE included in the Hanodver Request message and the Hanodver Request Acknowledge message in the BLCR while RAN2 did not touch the INM HandoverCommand and HandoverPreparationInformation for inter-CU MCG LTM, there will be some different handling in XnAP for MCG LTM and SCG LTM similar to the CHO and CPAC, respectively. If the above is agreeable, a similar handling in F1AP may also be needed for the candidate gNB or candidate SgNB that the reference configuration signaled from the CU to the DU is in a dedicated F1AP IE/field (i.e., the LTM Reference Configuration or the Reference Configuration Information) and in the CG-ConfigInfo (i.e., in the CU to DU RRC Information IE), respectively.

The corresponing TPs regarding the reference configuration issue for the inter-CU MCG LTM for TS 38.300 and both the inter-CU MCG LTM and inter-CU SCG LTM for TS 38.423 are attached.

# 2	Conclusion

In this contribution, we discussed the reference configuration for both the inter-CU MCG LTM and the inter-CU SCG LTM. The related observations and proposals are summarized below:

Observation 1: For LTM, when cell switch is triggered, the handling of ltm-CandidateConfig and/or ltm-ReferenceConfiguration is more like Full Configuration at the UE side.

Proposal 1: The source gNB can either request a reference configuration or, if available, provide a reference configuration for LTM in a Handover Request message.

Proposal 2: A candidate gNB, if being requested by the source gNB, provides one reference configuration for LTM in a Handover Request Acknowledge message.

Proposal 3: The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge message.

Observation 2: For Inter-CU SCG LTM, the ltm-ReferenceConfiguration has been included in the Inter-Node RRC messages CG-Config and CG-ConfigInfo in the RRC running CR for LTM.

Proposal 4: S-CPAC-similar principles can be applied for inter-CU SCG LTM as baseline.

Proposal 5: The MN can either request a reference configuration or, if has acquired from the source SN in the SN Change Required message, provide a reference configuration for LTM in a SN Addtion Request message to the candidate SN.

Proposal 6: A candidate SN, if being requested by the MN, provides one reference configuration for LTM in a SN Request Acknowledge message.

Proposal 7: The candidate SN indicates whether a LTM candidate configuration is a complete candidate configuration in the SN Request Acknowledge message.

# 3. Text Proposal to Inter-CU LTM BL CR to TS 38.300

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration. The source gNB may provide a reference configuration to the one or more candidate gNB(s). The source gNB may request a candidate gNB to provide the reference configuration.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request. The candidate gNB provides the reference configuration in the HANDOVER REQUEST ACKNOWLEDGE message if being requested by the source gNB.

6.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs.

Editor’s Note: The new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs. Details are FFS.

21. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

# 4. Text Proposal to Inter-CU LTM BL CR to TS 38.423

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

<<<<<<<<<<<<<<<<<<<< Skip Unchanged Part >>>>>>>>>>>>>>>>>>>>

If the LTM Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the LTM information for the UE in the target NG-RAN node.

If the LTM Information Response IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, store it and use it as defined in TS 38.300 [9].

If the Early Sync Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the early synchronization information for the UE in the target NG-RAN node.

If the Early Sync Information Response IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, store it and use it as defined in TS 38.300 [9].

If the Request For CSI-RS Configuration IE is contained in the LTM Information Request IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Configuration IE within the LTM Information Response in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Reference Configuration Request IE set to "true" is contained in the LTM Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the Reference Configuration IE for LTM in the HANDOVER REQUEST ACKNOWLEDGE message.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Skip Unchanged Part >>>>>>>>>>>>>>>>>>>>

If the Conditional PSCell Addition Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request concerns CPAC, as described in TS 37.340 [8]. Accordingly, the S-NG-RAN node shall, if supported, include the Conditional PSCell Addition Information Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the S-CPAC Request Information IE is contained in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the procedure is triggered for S-CPAC preparation. If the S-NG-RAN node accepts the request as a S-CPAC preparation, it shall include the Candidate PSCell with Other Information List IE in the Conditional PSCell Addition Information Acknowledge IE.

If the S-CPAC Reference Configuration Request IE set to "request" is contained in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, provide the SCG reference configuration for S-CPAC.

If the S-CPAC Multiple Target S-NG-RAN Node List IE is contained within the S-CPAC Request Information IE in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the information pertains to a list of PSCells suggested for other candidate SN(s) may also be prepared for S-CPAC, and act as described in TS 37.340 [8].

If the Conditional PSCell Addition Information Acknowledge IE is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, consider the indicated PSCells are selected by the target SN as candidate PSCells for CPAC or S-CPAC.

If the S-NG-RAN node applied a complete candidate configuration for a specific PSCell, e.g., as part of preparation of S-CPAC, the S-NG-RAN node shall inform the M-NG-RAN node by including the S-CPAC Complete Candidate Configuration Indicator IE in the Candidate PSCell with Other Information Item IE in the Conditional PSCell Addition Information Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the CG-CandidateList is included in the S-NG-RAN node to M-NG-RAN node Container IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it for the purpose of CPAC or S-CPAC.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC or S-CPAC procedure.

If the LTM Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request concerns LTM, as described in TS 37.340 [8]. Accordingly, the S-NG-RAN node shall, if supported, include the LTM Information Response IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the Reference Configuration Request IE set to "true" is contained in the LTM Information Request IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, provide the SCG reference configuration for LTM in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

<<<<<<<<<<<<<<<<<<<< Skip Unchanged Part >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

#### 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE CHANGE REQUIRED message may contain

-	the S-NG-RAN node to M-NG-RAN node Container IE.

If the M-NG-RAN node is able to perform the change requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE CHANGE CONFIRM message to the S-NG-RAN node. For DRBs configured with the PDCP entity in the S-NG-RAN node, the M-NG-RAN node may include data forwarding related information in the Data Forwarding Info from target NG-RAN node IE.

If the S-NODE CHANGE CONFIRM message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8].

The S-NG-RAN node may start data forwarding and stop providing user data to the UE and shall stop the timer TXnDCoverall upon reception of the S-NODE CHANGE CONFIRM message.

If the S-NODE CHANGE REQUIRED message includes the SCG UE History Information IE, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

If the S-NODE CHANGE REQUIRED message includes the SN Mobility Information IE, the M-NG-RAN node shall, if supported, store this information and use it as defined in TS 37.340 [8].

If the S-NODE CHANGE REQUIRED message includes the Source PSCell ID IE, the M-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

If the Conditional PSCell Change Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the requirement concerns CPAC, as described in TS 37.340 [8]. If the S-CPAC Request IE set to "initiation" is also contained in the Conditional PSCell Change Information Required IE included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the procedure is triggered for S-CPAC preparation, as described in TS 37.340 [8]. The S-NG-RAN node to M-NG-RAN node Container IE within the Conditional PSCell Change Information Required IE contains at least the suggested PSCell list for each candidate target S-NG-RAN node. Accordingly, the M-NG-RAN node may include the Conditional PSCell Change Information Confirm IE in the S-NODE CHANGE CONFIRM message. If the CPAC Preparation Type IE is included in the Conditional PSCell Change Information Confirm IE for a candidate target S-NG-RAN node, the S-NG-RAN node shall, if supported, consider that the candidate target S-NG-RAN node accepted the S-CPAC request and that the S-NG-RAN node may remain prepared for S-CPAC after PSCell change execution to that candidate target S-NG-RAN node, as described in TS 37.340 [8].

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Change Information Required IE included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, forward this information to the candidate target S-NG-RAN node, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC or S-CPAC procedure.

If the Multiple Target S-NG-RAN Node List IE is included in the S-NODE CHANGE REQUIRED message, if multiple Target S-NG-RAN nodes are prepared, the M-NG-RAN node may include the Additional List of PDU Session Resource Change Confirm Info – SN Terminated IE in the S-NODE CHANGE CONFIRM message to provide different data forwarding addresses for different Target S-NG-RAN nodes.

If the Source SN to Target SN QMC Information IE is contained in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, use it for QoE measurements handling, as specified in TS 37.340 [8].

If the LTM Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the procedure is triggered for LTM, as described in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<< Skip Unchanged Part >>>>>>>>>>>>>>>>>>>>

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 | This IE is ignored if the S-CPAC Request Information IE is present in the Conditional PSCell Addition Information Request IE. | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node, or the source SN in e.g. NR-DC to NR-DC (conditional) handover. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >S-CPAC Request Information | O |  | 9.2.3.192 | Indicates that SN addition is for S-CPAC preparation. | YES | reject |
| >S-CPAC Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for S-CPAC is requested. | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Selected NID | O |  | NID 9.2.2.65 | This IE, together with the Selected PLMN IE, indicates the SNPN proposed for the SCG to the S-NG-RAN node. | YES | ignore |
| QMC Coordination Request | O |  | 9.2.3.197 | This IE contains information for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements at the S-NG-RAN node subject to addition. | YES | ignore |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Source M-NG-RAN node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source M-NG-RAN node in e.g. NR-DC to NR-DC handover. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted to be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Setup Response Info – SN terminated | O |  | 9.2.1.6 |  | – |  |
| >>PDU Session Resource Setup Response Info – MN terminated | O |  | 9.2.1.8 |  | – |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |
| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source NG-RAN node for intra-system handover, or between the target S-NG-RAN node and the source SN in e.g.NR-DC to NR-DC (conditional) handover. | YES | ignore |
| SCG Activation Status | O |  | 9.2.3.155 |  | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  | Ignored, if the Candidate PSCell with Other Information List IE is included. | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >Candidate PSCell with Other Information List |  | 0..1 |  |  | YES | reject |
| >>Candidate PSCell with Other Information Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>>S-CPAC Complete Candidate Configuration Indicator | O |  | Complete Candidate Configuration Indicator 9.2.3.194 |  | – |  |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; T-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| QMC Coordination Response | O |  | 9.2.3.198 | This IE contains the response of the S-NG-RAN node to the QMC coordination request. | YES | ignore |
| CHO Information SN Addition Acknowledge | O |  |  |  | YES | reject |
| >PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 | PCell indicated in the corresponding S-NODE ADDITION REQUEST message. | – |  |
| Direct Forwarding Path Availability with source M-NG-RAN node | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source M-NG-RAN node. | YES | ignore |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session SN Change Required List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Required Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  | 9.2.1.18 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; S-NG-RAN node provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| Source PSCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |
| Conditional PSCell Change Information Required | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>CPC Indicator | M |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancellation, ...) |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |
| >S-CPAC Request | O |  | ENUMERATED (initiation, …) | Indicates that SN change is for S-CPAC preparation. | YES | reject |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| LTM Information Required | O |  | 9.2.1.xxY |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.xx1	LTM Information Request

This IE contains a request for LTM information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Configuration | O |  | OCTET STRING | The reference configuration for inter-CU MCG LTM; Includes the RRCReconfiguration message (TS 38.331 [10]) |
| CSI Resource Configuration | O |  | OCTET STRING |  |
| LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  |
| Request For CSI-RS Configuration | O |  | ENUMERATED (true, …)(FFS) |  |
| Reference Configuration Request | O |  | ENUMERATED (true, …) | Indicates that the reference configuration for LTM is requested. |
| SCG LTM Information Request | O |  |  | Specifically for inter-CU SCG LTM |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare for inter-CU SCG LTM. |

#### 9.2.1.xx2	LTM Information Response

This IE contains LTM related information to be provided in response to information provided in the LTM Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target NG-RAN node To Source NG-RAN node Transparent Container | FFS |  | FFS |  |
| Reference Configuration | O |  | OCTET STRING | The reference configuration for inter-CU MCG LTM; Includes the RRCReconfiguration message (TS 38.331 [10]) |
| CSI-RS Configuration | O |  | OCTET STRING |  |
| Complete Configuration Indicator | O |  | Complete Candidate Configuration Indicator 9.2.3.194 | The complete configuration indicator for the candidate configuration for inter-CU MCG LTM |
| SCG LTM Information Acknowledge | O |  |  | Specifically for inter-CU SCG LTM |
| >Candidate PSCell List |  | 1 |  |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >>>Complete Candidate Configuration Indicator | M |  | Complete Candidate Configuration Indicator 9.2.3.194 |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.xxY	LTM Information Required

This IE contains a requirement for Inter-CU SCG LTM.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multiple Target S-NG-RAN Node List |  | 1 |  |  |
| >Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  |
| >>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  |
| >>LTM Indicator | M |  | ENUMERATED (LTM-initiation, LTM-modification, LTM-cancellation, ...) |  |
| >>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. |
| >>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |

<< End of Changes >>