3GPP TSG-RAN3 Meeting #127	R3-250240

Athens, Greece, Feb 17-21, 2025

Agenda Item:	14.3

Source: 		Qualcomm Incorporated, Vodafone Group Plc, Vodafone Telekomünikasyon A.S.

Title: 	Discussion on Reply LS on OAM requirements to support Regenerative payload

Document for:	Discussion/Decision

# Background

In RAN3#125bis it was agreed that TAI list for onboard gNB will be provided to AMF via OAM for NTN regenerative payload. Corresponding LS R3-245845 was sent to SA2 and SA5 with RAN3 agreement. SA2 has sent a response LS R3-250023 stating that they have concerns on the RAN3 agreement on TAI.

R3-250023 SA2 discussed the OAM based solution and aspects related to it are unclear to SA2. SA2 understands that the TAIs broadcast by an NTN gNB are earth static, and the TAIs broadcast by the NTN gNB may change with the movement of the satellite. SA2 also understands the NG Removal procedure is defined to remove interface between a gNB and an AMF. SA2 believes after receiving a NG Removal message, the AMF will release the information of supported TAIs and it is the gNB to determine the time to initiate the NG Removal procedure based on the AMFs serving area (e.g., the serving PLMN allowed operation area). Therefore the number of TAIs and related updates will be bounded by those which can be covered by a satellite while using a NTN gateway. SA2 would like confirm these understanding with RAN3. - SA2 discussed whether the OAM solution would have impacts to the existing procedures related to paging and sequencing of OAM updates and RAN3 updates towards the AMF, e.g. in NGAP Setup/configuration update etc. However, it was unclear to SA2 how the OAM solution would work based on the description in the incoming LS to be able to determine whether there are impacts. - The frequency of updates by OAM was not clear to SA2 and whether the information provided to the AMF is a timetable of which TAIs the gNB will support at what time, or whether the updates are provided frequently as the satellite moves updating the AMF each time a change occurs. SA2 would like to confirm whether RAN3 and SA5 are aligned on the frequency and contents of the OAM updates to the AMF. Some companies in SA2 have concerns with the solutions agreed by RAN3, which changes the functional split between RAN and the core network.

In this paper we discuss our views on the SA2 LS response.

# Discussion

As per the LS from SA2 above we observe the following concerns on the RAN3 agreement.

RAN3 agreement introduces additional impacts to AMF (SA2), as AMF now have to receive TA updates via OAM instead of RAN.

Supported TA list is a mandatory IE in NG Setup message. If TAI list is provided via OAM what should be provided in the NG Setup? If RAN also provides TA list in NG setup and OAM also provided TA list which list should AMF consider? What happens to the legacy framework of TA update provided by RAN to AMF via NG. Is the legacy NG procedure obsolete with the OAM solution or OAM and NG based solution can co-exists, or if both the RAN and OAM provides TA list which TA list takes precedence over the other?

How frequent OAM provide TAI update to AMF? For example, should OAM send this list once when AMF comes up or every time a new RAN is covering a particular TA etc

The following gaps are observed on RAN3 agreement to send TAI list to AMF via OAM instead of NGAP for NTN Regenerative Payload –

- Supported TA list is a mandatory IE in NG Setup message. If TAI list is provided via OAM what should be provided in the NG Setup? If RAN also provides TA list in NG setup and OAM also provided TA list which list should AMF consider

- Impact to the legacy framework of TA update provided by RAN to AMF via NGAP is unclear. Is the legacy NG procedure obsolete with the OAM solution or OAM and NG based solution can co-exists, or if both the RAN and OAM provides TA list which TA list takes precedence over the other?

- Frequency of the TAI update from OAM to AMF is unknown. For example, should OAM send this list once when AMF comes up or every time a new RAN is covering a particular TA etc

To avoid the above issues, we think the following solution options are possible –

- Option 1 - Enhance NG Setup procedure to provide the complete TAI list served by NTN Regenerative payload gNB to AMF instead of OAM solution.

- Option 2 - For NTN TAI update RAN3 provides requirements on the TAI format and frequency to SA5 and SA2. Also mention in TS 38.413 or in TS 38.300 specification that TA update for NTN Regenerative payload is performed only via OAM and the legacy NGAP Configuration Update for TA update is not used.

- Option 3 – “DO NOTHING” stick to legacy procedure of TA update over NG to avoid AMF impacts and uniform solution for NTN Transparent and Regenerative payload.

Our preference is Option 3. As we want to have a uniform solution across NTN Transparent and regenerative payloads. Corresponding LS response is provided in the Annex.

- We prefer Option 3 “DO NOTHING” approach and stick with legacy procedure of TA update over NG to avoid AMF impacts and have uniform solution across NTN transparent and Regenerative payloads. Corresponding LS response is provided in the annex.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	The following gaps are observed on RAN3 agreement to send TAI list to AMF via OAM instead of NGAP for NTN Regenerative Payload –

	Supported TA list is a mandatory IE in NG Setup message. If TAI list is provided via OAM what should be provided in the NG Setup? If RAN also provides TA list in NG setup and OAM also provided TA list which list should AMF consider

	Impact to the legacy framework of TA update provided by RAN to AMF via NGAP is unclear. Is the legacy NG procedure obsolete with the OAM solution or OAM and NG based solution can co-exists, or if both the RAN and OAM provides TA list which TA list takes precedence over the other?

	Frequency of the TAI update from OAM to AMF is unknown. For example, should OAM send this list once when AMF comes up or every time a new RAN is covering a particular TA etc

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	We prefer Option 3 “DO NOTHING” approach and stick with legacy procedure of TA update over NG to avoid AMF impacts and have uniform solution across NTN transparent and Regenerative payloads. Corresponding LS response is provided in the annex.

# Annex - LS Response

# Title:	Reply LS on OAM requirements to support regenerative payload

# Response to:	LS on OAM requirements to support regenerative payload from RAN3 (R3-250023)

# Release:	Release 19

# Work Item:	5GSAT_Ph3-ARC

Source:	Qualcomm [To be RAN3]

To:	SA2, SA5

Cc:	CT1

Contact Person:

Name:	Geetha Rajendran

Tel. Number:

E-mail Address:	geethapr@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 thanks SA2 for their reply to LS on OAM requirements to support regenerative payload.

RAN3 discussed the issues pointed out by SA2 and understands that the requirements for OAM, to provide the frequency of TAI update and the impact to legacy NGAP Setup and Config Update procedures is unclear. RAN3 agrees to use the legacy behaviour of RAN informing the AMF on TAI list for NTN regenerative payload to avoid additional impacts and to have a common solution for NTN Transparent and Regenerative payload.

2. Actions:

To RAN2, RAN3:

ACTION: 	RAN3 asks SA2 to kindly take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

RAN3#127bis		07-11 April 2025	Wuhan, China

RAN3#128		19-23 May 2025	Malta, Malta