3GPP TSG-RAN3 Meeting #127	R3-250241

Athens, Greece, Feb 17-21, 2025

Agenda Item:	14.3

Source: 	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Xiaomi

Title: 	(TP for TS 38.300) Discussion on NR NTN Regenerative Payload

Document for:	Discussion/Decision

# Background

In RAN3#126 meeting, only inactive state discussion happened which was not very fruitful. In this paper we provide our views on Inactive UE support and feeder link switch over due to moving gNB on satellite.

RAN3#125bis The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. RAN3#126 Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

# Discussion

## Feeder Link Switch Over

In the last meeting we agreed that current mechanisms like multiple TNLA association is sufficient to address TNLA change if any during feeder link switch over. We propose to update this text to TS38.300 as provided in the annex of this paper.

Capture the agreement on TNLA “The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch over” in TS 38.300. Agree to TP in Annex

Feeder Link Switch over without AMF relocation

It was agreed to consider only the scenario where FLSO happens without AMF relocation. In the last meeting it was agreed that Legacy NG HO can be used if AMF also changes with Feeder Link switch over which does not have any specification impacts.

For soft and hard feeder link without AMF relocation the AMF and the gNB remains the same. Hence the UE context and NG context in AMF and gNB remains intact. Just the NTN GW IP address changes due to FLSO. Since the Feeder link and the NTN GW is out of 3GPP scope, the NTN operators can handle it via existing mechanisms like mobike, SCTP multihoming etc.

For soft and hard feeder link switch over without AMF relocation, the context in gNB and AMF remains intact. For soft FLSO, since AMF and gNB remains connected, current HO mechanisms can be used without any specification impact.

For hard FLSO, there would be a break in NGAP connection between AMF and gNB. The gNB and AMF needs to stop transmission over NG (NG-C and NG-U) at time T and restart at time T1 after hard FLSO. This interruption of NG due to hard FLSO needs to be handled in NG application layer as this needs handshaking between peers at both the end of NG connection to stop and start transmission synchronously.

For hard feeder link switch over without AMF relocation, the NG connection will be interrupted and both gNB and AMF should stop and start transmission over NG-C and NG-U before and after hard FLSO. This needs to be handled over NGAP to stop and start transmission synchronously.

Existing NGAP interface management procedures (e.g RAN/AMF Config Update procedures) could be enhanced with an IE to indicate the interruption in the NG connection due to hard FLSO. When AMF receives the indication, it can inform SMF/UPF accordingly.

Existing NGAP interface management procedures like RAN/AMF Config Update procedures could be enhanced with an IE to indicate interruption on NG connection due to hard FLSO.

## RRC_Inactive UE Support

In the last RAN3 meeting it was agreed to reuse the existing mechanism and procedures to support RRC Inactive state in Regenerative payload. Due to the movement of the gNB on satellite, UE may have been moved to RRC inactive state in gNB1 and UE might resume on gNB4 in the same geographical location. As Xn connection may not be available between gNB1 and gNB4, it is not possible for gNB4 to retrieve UE context from gNB1.

Due to the satellite movement, UE may go inactive in gNB1 and resume in gNB4 in the same geographical location. Due to missing Xn between gNB1 and gNB4, it is not possible to retrieve the inactive UE context from gNB1 to gNB4 using the current mechanism.

To address the problem of inactive UE context retrieval, the following solutions are proposed.

Option 1 – RAN Paging to relocate Inactive UE context

This option is described in HW paper in R3-244548. In this solution, when the gNB moves away from a particular geographical area, the gNB pages all the RRC inactive UEs wakes them up and moves the UEs to the subsequent gNB which covers the same geographical area. This solution defeats the purpose of RRC Inactive state and there is no UE power savings.

Option 2 – Proactive Inactive UE Context movement

In this option the inactive UE context is proactively moved from the gNB leaving a particular geographical area to an incoming gNB covering the same geographical area. This way when the UE resumes, the UE context is available in the nearby gNB to retrieve the context.

Option 3 – Hop by hop Retrieve UE context

In this option, the gNB where the UE resumes sends Retrieve UE Context Request to its neighbouring gNBs and the neighbouring gNBs forwards the request to its neighbours until the gNB which has the inactive UE context get the Retrieve UE context request. This solution does not have much standards impact. But this solution causes latency and signaling storm.

Option 4 – Move the UE Context to ground

In this option, the inactive UE context is moved to AMF or NTN GW on the ground. When the UE resumes, the context is retrieved from the AMF or NTN GW by the gNB where the UE resumes.

These are the few options listed, however there could be other options as well which are not precluded. RAN3 can discuss further on the solutions to solve this issue.

RAN3 to discuss the above options to address the issue of retrieving RRC_Inactive UE context from anchor gNB in NTN Regenerative payload. Detailed analysis of the solution options and the preferred solution is provided in R3-250152

## Mapped Cell ID

It was agreed to use the Mapped Cell ID in Regenerative payload as it is used in transparent payload. The Mapped Cell ID IE structure contains gNB ID + Cell ID. For regenerative payload, since the gNB covering a geographical area changes with time, it is unclear whether the gNB ID in the Mapped Cell ID will change or will it remain same for a geographical area. It needs to be clarified that gNB ID within a mapped cell ID remains constant in a geographical area.

Since the Mapped Cell ID IE contains gNB ID + Cell ID, it is unclear whether gNB ID in Mapped Cell ID changes with the movement of the gNB for regenerative payload.

The gNB ID within Mapped Cell ID remains constant in a geographical area for regenerative payload. Agree TP to TS 38.300 in Annex section.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	For soft and hard feeder link switch over without AMF relocation, the context in gNB and AMF remains intact. For soft FLSO, since AMF and gNB remains connected, current HO mechanisms can be used without any specification impact.

Observation 2.	For hard feeder link switch over without AMF relocation, the NG connection will be interrupted and both gNB and AMF should stop and start transmission over NG-C and NG-U before and after hard FLSO. This needs to be handled over NGAP to stop and start transmission synchronously.

Observation 3.	Due to the satellite movement, UE may go inactive in gNB1 and resume in gNB4 in the same geographical location. Due to missing Xn between gNB1 and gNB4, it is not possible to retrieve the inactive UE context from gNB1 to gNB4 using the current mechanism.

Observation 4.	Since the Mapped Cell ID IE contains gNB ID + Cell ID, it is unclear whether gNB ID in Mapped Cell ID changes with the movement of the gNB for regenerative payload.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Capture the agreement on TNLA “The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch over” in TS 38.300. Agree to TP in Annex

Proposal 2:	Existing NGAP interface management procedures like RAN/AMF Config Update procedures could be enhanced with an IE to indicate interruption on NG connection due to hard FLSO.

Proposal 3:	RAN3 to discuss the above options to address the issue of retrieving RRC_Inactive UE context from anchor gNB in NTN Regenerative payload. Detailed analysis of the solution options and the preferred solution is provided in R3-250152

Proposal 4:	The gNB ID within Mapped Cell ID remains constant in a geographical area for regenerative payload. Agree TP to TS 38.300 in Annex section.

# Annex (TP for TS 38.300)

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN.

16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

The current mechanisms including, e.g. multiple TNLA association, can be used for TNL management during feeder link switch for NTN regenerative Payload.

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions). gNB ID in a Mapped Cell ID remains constant in a geographical area.

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.