3GPP TSG-RAN3 Meeting #127	R3-250242

Athens, Greece, Feb 17-21, 2025

Agenda Item:	15.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on Support for IoT NTN for Regenerative Payload

Document for:	Discussion/Decision

# Background

In the last RAN3 meeting, there was no clear understanding among the companies if the objective of the work item included regenerative payload as well. In December RAN plenary RANP#106, the work item document RP-243278 was updated to include full eNB as regenerative payload.

RP-243278 - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required RAN3#126 Introduce the stage 2 description of Split MME architecture and Full CN on board architecture following the architecture assumptions and principles? How to support the eNB as regenerative payload on board without S&F satellite operation? Check progress in RAN1/2…

In this paper we discuss our views on RAN impacts for eNB in Store and forward mode and full eNB as regenerative payload.

# Discussion

## Store and Forward support

The architecture options agreed in SA2 for regenerative payload in S2-2407186 has whole CN on board in the satellite and proxies of CN deployed in the ground. The proxies deployed in the ground are out of 3GPP scope. Another option is split MME with MME onboard and MME on ground. The MME on ground and the interface between the MMEs is out of 3GPP scope as agreed in S2-2407191.

S2-2407186 The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. - The implementation of the proxies and the interface between them is out of 3GPP scope. S2-2407191 1) In the Split MME architecture, HSS is on the ground. 2) MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground – the MME part which is on the ground network with an interface out of scope of 3GPP.

In both the architecture options agreed in SA2, the whole eNB is on board and the S1 interface between eNB and MME is not impacted. In the last meeting companies proposed a third architecture with eNB only on board which is not in the SA2 concluded architecture options for Store and Forward mode. Hence RAN3 should only analyse full CN on board and split MME architecture options for RAN3 impact for S&F.

The architecture options in SA2 for S&F for Regenerative payload has whole CN on board as one option and split MME on board in another option. eNB only on board is not proposed in SA2 concluded architecture option for S&F.

RAN3 to consider only full CN on board and split MME architecture options agreed in SA2 for RAN3 impact.

In both full CN on board and split MME architecture options, we see no impacts to S1-C as S1-C is on-board in the satellite. For full CN on board, the S1-U is also on the satellite and we see no impacts to S1-U. For split MME case, the S1-U is via Feeder link to the SGW on the ground, when the satellite is not operating in S&F mode. However, when the satellite is in S&F mode there will not be any data transmission. Also, in SA2 is the agreed that the connection between MME on board and the other CN components on ground is out of standards scope. Hence, we do not see any impact to S1-U in split MME architecture as well.

In both full CN on board and split MME architecture option, the whole eNB is on board and the S1-C interface is also on board the satellite. Hence, we see no impact to S1-C for both the architecture options.

S1-U interface is on board satellite for full CN architecture option whereas S1-U interface is connected to ground SGW via feeder link for Split MME architecture. For Split MME architecture, SA2 has agreed that connection between MME on board and other CN components on ground are out of standards scope. Hence, we see no impact to S1-U in both the architecture options.

RAN3 to agree that there are no RAN3 impacts for both full CN on board and split MME architecture options.

## Regenerative Payload

### S1 Connection Management

The agreement in NR NTN Regenerative Payload WI for NG Connection management can be replicated for S1 as well. The NG Removal procedure agreed in NR NTN can be mirrored for IoT NTN to support SI connection management for Regenerative payload

Agreement in NR NTN Regenerative payload to introduce NG Removal procedure for NG connection management can be mirrored for S1 as well to support S1 connection management for IoT NTN for Regenerative Payload.

### Feeder Link Switch over

Feeder link switch over scenarios can be discussed in NR NTN. The agreements achieved in NR NTN can be replicated in IoT NTN as well. In S1, there is no support for multiple TNLs. In the last RAN3 meeting it was agreed for NR NTN that “The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.” Similar agreement can be replicated for IoT NTN and can be discussed further on how to address the lack of multiple TNL support.

Feeder link switch over scenarios can be discussed in NR NTN and the agreements in NR NTN can be replicated in IoT NTN.

### Mapped Cell ID

The usage of Mapped Cell ID for Regenerative Architecture can be similar to that of Transparent Payload. NR NTN agreements on Mapped Cell ID can be replicated for IoT NTN as well.

The usage of Mapped Cell ID in Regenerative Payload can be similar to that of transparent payload.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	The architecture options in SA2 for S&F for Regenerative payload has whole CN on board as one option and split MME on board in another option. eNB only on board is not proposed in SA2 concluded architecture option for S&F.

Observation 2.	In both full CN on board and split MME architecture option, the whole eNB is on board and the S1-C interface is also on board the satellite. Hence, we see no impact to S1-C for both the architecture options.

Observation 3.	S1-U interface is on board satellite for full CN architecture option whereas S1-U interface is connected to ground SGW via feeder link for Split MME architecture. For Split MME architecture, SA2 has agreed that connection between MME on board and other CN components on ground are out of standards scope. Hence, we see no impact to S1-U in both the architecture options.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to consider only full CN on board and split MME architecture options agreed in SA2 for RAN3 impact.

Proposal 2:	RAN3 to agree that there are no RAN3 impacts for both full CN on board and split MME architecture options.

Proposal 3:	Agreement in NR NTN Regenerative payload to introduce NG Removal procedure for NG connection management can be mirrored for S1 as well to support S1 connection management for IoT NTN for Regenerative Payload.

Proposal 4:	Feeder link switch over scenarios can be discussed in NR NTN and the agreements in NR NTN can be replicated in IoT NTN.

Proposal 5:	The usage of Mapped Cell ID in Regenerative Payload can be similar to that of transparent payload.