3GPP TSG-RAN3 Meeting #127	R3-250245

Athens, Greece, Feb 17-21, 2025

Agenda Item:	11.4

Source: 	Qualcomm Incorporated

Title: 	Discussion on Continuous MDT

Document for:	Discussion/Decision

# Background

In RAN3#126, it was agreed to send LS to SA5 on the requirements for Continuous MDT and let SA5 choose a solution based on the requirements.

RAN3#126 Continuous MDT collection targeting the same UE across RRC states In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs. Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS.

In this paper we discuss provide our views on the requirements for Continuous MDT to be sent to SA5 in the LS.

# Discussion

## Continuous MDT Problem Statement

The agreed problem statement for Continuous MDT as captured in the TR 38.743 can be sent to SA5 in the LS for SA5 to have a better understanding of the problem statement and to come up with a solution.

Problem statement from TR 38.743

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

Add Problem statement - Problem A (measurement continuity) and Problem B (trace correlation) from TR 38.743 in the LS to SA5 for details on the problem to be addressed.

## Requirements for Continuous MDT

It was agreed in RAN3 to avoid solutions that impacts UE, so that MDT reports from legacy UE can also be collected for AI/ML training purposes. Hence any solution chosen by SA5 should take the legacy UEs into consideration.

Requirement 1: Solution for Continuous MDT should not impact UE as MDT reports from legacy UEs can also be used for AI/ML training purposes.

Currently the MDT job type from TCE can identify only immediate or logged MDT. To satisfy the Continuous MDT requirements RAN should be able to identify if the MDT job type is for continuous MDT or other MDTs. With the existing MDT configuration, SA5 to provide a solution for RAN to identify a continuous MDT session to avoid UE impacts.

Requirement 2: TCE to enable RAN to be aware that the MDT configuration is for Continuous MDT without UE impact.

When the immediate or logged MDT reports are sent from RAN to TCE, TCE cannot correlate that the reports are from the same UE for continuous MDT. Hence either TCE provide a Continuous MDT identifier to RAN or TCE can request RAN to allocate an continuous MDT identifier and report the same in the immediate and logged MDT reports for MDT report correlation.

Requirement 3: TCE to provide a Continuous MDT identifier to RAN or TCE to request RAN to allocate a continuous MDT identifier in immediate and logged MDT configuration. So that RAN can report the same Continuous MDT identifier to TCE in immediate and logged MDT reports for trace correlation at TCE.

When the UE has moved from RRC_Idle to RRC_Connected State, RAN should be able to configure the UE with immediate MDT reports without waiting for TCE to provide the MDT Configuration again. This way measurement gaps can be avoided.

Requirement 4: TCE to provide Continuous MDT configuration to a group of neighbouring gNBs to store the MDT configuration in gNB and configure the UE with MDT logging across RRC state transitions.

Add the below Requirements to the SA5 LS –

- Requirement 1: Solution for Continuous MDT should not impact UE as MDT reports from legacy UEs needs to be used for AI/ML training purposes.

- Requirement 2: TCE to enable RAN to be aware that the MDT configuration is for Continuous MDT without UE impact.

- Requirement 3: TCE to provide a Continuous MDT identifier to RAN or TCE to request RAN to allocate a continuous MDT identifier in immediate and logged MDT configuration for RAN to report the same Continuous MDT identifier to TCE in immediate and logged MDT reports for trace correlation at TCE.

- Requirement 4: TCE to provide Continuous MDT configuration to a group of neighbouring gNBs to store the MDT configuration and configure the UE with MDT logging across state transitions.

RAN cannot identify the UE across RRC state Transitions. Hence it is not possible for RAN to configure the same UE for MDT reporting across RRC state Transitions for Continuous MDT purposes. Only AMF can identify the UE across State Transitions. Hence gNB can request AMF to store the UE’s continuous MDT configuration when UE goes idle and provide the same configuration back to RAN when goes connected again.

RAN cannot identify UE across state Transitions to configure the same UE with continuous MDT session.

RAN3 to send LS to SA2 to check feasibility on whether AMF can store Continuous MDT configuration upon request from gNB, when UE goes to RRC_IDLE state and AMF to forward the Continuous MDT configuration back to gNB where the same UE moves to RRC_Connected state.

## User Consent for Continuous MDT

User consent is required for MDT report collection and user consent is provided for specific use cases for MDT report collection. It is unclear whether the user consent received for logged and immediate MDT can be used to collect data for AI/ML training as well or a separate user consent is needed. SA3 clarification is also needed to know if user consent received for logged and immediate MDT can be used for Continuous MDT collection.

RAN3 to discuss if user consent received for logged and immediate MDT report collection can be used for MDT report collection for AI/ML training and for continuous MDT collection.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	RAN cannot identify UE across state Transitions to configure the same UE with continuous MDT session.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Add Problem statement - Problem A (measurement continuity) and Problem B (trace correlation) from TR 38.743 in the LS to SA5 for details on the problem to be addressed.

Proposal 2:	Add the below Requirements to the SA5 LS –

	Requirement 1: Solution for Continuous MDT should not impact UE as MDT reports from legacy UEs needs to be used for AI/ML training purposes.

	Requirement 2: TCE to enable RAN to be aware that the MDT configuration is for Continuous MDT without UE impact.

	Requirement 3: TCE to provide a Continuous MDT identifier to RAN or TCE to request RAN to allocate a continuous MDT identifier in immediate and logged MDT configuration for RAN to report the same Continuous MDT identifier to TCE in immediate and logged MDT reports for trace correlation at TCE.

	Requirement 4: TCE to provide Continuous MDT configuration to a group of neighbouring gNBs to store the MDT configuration and configure the UE with MDT logging across state transitions.

Proposal 3:	RAN3 to send LS to SA2 to check feasibility on whether AMF can store Continuous MDT configuration upon request from gNB, when UE goes to RRC_IDLE state and AMF to forward the Continuous MDT configuration back to gNB where the same UE moves to RRC_Connected state.

Proposal 4:	RAN3 to discuss if user consent received for logged and immediate MDT report collection can be used for MDT report collection for AI/ML training and for continuous MDT collection.

# Annex (LS To SA5)

# Title:	LS on OAM requirements to support Continuous MDT

# Response to:	-

# Release:	Release 19

# Work Item:	NR_AIML_NGRAN_enh-Core

Source:	Qualcomm [To be RAN3]

To:	SA5

Cc:

Contact Person:

Name:	Geetha Rajendran

Tel. Number:

E-mail Address:	geethapr@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

For NG-RAN AI/ML use cases, RAN3 identified requirement to collect MDT reports from same group of UEs across RRC state transitions for AI/ML training in OAM. RAN3 agreed to have a solution for Continuous MDT collection and defined the problem statement for the same as below in TR 38.743.

Continuous MDT problem statement from TR 38.743

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

To address the above problem statements RAN3 has identified the below requirements, to be considered for Continuous MDT solution.

- Requirement 1: Solution for Continuous MDT should not impact UE as MDT reports from legacy UEs needs to be used for AI/ML training purposes.

- Requirement 2: TCE to enable RAN to be aware that the MDT configuration is for Continuous MDT without UE impact.

- Requirement 3: TCE to provide a Continuous MDT identifier to RAN or TCE to request RAN to allocate a continuous MDT identifier in immediate and logged MDT configuration for RAN to report the same Continuous MDT identifier to TCE in immediate and logged MDT reports for trace correlation at TCE.

- Requirement 4: TCE to provide Continuous MDT configuration to a group of neighbouring gNBs to store the MDT configuration and configure the UE with MDT logging across state transitions.

Since MDT is in SA5 scope, RAN3 would kindly request SA5 to consider the Continuous MDT problem statement and the requirements and provide a solution for Continuous MDT collection.

2. Actions:

To SA5:

ACTION: 	RAN3 asks SA5 to kindly take the above information into account and provide a suitable response.

3. Date of Next TSG SA WG2 Meetings:

RAN3#127bis		07-11 April 2025	Wuhan, China

RAN3#128		19-23 May 2025	Malta, Malta

# Annex (LS To SA2)

# Title:	LS on Continuous MDT configuration store and forward

# Response to:	-

# Release:	Release 19

# Work Item:	NR_AIML_NGRAN_enh-Core

Source:	Qualcomm [To be RAN3]

To:	SA2

Cc:	SA5

Contact Person:

Name:	Geetha Rajendran

Tel. Number:

E-mail Address:	geethapr@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

For NG-RAN AI/ML use cases RAN3 identified a requirement to collect MDT reports from same group of UEs across RRC state transitions for AI/ML training in OAM. RAN3 agreed to have a solution for Continuous MDT collection and defined the problem statement for the same as below in TR 38.743.

Continuous MDT problem statement from TR 38.743

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

To address the Measurement Continuity problem, as gNB cannot identify the UE across RRC state transitions, gNB can request AMF to store the continuous MDT configuration when UE moves to RRC_IDLE state and AMF can forward the continuous MDT configuration to gNB when UE moves to RRC_CONNECTED state.

RAN3 would like to request SA2 to let RAN3 know the feasibility of storing and forwarding Continuous MDT configuration of the UE by AMF.

2. Actions:

To SA2:

ACTION: 	RAN3 asks SA2 to kindly take the above information into account and provide a response on the feasibility of the solution.

3. Date of Next TSG SA WG2 Meetings:

RAN3#127bis		07-11 April 2025	Wuhan, China

RAN3#128		19-23 May 2025	Malta, Malta