3GPP TSG-RAN WG3 Meeting #127	R3-250256

Athens, GR, 17-21 Feb, 2025

Title:	(TP for BLCR 38.473) Authorization for multi-hop relay

Agenda Item:	22.2

Source:	Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, NEC, CATT, Ericsson, Samsung, ZTE

Document for:	other

# 1	Introduction

# 1. Introduction

This paper introduces a TP to TS 38.473 to support authorization information for SL Multi-hop Relay, based on the progress made by SA2. The following service authorization details should be transferred from the gNB-CU to the gNB-DU over F1.

- 5G ProSe Layer-3 Multi-Hop UE-to-Network Relay

- 5G ProSe Layer-2 Multi-Hop UE-to-Network Relay

- 5G ProSe Layer-2 Multi-Hop Intermediate UE-to-Network Relay

- 5G ProSe Layer-2 Multi-Hop Remote

# Annex3	TP for 38.473

#### 9.3.1.233	5G ProSe Authorized

This IE provides information on the authorization status of the UE to use the 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery | - |  |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication | - |  |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay | - |  |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay | - |  |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE | - |  |
| 5G ProSe Layer-2 Multipath | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the 5G ProSe Layer-2 Remote UE is authorized for 5G ProSe multipath transmission | YES | ignore |
| 5G ProSe Layer-2 UE-to-UE Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Relay UE | YES | ignore |
| 5G ProSe Layer-2 UE-to-UE Remote | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Remote UE. | YES | ignore |
| 5G ProSe Layer-3 Multi-Hop UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 Multi-Hop UE-to-Network Relay. | YES | ignore |
| 5G ProSe Layer-2 Multi-Hop UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Multi-Hop UE-to-Network Relay. | YES | ignore |
| 5G ProSe Layer-2 Multi-Hop Intermediate UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Multi-Hop Intermediate UE-to-Network Relay. | YES | ignore |
| 5G ProSe Layer-2 Multi-Hop Remote | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Multi-Hop Remote. | YES | ignore |

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

[snip]

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-FiveGProSeLayer3MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay,

id-FiveGProSeLayer2MHRemote,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

[snip]

FiveG-ProSeAuthorized ::= SEQUENCE {

fiveG-proSeDirectDiscovery					FiveG-ProSeDirectDiscovery									OPTIONAL,

fiveG-proSeDirectCommunication				FiveG-ProSeDirectCommunication								OPTIONAL,

fiveG-ProSeLayer2UEtoNetworkRelay			FiveG-ProSeLayer2UEtoNetworkRelay							OPTIONAL,

fiveG-ProSeLayer3UEtoNetworkRelay			FiveG-ProSeLayer3UEtoNetworkRelay							OPTIONAL,

fiveG-ProSeLayer2RemoteUE					FiveG-ProSeLayer2RemoteUE									OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { {FiveG-ProSeAuthorized-ExtIEs} }	OPTIONAL,

...

}

FiveG-ProSeAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-FiveG-ProSeLayer2Multipath	CRITICALITY ignore	EXTENSION FiveG-ProSeLayer2Multipath		PRESENCE optional		}|

{ ID id-FiveG-ProSeLayer2UEtoUERelay	CRITICALITY ignore	EXTENSION FiveG-ProSeLayer2UEtoUERelay		PRESENCE optional		}|

{ ID id-FiveG-ProSeLayer2UEtoUERemote	CRITICALITY ignore	EXTENSION FiveG-ProSeLayer2UEtoUERemote		PRESENCE optional		}|

{ ID id-FiveGProSeLayer3MHUEtoNetworkRelay				CRITICALITY ignore	EXTENSION FiveGProSeLayer3MHUEtoNetworkRelay		PRESENCE optional}|

{ ID id-FiveGProSeLayer2MHUEtoNetworkRelay				CRITICALITY ignore	EXTENSION FiveGProSeLayer2MHUEtoNetworkRelay		PRESENCE optional}|

{ ID id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay	CRITICALITY ignore	EXTENSION FiveGProSeLayer2MHIntermediateUEtoNetworkRelay		PRESENCE optional}|

{ ID id-FiveGProSeLayer2MHRemote						CRITICALITY ignore	EXTENSION FiveGProSeLayer2MHRemote					PRESENCE optional},

...

}

FiveG-ProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer3UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2RemoteUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2Multipath ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2UEtoUERelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2UEtoUERemote ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer3MHUEtoNetworkRelay	::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2MHUEtoNetworkRelay	::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2MHIntermediateUEtoNetworkRelay	::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2MHRemote	::= ENUMERATED {

authorized,

not-authorized,

...

}

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

[snip]

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-FiveGProSeLayer3MHUEtoNetworkRelay				ProtocolIE-ID ::= 996 – to be assigned by MCC

id-FiveGProSeLayer2MHUEtoNetworkRelay				ProtocolIE-ID ::= 997 – to be assigned by MCC

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay	ProtocolIE-ID ::= 998 – to be assigned by MCC

id-FiveGProSeLayer2MHRemote							ProtocolIE-ID ::= 999 – to be assigned by MCC