3GPP TSG-RAN WG3 Meeting #127	R3-250261

# Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.415 | CR | 0049 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the PDU set information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, Ericsson, CATT, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R18 XR, in order to transfer the PDU set information for DL packets, the frame format for DL PDU SET INFORMATION was introduced to TS 38.415 in section 6.5. However, the explanation of the information elements in the frame is not complete. The description for the Padding field is absent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description for the Padding field in the DL PDU SET INFORMATION frame. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release). And, there is no functional impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to set the padding bits for the DL PDU SET INFORMATION frame. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

## 6.5	Elements for the PDU Set Information user plane protocol

### 6.5.1	General

The structure of frames is specified as in the section 5.5.1.

### 6.5.2	Frame format for the PDU Set Information user plane protocol

#### 6.5.2.1	DL PDU SET INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN node or gNB-DU to receive PDU Set Information and indication of End of Data Burst of a QoS flow.

The following shows the respective DL PDU SET INFORMATION frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | EDB | EPDU | PSSI | Spare | 1 |
| QoS Flow Identifier (QFI) |  |  |  |  |  | PSSN |  | 2 |
| PSSN |  |  |  |  |  |  |  |  |
| Spare |  |  |  | PSI |  |  |  | 1 |
| PSN |  |  |  |  |  |  |  | 1 |
| PSSize |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 6.5.2.1-1: DL PDU SET INFORMATION (PDU Type 0) Format

### 6.5.3	Coding of information elements in frames

#### 6.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU Set UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SET INFORMATION, 1-15=reserved for future PDU type extensions}.

Field length: 4 bits.

#### 6.5.3.2	Spare

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

#### 6.5.3.3	QoS Flow Identifier (QFI)

Description: When present this parameter indicates the QoS Flow Identifier of the QoS flow to which the transferred packet belongs.

Value range: {0..26-1}.

Field length: 6 bits.

#### 6.5.3.5	PSSI (PDU Set Size Indicator)

Description: This parameter indicates the presence of PDU Set Size (PSSize).

Value range: {0= PSSize not present, 1= PSSize present}.

Field length: 1 bit.

#### 6.5.3.6	Void

#### 6.5.3.7	End PDU of the PDU Set (EPDU)

Description: This parameter indicates whether the current PDU is the last PDU of the PDU set.

Value range: {0= all other PDUs of the PDU Set, 1= last PDU of the PDU set}.

Field length: 1 bit.

#### 6.5.3.8	End of Data Burst (EDB)

Description: This parameter indicates the end of a Data Burst.

Value range: {0 =all other PDUs, 1= the last PDU of a data burst}.

Field length: 1 bit.

#### 6.5.3.9	PDU Set Importance (PSI)

Description: This parameter indicates the importance of the current PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance with the exception that value “0” means sender cannot define importance. PDU Set with the highest importance PDU Set is indicated by 1 and the lowest importance PDU Set is indicated by 15.

Value range: {0..24-1}.

Field length: 4 bits.

#### 6.5.3.10	PDU Set Sequence Number (PSSN)

Description: This parameter indicates the sequence number of the PDU Set to which the current PDU belongs acting as an identifier for the PDU Set.

Value range: {0..210-1}.

Field length: 10 bits.

#### 6.5.3.11	PDU Sequence Number within a PDU Set (PSN)

Description: This parameter indicates the sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..28-1}.

Field length: 1 octet.

#### 6.5.3.12	PDU Set Size (PSSize)

Description: This parameter indicates the total size of all PDUs in bytes of the PDU Set to which the current PDU belongs.

Value range: {0..224-1}.

Field length: 3 octets.

6.5.3.x	Padding

Description: The padding is included at the end of the frame to ensure that the PDU Set Information user plane protocol PDU length (including padding and the future extension) is (n*4– 2) octets, where n is a positive integer. If there is any future extension, the padding should be added after the future extensions.

Field Length: 0–3 octets.

CHANGES END