3GPP TSG-RAN WG3 Meeting #127	R3-250262

Athens, Greece, 17 - 21 February 2025

Agenda Item:	31.1

Source:	Huawei, CMCC, CATT, Nokia, Nokia Shanghai Bell, BT, Vodafone

Title:	(TP for XR BL CR for TS37.340) Support for ECN marking status change

Document for:	Approval

1	Introduction

During RAN3#125 meeting, it was agreed in R3-244771[1] to allow the SN to change the ECN marking status for the DRB using SCG resources, via the SN initiated S-NG-RAN node Modification procedure. Corresponding IEs are added to the S-NODE MODIFICATION REQUIRED messages.

However, the corresponding description for the ECN marking status change initiated by SN is absent in the current stage-2 BL CR for TS 37.340, i.e., R3-245034[2].

This paper provides the TP for capturing the stage 2 description for the SN initiated ECN marking status change procedure.

2	References

[1] R3-244771, (TP to BL CR for TS 38.423) Update on ECN marking in DC #, CATT, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, CMCC, RAN3 meeting #125.

[2] R3-245034, (BL CR to 37.340) Support of XR enhancements, ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi, NEC, RAN3 meeting #125bis.

Annex 1——TP for BL CR for TS 37.340

CHANGES START

### 13.y	ECN marking for L4S and congestion information exposure

The ECN marking for L4S and congestion information exposure as described in TS 38.300 [3] is extended to address the NR-DC operation.

For ECN marking for L4S at gNB or UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the ECN marking request at gNB or ECN marking request at UPF to SN and SN reports the status indication back to the MN.  The SN may change the status and report the new status indication to the MN during the SN initiated SN Modification procedure.

For congestion reporting from gNB to UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the congestion information request to SN and SN reports the status indication back to the MN.

CHANGES END