3GPP TSG-RAN WG3 Meeting #127	R3-250269

Athens, Greece, Feb.17–21, 2025

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 38.473) Discussion on MRO for LTM

Document for:		Discussion and Decision

1	Introduction

RAN3#124 meeting agreed that:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

RAN3#125 meeting agreed that:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure.

RAN3#125bis meeting agreed that:

For TP to BL CR to TS 38.300:

- Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

- For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

- Change “successful handover" definition, as proposed in R3-245230.

For TP to BL CR to TS 38.401:

- Capture wrong LTM prepared cell selection.

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

For TP to BL CR to TS 38.401:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

In this paper, we would further discuss the details of MRO for LTM.

2	Discussion

Based on agreements achieved so far, we can sumarize the failure cause analysis and optimisation as well as RLF report forwarding in F1 as below:

- For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization, CU does not forward the RLF Report to DU;

- For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …), CU initiates analysis and may forward the RLF report to the DU responsible for the failure (e.g. CU forwards RLF Report to the failure DU in case of too late LTM, or, CU forwards RLF Report to the source DU in case of too early LTM or LTM to wrong cell) , the DU responsible for the failure performs optimization.

For the RLF shortly after successful LTM cell switch, C-RNTI of target cell is included in RLF report, however, in case of too early LTM or LTM to wrong cell, the C-RNTI of source PSCell needs to be transferred in F1 interface together with the RLF report to enable source DU to identify UE context. Compared with adding description in TS38.401, it is more straightforward to clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473 to improve text readability. The TP for TS38.473 is attached in the Annex.

Proposal 1: Clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473.

RAN2#127bis meeting agreed that unless RAN3 defines a NW-based solution, the UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). Since it is network that configures the UE to perform UE based TA measurement or PDCCH ordered early TA acquisition, network can identify how the UE got the TA value when receiving the RLF report, based on the stored or retrieved UE context (e.g. configuration related to early TA acquisition, configuration related to UE based TA measurement), it is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

Proposal 2: It is not needed for the UE to report whether and how the UE got the TA value used for a failed LTM cell switch in RLF report.

Similarly, network configures the LTM candidate cell(s), it can identify whether a measured neighbour cell included in the RLF report is an LTM candidate cell or not, based on the stored or retrieved UE context e.g. via the timeConnFailure and C-RNTI IE reported in RLF report, therefore, it is not needed for the UE to log and report the LTM candidate cell list or an indication concerning whether a measured neighbour cell is an LTM candidate cell or not.

Proposal 3: It is not needed for UE to report the LTM candidate cell list or an indication concerning whether a measured neighbour cell is an LTM candidate cell or not in RLF Report.

For an intra-DU LTM or inter-DU intra-CU LTM procedure, ping-pong issue may occur especially when subsequent LTM is supported. Upon LTM cell switch is successful, CU may receive the ACCESS SUCCESS message, to avoid or solve ping-pong issue, CU collects or updates the UHI upon receiving the ACCESS SUCCESS message.

Proposal 4: CU collects or updates the UHI upon receiving the ACCESS SUCCESS message.

To detect ping-pong event,

- Option 1: CU detects ping-pong event based on the UHI, then CU indicates the detected ping-pong event to source DU, e.g. via the ACCESS AND MOBILITY INDICATION message.

- Option 2: CU forwards the UHI to source DU e.g. via the ACCESS AND MOBILITY INDICATION message, then source DU detects ping-pong event.

Based on the indicated or detected ping-pong event, source DU performs optimisation for LTM parameters, and, the UHI is taken into account for target cell selection for LTM cell switch to avoid ping-pong.

Proposal 5: RAN3 to discuss ping-pong event detection,

- Option 1: CU detects ping-pong event based on the UHI, then CU indicates the detected ping-pong event to source DU, e.g. via the ACCESS AND MOBILITY INDICATION message.

- Option 2: CU forwards the UHI to source DU e.g. via the ACCESS AND MOBILITY INDICATION message, then source DU detects ping-pong event.

3	Conclusion

In this paper, MRO for LTM are further discussed. We have the following proposals:

Proposal 1: Clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473.

Proposal 2: It is not needed for the UE to report whether and how the UE got the TA value used for a failed LTM cell switch in RLF report.

Proposal 3: It is not needed for UE to report the LTM candidate cell list or an indication concerning whether a measured neighbour cell is an LTM candidate cell or not in RLF Report.

Proposal 4: CU collects or updates the UHI upon receiving the ACCESS SUCCESS message.

Proposal 5: RAN3 to discuss ping-pong event detection,

- Option 1: CU detects ping-pong event based on the UHI, then CU indicates the detected ping-pong event to source DU, e.g. via the ACCESS AND MOBILITY INDICATION message.

- Option 2: CU forwards the UHI to source DU e.g. via the ACCESS AND MOBILITY INDICATION message, then source DU detects ping-pong event.

Annex: TP for TS38.473

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU in case that the connection Failure type IE in the nr-RLF-Report-r16 IE is set to “RLF” and RLF Report Failure Type IE is set to “too early LTM” or “LTM to wrong cell” | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |