3GPP TSG-RAN WG3 Meeting #127	R3-250271

Athens, Greece, Feb.17–21, 2025

Agenda item:	 10.3.1

Source:		Lenovo

Title:		Discussion on MRO for intra-NTN mobility

Document for:		Discussion and Decision

1	Introduction

RAN3#125 meeting agreed that:

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

The RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

-      Measured UTC time when RLF CHO is executed

Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN?

RAN3#125bis meeting further agreed that:

For time based trigger condition:

WA: down-select among below three alternatives.

Alt 1: the RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

Alt 2: a flag to indicate whether the RLF happens before T1 or after T1

Alt 3: based on existing timers in RLF report.

Further check the granularity of the current timers.

For location based trigger condition:

Too late CHO Execution: RLF happens in the source cell before CHO execution. The UE re-establishes in a cell other than the source cell.

Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

For earth fixed case, there is no need to report the measured distance from the UE.

FFS on the earth moving case.

For joint trigger condition:

Agree on the following definition of the use cases and agreement:

- Case 1: Too Late CHO Execution: RLF happens in the source cell. The UE re-establishes in the cell other than the source cell.

- Case 2: Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

RAN3#126 meeting further agreed that:

For single time based trigger condition:

Existing timers can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, FFS if the accuracy is a problem to be solved.

For single location based trigger condition:

WA: There is no need to report any other info in the RLF report for the location based CHO.

For multiple trigger conditions:

No consensus on further enhancement.

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

In this paper, we would further discuss MRO for intra-NTN mobility.

2	Discussion

RAN3#124 meeting agreed MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover, and the following trigger conditions for MRO for intra-NTN mobility shall be considered:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

For time based trigger condition, or time-based and measurement-based combined trigger condition:

RAN3#125 meeting agreed to consider Case 1 Too Late CHO Execution for CHO with a time-based trigger condition: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. And it was also agreed to distinguish if the RLF happens before or after T1. One left issue from RAN3#125bis meeting was how network can distinguish it, there are three alts on table:

- Alt 1: the RLF report includes for single time-based trigger condition:

  - Absolute time (measured UTC time) when RLF occurred

- Alt 2: a flag to indicate whether the RLF happens before T1 or after T1

- Alt 3: based on existing timers in RLF report.

In Alt3, network can derive the time when the RLF failure happens based on the existing time information IE (e.g. timeSinceFailure IE) in RLF report, and network can distinguish whether the RLF happens before or after T1 because it can also know the absolute time T1 e.g. based on UE context. Alt3 can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, the 100ms accuracy is sufficient for MRO purpose, and there is no critical issue to be solved. On the other hand, the size of absolute time (measured UTC time) is big, and reporting absolute time (measured UTC time) when RLF occurred in the RLF report would cause UE complexity, especially considering that RAN2 want to prioritize RAN3-based solutions and RAN2 agree to pause and wait if any for RAN3 LS for SON/MDT for NTN. Alt 2 is a comprised way but also a UE based solution, it needs further work in RAN2. Therefore, there is no need to report additional information concerning whether the RLF happens before or after T1 (e.g. absolute time (measured UTC time) when RLF occurred, or a flag to indicate whether the RLF happens before T1 or after T1) in the RLF report.

Observation: In case of Too Late CHO Execution for CHO with a time-based trigger condition, existing timers can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, the accuracy is not a critical issue to be solved.

Proposal 1: In case of Too Late CHO Execution for CHO with a time-based trigger condition, there is no need to report additional information concerning whether the RLF happens before or after T1 (e.g. absolute time (measured UTC time) when RLF occurred, or a flag to indicate whether the RLF happens before T1 or after T1) in the RLF report.

For CHO configured with a time-based trigger condition, or CHO configured with time-based and measurement-based combined trigger condition, a connection failure may happen due to configuring inappropriate parameter(s), besides the time when the connection failure happens, the time when CHO is executed may also be derived based on the existing time information IE (e.g. timeSinceCHO-Reconfig IE, timeConnFailure IE, or timeSinceFailure IE) in RLF report. It is unnecessary to include the absolute time (measured UTC time) when CHO is executed in the RLF report.

Proposal 2: It is unnecessary to include the absolute time (measured UTC time) when CHO is executed in the RLF report.

For CHO configured with time-based and measurement-based combined trigger condition, there may be a case that an RLF happens due to the absolute time value t1-Threshold + duration arrives, but measurement based CHO execution condition is not fulfilled. Since signal strength/quality difference between NTN cell center and cell edge is small, time-based trigger condition has bigger impacts to mobility performance compared with measurement-based trigger condition in CHO NTN, it is beneficial to enable network know that measurement based CHO execution condition is setting inappropriately to avoid such a failure case, therefore, measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold + duration, may be stored or reported by the UE in RLF report.

Proposal 3: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold + duration.

For location based trigger condition, or location-based and measurement-based combined trigger condition:

For CHO configured with a location-based trigger condition, RAN3#125bis meeting agreed there is no need to report the measured distance from the UE for earth fixed case, and it is FFS on the earth moving case. RAN3#126 meeting made a WA that there is no need to report any other info in the RLF report for the location based CHO. We can find that in earth moving case, network can also derive the distance between UE and source cell/candidate cell(s) when HOF/RLF happens, e.g. based on the UE location information included in the RLF report and the moving reference location when HOF/RLF happens determined based on referenceLocation1.

Proposal 4: It is unnecessary to include the distance between UE and source cell/candidate cell(s) when HOF/RLF happens in the RLF report.

For both earth fixed case and earth moving case, for CHO configured with location-based and measurement-based combined trigger condition, there may be a case that location based CHO execution condition is fulfilled but measurement based CHO execution condition is not fulfilled before RLF happens, since signal strength/quality difference between NTN cell center and cell edge is small, location-based trigger condition has bigger impacts to mobility performance compared with measurement-based trigger condition in CHO NTN, to enable network know that measurement based CHO execution condition is setting strictly, measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens, may be stored or reported by the UE in RLF report.

Proposal 5: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens.

Fallback from RACH-less HO to RACH-based HO

Fallback from RACH-less HO to RACH-based HO is supported in NTN. If RA-InformationCommon is included in RLF report, it means that fallback from RACH-less HO to RACH-based HO is performed, then network may optimize RACH-less HO related configurations. So, an explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in the RLF report.

Proposal 6: An explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in the RLF report.

3	Conclusion

In this paper, MRO enhancements for intra-NTN mobility are discussed. We have the following observation and proposals:

Observation: In case of Too Late CHO Execution for CHO with a time-based trigger condition, existing timers can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, the accuracy is not a critical issue to be solved.

Proposal 1: In case of Too Late CHO Execution for CHO with a time-based trigger condition, there is no need to report additional information concerning whether the RLF happens before or after T1 (e.g. absolute time (measured UTC time) when RLF occurred, or a flag to indicate whether the RLF happens before T1 or after T1) in the RLF report.

Proposal 2: It is unnecessary to include the absolute time (measured UTC time) when CHO is executed in the RLF report.

Proposal 3: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold + duration.

Proposal 4: It is unnecessary to include the distance between UE and source cell/candidate cell(s) when HOF/RLF happens in the RLF report.

Proposal 5: RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled before RLF happens.

Proposal 6: An explicit indication regarding fallback from RACH-less HO to RACH-based HO is not needed in the RLF report.