3GPP TSG-RAN WG3 Meeting #127	R3-250280

Athens, Greece, 17th – 21th Febuary, 2025

Agenda Item:	12.2

Source: 	Lenovo

Title: 	(TP to BL CR 38.401) Discussion on ULI and WAB node migration

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) focus on RAN3.

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management.

In this contribution, the ULI issues and procedures for WAB-node migration are discussed, including WAB-MT and WAB-gNB migration.

- Discussion

User Location Information

RAN3 #126: WA: Additional ULI for WAB consists of TAI and NR CGI, which are determined by the WAB-node, reflecting the WAB-node’s physical location. This solution is compliant with Opton1 and Option3. It is up to SA2 to support one of Opton1 and Option3 or both. It is FFS whether there are issues with applying the solution to MT access via NTN. It is FFS whether further information needs to be included in the additional ULI Clause 8.5 of TR 23.700-06: - The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is: - TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN. - If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative phase. - option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM. - option 2: geo-location of the MWAB. - option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN. - The AMF may provide the received Additional ULI to LMF in LCS request.

In case of NTN case, as defined in TS 38.300, the Cell Identity used in ULI of NTN is a Mapped Cell ID rather than legacy Cell ID. And the TAI IE in ULI is ignored in case of NTN and the gNB reports NR NTN TAI Information instead as in TS 38.413.

In addition, based on the LS from SA2, Option 1 is preferred than Option 3 which cannot be applied for NTN backhaul. Therefore, to align with SA2, the WAB-gNB should report mapped TAI and NR CGI reflecting the WAB-node’s physical location in the additional ULI in case the PLMN serving the WAB-MT and the PLMN broadcasted by the WAB-gNB are not the same, and the mapped TAI and NR CGI are determined by the WAB-gNB based on input from the OAM.

Proposal 1: WAB-gNB should report mapped TAI/NR CGI reflecting the WAB-node’s physical location in the additional ULI in case the PLMN serving the WAB-MT and the PLMN broadcasted by the WAB-gNB are not the same, and the mapped TAI/NR CGI are determined by the WAB-gNB based on input from the OAM.

WAB-node migration

In RAN3 #124 meeting, following agreements had been achieved for scenarios of WAB-node migration.

Support the following scenarios for intra-PLMN WAB-node mobility: - UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN. - UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN. - BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN. - BH UPF/AMF changes as WAB-MT moves inside a PLMN.

Case 1: BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN

In case of WAB-MT migration without BH UPF/AMF changing, the WAB-MT performs handover from a source BH RAN node to a target BH RAN node, which both connect to the same BH AMF, and the WAB-MT anchors at the same BH UPF in the source and target path. A brief flow chart and migration procedure are as follow.

Figure 1: An example of WAB-MT migration procedure with BH 5GC unchanged

1.	The WAB-MT sends a MeasurementReport message to the source BH RAN node. This report is based on a Measurement Configuration the WAB-MT received from the source BH RAN node before.

2.  The source BH RAN node sends a HANDOVER REQUEST message to the target BH RAN node over the Xn interface.

3.	The target BH RAN node performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message to the source BH RAN node.

4.	The source BH RAN node forwards the received RRCReconfiguration message to the WAB-MT.

5.	A random access procedure is performed at the target BH RAN node.

6.	The WAB-MT responds to the target BH RAN node with an RRCReconfigurationComplete message.

7.	The target BH RAN node triggers the path switch procedure for migration of the WAB-MT, then all the PDU sessions of the WAB-MT are switched to the target path, and all NG and Xn traffic are transported via the target path from/to the BH UPF over the PDU sessions.

8.	The target BH RAN node sends UE CONTEXT RELEASE message to the source BH RAN node after WAB-node migration.

Proposal 2: Agree the procedure for WAB-MT migration with BH UPF/AMF unchanged.

Case 2: BH UPF/AMF changes as WAB-MT moves inside a PLMN

For this scenario, it may be further divided into follow sub-scenarios:

- BH AMF remains unchanged and BH UPF changes as WAB-MT moves inside a PLMN

- Both BH AMF and BH UPF change as WAB-MT move inside a PLMN

Because all the NG/Xn traffic of the WAB-gNB and UP traffic of served UEs need to be backhauled via the BH PDU session, when the BH UPF changing during WAB-MT migration, the WAB-MT needs to setup new PDU session with the target BH UPF and release the old PDU session with the old BH UPF. The detail procedure has main impact on the core network and can be captured in SA2.

Case 3: UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN

As the UE’s AMF remains unchanged, the gNB ID for the served UEs of the WAB-node also remains unchanged. While for the cell ID and TAC, they can be also remains unchanged and no additional impacts on UE are introduced.

In some other case, the cell ID, PCI and TAC may be changed without the change of the UE’s AMF(s). For example, if the TAC changes, the WAB-gNB needs to inform its AMF with the new TAC, and the served UE may need to perform a Mobility Registration Update procedure. Then the legacy procedure can be reused for the case of WAB-gNB migration without UE’s AMF/UPF changed.

Proposal 3: The legacy procedures for change of cell ID, PCI and TAC can be reused for the case of WAB-gNB migration without UE’s AMF/UPF changed.

Case 4: UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN

RAN3 #126: The “two logical gNB solution” can support UE’s AMF change during WAB-gNB mobility.

This scenario is similar to mobile IAB-DU migration, where the mobile IAB-node supports two logical mobile IAB-DUs having F1 connections with the source F1-terminating IAB-donor-CU and target F1-terminating IAB-donor-CU, respectively. To realize WAB-gNB inter-AMF migration, the WAB-node needs to support two logical WAB-gNB concurrently during the migration. For the UE(s) served by the WAB-node, they are handed over from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB. And after all UEs are handed over, the NG connection between source logical WAB-gNB and source AMF may be removed. A brief flow chart and migration procedure are as follow.

Figure 2: An example of WAB-gNB migration procedure with UE´s AMF/UPF change.

1.	The target logical WAB-gNB sends a NG SETUP REQUEST message to the target UE’s AMF in the UE’s 5GC.

2.  The target UE’s AMF responses to the target logical WAB-gNB with a NG SETUP RESPONSE message. After the NG is set up, the target logical WAB-gNB can start serving UE(s).

3.	The source logical WAB-gNB performs NG-based UE handover from from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB.

4.	After all the UEs are handed over, the NG connection between the source logical WAB-gNB and the source UE’s AMF is removed.

Proposal 4: Agree the procedure for WAB-gNB migration with UE´s AMF/UPF change.

Proposal 5: After all the UEs are handed over, the NG connection between the source logical WAB-gNB and the source UE’s AMF is removed.

RAN3 #126: It is for further discussions which solution to adopt, if the single gNB solution, the “two gNB solution” or both. It is to be further checked whether a single gNB solution is feasible via an LS to SA2, if the LS is agreeable.

As for the single logical WAB-gNB solutions, an LS was sent to SA2 (and CC RAN2) in last meeting to check whether these single-gNB solutions can be achieved without CN impact. Based on the Reply LS, there is no consensus on the support of a single-gNB solution in SA2. Therefore, the single-gNB solution may be deprioritized unless further inputs from other WGs.

# Conclusion

This contribution aims to discuss ULI for WAB and WAB migration procedures. And following observations and proposals are concluded.

Proposal 1: WAB-gNB should report mapped TAI/NR CGI reflecting the WAB-node’s physical location in the additional ULI in case the PLMN serving the WAB-MT and the PLMN broadcasted by the WAB-gNB are not the same, and the mapped TAI/NR CGI are determined by the WAB-gNB based on input from the OAM.

Proposal 2: Agree the procedure for WAB-MT migration with BH UPF/AMF unchanged.

Proposal 3: The legacy procedures for change of cell ID, PCI and TAC can be reused for the case of WAB-gNB migration without UE’s AMF/UPF changed.

Proposal 4: Agree the procedure for WAB-gNB migration with UE´s AMF/UPF change.

Proposal 5: After all the UEs are handed over, the NG connection between the source logical WAB-gNB and the source UE’s AMF is removed.

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- 3GPP TR 38.799.

# TP to BL CR 38.401

# X	Wireless Access Backhaul

NEXT CHANGE

## X.3	WAB-node migration procedure

### X.3.a	Migration of WAB-MT with BH 5GC unchanged

The WAB-MT of the WAB-node can be migration from a source BH RAN node to a target BH RAN node. Both the source BH RAN node and the target BH RAN node connect to the same BH AMF, and the WAB-MT anchors at the same BH UPF in the source and target path. Figure X.3.a-1 shows an example of WAB-MT migration procedure with BH 5GC unchanged.

Figure X.3.a-1: An example of WAB-MT migration procedure with BH 5GC unchanged

1.	The WAB-MT sends a MeasurementReport message to the source BH RAN node. This report is based on a Measurement Configuration the WAB-MT received from the source BH RAN node before.

2.  The source BH RAN node sends a HANDOVER REQUEST message to the target BH RAN node over the Xn interface.

3.	The target BH RAN node performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message to the source BH RAN node.

4.	The source BH RAN node forwards the received RRCReconfiguration message to the WAB-MT.

5.	A random access procedure is performed at the target BH RAN node.

6.	The WAB-MT responds to the target BH RAN node with an RRCReconfigurationComplete message.

7.	The target BH RAN node triggers the path switch procedure for migration of the WAB-MT, then all the PDU sessions of the WAB-MT are switched to the target path, and all NG and Xn traffic are transported via the target path from/to the BH UPF over the PDU sessions.

8.	The target BH RAN node sends UE CONTEXT RELEASE message to the source BH RAN node after WAB-node migration.

NEXT CHANGE

### X.3.b	Migration of WAB-gNB with UE´s AMF/UPF change

To realize WAB-gNB inter-AMF migration, the WAB-node needs to support two logical WAB-gNB concurrently during the migration. For the UE(s) served by the WAB-node, they are handed over from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB. Figure X.3.b-1 shows an example of WAB-gNB migration procedure with UE’s AMF/UPF change.

Figure X.3.b-1: An example of WAB-gNB migration procedure with UE´s AMF/UPF change

1.	The target logical WAB-gNB sends a NG SETUP REQUEST message to the target UE’s AMF in the UE’s 5GC.

2.  The target UE’s AMF responses to the target logical WAB-gNB with a NG SETUP RESPONSE message. After the NG is set up, the target logical WAB-gNB can start serving UE(s).

3.	The source logical WAB-gNB performs NG-based UE handover from from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB.

4.	After all the UEs are handed over, the NG connection between the source logical WAB-gNB and the source UE’s AMF is removed.