3GPP TSG RAN WG3 #127	R3-250289

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	17.2

Source: 	Lenovo

Title: 	Discussion on On-demand SSB for SCell

Document for:	Discussion and Decision

Introduction

In RAN#102, a new work item “Enhancements of network energy savings for NR” was agreed [1]. The one objective of the work item is the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we analyse RAN3 impact and high-level solution based on NW-triggered on-demand SSB operation.

Discussion

2.1 RAN1 progress

In RAN1#116, RAN1 has discussed 3 triggering methods of on-demand SSB:

- Method 1: UE uplink wake-up-signal using an existing signal/channel

- Method 2: Cell on/off indication via backhaul

- Method 3: SCell activation/deactivation signaling

The 2nd and 3rd methods can be categorized into NW-triggered on-demand SSB operation. Many companies have not been convinced yet whether to support UE-triggered on-demand SSB operation. In high level, NW-triggered on-demand SSB operation is adopted, how to design NW triggering message and when to transmit message is FFS. And it was agreed:

- RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations. - For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission - Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for

In RAN1#116bis, RAN1 has further discussed on-demand SSB triggered by gNB, reached agreement on:

- identified scenarios and cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1, - Scenario #2A and Case #2 - Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - when and how to transmit that on-demand SSB burst(s) after trigger on-demand SSB SCell operation by NW. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

In RAN1#117, RAN1 has further discussed on-demand SSB triggered by gNB, reached agreement on:

- For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

In RAN1#118bis, RAN1 has further discussed on-demand SSB triggered by gNB, reached agreement on:

For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

In RAN1#119, RAN1 has further discussed on-demand SSB triggered by gNB, reached agreement on:

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Observation 1: NW-triggered on-demand SSB operation has been agreed by RAN1 for network energy saving:

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A, wherein MAC CE can indicate one of multiple candidate values configured by RRC.

Observation 2: Two Scenarios of NW triggering message for on-demand SSB have been identified by RAN1:

  - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command.

  - Scenario #2A: SCell is configured to a UE when a UE receives SCell activation command.

Observation 3: Two cases of SSB transmission on a cell supporting on-demand SSB Scell include: 1) no always-on SSB transmission, 2) SSB transmission with long periodicity.

2.2 on-demand SSB operation

RAN3#126 has discussed and reached agreement on RRC based signaling for on-demand SSB operation at following:

- For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

According to the agreement, if DU makes decision to activate/deactivate OD-SSB, DU may start/stop transmitting OD-SSB to UE first and convey the activation state to CU, then CU transmits it in RRC message to UE via DU. There may be gap between transmission of RRC message and OD-SSB transmission. No critical issue is foreseen.

RAN1 has agreed to support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Scenario #2A refers to “When UE receives SCell activation command”, and Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command. In both Scenario #2A and Scenario #2, if DU makes decision to activate/deactivate OD-SSB, it transmits MAC-CE for on-demand SSB transmission indication to UE and start/stop transmitting OD-SSB to UE, and when and how long to transmit OD-SSB is up to RAN1 decision. There is no RAN3 impact.

For the MAC CE based signaling, RAN3 discusses whether CU node needs to decide to trigger on-demand SSB. If CU determines to trigger on-demand SSB, e.g., on/off the SSB transmission, or switch to long or short SSB transmission period, CU transmits a command to DU, accordingly DU transmits MAC CE message to UE and transmits SSB based on the command.

Proposal 1: RAN3 discusses whether CU node could decide to trigger on-demand SSB. E.g., CU determines to trigger on-demand SSB, e.g., on/off the SSB transmission, or switch to long or short SSB transmission period, CU transmits a command to DU, accordingly DU transmits MAC CE message to UE and transmits SSB based on the command.

NG-RAN node may determine to trigger on-demand SSB transmission based on request/indication by the neighbor NG-RAN node. For example, if NG-RAN node enters energy saving mode by switch off SSB transmission or switch to long SSB transmission period for a NES cell and it transmits an indication to indicate that the concerned cell is in this energy saving mode to the neighbour NG-RAN node. The neighbour NG-RAN requests to trigger On-demand SSB transmission of the cell if it needs to be used for layer 3 measurement.

A straightforward for the high-level solution from RAN3 point of view is introduce an on-demand SSB trigger IE, which is separate from the Cell and SSB activation IE to implement the on-demand SSB operation between NG-RAN nodes.

Proposal 2: RAN3 to introduce an on-demand SSB trigger IE which is separate from the Cell and SSB activation IE to implement the on-demand SSB operation between NG-RAN nodes.

Conclusion

In summary, we have following observations and proposals for on-demand SSB for SCell in carrier aggregation:

Observation 1: NW-triggered on-demand SSB operation has been agreed by RAN1 for network energy saving:

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A, wherein MAC CE can indicate one of multiple candidate values configured by RRC.

Observation 2: Two Scenarios of NW triggering message for on-demand SSB have been identified by RAN1:

  - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command.

  - Scenario #2A: SCell is configured to a UE when a UE receives SCell activation command.

Observation 3: Two cases of SSB transmission on a cell supporting on-demand SSB Scell include: 1) no always-on SSB transmission, 2) SSB transmission with long periodicity.

Proposal 1: RAN3 discusses whether CU node could decide to trigger on-demand SSB. E.g., CU determines to trigger on-demand SSB, e.g., on/off the SSB transmission, or switch to long or short SSB transmission period, CU transmits a command to DU, accordingly DU transmits MAC CE message to UE and transmits SSB based on the command.

Proposal 2: RAN3 to introduce an on-demand SSB trigger IE which is separate from the Cell and SSB activation IE to implement the on-demand SSB operation between NG-RAN nodes.

References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, RAN#102.

- R3-247440, Discussion on On-demand SSB for SCell, Lenovo