3GPP TSG-RAN WG3 #127	R3-250291

Athens, GR, 17th – 21st Feb 2025

Agenda Item:	20.2

Source:	Lenovo

Title: 		AIML for gNB assisted positioning

Document for:	Discussion and Approval

# Introduction

In the last RAN3 meeting, the following agreements were made. In this paper, we further discuss the rest issues.

Case 3b:

Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

Case 3a:

The AI/ML model (inference and training functions) is at the gNB.

Model Training: Label information as training data from LMF to gNB? Data collection from UE or TRP or from LMF?

Model performance monitoring: OptionA or OptionB?

Option A. gNB performs monitoring metric calculation for its own model.

Option B. LMF performs monitoring metric calculation for the model located at the gNB.

The existing LCS framework is re-used for AI/ML positioning.

Case 3a agreements:

- LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

- How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

- NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

- WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For case 3b:

- NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

Turn WA to agreement:

RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

FFS if LMF should be aware that the reported measurements are AI/ML generated.

# Discussion

## Case 3a: AIML for NG-RAN node assisted positioning with gNB-side model

In the previous meeting, RAN3 discussed how gNB can collect data from LMF, and a working assumption was made:

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

This would assume that a positioning session is on-going for a UE, only that gNB could request the demanded part B information from LMF for the associated UE. To do that gNB would generate an NAPPa message (e.g., UL UE ASSOCIATED NRPPA TRANSPORT) that contains the NRPPa-PDU as well as AMF/RAN UE NGAP ID to identify the UE. Then AMF will translate the UE NGAP ID to the UE correlation ID when forwarding the NRPPa-PDU to LMF over SBI.

9.2.9.2 UPLINK UE ASSOCIATED NRPPA TRANSPORT This message is sent by the NG-RAN node and is used for carrying NRPPa message over the NG interface. Direction: NG-RAN node AMF IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. Routing ID; M; 9.3.3.13; YES; reject. NRPPa-PDU; M; 9.3.3.14; YES; reject.

For case 3a, RAN3 supports gNB to request part B information for a UE that is under an on-going UL based positioning session, i.e., with assigned AMF/RAN UE NAGP ID.

In the last RAN3 meeting, it was agreed that LMF provides ground truth label and related data to NG-RAN node, while it is unclear how it can be done over NRPPa interface. In our view, considering there is no existing NRPPa procedure that can be reused for such purpose, the easiest way would be introduce a new NRPPa procedure for gNB to request the ground truth label and related data to NG-RAN node.

For Case 3a, RAN3 introduces a new UE associated NRPPa procedure for gNB to request and receive the UE part B information from LMF.

The concerned NRPPa message in Proposal 2 is contained in the NGAP UL/DL UE ASSOCIATED NRPPA TRANSPORT messages, wherein the AMF/RAN UE NGAP ID is used to identify the UE.

RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For activating a gNB-sided model, one needs to know in which scenario the trained AI model will be applicable. This is by nature known by gNB itself, but difficult to transfer the meta data about the gNB-sided AI model to LMF. Besides, since last meeting, RAN3 agreed to only support RAN based monitoring for case 3a, then it should be upon gNB’s own decision if an AIML model will be activated/deactivated and used to improve the positioning accuracy for case 3a.

FFS if LMF should be aware that the reported measurements are AI/ML generated.

Since there is no mean for LMF to understand if the measurements reported by gNB based on AIML is good or not, there seems no need for LMF to be aware if the measurements is generated by AIML neither.

For Case 3a, it is upon gNB’s decision if an AIML based method will be used to improve the positioning accuracy.

LMF does not need to explicitly know if the measurements reported by gNB are AIML generated or not.

## Case 3b: AIML for NG-RAN node assisted positioning with LMF-side model

RAN1 agreements relevant to Case 3b: Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

SA2 has sent an LS to RAN3 in R3-250074 asking RAN3’s feedback related to

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

According to RAN1 progress so far, for case 3b, the only RAN3 impact would be any enhancement to support new measurement exchange between gNB and LMF. For example,

For sample-based measurement (if supported), LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

When gNB reports the channel measurement result to LMF, gNB may also report timing information and paired timing information and power information.

For Case 3b, based on RAN1 discussion, RAN3 supports exchanging the following information between gNB and LMF over NRPPa:

  - From LMF to gNB, if sample-based measurement is supported, LMF can signal parameter values of Nt, Nt', k to gNB

  - From gNB to LMF, gNB will report (mandatory) channel measurement, (optional) quality of the channel measurement, and (mandatory) time stamp of the channel measurement.

In legacy, NRPPa procedures such as Positioning Information Exchange, TRP Information Exchange, Measurement procedures were specified for gNB to perform UL SRS based measurement according to LMF’s demand and send the result to LMF for UL based positioning (UL-TDOA, UL-AoA). Those existing NRPPa procedures can be used with possible enhancement to provide input for the LMF-sided model in case 3b.

For Case 3b, existing NRPPa procedures such as Positioning Information Exchange, TRP Information Exchange, Measurement procedures can be used with possible enhancement to provide input for the LMF-sided model.

From RAN load perspective, there seems no fundamental difference compared to when gNB is involved in legacy UL based positioning (UL-TDOA, UL-AoA). Therefore, no specific enhancement is foreseen.

For Case 3b, RAN3 assumes the load experienced by RAN is similar as when RAN is involved in legacy UL based positioning.

# Conclusion

Based on the discussion above, we observe:

Based on the discussion above, we propose:

Proposal 1	For case 3a, RAN3 supports gNB to request part B information for a UE that is under an on-going UL based positioning session, i.e., with assigned AMF/RAN UE NAGP ID.

Proposal 2	For Case 3a, RAN3 introduces a new UE associated NRPPa procedure for gNB to request and receive the UE part B information from LMF.

Proposal 3	The concerned NRPPa message in Proposal 2 is contained in the NGAP UL/DL UE ASSOCIATED NRPPA TRANSPORT messages, wherein the AMF/RAN UE NGAP ID is used to identify the UE.

Proposal 4	For Case 3a, it is upon gNB’s decision if an AIML based method will be used to improve the positioning accuracy.

Proposal 5	LMF does not need to explicitly know if the measurements reported by gNB are AIML generated or not.

Proposal 6	For Case 3b, based on RAN1 discussion, RAN3 supports exchanging the following information between gNB and LMF over NRPPa:

a.	From LMF to gNB, if sample-based measurement is supported, LMF can signal parameter values of Nt, Nt', k to gNB

b.	From gNB to LMF, gNB will report (mandatory) channel measurement, (optional) quality of the channel measurement, and (mandatory) time stamp of the channel measurement.

Proposal 7	For Case 3b, existing NRPPa procedures such as Positioning Information Exchange, TRP Information Exchange, Measurement procedures can be used with possible enhancement to provide input for the LMF-sided model.

Proposal 8	For Case 3b, RAN3 assumes the load experienced by RAN is similar as when RAN is involved in legacy UL based positioning.