3GPP TSG-RAN WG3 Meeting #127	R3-250294

Athens, Greece, 17 – 21 February, 2025

Agenda item:	8.1

Source: 	Huawei

Title: 	Discussion on the XR available data rate report

Document for:	Discussion and Decision

# 1. Introduction

During RAN3#125 meeting, RAN3 provided feedback on RAN available data rate exposure. SA2 replied RAN3 with further clarification and explanation on the solution in S2-2412720/ R3-250018 [1]. In this contribution, we will analyse the RAN3 impacts based on the reply from SA2. Some potential specification impacts will be provided in TPs [4] and [5] based on the discussion in this contribution.

# 2 Discussion

SA2 has investigated the solution to expose RAN available data rate to the application layer so that the application can adjust the data rate accordingly. RAN3 has replied to SA2 that it is feasible for NG-RAN node to provide the available data rate to CN via UP at least for semi-static periodicity traffic in R3-244844 [2]. Based on RAN3 feedback, SA2 provided more details of the solution in S2-2412720, which is excerpted below.

- For semi-static periodicity traffic, it is feasible for NG-RAN node to provide the available data rate to CN via UP. For dynamic traffic, some companies have concern that the reporting of available data rate may not be accurate. RAN3 would like SA2 to further clarify the definition of “available data rate”, e.g., it is only below the GFBR, or between GFBR and MFBR, or even above MBFR; whether it should be periodic reporting, on demand reporting, or threshold(s) defined reporting. Feedback from SA2: From SA2 perspective, it is expected that the available data rate is properly reflecting the resources that are currently provided by the NG-RAN for the QoS Flow. While it is up to the application layer how to make use of it, we can however assume that the application will typically exchange data with a somewhat lower rate than what is reserved or measured at NG-RAN. Regarding the definition of “available data rate”, it refers to the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR. The “available data rate” can be between the GFBR and the MFBR for a QoS Flow with MFBR>GFBR. The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP. The “available data rate” should however not be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR). Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR). Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly. The usage of the first packets after a change in the “available data rate” should be sufficient to transfer the information to the UPF. Similar to the logic described in 23.501 clause 5.7.2.4.1a, the NG-RAN is expected to immediately report a change to a lower threshold value for the “available data rate” and a change to a higher threshold value with some hysteresis.

Regarding the details on the available data rate report, SA2 expects NG-RAN to provide the available data rate based on one or more thresholds provided by the application. In our understanding, the thresholds actually determine several ranges of data rate. The NG-RAN node can use the bits referred to these thresholds to indicate the range of the available data rate. Such available data rate report by indicating a reference of the respective threshold can be carried via UP.

For example, assuming that there are three thresholds, e.g. T1, T2 and T3 (T3>T2>T1), the last reported available data rate is T1, which means the available data rate is in the interval between T1 and T2. After a while, if the NG-RAN node is able to allocate more resources to the QoS flow, the available data rate is increased and reaches the interval between T2 and T3. Then the NG-RAN node should notify the CN with the updated value. For example, it can send a reference corresponding to T2 to the CN via UP to indicate that the latest available data rate is within the range T2~T3.

The thresholds can be provided to NG-RAN by the CN via CP, e.g. as part of the QoS flow level QoS parameters. A potential implementation is that the CN provides NG-RAN with one or more thresholds along with their corresponding index(es). When the NG-RAN node needs to indicate the available data rate, it can send an index corresponding to one of the thresholds, where the index i means the available data rate is in the range between the ith threshold and the i+1th threshold. The thresholds can be provided for UL and DL separately.

Proposal 1: CN provides one or more thresholds to NG-RAN via CP. NG-RAN indicates the available data rate based on these configured thresholds via UP.

According the SA2’s LS R3-250018, the QoS requirements of the QoS flow should also be considered when determining the available data rate. For each QoS flow, the indicated available data rate should not over the MFBR. If the QoS flow is not configured with QNC or it is enabled with QNC but without AQP, the available data rate reported via UP can be smaller than GFBR. If the QoS flow is configured with AQP, the available data rate reported via UP should not be below the GFBR. This provides the boundary of the available data rate. From RAN3 perspective, the above limitations can be considered by the CN when providing the thresholds to NG-RAN. There is no need for NG-RAN to check whether the indicated available data rate is aligned with the QoS requirements.

Proposal 2: MFBR and GFBR can be considered when setting the values of the thresholds.

---------Excerption from TS 23.501--------

##### 5.7.2.4.1a	Notification Control without Alternative QoS Profiles

---------unrelated parts are skipped--------

When the NG-RAN determines that the GFBR, the PDB and the PER of the QoS profile can be fulfilled again for a QoS Flow (for which a notification that the "GFBR can no longer be guaranteed" has been sent), the NG-RAN shall send a notification, informing the SMF that the "GFBR can be guaranteed" again and the SMF may forward the notification to the PCF, see TS 23.503 [45]. The NG-RAN shall send a subsequent notification that the "GFBR can no longer be guaranteed" whenever necessary.

NOTE 2:	It is assumed that NG-RAN implementation will apply some hysteresis before determining that the "GFBR can be guaranteed again" and therefore a frequent signalling of "GFBR can be guaranteed again" followed by "GFBR can no longer be guaranteed" is not expected.

Another point worth noting in the SA2’s latest LS is that NG-RAN is expected to provide the updated available data rate information with some hysteresis when it changes to a higher threshold. While when the available data rate changes to a lower threshold, NG-RAN is expected to report the change immediately. The motivation to do so is to prevent ping-pong. This should be considered in RAN3 specifications. However, it is difficult to modelling the gNB behaviour on when to report the available data rate after change. In our view, a proper way is to leave the hysteresis to network implementation. Some description can be added on the specification to clarify this. We noticed that in TS 23.501, there is a NOTE to describe such kind of RAN implementation based hysteresis for QNC mechanism, which is excerpted as below.

Thus, a straightforward way to capture such implementation-based hysteresis for reporting higher available data rate should also in SA2’s specification, RAN3 can coordinate with SA2 to capture this.

Proposal 3: RAN3 to ask SA2 to capture in their specification that NG-RAN should immediately report a change to a lower threshold for the “available data rate” and a change to a higher threshold value with some hysteresis.

Based on the above analysis, we provide some initial TPs in [4] and [5] for the solution, which involves TS 38.413 to enable the provision of the thresholds from CN to NG-RAN and TS 38.415 for report of the available data rate. It can be further updated according to the discussion and progress in RAN3 and SA2. Furthermore, a draft reply LS to SA2 is provided in [3].

Proposal 4: RAN3 to agree the TPs in R3-250660 and R3-250659 for supporting the available date rate report, and sent reply LS to SA2 by taking the content in R3-250295 as baseline.

# 3. Conclusion

In this contribution, we analyzed the latest SA2’s LS on available data rate report, and provide the following proposals:

Proposal 1: CN provides one or more thresholds to NG-RAN via CP. NG-RAN indicates the available data rate based on these configured thresholds via UP.

Proposal 2: MFBR and GFBR can be considered when setting the values of the thresholds.

Proposal 3: RAN3 to ask SA2 to capture in their specification that NG-RAN should immediately report a change to a lower threshold for the “available data rate” and a change to a higher threshold value with some hysteresis.

Proposal 4: RAN3 to agree the TPs in R3-250660 and R3-250659 for supporting the available date rate report, and sent reply LS to SA2 by taking the content in R3-250295 as baseline.

# 4. Reference

[1]. S2-2412720, TSG SA WG2, LS Reply on FS_XRM Ph2.

[2]. R3-244844, TSG RAN WG3, Response LS to SA2 on FS_XRM Ph2.

[3]. R3-250295, [draft] Reply LS on FS_XRM Ph2 for available data rate report, Huawei.

[4]. R3-250660, (TP for TS 38.413) XR available data rate report, Huawei.

[5]. R3-250659, (TP for TS 38.415) XR available data rate report, Huawei