3GPP TSG-RAN WG3 #127	R3-250296

Athens, GR, 17 – 21 Feb. 2025

Agenda Item:	18.2. Support LP-WUS Indicating Paging Monitoring

Source:	NTT DOCOMO INC.

Title:	Further discussion on LP-WUS

Document for:	Discussion, agreement

Introduction

In past two meetings, RAN3 achieved following:

[RAN3#125]

- New IEs should be introduced to support LP-WUS on NG, Xn and F1.

[RAN3#125bis]

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

- The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

- The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

- Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

- The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

[RAN3#126]

- WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

- It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

- NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

- WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

And also, some points noted:

- Take this into the CRs as editor notes, “The maximum number of CN assigned LP-WUS subgroups is 32.”

In this paper, to resolve remaining open points and make progress, we have further discussion for LP-WUS support in RAN3.

Discussion

General

For clarification, the remaining points pending on other WGs are summarized below.

- WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

Depending on the conclusion of the discussion on whether RAN2 supports RAN paging for UEs in LP-WUS state, i.e., supports LP-WUS continuation in neigbour cells, this WA will be revised.

- WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

Depending on the conclusion of the discussion on whether UE capability for LP-WUS is defined per freq/cell or per UE in RAN2, this WA will be revised.

Emergency call back

Several companies pointed out at past RAN3 meetings that the paging delay caused by LP-WUS may be a problem in emergency call backs, and also the following was captured in last RAN2 meeting.

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In an emergency call, the paging message is sent via normal PDU session, and the NW does not know whether the UE maintains the emergency PDU session, and UE can not predict the emergency call back in advance. Therefore, it is impossible for the NW to make it sure that UE is not in low power state and latency for emergency call back would be avoided.

The issue we want to address here is whether the NW should introduce a solution to avoid the wake up delay when an emergency PDU session has been established and the UE is expected to maintain it and an emergency call back may occur. Regardless of whether CN based subgrouping or UE_ID based subgrouping, the UE may avoid the wake up delay if UE’s implementation does not care about possibility emergency call back and goes to low power state in such case. One solution is that gNB sends paging for such time sensitive call at the first PO regardless of the UE’s LP-WUS capability based on NW implementation and the UE avoid entering low power state based on UE implementation. Another solution is to activate/deactivate the LP-WUS function of the UE from the NW (RAN or CN) explicitly and let the NW make it sure that UE is not in low power state. The SIB broadcasts whether LP-WUS can be used in the cell, but since this problem is UE dedicated, it is necessary to introduce a new UE dedicated signaling. Also, since this signaling may affect RRC signaling and NG signaling, coordination with RAN2 would be needed.

As mentioned above, RAN2 also discusses this issue and also discusses sending an LS to SA2 to check the requirement [1]. Depending on the conclusion of the discussion, we can discuss enhancement of RAN3 signaling, if necessary.

LP-WUS configuration generation

LS from SA2 [2] describes two main points. 1) LP-WUS terminology and 2) operation when LP-WUS capability of UE is not acquired in NW.

- Terminology

From our point of view, we do not need to change the LP-WUS terminology and this topic is led by RAN1, therefore RAN3 cannot discuss and decide new terminology for LP-WUS.

Proposal 1:	RAN3 doesn't require to change the terminology "LP-WUS".

- LP-WUS capability related issue

Based on the LS, this problem may be caused by the fact that if the UE goes to the LP-WUS cell when the NW node that the UE last reported capability is not aware of the release 19 LP-WUS capability, the UE try to enable LP-WUS even though there is no LP-WUS capability in the UE context maintained by the NW, causing a mismatch between the UE and the NW. In this case, UE will go low power state but NW considers that the UE is not LP-WUS UE and may send paging without WUS transmission. From operator perspective, if this issue is confirmed by WGs, it is better for us to specify standardized solution. However, this issue is mainly related to RAN2 perspective, therefore, we think at first this issue should be confirmed by RAN2 and if it is confirmed, RAN2 should also discuss whether they resolve this issue with standardized solution or left it up to NW/UE implementation and what the solution is, if needed. RAN3 can work on solution only after this issue and RAN3 impact would be confirmed, so that it should be pending on RAN2 discussions.

Proposal 2:	Regarding the problem of managing the LP-WUS capability of the UE pointed out by SA2, it is up to RAN2 what kind of actions would be needed. FFS on RAN3 work for this issue pending on RAN2 progress.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	RAN3 doesn't require to change the terminology "LP-WUS".

Proposal 2:	Regarding the problem of managing the LP-WUS capability of the UE pointed out by SA2, it is up to RAN2 what kind of actions would be needed. FFS on RAN3 work for this issue pending on RAN2 progress.

References

- R2-2501002, “Discussion on the LP-WUS handling for Emergency call back”, NTT Docomo.

- R3-250021, “LS Reply on LP-WUS subgrouping”, SA2