3GPP TSG-RAN WG3 Meeting #127	R3-250300

Athens, GR, 17-21 Feb, 2025

Title:	(TP for MDT BLCRs for TS38.413, TS38.423, TS37.320): SON for Intra-NTN mobility

Agenda Item:	10.3.1

Source:	Huawei

Document for:	other

1. Introduction

This document continues to discuss the enhancements for SON and MDT for NTN on top of the progress of last meeting.

2. Discussion

2.1	Location based trigger event

In the last meeting we had a working assumption as follows:

WA: There is no need to report any other info in the RLF report for the location based CHO.

This was based on the assumption that the node receiving the RLF report can retrieve the context and calaculate the distance based on time information (100ms accuracy) and ephemeris.

TS 38.331 contains the following information for D2:

Ml1 is the distance between UE and a moving reference location for this event, not taking into account any offsets. The moving reference location is determined based on movingReferenceLocation and the corresponding epoch time and satellite ephemeris for the serving cell broadcast in SIB19.

Ml2 is the distance between UE and a moving reference location for this event, not taking into account any offsets. The moving reference location is determined based on the parameter referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to this event.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for condEventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for condEventD2. Each step represents 50m.

#### EpochTime The IE EpochTime is used to indicate the epoch time for the NTN assistance information, and it is defined as the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. The reference point for EpochTime of the serving, target, or neighbour NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point when this field is provided in an NTN cell and the gNB when this field is provided in a TN cell. In case of handover or conditional handover, EpochTime is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the target cell. Otherwise, EpochTime is used based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the serving cell.

From above quotation, we may learn that the granularity of the UE measured distance to the reference location is 50m. And the granularity of the input epoch time is the starting time of a DL sub-frame which means a 1ms granularity.

Observation 1: The UE measured distance has a granularity of 50m with a 1ms granularity epoch time as input.

If we let the network to calculate the distance, the complexity is shown in the following Figure 1. The uncertainty is  mainly introduced by the timeSinceCHO-Reconfig in the RLF report when calculating the UTC time of RLF (step 1 in Figure 1. Because the granularity of the timeSinceCHO-Reconfig is 100ms. The uncertainty will be enlarged in step2 when calculating the satellite location, as the satellite usually is moving quite fast.

According to Table 7.3.2.1.4-1 in TS 38.821, we may assume that the relative velocity of the satellite is 7.56 km/s, meaning. It roughly indicates that the uncertainty introduced by the timeSinceCHO-Reconfig is 756m which is much higher than the granularity of UE measured distance, i.e, 50m.

Figure 1: distance calculation based on location info in RLF report by the network

Observation 2: The network calculated distance the distance to the reference location when RLF occurs has huge complexity and a large uncertainty of 756m within 100ms which is far large than the UE measured, i.e, 50m within 1ms.

The second aspect is that although the RLF already includes the location information. This information is optional in the RLF report. Relying on the location information in RLF report to calculate the distance to reference location when RLF occurred may mandate the UE to always include the location information into the RLF report.

On the other hand, the absence of the UE location information in the RLF report may make the network based solution not workable.

Observation 3: The network based solution to calculate the distance to the reference location when RLF occurs mandates the UE to include the location information in the RLF report and is not workable when the UE location information in the RLF report is absent.

Considering that collecting the measured distance when RLF occurs and including it in the RLF report by the UE bring very less cost to the UE, we propose:

Proposal 1:	To revert the WA and agree to include the following information in the UE RLF report for location based CHO:

- An indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither.

- The measured distanceFromReference1 when the condition to the candidate cell is fulfilled, or the measured distanceFromReference2 when the condition to the source cell is fulfilled.

2.2	MRO for earth moving cells

As discussed in R3-247131 the NTN creates a new scenario for MRO. In the past we have assumed MRO as a statistical process to learn the radio environment, i.e. adjust for obstruction (slow fading). For NTN there is typically less impact from variations due to obstacles in radio environment. As pointed out in R3-247131, there may still be some slow fading and it would make sense to introduce a mechanism to cope with this. This contribution mentions clouds, wbut we also foresee that large obstacles may introduce slow fading, especially for UEs located close to this obstacle.

The new scenario is however that the cell is not stationary, but moving across the ground. For a LEO satellite, the cell would perform one orbit in in 100 minutes and typically return to the same position on earth twice per day. This would make the collection of statistics more difficult.

Figure 1. Figure from showing slow fading (a cloud).

One approach could be to use location information and build a set of information related to different locations and the impacts on the mobility decisions. This solution would be completely implementation dependent and may be useful for a node with a pattern that periodically reurns to the same location and whith the same neighbours. One possible addition to this solution would be to exchange the information collected between different nodes in terms of handover trigger offset for geographical areas using e.g. mobility settings change.

In R3-247131 the proposal is instead to use UE related information. The neighobur node is informed that a different handover trigger is used for this specific UE and can use this to map to a certain problem area and even use it for future mobility. The solution is interesting since it removes the need to map changes into geographical locations when exchanged between nodes.

R3-247131 also propose to re-use the mobility settings change procedure for this. We think think this could be useful for cell-related information (e.g. the mapping between geographical area and HO trigger) as discussed above. But for UE related information a more lightweight solution would be to include an offset of the HO trigger in the HO preparation. The receiving node can use this to understand that the mobility trigger for this UE was adjusted when moving the UE from the source to target and the target may also use this for future mobility events to any cells.

Using a legacy non-UE associated solution based on  mobility change procedure would lead to the following drawbacks:

- Mobility change is a negotiation that propose a new HO trigger used by the receiving node (in this case: target)  when handing over back to the requesting node (in this case: serving) to avoid mobility problems between these cells. The HO trigger in the opposite direction (source->target) can also be indluded, but this is an optional IE, and was previously considered to be included for information.

- The validity of this negotiation is until the next mobility change message is received. Hence, we would need to either signal the same UEs multiple times, or restrict the negotitation to a certain time intercvall, which means the information cannot be updated timely.

- Using this method per UE may require a lot of messages, for which the procedure was not designed. It was previously assumed this procedure ius used to update neighbour nodes on MRO/MLB adjustments, which is typically considered to be non-frequent and based on statistics.

Based on the discussion above, we believe that the problem scenario is worth discussing.

Proposal 2: Agree that a solution for earth moving cell scenarios is beneficial.

2.3	Area scope enhancement for MDT for NTN

The following agreement is achieved and a LS was sent to RAN2 to check the feasiblility of reusing the geographical area defined for MBS NTN.

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

We propose to proceed the TPs in the annex to caputure above agreements. The geographical area definition could be marked with FFS.

Proposal 3: To proceed the TPs for the area scope enhancement for MDT for NTN in the annex.

3.	Conclusion

This document makes the following observations and proposals:

Observation 1: The UE measured distance has a granularity of 50m with a 1ms granularity epoch time as input.

Observation 2: The network calculated distance the distance to the reference location when RLF occurs has huge complexity and a large uncertainty of 756m within 100ms which is far large than the UE measured, i.e, 50m within 1ms.

Observation 3: The network based solution to calculate the distance to the reference location when RLF occurs mandates the UE to include the location information in the RLF report and is not workable when the UE location information in the RLF report is absent.

Proposal 1:	To revert the WA and agree to include the following information in the UE RLF report for location based CHO:

- An indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither.

- The measured distanceFromReference1 when the condition to the candidate cell is fulfilled, or the measured distanceFromReference2 when the condition to the source cell is fulfilled.

Proposal 2: Agree that a solution for earth moving scenarios is beneficial.

Proposal 3: To proceed the TPs for the area scope enhancement for MDT for NTN in the annex.

The corresponding TPs are provided in the annex.

Annex	TP for MDT for 38.413

/*********************Start of changes************************************/

9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL |  |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.3.3.65 |  | - |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | - |  |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the TAI IE. | - |  |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >>SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | - |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity IE. | - |  |
| >GeoArea Based MDT |  |  |  |  | YES | ignore |
| >>GeoArea |  |  | Detailed definition is pending to RAN2. |  |  |  |
| >Mapped Cell based |  |  |  |  | YES | ignore |
| >>Mapped Cell ID List for MDT |  | 1..<maxnoofMappedCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging Interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | Corresponds to the LoggingInterval IE as defined in TS 38.331 [18]. | - |  |
| >>Logging Duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | Corresponds to the LoggingDuration IE as defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  | This IE is ignored if the Area Scope of MDT IE is set to "PLMN Wide" | YES | ignore |
| >CAG List for MDT | M |  | 9.3.3.65 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is 16. |
| maxnoofMappedCellIDforMDT | Maximum no. of Mapped Cell ID subject for MDT scope. Value is 32. |

/*********************End of changes************************************/

Annex	TP for MDT for 38.423

/*********************Start of changes************************************/

9.2.3.126	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Immediate MDT and Trace, Logged MDT only, ...) |  | – |  |
| CHOICE Area Scope of MDT-NR | O |  |  |  | – |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT-NR |  | 1 .. <maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. | – |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1 |  |  | – |  |
| >>>TAI List for MDT Item |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>>TAC | M |  | 9.2.2.5 |  | – |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.2.3.191 |  | – |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | – | - |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>TAC | M |  | 9.2.2.5 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >> SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >GeoArea Based MDT |  |  |  |  | YES | ignore |
| >>GeoArea |  |  | Detailed definition is pending to RAN2. |  |  |  |
| >Mapped Cell based |  |  |  |  | YES | ignore |
| >>Mapped Cell ID List for MDT |  | 1..<maxnoofMappedCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| CHOICE MDT Mode | M |  |  |  | – |  |
| >Immediate MDT-NR |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [43]. First Bit = M1, Second Bit= M2, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 3. | – |  |
| >>M1 Configuration | C-ifM1 |  | 9.2.3.128 |  | – |  |
| >>M4 Configuration | C-ifM4 |  | 9.2.3.129 |  | – |  |
| >>M5 Configuration | C-ifM5 |  | 9.2.3.130 |  | – |  |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [43]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". | – |  |
| >>M6 Configuration | C-ifM6 |  | 9.2.3.131 |  | – |  |
| >>M7 Configuration | C-ifM7 |  | 9.2.3.132 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >Logged MDT-NR |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 , ms61440, infinity, ...) | Corresponds to information provided in the LoggingInterval IE as defined in TS 38.331 [10]. The value "infinity" represents one shot logging, i.e., only one log per event in the logged MDT report. | – |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90, 120) | Corresponds to information provided in the LoggingDuration IE as defined in TS 38.331 [10]. Unit: [minute]. | – |  |
| >>CHOICE Report Type | M |  |  |  | – |  |
| >>>Periodical |  |  |  |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Logged Event Trigger Config | M |  | 9.2.3.137 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >>Area Scope of Neighbour Cells | O |  | 9.2.3.140 |  | – |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [10]. | – |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | – |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  |  | YES | Ignore |
| >CAG List for MDT | M |  | 9.2.3.191 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is 16. |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |

/*********************End of changes************************************/

Annex – TP for MDT BLCR for TS37.320

/*********************Start of changes************************************/

5.1.1.1.1	Configuration parameters

The logged measurement configuration consists of:

-	configuration of downlink pilot strength measurements logging for (E-)UTRA and NR.

-	configuration of MBSFN measurement logging for E-UTRA.

-	configuration of the triggering of logging events:

-	for (E-)UTRAN:

-	periodic measurement trigger is supported, for which the logging interval is configurable. The parameter specifies the periodicity for storing MDT measurement results. It should be configured in seconds in multiples of the applied IDLE mode DRX, i.e. multiples of 1.28s which is either a factor or multiple of the IDLE mode DRX. The UE behaviour is unspecified when the UE is configured with a DRX cycle larger than the logging interval.

-	for NR:

-	periodic measurement trigger is supported, for which the logging interval is configurable. The parameter specifies the periodicity for storing MDT measurement results.

-	for E-UTRAN and NR:

-	event-based trigger is supported, for which the logging interval is configurable, which determines periodical logging of available data (e.g. time stamp, location information), and the following two types of events are supported:

-	measurement quantity-based event L1, for which the event threshold, hysteresis, and time to trigger are configurable. If the configured time to trigger is not a multiple of the DRX cycle, then the UE uses the next multiple of DRX cycle duration that is larger than the time to trigger for evaluating the event L1;

-	out-of-coverage detection trigger.

NOTE:	The logging configuration for event-based and periodical DL pilot strength logged measurements can be configured independently. Only one type of event can be configured to the UE.

-	configuration of the logging duration. This configuration parameter defines a timer activated at the moment of configuration, that continues independent of state changes, RAT or RPLMN change. When the timer expires the logging is stopped and the configuration is cleared (except for the parameters that are required for further reporting e.g. network absolute time stamp, trace reference, trace recording session reference and TCE Id).

-	network absolute time stamp to be used as a time reference to UE.

-	Trace Reference parameter as indicated by the OAM configuration as specified in TS 32.422 [6].

-	Trace Recording Session Reference as indicated by the OAM configuration as specified in TS 32.422 [6].

-	TCE Id as indicated by the OAM configuration as specified in TS 32.422 [6].

-	(optionally) MDT PLMN List, indicating the PLMNs where measurement collection and log reporting is allowed. It is either the Management Based MDT PLMN List or the Signalling Based MDT PLMN List, depending on how the Logged MDT task was initiated (see 5.1.3).

-	(optionally) configuration of a logging area. A UE will log measurements as long as it is within the configured logging area. The scope of the logging area may consist of one of:

-	a list of up to 32 global cell identities for PLMN, and, for NR, additionally a list of up to 144 PNI-NPNs with a maximum of 12 different PLMN identities. If one or both of these lists are configured, the UE will only log measurements when camping in any of the cells belonging to the list of global cell identities, or in any of the cells belonging to the listed PNI-NPNs.

-	a list of up to 8 TAs or 8 LAs or 8 RAs for PLMN, and, for NR, additionally a list of up to 256 PNI-NPNs. If one or both of these lists are configured, the UE will only log measurements when camping in any cell belonging to the preconfigured TA/LA/RAs, or in any of the cells belonging to the listed PNI-NPNs.

-	for NR, a list of inter-frequency neighbouring cells per frequency.

-	for NR, a list of up to 144 PNI-NPNs with a maximum of 12 different PLMN identities.

-	for NR, a list of up to 16 SNPNs.

-	for NR, a list of up to 32 global cell identities for SNPN. If this list is configured, the UE will only log measurements when camping in any of these cells.

-	for NR, a list of up to 8 TAs for SNPN. If this list is configured, the UE will only log measurements when camping in any cell belonging to the configured TAs.

-	for NR, a geographical area. If this geographical area is configured, the UE ill only log measurements when comping in any cell belongs to the configured geographical area.

The configured logging area can span one of:

-	PLMNs in the MDT PLMN List. If no area is configured, the UE will log measurements throughout the PLMNs of the MDT PLMN list.

-	Any configured SNPN area.

-	(optionally) for NR, configuration of a list of neighbouring frequencies and/or cells, indicating the UE to include neighbouring cell's measurements as indicated in the list in the logged MDT report.

-	(optionally) for E-UTRA, configuration of target MBSFN area(s) for MBSFN measurement logging. If target MBSFN area(s) is configured, UE applies it in addition to other restrictions such as the logging area. The UE will log measurements as long as it receives MBMS service from an indicated target MBSFN area and is within the configured logging area. The target MBSFN area(s) is defined by a list of up to 8 entries, where each entry indicates a carrier frequency and optionally indicates a specific MBSFN area on a carrier frequency.

-	(optionally) configuration of the WLAN access point names, indicating the UE to attempt to obtain WLAN measurements associated to these access points.

-	(optionally) configuration of the Bluetooth beacon names, indicating the UE to attempt to obtain Bluetooth measurements associated to these beacons.

-	(optionally) for NR, configuration of the sensor names, indicating the UE to attempt to obtain sensor measurements.

-	(optionally) for E-UTRA, configuration indicating the UE to attempt to obtain uncompensated barometric pressure measurements.

-	(optionally) for NR, the network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes, indicating the UE is allowed to log the measurement results related to early measurement frequencies in the logged MDT report.

-	(optionally) for NR and E-UTRA, logged MDT type flag, indicating the logged measurement configuration is the signalling based MDT (see 5.4.0).

5.4.2	RRC_IDLE & RRC_INACTIVE

5.4.2.1	General

For UE in RRC_IDLE and RRC_INACTIVE states Logged MDT procedures as described in 5.1.1 apply.

For Logged MDT measurement collection for RRC INACTIVE UEs, the actual process of logging within the UE, takes place in RRC INACTIVE state and may be continued in RRC IDLE state; or vice versa.

The logged measurement stored in UE during RRC INACTIVE and RRC IDLE state are kept for a given common period before they are deleted as in LTE MDT.

If the signalling based logged or immediate MDT configuration received by the NG-RAN when UE is in RRC_INACTIVE:

-	The NG-RAN stores the MDT configuration in the UE context;

-	When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE;

-	When the UE resumes the RRC connection in one new NG-RAN, the new NG-RAN can configure the MDT configuration for the UE, only if the signalling based logged MDT was received by the new NG-RAN from the previous NG-RAN or AMF.

If the management based MDT configuration is received by the NG-RAN when UE is in RRC_INACTIVE,

-	No requirement for the NG-RAN to store the MDT configuration in the UE context;

-	When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE;

-	When the UE resumes the RRC connection in another NG-RAN, the source NG-RAN will not propagate the management based MDT configuration. The source NG-RAN should inform the target NG-RAN of UE consents.

Logged MDT measurements are sent on Signalling Radio Bearer SRB2 in RRC_CONNECTED state.

If a list of mapped cell identities is received from AMF as the area scope of logged MDT in NTN, the gNB based on OAM configuration and its implementation will transform it into a geographical area when it is configured to the UE.