3GPP TSG RAN WG3#127	R3-250338

Athens, Greece, 17 - 21 Feb. 2025

Agenda item:	17.3

Source: 	China Telecom

Title: 	On support on-demand SIB1

Document for:	Discussion and decision

- Introduction

In RAN3#127, the good progress for OD-SIB1 support was made and following agreements were achieved:

- The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

- The procedure uses non UE-associated signaling.

- Cell A stores the UL WUS configuration information after it has received it.

- A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- The procedure uses non UE-associated signalling.

- Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

As a result, two CRs were agreed as Baseline CR, i.e. R3-247857 and R3-247858.

There are still some issues need to be discussed as follows:

- Whether CellA can stop and resume the broadcasting of UL WUS configuration?

- Whether the activation/deactivation towards CellA on UL WUS configuration broadcasting is needed?

In this paper, we discuss on the remaining issues and present our proposals.

- Discussion

2.1 Deactivation of CellA UL WUS configuration broadcasting

The procedure for On-demand SIB1(OD-SIB1) is as follows: When a Cell is ready to enter on-demand SIB1 transmission mode and become a NES Cell, it sends a WUS configuration to a Cell A. Upon receiving the WUS configuration, Cell A broadcasts it to the UEs camping on Cell A. A UE in RRC Idle/Inactive that receives the WUS configuration can send a WUS to the NES Cell and, if needed, receive SIB1 from the NES Cell.

Throughout the entire OD-SIB1 procedure, the NES cell is the initiator. It actives the procedure, so it should have the ability to deactivate it when necessary, such as more UEs begin connecting to it.

Proposal 1: A NES cell should have the ability to deactivate Cell A UL WUS configuration broadcast.

2.2 Cell A Stopping/Resuming UL WUS configuration broadcast

In the On-demand SIB1 procedure, Cell A should evaluate its workload and response to requests for broadcasting UL WUS configuration from NES Cell. If the workload of Cell A is low, it can broadcast the UL WUS configuration for the NES Cell without impacting its own services. However, if Cell A’s workload becomes high and broadcasting the UL WUS configuration starts to interfere with its own services, Cell A must send a signalling message to inform the NES Cell that it will stop broadcasting the UL WUS configuration. Similarly when Cell A is able to resume the broadcasting, it also could send a signalling to notify the NES Cell that.

Proposal 2: Cell A should have the capability to stop and resume broadcasting UL WUS configuration and inform the NES Cell of these actions.

2.3 Specification Impact on Xn

To support the function of deactivation of Cell A’s UL WUS configuration broadcast, a signalling mechanism is needed. This signalling would allow the NES Cell to instruct Cell A to stop broadcasting the UL WUS configuration. The procedure of stopping the UL WUS configuration broadcast (referred to as the UL WUS Configuration Provision Deactivation procedure) could be classified as a phase 3 procedure as the NES Cell can independently decide to operate in the normal SIB1 broadcast state without requiring feedback from Cell A, as illustrated in Fig. 1.

Proposal 3: A Phase 3 procedure (e.g. UL WUS Configuration Provision Deactivation) should be introduced to deactivate the Cell A’s UL WUS Configuration Provision.

Similarly, when Cell A needs to stop or resume the UL WUS Configuration broadcast, it should send signalling to the NES cell to inform it of the change in broadcast state. The procedures for stopping and resuming the UL WUS Configuration broadcast could be combined into a single phase 3 signalling procedure, such as the procedure for UL WUS Configuration Provision Update.

Proposal 4: A phase 3 procedure (e.g. UL WUS Configuration Provision Update) should be introduced to handle the stopping and resumption of the UL WUS Configuration Provision.

Figure 1: Procedure of UL WUS Configuration Provision Deactivation

Figure 2: Procedure of UL WUS Configuration Provision Update

2.4 Specification Impact on F1

In the CU-DU architecture, to support the deactivation mechanism of Cell A’s UL WUS configuration broadcast, the gNB-CU of Cell A should send a signalling message to gNB-DU, instructing it to stop the UL WUS configuration broadcast when the gNB-CU receives a deactivation signalling from the NES Cell. Therefore, a new message (e.g. UL WUS Configuration Broadcasting Deactivation message) should be introduced in the F1 interface, as shown in Fig.3.

Proposal 5: A phase 3 procedure (e.g. UL WUS Configuration Broadcast Deactivation) should be introduced to inform the gNB-DU to stop Cell A’s UL WUS Configuration broadcast.

Figure 3: Procedure of UL WUS Configuration Broadcast Deactivation

Similarly, to support the stopping and resuming mechanism of UL WUS configuration broadcast, the gNB-DU of Cell A should send a message to the gNB-CU to notify the UL WUS configuration broadcast will be stopped, since the gNB-DU is responsible for broadcasting the UL WUS configuration of NES Cell, as shown in Fig.4. Once the gNB-CU receives the signalling from the gNB-DU, it can then send a provision update message to the NES cell.

Proposal 6: A phase 3 procedure (e.g. UL WUS Configuration Broadcast Update) should be introduced to inform gNB-CU when gNB-DU is about to stop or resume the UL WUS Configuration broadcast.

Figure 4: Procedure of UL WUS Configuration Broadcast Update

- Conclusion

Based on the above discussion regarding support for on-demand SIB1 operation, we have the following proposals:

Proposal 1: A NES cell should have the ability to deactivate Cell A UL WUS configuration broadcast.

Proposal 2: Cell A should have the capability to stop and resume broadcasting UL WUS configuration and inform the NES Cell of these actions.

Proposal 3: A Phase 3 procedure (e.g. UL WUS Configuration Provision Deactivation) should be introduced to deactivate the Cell A’s UL WUS Configuration Provision.

Proposal 4: A phase 3 procedure (e.g. UL WUS Configuration Provision Update) should be introduced to handle the stopping and resumption of the UL WUS Configuration Provision.

Proposal 5: A phase 3 procedure (e.g. UL WUS Configuration Broadcast Deactivation) should be introduced to inform the gNB-DU to stop Cell A’s UL WUS Configuration broadcast.

Proposal 6: A phase 3 procedure (e.g. UL WUS Configuration Broadcast Update) should be introduced to inform gNB-CU when gNB-DU is about to stop or resume the UL WUS Configuration broadcast.

- Reference

[1] RP-242354, New WID: Enhancements of network energy savings for NR, Ericsson.