3GPP TSG RAN WG3#127	R3-250339

Athens, Greece, 17 – 21 Feb. 2025

Agenda item:	12.3

Source: 	China Telecom

Title: 	On Mobility for NR Femto

Document for:	Discussion and decision

- Introduction

In RAN3#127, some agreements were made and captured in TPs i.e. R3-247829, R3-247832, R3-247799, R3-247833 and R3-247834. But the UE mobility between NR Femtos, as well as between NR Femto and gNB has not been sufficiently discussed.

In this paper, we analyze the UE mobility for NR Femto and present our proposals.

- Discussion

2.1 UE Mobility between NR Femtos

If NR Femtos connect directly to the 5GC, shown as Fig. 1, applies the mobility procedure for NR Femto is the same as that of the normal network controlled mobility.

If NR Femtos connect with 5GC through an NR Femto GW, shown as Fig. 2, the intra-NR Femto handover follows the preparation and execution phase of the handover procedure performed without involvement of the NR Femto GW and 5GC, i.e. handover preparation messages are directly exchanged between NR Femtos. During the handover completion phase, the target NR Femto triggers the release of the resources at the source NR Femto. All above procedures are similar to a normal gNB handover procedure.

The difference lies in the path switch phase. In a normal gNB handover procedure, the target gNB sends a PATH SWITCH REQUEST message to the AMF triggering the 5GC to switch the DL data path toward the target gNB and establish an NG-C interface instance with it.

However, in an NR Femto handover procedure, the target NR Femto first sends a PATH SWITCH REQUEST message to NR Femto Gateway(GW). If the NR Femto GW determines that both the source and target NR Femtos are connected to it, it does not forward the message to the AMF. Instead, it switches the DL data path to the target NR Femto and establishes an NG-C interface instance with it. The NR Femto GW then sends a PATH SWITCH REQUEST ACKNOWLEDGE message to the target NR Femto, confirming the completion of path switch.

From the perspective of 5GC, the NG-C path between UE and AMF remains unchanged.

The AMF UE NGAP ID 2 IE, introduced in last RAN3 meeting, can be used to identify the path between NR Femto and NR Femto GW, while the AMF UE NGAP ID is used to identify the path between NR Femto and AMF. When UE mobility occurs within the same NR Femto GW, the AMF UE NGAP ID will not change, but the AMF UE NGAP ID 2 differs before and after NR Femto handover.

| Figure 1 NR Femto connects directly to 5GC | Figure 2 UE mobility within a NR Femto GW |
| --- | --- |

If UE mobility occurs between different NR Femto GWs, the target NR Femto GW forwards the PATH SWITCH REQUEST message received from the target NR Femto to AMF. The AMF then switches the control and data path toward the target NR Femto GW, and the target NR Femto GW switches the control and data path toward the target NR Femto.

In this case, both AMF UE NGAP ID and AMF UE NGAP ID 2 change before and after NR Femto handover.

Figure 3 UE mobility between different NR Femto GWs

Proposal 1: The stage2 description of mobility procedure within NR Femtos and the usage of AMF UE NGAP ID 2 should be added to TS 38.300.

2.2 UE Mobility from gNB to NR Femto

In the case of UE Mobility from a gNB to an NR Femto, as described in Section 16.7.2 of [3] on PNI-NPN mobility, shown in Fig.4, the source NG-RAN node is aware of the list of CAG IDs supported by the candidate target NR Femtos. During the handover, the source NG-RAN node selects a target cell from the candidates that complies with the received PNI-NPN restrictions.

Upon an incoming handover, the target NR Femto node receives the PNI-NPN mobility restrictions and verifies that the selected target cell is compatible with the received mobility restrictions.

When UE is allowed to access CAG cells of NR Femto, the target NR Femto sets up the UE-associated logical NG-connection with AMF via NR Femto Gateway.

Figure 4 UE mobility from gNB to NR Femto

2.3 UE Mobility from NR Femto to gNB

In the case of UE Mobility from an NR Femto to a gNB, when a UE leaves a CAG cell in active mode, the normal network controlled mobility applies.

Figure 5 UE mobility from NR Femto to gNB

Proposal 2: The stage2 description of mobility to/from NR Femto should be added to TS 38.300.

- Conclusion

Based on the above discussion on UE mobility for NR Femto, we provide the following proposals:

Proposal 1: The stage2 description of mobility procedure within NR Femtos and the usage of AMF UE NGAP ID 2 should be added to TS 38.300.

Proposal 2: The stage2 description of mobility to/from NR Femto should be added to TS 38.300.

- Reference

[1] RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, AT&T.

[2] RP-241925, TR 38.799 v2.1.0 NR; Study on additional topological enhancements for NR, NTT DOCOMO INC., AT&T.

[3] TS 38.300, NR; NR and NG-RAN Overall description; Stage-2 Rel-18, 3GPP.

[4] TS 38.413, NG-RAN; NG Application Protocol (NGAP) Rel-18, 3GPP.

[5] TS 36.300, E-UTRAN; Overall description; Stage 2 Rel-18, 3GPP.

[6] TS 36.413, E-UTRAN; S1 Application Protocol (S1AP) Rel-18, 3GPP.