3GPP TSG-RAN WG3 Meeting #127	R3-250347

Athens, Greece, 17th - 21th Feb 2025

Title:	Discussion on AIML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Discussion and Decision

1. Introduction

In RAN3#125bis and RAN3#126 meetings, the following agreements were made with several FFSs.

RAN3#125bis: Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message. The Coverage Modification Cause reflects the predicted CCO issue. WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1. Introducing separate time information for future coverage state and predicted CCO issue. Time for future coverage state: The point in time when the future coverage state will be applied. Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen. RAN3#126: Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1. Introducing new IE for Future Coverage Modification List over Xn. Introducing new IE for Future Coverage Modification Notification over F1. Introducing new IE for predicted CCO assistance information over F1. WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE. Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1. Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node Configuration Update message over Xn. FFS on whether to send time information related to coverage modification cause over Xn. Further discussion on whether Value 0 needed for Future Cell Coverage State IE. Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds.

In this document, we continue discussing the AI/ML based Coverage and Capacity Optimization (CCO) and provide our proposals.

2. Discussion

In RAN3#125bis meeting, it was agreed to transfer “Future Coverage State and the associated Coverage Modification Cause” in the NG-RAN NODE CONFIGURATION UPDATE message”. The Coverage Modification Cause reflects the predicted CCO issue.

In RAN3#126 meeting, RAN3 reached the following agreements:

Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node Configuration Update message over Xn.

Based on the agreements above, we have following overview of the solutions for AI/ML based CCO.

Figure 1 – Overview of the solution for AI/ML based CCO

However, it is still FFS on whether to send time information related to coverage modification cause over Xn. In this contribution, we describe our view and proposals for this FFS issue.

As described in Figure 1, we assume gNB1 as the node where the CCO issue is predicted, and gNB2 as a neighbouring node of gNB1. When gNB2 receives the future coverage status and related coverage modification cause derived at gNB1, gNB2 needs to decide when to take appropriate actions to match the future CCO state of gNB1. In our opinion, time information for Future Coverage Status is enough for gNB2 to decide when to take the appropriate actions. Based on the timing information for Future Coverage Status, gNB2 can know when the gNB1 will apply the future coverage status, then gNB2 can make decision on when to take appropriate action.

Therefore, we have the following proposal:

Proposal 1: Do not need to send time information related to coverage modification cause over Xn.

3. Conclusion

In this contribution, we further discuss AI/ML based Coverage and Capacity Optimization (CCO). We have the following proposal:

Proposal 1: Do not need to send time information related to coverage modification cause over Xn.