3GPP TSG-RAN WG3 Meeting #127	   				         R3-250349

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:		15.2

Source:			China Telecom

Title:		Support for IoT NTN with regenerative payload

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 IoT NTN enhancements WI contains the following objective[1]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

  - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Last RAN3 meeting has discussed the architecture of IoT NTN regenerative payload, RAN3 decides to work on Split MME architecture and Full CN on board architecture. In this contribution, we will continue to discuss the potential standard impacts on support of regenerative payload for IoT NTN.

- Discussion

2.1 Regenerative Architecture

SA2 has confirmed that their conclusions on regenerative-based satellite access apply to both NG-RAN/5GC and E-UTRAN/EPC. On this foundation, some revisions were made to the IoT NTN WID at the RAN # 106 meeting, including specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed.

Firstly, we would like to clarify the scenario of Store&Forward (S&F) satellite operation with full eNB as regenerative payload. When the feeder link is available, the onboard eNB can connect with the ground MME to provide instant communication services, and the feeder link transports the S1 interface. When feeder link is not available and the network supports S&F operation, the satellite can activate the S&F operation mode according to the situation.

Observation: S&F is a mode of operation that may be switched on or off based on the availability of feeder link. When the feed link is available, the onboard eNB can directly connect with the ground MME.

It is beneficial for the eNB to dynamically shut down S1 toward the ground/onboard MME when performing S&F switching on or off switching off operations. For the enhancement of the S1 interface, we can refer to the agreements in NR NTN topic.

In previous discussions, some companies have pointed out that multiple SCTP associations are currently not supported for E-UTRAN, so eNB IP address changes due to soft feeder link switch over are not currently supported for S1. One possibility would be to specify multiple SCTP associations for S1 in a similar manner as for NG, or to address it based on implementation. Some companies believe that RAN3 has decided to adopt the Split MME architecture and the Full CN onboard architecture, due to the current architecture scenario requiring support core network elements on board, the feeder link switch over only affects the interface between MME-onboard and MME-on ground (in Split MME on board), and the interface between proxies (in Full CN on board). SA2 agreed these interfaces are out of 3GPP scope. Now, due to the emergence of new requirement, we may need to re-examine the potential standard impact of this issue.

We propose to discuss the following:

- S1 Removal procedure.

- Multiple SCTP associations for S1.

Proposal 1: Regarding how to support regenerative payload, RAN3 needs to discuss the following:

- S1 Removal procedure.

- Multiple SCTP associations for S1.

The stage-2 CR of IoT NTN should include two aspects: the regenerative payload, and the S&F operation. The BLCR for NR NTN to support regenerative payload could be used as baseline.

Proposal 2: It is proposed to specify the support of eNB on board in TS 36.300, the BLCR for NR NTN could be used as baseline.

2.1 S&F operation

Based on the conclusion of the last meeting, RAN3 has decided to prioritize the split MME architecture and the full CN onboard architecture. To align among other WGs and avoid duplication work, we suggest RAN3 only work on split MME and full CN on board architecture during S&F satellite operation.

Proposal 3: RAN3 only work on split MME and full CN on board architecture during S&F satellite operation.

Split MME on board

In legacy CIoT, TS 36.300 specifies both CP solution and UP solution. For CP solution, the UE data is transferred via NAS message and handled by MME. And for UP solution, the UE data is transferred via S1-U tunnels and handled by SGW.

In split MME architecture, for the CP solution, the S&F operation can be implemented by MME onboard and MME on ground, due to the MO/MT data is buffered in MME, which may not affect eNB. For the UP solution, since the SGW is on the ground, it may cause some impact on the S1-U. Currently, considering that SA2 may not support the UP solution for IoT NTN, it is expected that it will not have an impact on S1AP.

In addition, whether is it necessary for MME to transfer the S&F monitoring list of satellites IDs to the eNB for assisting ephemeris broadcast, RAN3 can wait for the progress of SA2 and RAN2.

Proposal 4: For the split MME architecture, there may be no specification impact on S1AP.

Full CN on board

For the full CN on board architecture, including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Since all CN components are onboard, the UE attach procedure can be directly finished as legacy TN or NTN, and current MO/MT procedure can be reused. If multiple satellites are used, how to share UE context between satellites belong to SA2 scope, it seems have no impact to RAN3.

Proposal 5: For the full CN on board architecture, there may be no specification impact on S1AP.

- Conclusions

In this paper, we have the following proposals:

Observation: S&F is a mode of operation that may be switched on or off based on the availability of feeder link. When the feed link is available, the onboard eNB can directly connect with the ground MME.

Proposal 1: Regarding how to support regenerative payload, RAN3 needs to discuss the following:

- S1 Removal procedure.

- Multiple SCTP associations for S1.

Proposal 2: It is proposed to specify the support of eNB on board in TS 36.300, the BLCR for NR NTN could be used as baseline.

Proposal 3: RAN3 only work on Split MME and Full CN on board architecture during S&F satellite operation.

Proposal 4: For the split MME architecture, there may be no specification impact on S1AP.

Proposal 5: For the full CN on board architecture, there may be no specification impact on S1AP.

- References

[1]	RP-243278, Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc.

[2]	TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3.

[3] 	R3-247016, Reply LS on Support of Regenerative-based Satellite Access.