3GPP TSG-RAN WG3 Meeting #127	   				         R3-250352

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:		16.2

Source:			China Telecom

Title:		A-IoT architecture aspects

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 Ambient IoT WID contains the following objective[1]:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we will provide some initial consideration on RAN architecture aspects from the perspective of RAN3.

- Discussion

We completed the TR 38.769 in the last meeting. The architecture and protocol stack of topology 1 are as follows [2]:

Figure 6.4.1-1 Logical system architecture for topology 1

Figure 6.4.1-2. Protocol Stack for Topology 1

The network elements in A-IOT CN represent AMF and/or AIOTF. The A-IOT RAN can communicates with AIOTF via a direct path or indirect path (via AMF) [3]. We think both direct path and indirect paths should be supported in this Release.

Proposal 1: The A-IOT RAN can communicates with AIOTF via a direct path or indirect path.

The XX interface in the above figure is NG interface. One remaining issue in TR is Whether "XXAP" is realised by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP. In both direct and indirect path modes, the AMF will encode/decode the NGAP messages, with the difference being whether AIOTF messages is transparent to AMF. Anyway, considering the inevitable impact on NGAP under both path modes, we slightly prefer to reuse existing NGAP and enhance it by introducing new signaling to support A-IoT Services.

In this case, using a unified Nnetwork architecture figure is beneficial, the protocol stack of topology 1 in TR can be used as a baseline, where XXAP is replaced with NGAP.

Proposal 2: NGAP between A-IOT RAN and A-IOT CN is enhanced to support Ambient IoT Services.

Proposal 3: XXAP is NGAP.

Currently, SA2 has completed the conclusion on Key Issue #1. For topology 1, the AIOTF selects the A-IOT RAN node(s) and optionally provides a list of the BS readers to A-IOT RAN. SA2 assumes that OAM can configure the AIOTF with needed information for this selection. SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information The final conclusion depends on RAN3 discussion.

In general, pre-configured or signaling based solution are feasible for AIoTF aware BS readers. For signaling based solutions, it is necessary to explicitly report BS reader information in NG interface management related messages, such as BS reader IDs and their corresponding service areas.

Observation 1: Both pre-configured and signaling based solution are feasible for AIoTF aware BS readers.

Therefore, the AIOTF will be able to get the BS Reader IDs and corresponding service areas, which will be used for subsequent A-IOT operations.

Proposal 4: the AIOTF selects the AIOT RAN node(s) and provides a list of the BS readers.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: The A-IOT RAN can communicates with AIOTF via a direct path or indirect path.

Proposal 2: NGAP between A-IOT RAN and A-IOT CN is enhanced to support Ambient IoT Services.

Proposal 3: XXAP is NGAP.

Observation 1: Both pre-configured and signaling based solution are feasible for AIoTF aware BS readers.

Proposal 4: the AIOTF selects the AIOT RAN node(s) and provides a list of the BS readers.

- References

[1] 	RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.

[2]	3GPP TR 38.769, Study on solutions for Ambient IoT in NR.

[3]	3GPP TR 23.700-13, V1.3.0 (2025-01).