3GPP TSG-RAN WG3 Meeting #127	   				         R3-250354

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:		22.2

Source:			China Telecom

Title:	Discussion on control plane procedures for multi-hop relay

Document for:	 	Discussion, Decision

- Introduction

In the newest WID, the objective of service continuity is as follows [1]:

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE):

First Priority:

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

Second Priority in order of importance:

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

In this contribution, we will provide our initial thoughts about control plane procedures and how to support these intra-gNB service continuity scenarios for multi-hop U2N relay.

- Discussion

Control plane procedures

According to the agreements of RAN2, there are two approaches[2]:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

Due to ongoing RAN2 discussions on whether and how to support approach 2, we can currently focus on approach 1. The specific impacts of approach 2 can wait for further progress from RAN2.

In legacy L2 U2N Remote UE initial access procedure is as follows[3]:

- Figure 8.19.1-1: Remote UE Initial Access procedure

Generally, we understand that most steps of the procedure may also be used in multi-hop Remote UE initial access with some adaptive modifications.

For steps 2 and 3, the L2 U2N Remote UE sends the RRCSetupRequest message for its connection establishment with gNB-DU via the first Relay UE. Then the first Relay UE withholds the received RRC message and sends the SidelinkUEInformationNR message via the relay path to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the first relay UE, intermediate relay UE, or last relay UE is not in RRC_CONNECTED, it should trigger the RRC establishment/resume procedure to enter RRC_CONNECTED at first, as though they are acting as a remote UE. It should be noted that if the above steps of the relay UEs entering the RRC_CONNECTED state and configuring the RLC relay channel (current spec only support it during UE CONTEXT MODIFICATION procedure) are executed as a "cascade" sequence, the delay of the above steps may be quite significant. So it is suggested to support the configuration of PC5 RLC Channel for relaying Remote UE’s SRB1 in the UE CONTEXT SETUP procedure of intermediate Relay UE.

After receiving the SUI information from at least the first relay UE(or from last Relay UE which on behalf of all other relay UEs), the gNB-CU can know each hop path of the L2 U2N Remote UE. Subsequently, gNB-CU send the UE CONTEXT MODIFICATION REQUEST messages for each U2N Relay UE to gNB-DU. Such message may request the establishment of Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0 and SRB1. In step 5~10/15/30, the gNB, last Relay UE, intermediate Relay UE may perform relaying channel setup procedure at the same time.

For L2 U2N Remote UE, when its RRCSetupRequest message is forwarded to gNB, gNB-DU needs to know the first relay UE ID in order to generate at least the PC5 Relay RLC channel configuration send it to gNB-CU for relaying of U2N Remote UE’s SRB1. This can be obtained via the gNB-CU and the SRAP header. The potential impact of RAN3 can wait for RAN2 progress.

After the above analysis, we think most steps of the legacy L2 U2N Remote UE initial access procedure can be reused for multi-hop scenario, with some different aspects we have the following proposals:

Proposal 1: When a U2N remote UE initiates access, it should support the case where each relay UE is in RRC_CONNECTED state. FFS on whether the intermediate relay UE can remain in RRC_IDLE/RRC_INACTIVE state.

Proposal 2: The gNB can configure SRB1 relaying PC5 RLC channel during UE CONTEXT SETUP procedure of relay UE.

Intra-gNB multi-hop service continuity

For Intra-gNB multi-hop indirect to direct path switching and Intra-gNB multi-hop indirect to single-hop indirect path switching(scenarios A and B), the existing single-hop procedures can be reuse in multi-hop scenario.

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell in WID. However, in the post-email discussion about the approach 1 and 2[2], some companies raised concerns about the validity of scenario C and D when applying approach 2.

Considering that scenarios C and D are the second priority, RAN3 can further discuss them after the structure of approach 1 or 2 is determined by RAN2.

Proposal 3: The existing single-hop path switching procedures can be reuse in multi-hop scenarios A and B. How to support scenarios C and D depend on the progress of other WGs.

- Conclusions

Based on the above discussion, we propose the following:

Proposal 1: When a U2N remote UE initiates access, it should support the case where each relay UE is in RRC_CONNECTED state. FFS on whether the intermediate relay UE can remain in RRC_IDLE/RRC_INACTIVE state.

Proposal 2: The gNB can configure SRB1 relaying PC5 RLC channel during UE CONTEXT SETUP procedure of relay UE.

Proposal 3: The existing single-hop path switching procedures can be reuse in multi-hop scenarios A and B. How to support scenarios C and D depend on the progress of other WGs.

- References

[1]	RP-242349, Revised WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet, 3GPP TSG RAN Meeting #105.

[2]	R2-2410006, Report of [POST127][402][Relay] Multi-hop relay control plane, InterDigital.

[3]	3GPP TS 38.401, V18.4.0.