3GPP TSG-RAN WG3 Meeting #127                                                                   R3-250355

Athens, Greece, Feb 17th – 21st, 2025

Source:	CATT, China Telecom, CMCC

Title:	(draft CR to F1AP) Discussion on L3 measurement report for SCell activation

Agenda Item:	9.2

Document for:	Discussion and Approval

1 Introduction

In the Rel-18 NR RRM enhancement topic, RAN4 had some conclusions on unknown FR2 SCell activation enhancement with L3 reporting. RAN4 sent a LS[1] to RAN2 in RAN4#108 meeting (August 2023) for designing the corresponding signaling to support the enhancements.

Following details are captured in the LS to RAN2:

Enhancement 1: network triggering report (of L3 measurement result) during FR2 unknown SCell activation

Enhancement for multiple FR2 unknown SCells’ activation Agreements: - Define requirements for multiple SCell activation (with and without PUCCH SCell) considering at least enhancement of L3 measurement reporting triggered by SCell activation command when all to-be-activated SCells are in the same band. - Note: If requirements are not finalized in RAN4 #109 then they will be deprioritized TCI activation command transmitted is based on L3 measurement results or L1 measurement results? Agreements: - If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on: - the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command. - the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

RAN4 introduced an enhancement to support L3-measurement report based TCI state activation during FR2 unknown SCell activation. When SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported on the unknown SCell, the L3 measurement report will be initiated which includes the SCell with SSB level measurement result for that SCell. Then the L3 measurement report is used for TCI state activation in the unknown SCell.

To support above enhancements, RAN2 has agreed to introduce a new report type reportOnActivation. According to the CR[2] to RRC spec and CR[3] to MAC spec in RAN2, for the unknown SCell, L3 measurement results are reported upon UE receiving the SCell activation command MAC CE when the new report type is configured to UE.

2 Discussion

According to RAN4, after the L3 report is triggered for SCell activation, gNB can activate TCI state based on the reported L3 measurement results to activate SCell fast. However, in split architecture, the node receiving the L3 measurement report is not that transmitting TCI activation command, i.e., it is CU to receive the L3 measurement report while DU takes charge of sending TCI activation command to UE.

Obviously, CU and DU need some coordination to support the L3-measurement based TCI activation, otherwise, the unknown SCell activation enhancements introduced by RAN4/2 will not be supported in Rel-18 in case of split architecture.

It is proposed to introduce some enhancement to F1 to support L3-measurement based TCI activation from Rel-18.

Proposal 1: RAN3 introduce F1 enhancement to support L3-measurement based TCI activation from Rel-18.

For gNB-DU to send the TCI activation command based on L3-measurement report, there are two candidate solutions with respect to CU/DU coordination:

- Option 1: When gNB-CU receives L3 report triggered by SCell activation command, it determines the beam (SSB) used by UE for SCell activation based on the beam level L3 report, and indicates the selected beam to gNB-DU

- Option 2: When gNB-CU receives L3 report triggered by SCell activation command, it notifies the beam level L3 report to gNB-DU for gNB-DU to determine the beam and TCI state used for SCell activation

Difference between the two solutions resides in which node decide the beam used for SCell activation.

In Rel-18 RACH-less HO, the RRC handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. As specified by TS 38.300[4], the beam used by UE can be determined based on UE beam level measurement report.

Definition of RACH-less HO command in TS 38.331[5]:

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need N

beamIndication-r18                  CHOICE {

tci-StateID-r18                     TCI-StateId,

ssb-Index-r18                       SSB-Index

}                                                                                               OPTIONAL,   -- Need N

...

}

In split architecture, the RRC handover command is generated by gNB-DU and gNB-DU can read the candidate cells’ L3 measurement report which is transferred via the CU to DU RRC information in Handover Preparation Information IE to determine the selected beam (TCI state or SSB) for UE.

It can be understood that gNB-DU determining the beam for UE based on L3 measurement report is already there from Rel-18. Also, L3 measurement report transfer from CU to DU is a legacy approach, though it is specifically used for RRC handover. Thus, Option 2 aligns with the DU and CU functionality split.

For option 1, CU selects the beam for UE using L3 measurement report. However, per legacy function split, CU is only responsible for cell level determination for UE, for example, during handover preparation or CA configuration, while the beam level determination for UE is a new functionality to CU and thus need further study. Compared with Option 1, Option 2 is more aligned with the common understanding for CU/DU functionality split. It is proposed that RAN3 adopt Option 2.

Proposal 2: gNB-CU notifies the beam level L3 report to gNB-DU for gNB-DU to determine the beam and the TCI state for SCell activation.

Further, according to RAN4’s conclusion, if both L3 report and L1-RSRP report is received before transmitting the TCI activation command, gNB can use ether L3 report of L1-RSRP report for TCI activation up to implementation. In split architecture, given L1-RSRP measurement is reported to gNB-DU, if the L3 measurement report is transferred from CU to DU, gNB-DU can decide to use either L3 report or L1-RSRP report for TCI activation.

Proposal 3: If both L3 measurement report and L1-RSRP report for SCell is received, gNB-DU can use either of them for TCI state activation.

If Option 2 is adopted, the beam level L3 measurement report need be transferred from CU to DU. In the current F1AP and RRC spec, the L3 measurement report of candidate cells is included in Handover Preparation Information IE of CU to DU RRC information which is transferred by UE CONTEXT SETUP message for RRC handover usage. This procedure cannot be reused for transferring L3 measurement report of SCells from CU to DU for SCell activation. F1AP needs enhancement, and there are two possible solutions:

- Solution 1

UE Context Modification procedure is reused. CU transfers L3 measurement report of SCell(s) via the CU to DU RRC information in the UE CONTEXT MODIFICATION message.

Introduce a new IE for L3 measurement report of SCell(s) in the CU to DU RRC information.

- Solution 2

Introduce a new Class 2 F1AP procedure and a new F1AP message for transferring L3 measurement report of of SCell(s).

RAN3 can discuss which solution to adopt.

Proposal 4: RAN3 to discuss whether to reuse the UE Context Modification procedure or introduce a new F1AP procedure to transfer the L3 measurement report from CU to DU for the usage of SCell activation.

In RAN2#123bis meeting[6], RAN2 made the following agreements on multiple SCell activation case:

Agreements:

1.	If the network activates multiple Scells within same MAC CE the UE may send only one measurement report.

Per this conclusion, UE may send one measurement report for L3 measurement results of multiple SCells which are activated by the same MAC CE. The measurement results for multiple SCells can be reported by UE via measResultServingMOList in MeasResults IE in RRC spec. So, this IE can be reused for notifying the L3 measurement results of multiple SCells.

Proposal 5: measResultServingMOList in MeasResults IE in RRC spec is reused for containing the L3 measurement results of SCell(s).

Draft CRs for above solutions are provided in Annexes.

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1: RAN3 introduce F1 enhancement to support L3-measurement based TCI activation from Rel-18.

Proposal 2: gNB-CU notifies the beam level L3 report to gNB-DU for gNB-DU to determine the beam and the TCI state for SCell activation.

Proposal 3: If both L3 measurement report and L1-RSRP report for SCell is received, gNB-DU can use either of them for TCI state activation.

Proposal 4: RAN3 to discuss whether to reuse the UE Context Modification procedure or introduce a new F1AP procedure to transfer the L3 measurement report from CU to DU for the usage of SCell activation.

Proposal 5: measResultServingMOList in MeasResults IE in RRC spec is reused for containing the L3 measurement results of SCell(s).

4 Reference

- R2-2309450, LS on FR2 SCell activation enhancements, RAN2#123bis, RAN4, to RAN2, cc RAN1

- R2-2313936, Introduction of FR2 SCell enhancements, RAN2#124, Apple, CATT, Ericsson, Xiaomi, Qualcomm Incorporated, Huawei, HiSilicon, ZTE

- R2-2313937, Introduction of FR2 SCell enhancements, RAN2#124, Xiaomi, Apple, CATT, Ericsson, Qualcomm Incorporated, Huawei, HiSilicon, ZTE

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.4.0 (2024-12)

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v18.4.0 (2024-12)

- Report of 3GPP TSG RAN2 meeting #123bis, MCC

Annex A - draft CR to F1AP for solution 1

3GPP TSG-RAN WG3 Meeting #127	R3-24xxxx

# Athens, Greece, Feb 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | xxxx | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of L3 measurement report notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, China Telecom, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, the enhancement of L3 measurement report during FR2 unknown SCell activation has been introduced. When SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported on the unkown SCell, the RRC measurement report will be initiated which includes the SCell with beam level measurement result for that SCell. RAN2 has introduced a new report type to support SCell activation triggered L3 report in Rel-18. According to RAN4’s conclusion, after the L3 report is triggered for SCell activation, gNB can activate TCI state based on the reported L3 measurement results to activate SCell fast. However, in split architecture, the node receiving the L3 measurement report is not that transmitting TCI activation command, i.e., it is CU to receive the L3 measurement report while DU takes change of sending TCI activation command to UE. So, CU and DU need some coordination to support the L3-measurement based TCI activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new IE for L3 measurement report of SCell(s) in the CU to DU RRC information. For UE Context Modification procedure, add the behaviour of gNB-DU on receiving the new IE in the CU to DU RRC information in the UE CONTEXT MODIFICATION message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because the change only introduces impact to UE Context Modification procedure. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unknown SCell activation enhancements introduced by RAN4/2 will not be supported in Rel-18 in case of split architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.2, 9.3.1.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes----------------------------------------------

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

>>>>>>>>>>>>>>>>>>>>>>>>>>Unchanged part is skipped<<<<<<<<<<<<<<<<<<<<<<<<<

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall deduce that changes to the measurements’ configuration need to be applied. The gNB-DU shall take the received info, e.g. the measObjectToAddModList IE, and/or the measObjectToRemoveList IE into account, when generating measurement gap and when deciding if a measurement gap is needed or not.

If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForInterruptionInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For DC operation, if the gNB-CU includes the CG-Config IE in the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may initiate low layer parameters coordination taking this information into account.

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information or NR sidelink information as defined in TS 38.331 [8].

If the SCell L3 Measurement Report IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, deduce that a L3 measurement report was triggered by SCell activation command and may use the beam level L3 measurement report indicated by this IE for activating the TCI state used by the UE at the SCell(s).

>>>>>>>>>>>>>>>>>>>>>>>>>>Unchanged part is skipped<<<<<<<<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change----------------------------------------------

#### 9.3.1.25	CU to DU RRC Information

This IE contains the RRC Information that are sent from gNB-CU to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CG-ConfigInfo | O |  | OCTET STRING | Includes the CG-ConfigInfo message, as defined in TS 38.331 [8]. | - |  |
| UE-CapabilityRAT-ContainerList | O |  | OCTET STRING | This IE is used in the NG-RAN and it includes the UE-CapabilityRAT-ContainerList IE, as defined in TS 38.331 [8]. | - |  |
| MeasConfig | O |  | OCTET STRING | Includes the MeasConfig IE, as defined in TS 38.331 [8] (without the MeasGapConfig IE). For EN-DC/NGEN-DC operation, includes the list of FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps. For NG-RAN, NE-DC and MN for NR-NR DC, includes the list of FR1 and/or FR2 frequencies, for which the gNB-CU requests the gNB-DU to generate gaps and the gap type (per-UE or per-FR). | - |  |
| Handover Preparation Information | O |  | OCTET STRING | Includes the HandoverPreparationInformation message, as defined in TS 38.331 [8]. | YES | ignore |
| CellGroupConfig | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | YES | ignore |
| Measurement Timing Configuration | O |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. In EN-DC/NGEN-DC, it is included when the gaps for FR2 are requested to be configured by the MeNB. For MN in NR-NR DC,it is included when the gaps for FR2 and/or FR1 are requested by the SgNB | YES | ignore |
| UEAssistanceInformation | O |  | OCTET STRING | Includes the UEAssistanceInformation message, as defined in TS 38.331 [8]. | YES | ignore |
| CG-Config | O |  | OCTET STRING | Includes the CG-Config message, as defined in TS 38.331 [8]. | YES | ignore |
| UEAssistanceInformationEUTRA | O |  | OCTET STRING | Includes the UEAssistanceInformation message, as defined in TS 36.331 [41]. | YES | ignore |
| Location Measurement Information | O |  | OCTET STRING | Includes the LocationMeasurementInfo IE, as defined in TS 38.331[8] | YES | ignore |
| MUSIM-GapConfig | O |  | OCTET STRING | Includes the MUSIM-GapConfig IE as defined in TS 38.331 [8]. | YES | reject |
| SDT-MAC-PHY-CG-Config | O |  | OCTET STRING | Includes the SDT-MAC-PHY-CG-Config IE, as defined in TS 38.331 [8]. | YES | ignore |
| MBSInterestIndication | O |  | OCTET STRING | Includes the MBSInterestIndication message as defined in TS 38.331 [8]. | YES | ignore |
| NeedForGapsInfoNR | O |  | OCTET STRING | Includes the NeedForGapsInfoNR IE, as defined in TS 38.331 [8]. | YES | ignore |
| NeedForGapNCSG-InfoNR | O |  | OCTET STRING | Includes the NeedForGapNCSG-InfoNR IE, as defined in TS 38.331 [8]. | YES | ignore |
| NeedForGapNCSG-InfoEUTRA | O |  | OCTET STRING | Includes the NeedForGapNCSG-InfoEUTRA IE, as defined in TS 38.331 [8]. | YES | ignore |
| ConfigRestrictInfoDAPS | O |  | OCTET STRING | Includes the ConfigRestrictInfoDAPS-r16 IE as defined in TS 38.331 [8]. This IE is used at the source node if DAPS HO is configured. | YES | ignore |
| Preconfigured measurement GAP Request | O |  | ENUMERATED(true, …) |  | YES | ignore |
| NeedForInterruptionInfoNR | O |  | OCTET STRING | Includes the NeedForInterruptionInfoNR IE, as defined in TS 38.331 [8]. | YES | ignore |
| musim-CapabilityRestrictionIndication | O |  | ENUMERATED (true, …) | Corresponds to the musim-CapRestrictionInd contained in the RRCSetupComplete message or the RRCResumeComplete message as defined in TS 38.331 [8]. | YES | ignore |
| musim-CandidateBandList | O |  | OCTET STRING | Includes the musim-CandidateBandList contained in the OtherConfig IE, as defined in TS 38.331 [8]. | YES | ignore |
| SCell L3 Measurement Report | O |  | OCTET STRING | Includes the measResultServingMOList contained in the MeasResults IE, as defined in TS 38.331 [8]. | YES | ignore |

<<<<<<<<<<<< ASN.1 part will be added if solution 2 is adopted>>>>>>>>>>>

-------------------------------------------End of changes----------------------------------------------

Annex B - draft CR to F1AP for solution 2

3GPP TSG-RAN WG3 Meeting #127	R3-24xxxx

# Athens, Greece, Feb 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | xxxx | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of L3 measurement report notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, China Telecom, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, the enhancement of L3 measurement report during FR2 unknown SCell activation has been introduced. When SCell activation command MAC CE is received and UE has valid L3 measurement results available but not reported on the unkown SCell, the RRC measurement report will be initiated which includes the SCell with beam level measurement result for that SCell. RAN2 has introduced a new report type to support SCell activation triggered L3 report in Rel-18. According to RAN4’s conclusion, after the L3 report is triggered for SCell activation, gNB can activate TCI state based on the reported L3 measurement results to activate SCell fast. However, in split architecture, the node receiving the L3 measurement report is not that transmitting TCI activation command, i.e., it is CU to receive the L3 measurement report while DU takes change of sending TCI activation command to UE. So, CU and DU need some coordination to support the L3-measurement based TCI activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Class 2 F1AP procedure for gNB-CU to notify the L3 measurement report triggered by SCell activation command to gNB-DU. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because the change introduces a new funtion and thus does not affect other functions. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unknown SCell activation enhancements introduced by RAN4/2 will not be supported in Rel-18 in case of split architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.3.x, 8.3.x.1, 8.3.x.2, 8.3.x.3, 8.3.x.4, 9.2.2.x… |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes----------------------------------------------

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| BAP Mapping Configuration | BAP MAPPING CONFIGURATION | BAP MAPPING CONFIGURATION ACKNOWLEDGE | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration | GNB-DU RESOURCE CONFIGURATION | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation | IAB TNL ADDRESS REQUEST | IAB TNL ADDRESS RESPONSE | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update | IAB UP CONFIGURATION UPDATE REQUEST | IAB UP CONFIGURATION UPDATE RESPONSE | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| Multicast Context Notification | MULTICAST CONTEXT NOTIFICATION INDICATION | MULTICAST CONTEXT NOTIFICATION CONFIRM | MULTICAST CONTEXT NOTIFICATION REFUSE |
| Multicast Common Configuration | MULTICAST COMMON CONFIGURATION REQUEST | MULTICAST COMMON CONFIGURATION RESPONSE | MULTICAST COMMON CONFIGURATION REFUSE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| DU-CU Cell Switch Notification | DU-CU CELL SWITCH NOTIFICATION |
| CU-DU Cell Switch Notification | CU-DU CELL SWITCH NOTIFICATION |
| DU-CU TA Information Transfer | DU-CU TA INFORMATION TRANSFER |
| CU-DU TA Information Transfer | CU-DU TA INFORMATION TRANSFER |
| QoE Information Transfer Control | QOE INFORMATION TRANSFER CONTROL |
| RACH Indication | RACH INDICATION |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| Mobile IAB F1 Setup Triggering | MIAB F1 SETUP TRIGGERING |
| Mobile IAB F1 Setup Outcome Notification | MIAB F1 SETUP OUTCOME NOTIFICATION |
| Broadcast Transport Resource Request | BROADCAST TRANSPORT RESOURCE REQUEST |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| DU-CU Access And Mobility Indication | DU-CU ACCESS AND MOBILITY INDICATION |
| CU-DU L3 Measurement Report Notification | CU-DU L3 MEASUREMENT REPORT NOTIFICATION |

-------------------------------------------Next change----------------------------------------------

### 8.3.x	DU-CU L3 Measurement Report Notification

#### 8.3.x.1	General

The purpose of the CU-DU L3 Measurement Report Notification procedure is to enable the gNB-CU to transfer the L3 measurement report triggered by SCell activation command to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.3.x.2	Successful Operation

Figure 8.3.x.2-1: CU-DU Measurement Report Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU L3 MEASUREMENT REPORT NOTIFICATION message.

Upon reception of the CU-DU L3 MEASUREMENT REPORT NOTIFICATION message, the gNB-DU shall, if supported, consider that a L3 measurement report was triggered by SCell activation command to UE and may use the beam level L3 measurement report indicated in the SCell L3 Measurement Report IE for activating the TCI state used by the UE at the SCell(s).

#### 8.3.x.3	Unsuccessful Operation

Not applicable.

#### 8.3.x.4	Abnormal Conditions

Not applicable.

-------------------------------------------Next change----------------------------------------------

#### 9.2.2.x	CU-DU L3 MEASUREMENT REPORT NOTIFICATION

This message is sent by the gNB-CU to notify the gNB-DU about the L3 measurement report triggered by SCell activation command.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SCell L3 Measurement Report | M |  | OCTET STRING | Includes the measResultServingMOList contained in the MeasResults IE, as defined in TS 38.331 [8]. | YES | ignore |

<<<<<<<<<<<< ASN.1 part will be added if solution 2 is adopted>>>>>>>>>>>

-------------------------------------------End of changes----------------------------------------------