3GPP TSG-RAN WG3 Meeting #127                                                                 R3-250356

Athens, Greece, Feb 17th–21st, 2025

Source:	CATT

Title:	(TP to BLCR for TS 38.401) On support of WAB

Agenda Item:	12.2

Document for:	Approval

1 Introduction

In RAN3#126 meeting [1], additional topological enhancements for NR are studied and some aspects including the access control and additional ULI of Wireless Access Backhaul (WAB) have not been finalized.

In this contribution, we will continue discussion about the open issues of the basic functions of WAB.

2 Discussion

2.1 Avoid multi-hop WAB topology

One LS [2] was sent from SA2 for asking RAN to discuss several issues that included the issue to restrict WAB-MT accessing a WAB-gNB cell:

In the last meeting, a solution was agreed to solve the issue in case of handover. The feedback in reply LS [3] is as below:

RAN3 has agreed the following solution to prevent multi-hop backhauling upon WAB-MT HO: For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling. The discussion on RAN based approach for preventing multi-hop backhauling at the WAB-MT’s initial access is still ongoing in RAN3.

However, the solution in case of initial access has not been agreed. There are two candidate solutions:

To be further discussed whether Solution 1 or Solution 3 (or both) need to be selected 1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells. 3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Solution 1 uses dedicated resources or configurations for WAB cells to prevent WAB-MT accessing. Solution 1 was preferred by some companies in the last meeting. However, this solution has following drawbacks or potential issues:

- It requires the different resources (such as frequencies, PCIs) or configurations (e.g., NR CGI) to be assigned to each WAB-gNB, which will certainly complicate operators’ network management and impose restrictions on resource allocation.

- There is an issue on how to make WAB-MT know those dedicated parameters. The possible method is that the OAM configures those parameters to the WAB-gNB collocated with the WAB-MT. That will introduce new requirement to OAM, and thus should be confirmed by SA5. But a potential challenge is that this method would not work when WAB-MT moves to a PLMN different to the PLMN served by the co-located WAB-gNB. Because in that case, OAM of the PLMN served by WAB-gNB has to configure the WAB-gNB with dedicated parameters used for WAB in other PLMNs. That’s an even more difficult target for OAM.

- Impact to WAB-MT implementation is unavoidable for the handling to the dedicated parameters during the cell (re-)selection. As diverse parameters are used for this usage, it’s hard to achieve a unified handling, thus the implementation is complicated.

- Not sure whether totally relies on implementation, i.e., without impact to stage-3 spec, such as TS 38.304.

Compared with Solution 1, Solution 3 is much easier for operators and without OAM requirement. The impacts of solution 3 include:

- A “WAB-Cell” indicator will be added to SIB1. From implementation perspective, only WAB-gNB need broadcast the indicator.

- WAB-MT handling for “WAB-Cell” indicator will be captured in TS 38.304, i.e., if this indicator is present, WAB-MT will treat this cell as barred during cell (re-)selection.

It can be observed Solution 3 has very limited impact to specs (TS 38.331/TS 38.304), without restriction to network deployment, and the handling is easy for WAB-node.

So, solution 3 can be adopted for avoiding multi-hop for idle mode WAB-MT.

Proposal 1-1: Multi-hop topology for idle mode WAB-MT can be avoided by solution 3.

RAN3 is proposed to send the LS to RAN2 on solution 3 for multi-hop topology avoidance.

Proposal 1-2: RAN3 send the LS to RAN2 on multi-hop topology avoidance as Annex A.

2.2 Additional ULI

According to SA2’s conclusion [4], indication of an additional ULI from the WAB-gNB to the AMF for UE is required. One LS [2] was sent from SA2 for asking RAN to discuss the issue of additional ULI and feedback:

According to clause 8.5 of TR 23.700-06 [4], 3 potential options are listed for the content of additional ULI in case the PLMN serving the WAB-MT and the PLMN serving the UE are not the same PLMN.

clause 8.5 of TR 23.700-06 - The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is: - TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN. - If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative in phase. - option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM. - option 2: geo-location of the MWAB. - option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN. - The AMF may provide the received Additional ULI to LMF in LCS request. NOTE: If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

In the last meeting, some progress has been made. The additional ULI includes TAI and NR CGI. But, two FFS were still captured.

WA: Additional ULI for WAB consists of TAI and NR CGI, which are determined by the WAB-node, reflecting the WAB-node’s physical location. This solution is compliant with Opton1 and Option3. It is up to SA2 to support one of Opton1 and Option3 or both. It is FFS whether there are issues with applying the solution to MT access via NTN. It is FFS whether further information needs to be included in the additional ULI

During the meeting, some companies believe that the solution can only be applied to the case when the WAB-MT accesses the network via terrestrial network. In case WAB-MT corrects to NTN, the cell serving the WAB-MT is relatively large, e.g., one cell may cover multiple tracking areas. Therefore, it was considered by some companies that the WAB-node should report the multiple TAIs which the NTN cell broadcasts as the additional ULI to UE’s AMF, and thus additional ULI should support multiple TAIs.

However, SA2 has concluded that Option 1 is selected s the additional ULI solution and sent a LS[5] to RAN3. For Option1, the physical location of WAB-node should only be mapped to one set of TAI and NRCGI based on input from OAM, then WAB-node can report the mapped TAI/NR CGI via the additional ULI. If the additional ULI contains multiple TAIs in the case of non-terrestrial network, it cannot help the core network to locate the UE, so it does not make sense to report multiple sets of TAI and NRCGI.

So, there is no issue for including one set of TAI/NR CGI in additional ULI.

Observation: There is no issue for including one set of TAI/NR CGI in additional ULI.

No further information needs to be included in the additional ULI.

Proposal 2: No further information needs to be included in the additional ULI.

2.3 WAB authorization

During the WAB-node integration procedure, the WAB-MT establishes the BH PDU session(s) and the WAB-gNB sets up the NG/Xn connections. However, if the WAB-MT is not authorized, it should not establish the BH PDU session(s). If the WAB-gNB is not authorized by OAM/SeGW, it should not set up the NG/Xn connections.

After the WAB-node integration, if the authorization status of WAB-gNB is changed to “not authorized”, the WAB-gNB can remove the NG/Xn connections after releasing/handing over all the UEs. According to the agreement of SA2 [6] in the SA2#166 meeting, when the WAB-gNB stops operating under OAM control, the WAB-gNB may trigger the co-location WAB-MT to release the BH PDU session(s).

On the other hand, when the authorization status of WAB-gNB is changed from “not authorized” to “authorized”, the WAB-gNB should re-establish NG connections (and Xn connections if needed), as the BH PDU session(s) have been set up in that case.

After the WAB-node integration, if the authorization status of WAB-MT is changed to “not authorized”, the WAB-MT needs to request the co-location WAB-gNB to release/hand over all the UEs and removing the NG/Xn connections. The BH PDU session(s) are removed by AMF as specified in TS 23.501.

Accordingly, the WAB-MT should re-establish the BH PDU session(s) and request the co-location WAB-gNB to set up NG connections (and Xn connections as needed) if the authorization status of WAB-MT is changed back to “authorized”.

TS 38.401 should capture above behaviors with respect to the WAB-gNB/WAB-MT authorization.

Proposal 3-1: During the WAB-node integration procedure, if the WAB-gNB is authorized, the WAB-gNB establishes NG/Xn connections, otherwise, it does not establish NG/Xn connections.

Proposal 3-2: If the authorization status of WAB-gNB is changed to “not authorized”, the WAB-gNB removes the NG/Xn connections after releasing/handing over all the UEs and the WAB-gNB may request the co-located WAB-MT to release the BH PDU session(s). If the authorization status of WAB-gNB is changed to “authorized”, the WAB-gNB re-establishes the NG/Xn connections.

Proposal 3-3: During the WAB-node integration procedure, if the WAB-MT is authorized, the WAB-MT establishes the BH PDU session(s), otherwise, it does not establish the BH PDU session(s).

Proposal 3-4: If authorization status of the WAB-MT is changed to “not authorized”, the WAB-MT can request the WAB-gNB to hand over or release all UEs and remove the NG/Xn connections. The BH PDU session(s) are removed by AMF as specified in TS 23.501. If the authorization status of WAB-MT is changed to “authorized”, the WAB-MT can re-establish the BH PDU session(s) and request the WAB-gNB to set up NG/Xn connections.

Proposal 3-4: Capture in TS 38.401 the behaviors with respect to WAB-gNB/WAB-MT authorization as Annex B.

2.4 WAB-MT mobility

According to WID [7], handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-MT’s mobility, including the case where the WAB-MT’s BH PDU session changes, needs to be specified. However, this aspect has not been discussed during SI stage.

During the WAB-MT’s mobility, the WAB-MT’s BH PDU session used for backhauling the WAB-gNB’s traffic may be re-established, for example, due to moving to a new region or moving between TN and NTN, The existing mechanisms defined in TS 23.501/TS 23.502 can be used for re-establishment of BH PDU session(s). In that case, the (outer) IP addressed used for transmitting the WAB-gNB’s traffic will be changed.

The WAB-MT should indicate the new IP address(s) to the WAB-gNB. The WAB-gNB may have to establish new SCTP connections using the new TNL address for NG/Xn connection. On NG interface, the WAB-gNB initiates RAN Configuration Update procedure/PDU Session Resource Modify Indication procedure to update the TNL addresses for NG-C and NG-U respectively. On Xn interface, the WAB-gNB can initiate NG-RAN Node Configuration Update procedure to update the TNL addresses for Xn-C and Xn-U.

TS 38.401 should capture the behavior of changing the IP addresses of BH PDU session(s) used for WAB-gNB’s traffic transporting.

Proposal 4-1: During the WAB-MT mobility, the IP address(s) of the BH PDU session(s) may be changed. WAB-MT should indicate the new IP address(s) to WAB-gNB. WAB-gNB can update the TNL addresses used for NG or Xn connection using the new IP address of the BH PDU session. The TNL addresses for NG or Xn connection can be updated using existing mechanism.

Proposal 4-2: Capture the behavior of WAB-node for changing of the IP address(s) of the BH PDU session(s) during the WAB-MT mobility in TS 38.401 as Annex B.

2.5 WAB-gNB mobility

2.5.1 WAB-gNB mobility without change of UE’s AMF

During WAB-gNB movement, radio configuration parameters of the cell(s) served by the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s).

This change of PCI for the cell served by WAB-gNB may trigger the WAB-gNB to initiate the intra-gNB handover to the UEs in RRC_CONNECTED state. After the PCI is changed, cell re-selection is performed by the UEs in RRC_IDLE or RRC_INACTIVE state camping on the WAB-gNB’s cell.

The change of TAC may trigger the Mobility Registration Update procedure for all the UEs moving together with the WAB-gNB.

Proposal 5-1: If the PCI of WAB-gNB cell is changed during WAB-gNB mobility without change of UE’s AMF, intra-WAB-gNB handover may be triggered for the connected mode UEs and cell re-selection is performed by the UEs in RRC_IDLE/RRC_INACTIVE state camping on the WAB-gNB.

Proposal 5-2: If the TAC of WAB-gNB cell is changed during WAB-gNB mobility without change of UE’s AMF, Mobility Registration Update procedure is triggered at the UE connected to or camping on the WAB-gNB.

2.5.2 WAB-gNB mobility with change of UE’s AMF

In the last meeting, RAN3 sends a reply LS [8] to SA2, and the LS confirms the feasibility of the two logical gNB solutions and request SA2 to check the single logical gNB solutions to support UE’s AMF change as following.

Solution 1: Using single WAB-gNB with single cell and changing TAC.

Solution 2: Using single WAB-gNB with two cells and different TACs.

In the last SA2 meeting, there is no consensus on whether to support the single WAB-gNB solutions [9]. So, it’s up to RAN3’s decision whether to take the single WAB-gNB solution(s).

For solution 1, SA2 considers it has CN impact and new procedures needs to be introduced if MRU based AMF relocation is used to change the connected mode UE’s AMF. Thus solution 1 is not recommended.

However, SA2 has confirmed that it can address the UE mobility handling as long as there are two different cells/TACs corresponding to the source and target AMFs for the NG based HO during the WAB-node mobility with AMF change, which is similar to the two logical gNBs solution, since the source(old) AMF uses the Target TAI in TARGET ID IE for routing. It can be understood that the NG based HO can be used to change UE’s AMF without CN impact even if the source gNB and target gNB are the same node.

Actually, the intra-NG-RAN node NG-based handover for changing the AMF of connected UEs has been supported in NTN, as specified in TS 23.501 [10]:

The Inter NG-RAN node N2 based handover procedure specified in clause 4.9.1.3 may also be used for intra-NG-RAN node handover. NOTE: One use case for intra-NG-RAN handover to be performed by the Inter NG-RAN node N2 based handover procedure is when an NG-RAN node serves a satellite access system that covers more than one country. In such a situation, the UE might move from a "cell" in one country into a "cell" in another country and the NG-RAN node may need to cause the AMF to change to an AMF serving the UE's new country.

The same scenario is captured in TS 38.300 [11]:

16.14.6 AMF (Re-)Selection The gNB implements the NAS Node Selection Function specified in TS 38.410 [16]. For an RRC_CONNECTED UE, when the gNB is configured to ensure that the UE connects to an AMF that serves the country in which the UE is located, if the gNB detects that the UE is in a different country to that served by the serving AMF, then it should perform an NG handover to change to an appropriate AMF, or initiate an UE Context Release Request procedure towards the serving AMF (in which case the AMF may decide to de-register the UE). For the purpose of selecting an appropriate AMF, the 5GC may verify the UE location according to TS 23.501 [3] and TS 38.305 [42] after the UE has attached to the network. NOTE: UE location verification for AMF selection should not be necessary if NTN cell(s) do not extend across countries.

Proposal 5-3: NG-based handover can be used to change UE’s AMF even if the source and target gNBs are the same.

For NG-based HO in the single WAB-gNB solution, the WAB-gNB only report the new TAI to the new AMF. Some companies would have concern on single WAB-gNB connecting to multiple AMF sets and reporting different TACs to different AMF sets. However, we think that is a pure implementation applied by the RAN side, and the same implementation has exactly been supported by above example captured in TS 23.501 [10] /TS 38.300 [11] for NTN scenario. Having said that, for the WAB-gNB mobility scenario, even if there is any potential issue for the same WAB-gNB connecting to different AMFs sets, the impact would be neglected. Note that the concurrent connecting between WAB-gNB and two AMF sets are just utilized for handing over the connected UEs via NG. The old cell(s) as well as the NG connection towards the old AMF should be released as soon as the NG HOs are completed, i.e., it will be a short interval for WAB-gNB to keep two NG connections with different AMF sets.

Proposal 5-4: For NG-based HO in the single WAB-gNB solution, the WAB-gNB can connect with different AMFs with reporting different TACs to different AMFs without CN impact.

Based on above, RAN3 can confirm that Solution 2 is feasible for changing the AMF of the connected UEs during WAB-node mobility besides the two logical gNB solution.

When WAB-node moves into an area of a new AMF, the TAC broadcasted by the cell(s) served by the WAB-node is changed to a TAC served ty the new AMF, the cell ID is changed as well according to the OAM configuration. The WAB-gNB obtains the configuration parameters to establish the connection to the UE’s new AMF. During the NG setup, the WAB-gNB only report the new TAC to the new AMF. The WAB-gNB instantiates new cell(s) which broadcast the new TAI and the new cell ID(s). The connected UEs can be handed over from the old cell(s) to the new cell(s) via NG-based HO to change the AMF.

It’s up to configuration/implementation whether to use two logical WAB-gNBs. For example, if the WAB-gNB is configured with a different gNB-ID with regarding to the new AMF by the OAM, the WAB-gNB can install a new logical WAB-gNB to establish NG connection with the new AMF and the new cell(s) are assumed being served the new logical WAB-gNB, otherwise, the WAB-gNB does not install another logical gNB.

Proposal 5-5: Given two different cells with different TACs corresponding to the source and target AMFs for NG-based HO with AMF change, it’s up to configuration whether to use two logical WAB-gNBs or not.

2.6 NG connection management

During the WAB-node movement, if the AMF(s) that WAB-gNB should connect and the TAI(s) broadcasted by the WAB-gNB change, the WAB-gNB will not serve the old TAI(s) and the old AMF(s) any more. The WAB-gNB may stop using the NG connection with the old AMF(s) after all the UEs connected to the old AMF(s) are handed over to the new AMF(s). The old AMF(s) are still reachable because the BH PDU session providing the IP connection to NG connections with the old AMF(s) is available. The WAB-gNB can remove the NG connections and keep the underlying SCTP connection alive. The removal of NG connection during WAB-node’s movement should be specified. Noted that BL CRs to TS 38.300 and TS 38.413 on NG connection removal have been approved by NTN. We can specify NG removal for WAB scenario in TS 38.300.

In past meeting, the introduction of NG suspend/resume is proposed. Since the WAB-node does not serve the old AMF(s) anymore, the old AMF(s) should not use the corresponding NG connections, e.g., sending paging messages to the WAB-node. Therefore, the WAB-node has to notify the old AMF(s) that the NG interface is not available through either new NGAP signalling or existing NGAP signalling (i.e., implicitly). Also, the WAB-node has to inform those AMF(s) to resume the NG connection when the WAB-node moves back.

In the last meeting, NG connection suspend/resume has not been discussed. From our perspective, given the removal of NG connection is adopted, there is no need to introduce NG connection suspend/resume.

Proposal 6-1: Specify NG removal for WAB scenario in TS 38.300.

Proposal 6-2: RAN3 do not introduce NG suspend/resume.

2.7 PCI collision avoidance

For PCI collision avoidance, as the scenario is quite similar with mobile IAB, the method of mobile IAB can be used as baseline for WAB. And it has only impact to stage-2 spec.

TS 38.401 should capture the mechanism of WAB-node for PCI collision avoidance.

Proposal 7-1: The mechanism for PCI collision avoidance in mobile IAB can be reused for WAB.

Proposal 7-2: Capture the mechanism of WAB-node for PCI collision avoidance in TS 38.401 as Annex B.

3 Conclusion

In the previous sections we made the following proposals:

Avoid multi-hop WAB topology

Proposal 1-1: Multi-hop topology for idle mode WAB-MT can be avoided by solution 3.

Proposal 1-2: RAN3 send the LS to RAN2 on multi-hop topology avoidance as Annex A.

Additional ULI

Observation: There is no issue for including one set of TAI/NR CGI in additional ULI.

Proposal 2: No further information needs to be included in the additional ULI.

WAB authorization

Proposal 3-1: During the WAB-node integration procedure, if the WAB-gNB is authorized, the WAB-gNB establishes NG/Xn connections, otherwise, it does not establish NG/Xn connections.

Proposal 3-2: If the authorization status of WAB-gNB is changed to “not authorized”, the WAB-gNB removes the NG/Xn connections after releasing/handing over all the UEs and the WAB-gNB may request the co-located WAB-MT to release the BH PDU session(s). If the authorization status of WAB-gNB is changed to “authorized”, the WAB-gNB re-establishes the NG/Xn connections.

Proposal 3-3: During the WAB-node integration procedure, if the WAB-MT is authorized, the WAB-MT establishes the BH PDU session(s), otherwise, it does not establish the BH PDU session(s).

Proposal 3-4: If authorization status of the WAB-MT is changed to “not authorized”, the WAB-MT can request the WAB-gNB to hand over or release all UEs and remove the NG/Xn connections. The BH PDU session(s) are removed by AMF as specified in TS 23.501. If the authorization status of WAB-MT is changed to “authorized”, the WAB-MT can re-establish the BH PDU session(s) and request the WAB-gNB to set up NG/Xn connections.

Proposal 3-4: Capture in TS 38.401 the behaviors with respect to WAB-gNB/WAB-MT authorization as Annex B.

WAB-MT mobility

Proposal 4-1: During the WAB-MT mobility, the IP address(s) of the BH PDU session(s) may be changed. WAB-MT should indicate the new IP address(s) to WAB-gNB. WAB-gNB can update the TNL addresses used for NG or Xn connection using the new IP address of the BH PDU session. The TNL addresses for NG or Xn connection can be updated using existing mechanism.

Proposal 4-2: Capture the behavior of WAB-node for changing of the IP address(s) of the BH PDU session(s) during the WAB-MT mobility in TS 38.401 as Annex B.

WAB-gNB mobility

Proposal 5-1: If the PCI of WAB-gNB cell is changed during WAB-gNB mobility without change of UE’s AMF, intra-WAB-gNB handover may be triggered for the connected mode UEs and cell re-selection is performed by the UEs in RRC_IDLE/RRC_INACTIVE state camping on the WAB-gNB.

Proposal 5-2: If the TAC of WAB-gNB cell is changed during WAB-gNB mobility without change of UE’s AMF, Mobility Registration Update procedure is triggered at the UE connected to or camping on the WAB-gNB.

Proposal 5-3: NG-based handover can be used to change UE’s AMF even if the source and target gNBs are the same.

Proposal 5-4: For NG-based HO in the single WAB-gNB solution, the WAB-gNB can connect with different AMFs with reporting different TACs to different AMFs without CN impact.

Proposal 5-5: Given two different cells with different TACs corresponding to the source and target AMFs for NG-based HO with AMF change, it’s up to configuration whether to use two logical WAB-gNBs or not.

NG connection management

Proposal 6-1: Specify NG removal for WAB scenario in TS 38.300.

Proposal 6-2: RAN3 do not introduce NG suspend/resume.

PCI collision avoidance

Proposal 7-1: The mechanism for PCI collision avoidance in mobile IAB can be reused for WAB.

Proposal 7-2: Capture the mechanism of WAB-node for PCI collision avoidance in TS 38.401 as Annex B.

4 Reference

- RAN3#126 agenda, RAN3 Chair

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2#163

- R3-247826, Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI), Samsung

- 3GPP TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, v 19.0.0 (2024-06)

- S2-2501324, Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI), SA2#166AH-e

- S2-2412633, Support of UE served by a MWAB: authorization aspects, Nokia, LG Electronics, Huawei, ZTE, Samsung, Ericsson, Qualcomm Inc.

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105

- R3-247872, Reply LS on FS_VMR_Ph2 solution impacts to RAN, RAN3#127

- S2-2501336, Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility) , SA2#166AH-e

- 3GPP TS 23.501, System architecture for the 5G System (5GS), Stage 2, V19.2.1 (2025-01)

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage 2, V18.4.0 (2024-12)

5 Annex A (Proposed LS)

Title:	[DRAFT] LS on questions regarding FS_VMR_Ph2

Response to:

Release:	Rel-19

Work Item:	FS_NR_WAB_5GFemto

Source:	CATT (to be RAN3)

To:	RAN2

Cc:	SA2

Contact Person:

#### Name:	Yang Lu

Tel. Number:

E-mail Address:	luyang@catt.cn

Attachments:

1. Overall Description:

In RAN3#127 meeting, RAN3 had the conclusion on avoiding multi-hop WAB topology for initial access:

WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly asks RAN2 to take into account above conclusion for their work and design corresponding signaling to support above solution.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN3 Meeting	 #127bis, April 7 to 11, 2025          China, CN

TSG-RAN3 Meeting	 #128, May 19 to 23, 2025          Malta, MT

6 Annex B (TP to BL CR for TS 38.401)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## X.3	WAB-gNB authorization

WAB authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB. The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the traffic of the co-located WAB-gNB’s traffic via BH PDU session(s). Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

During the WAB-node integration procedure, if the WAB-MT is authorized, the WAB-MT establishes the BH PDU session(s), otherwise, it does not establish the BH PDU session(s). If the WAB-gNB is authorized, the WAB-gNB establishes NG/Xn connections, otherwise, it does not establish NG/Xn connections.

After the WAB-node integration, if the authorization status of WAB-MT is changed to “not authorized”, the WAB-MT can request the co-located WAB-gNB to release/hand over all the UEs and remove the NG/Xn connections. The BH PDU session(s) of the WAB-MT are removed by AMF as specified in TS 23.501 [6]. When the WAB-MT is changed to “authorized”, the WAB-MT initiates the BH PDU session(s) establishment and requests the co-located WAB-gNB to set up NG connections (and Xn connections as needed).

After the WAB-node integration, if the authorization status of WAB-gNB is changed to “not authorized”, the WAB-gNB releases/hands over all the UEs and removes the NG/Xn connections, and it may trigger the WAB-MT to release the BH PDU session(s).  If the WAB-gNB is changed to “authorized”, it sets up NG connections (and Xn connections as needed).

X.4		Mobility handling

X.4.1		WAB-MT mobility

The WAB-MT reuses legacy mobility handover procedures for the UE as it moves throughout the BH-RAN. During the WAB-MT’s movement, the BH PDU session(s) of WAB-MT may need to be re-established reusing existing mechanisms defined in TS 23.501 [6]/TS 23.502 [7], when the WAB-MT’s PSA UPF need to be changed., In that case, the (outer) IP addressed used for the transport of NG and Xn connections will be changed, and. the WAB-MT should indicate the new IP addresses to the WAB-gNB. The WAB-gNB can change the TNL addresses used for C-plane and U-plane NG interface via RAN Configuration Update procedure over NG interface and PDU Session Resource Modify Indication procedure respectively, and change the TNL addressed used for C-plane and U-plane Xn interface by the NG-RAN Node Configuration Update procedure over Xn interface.

X.4.2		WAB-gNB mobility

X.4.2.1 	WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the cell(s) served by the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of PCI for the cell served by WAB-gNB may trigger the intra-gNB handover performed to the UEs connected to the WAB-gNB and trigger the cell re-selection at the UEs camping on the WAB-gNB. The change of TAC may trigger the Mobility Registration Update procedure as defined in TS 23.502 [7] at the UEs connected to or camping on the WAB-gNB.

X.4.2.2 	WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. In that case, the radio parameters configured to the cell(s) served by the WAB-gNB are changed, including cell ID, PCI and TAC, and the WAB-node is configured with the parameters needed to establish the NG connection to the new AMF.

The WAB-gNB then obtains the configuration parameters and it establishes NG connection towards the new AMF(s). During the NG setup procedure towards the new AMF(s), the WAB-gNB only reports the new TAC to the new AMF(s).

The WAB-gNB powers up new cell(s) corresponding to the new radio parameters. The new cell(s) broadcast the new radio parameters configured for the new AMF(s). The old cell(s) may remain active.

The UEs connected to or camping on the WAB-gNB are handled as follows:

-	A UE in RRC_CONNECTED state is handed over from an old cell served to a new cell and the UE’s AMF is changed to the new AMF via NG-based handover with AMF relocation, as defined in TS 23.502 [7].

-	A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and the UE’s AMF is changed to the new AMF via Mobility Registration Update procedure as defined in TS 23.502 [7] initiated at the UE.

The NG connection between the WAB-node and the initial AMF are removed, and the old cell(s) are removed from service when all UEs in RRC-CONNECTED state are handed over to the new cell(s).

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## 7.8	PCI Optimisation Function

The PCI Optimization Function in non-split gNB case is specified in TS 38.300 [2].

In split gNB architecture, the OAM configures a PCI for each NR cell to the gNB-DU.

For centralized PCI assignment in split gNB architecture, the gNB-CU detects PCI conflict of NR cells and reports the NR cells suffering PCI conflict to OAM directly. The OAM is in charge of reassigning a new PCI for the NR cell subject to PCI conflict.

For distributed PCI assignment in split gNB architecture, the OAM assigns a list of PCIs for each NR cell and sends the configured PCI list to the gNB-CU. If the gNB-CU detects PCI conflict, the gNB-CU may select a new PCI value from the preconfigured PCI list for the NR cell and send it to the gNB-DU by either F1 Setup procedure or gNB-CU configuration update procedure.

For mobile IAB deployments, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between mobile IAB cells and stationary cells by implementation.

For WAB deployment, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between WAB cells and stationary cells by implementation.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<