3GPP TSG-RAN WG3 Meeting #127                                                                    R3-250358

Athens, Greece, Feb 17th–21st, 2025

Source:	CATT

Title:	Discussion on stage-2 NR Femto

Agenda Item:	12.3

Document for:	Discussion and Decision

1 Introduction

In the last meeting[1], NR Femto was studied and some TPs are produced. The architecture and protocol stack for NR Femto have been captured in the BLCR of TS 38.300.

In this contribution, we will the leftover issue and the impact on stage-2 for NR Femto.

2 Discussion

2.1 Function

A NR Femto node hosts the same functions as a gNB, with the following additional specifics in case of connection to the NR Femto GW:

- Discovery of a suitable Serving NR Femto GW;

- A NR Femto node shall only connect to a single NR Femto GW at one time. The NR Femto node should not simultaneously connect to another NR Femto GW, or other AMF/UPF.

- The TAC and PLMN ID used by the NR Femto node shall also be supported by NR Femto GW.

- Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto node. Upon reception of the GUAMI from a UE, the NR Femto node shall include it in the INITIAL UE MESSAGE message.

- NR Femto nodes may be deployed without network planning. A NR Femto node may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location.

- Signalling the GUAMI of the Source AMF to the NR Femto GW in the PATH SWITCH REQUEST message.

- A NR Femto node may release the UE-associated logical NG-connection with NR Femto GW via UE context release procedure.

If the NR Femto GW is deployed, we propose that the NR Femto GW hosts the following functions in TS 38.300:

- Relaying UE-associated NG application part messages between AMF serving the UE and the NR Femto node serving the UE, for example INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message, HANDOVER REQUEST ACKNOWLEDGE message, PATH SWITCH REQUEST message, PATH SWITCH REQUEST ACKNOWLEDGE message.

- Terminating non-UE associated NG application part procedures towards the NR Femto node and towards the AMF, for example NG SETUP REQUEST message, PWS RESTART INDICATION message, PWS FAILURE INDICATION message, OVERLOAD START message, OVERLOAD STOP message.

- Optionally terminating NG-U interface with the NR Femto node and with the UPF.

- Supporting TAC and PLMN ID used by the NR Femto node.

- Xn interfaces shall not be established between the NR Femto GW and other nodes.

- Routing the PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto node.

- Requesting AMF to release the resources related to the signalled NG UE context via UE CONTEXT RELEASE REQUEST message.

Proposal 1: Capture above functions of NR Femto node and functions of NR Femto GW in TS 38.300 as [2].

2.2 Proximity Indication

In the past meeting, some companies discussed the introduction of the proximity indication. In LTE, the proximity indication is introduced for mobility of HeNB.

According to TS 36.300 [3], during the UE handover from source eNB to HeNB target cell, the proximity indication plays an important role. When a UE determines it may be near a CSG member cell, it sends an "entering" proximity indication to the source eNB. And, after sending an "entering" proximity indication, if the UE determines that it is no longer near a CSG member cell, the UE sends a "leaving" proximity indication to the source eNB. During the period between “entering” and “leaving”, the source eNB can configure the UE to acquire the SI of a specific cell. The UE can report the CSG ID and the CSG membership status of the cell via reading SI of the cell. The UE reports whether the target cell is a CSG member cell based on the UE's Permitted CSG list stored in the UE. Then the source eNB and MME can verify and carry out the access control based on the CSI-ID and CSG membership status. The procedure can prevent the network from initiating a handover to the target cell which is not the CSG member cell of the UE.

According to TS 36.331 [4], proximity indication procedure is described. This procedure is used to indicate that the UE is entering or leaving the proximity of one or more CSG member cells. The detection of proximity is based on an autonomous search function as defined in TS 36.304.

However, such UE assisted network-controlled handover is not needed in NR, given the CAG mechanism is reused for access control of NR Femto. According to TS 38.300 [5], after UE accesses the network, the AMF sets up the UE-associated logical NG-connection and provides the NG-RAN node with a list of CAGs allowed for the UE. The source NG-RAN node can use this list of CAGs allowed for the UE to determine whether the candidate cell is a member of the allowed CAGs of the UE and select a suitable cell for UE’s handover.

There is no mechanism to provide the source eNB with a list of CSGs allowed for the UE in LTE. So, it can be understood that is the reason why the proximity indication is needed in LTE.

In summary, the introduction of the proximity indication is no need for NR Femto.

Proposal 2: No need to introduce the proximity indication into NR Femto.

2.3 NAT

In the last meeting [1], the Femto GW is considered to be role of routing at IP in case of user plane transport concentration at the Femto GW. But, whether the Femto GW support NAT is FFS.

NAT (Network Address Translation) is a technology which translates internal private IP addresses (such as an internal LAN address) into legitimate public IP addresses. It can realize that an entire system appears on the Internet with a public IP address.  In our view, Femto GW can be responsible for the internal IP routing. And Femto GW uses a public IP address to connect to 5GC. Other NR Femto nodes use the internal private IP addresses. Femto GW routes traffic from the external IP address to the correct internal IP address.

NAT is a technology of Internet Engineering Task Force (IETF), the Femto GW can use it base on internal implementation, which has no impact to RAN3 spec.

Proposal 3: NR Femto GW supports NAT for IP routing.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: Capture above functions of NR Femto node and functions of NR Femto GW in TS 38.300 as [2].

Proposal 2: No need to introduce the proximity indication into NR Femto.

Proposal 3: NR Femto GW supports NAT for IP routing.

4 Reference

- RAN3#126 meeting report, RAN3 Chair

- R3-250359, (TP to BLCR for TS 38.300) Introduction of Functional Split for NR Femto, CATT, RAN3#127

- 3GPP TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN), Overall description, Stage-2, V18.2.0 (2024-06)

- 3GPP TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA), Radio Resource Control (RRC), Protocol specification, V18.3.1 (2024-09)

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.4.0 (2024-12)