3GPP TSG-RAN WG3 Meeting #127	R3-250370

Athens, Greece, 17th – 21st February, 2025

Title:	Discussion on the reply LSs on UAV regulation from SA1 and SA2

Agenda Item:	8.1

Source:	Huawei

Document for:	Discussion

# 1	Introduction

This contribution discusses RAN aspects and impacts for UAV regulation based on the reply LSs on UAV regulation that RAN3 received from SA1 and SA2 in [4] and [3], respectively.

A draft reply LS to SA2 in [11] is also proposed to be agreed.

# 2	Discussion

In RAN3#125bis meeting (October 2024) the discussion paper in [1] on UAV regulation was submitted, highlighting the need for a network-based mechanism for identifying, detecting and reporting the flight status information associated to both Rel-18 UAV UEs and non-UAV UEs located onboard an aerial vehicle (Rel-18 as well as pre Rel-18 UEs) .

The proponents motivated the specification of such mechanism to fulfil the requirement from the Chinese government aiming at tracking the real-time motion status (e.g., altitude, direction and speed) of a legitimate UAV UE for effective control and management of such devices, other than for the identification of “spurious” devices for public safety purposes. More specifically, the flight status information should be reported from the NG-RAN to the CN/AMF which, in turn, will transfer this information to the UAV monitoring platform via the NEF so that proper actions can be taken.

Eventually RAN3 decided to put the solution discussion on hold and check with SA2 about the requirement as mentioned by the proponents of the network-based mechanism for UAV regulation, hence RAN3 agreed to send the LS in [2] to SA2 (Cc: RAN, SA, SA1), an excerpt of which is reported below for convenience:

- Overall Description: During RAN3#125bis meeting, RAN3 has discussed whether to convey flight information e.g. altitude and velocity, from RAN to CN for control and management of the R18 UAV UEs and legacy devices (i.e. a Rel-15/16/17 device and non-UAV Rel-18 device) equipped in an aerial vehicle, based on operator’s contribution. RAN3 would like to ask for the guidance from SA2 whether it is a requirement for NG-RAN node to provide flight information to CN for the following cases: 1. R18 UAV UEs; 2. Legacy devices (non-UAV device); 2. Actions: To SA2 group: ACTION: RAN3 kindly asks SA2 to provide guidance.

At this meeting RAN3 received a reply LS from SA2 in [3] and also a reply LS from SA1 in [4], and this contribution discusses both reply LSs jointly.

In [4] SA1 informs RAN3 that there are no explicit service requirements in Rel-19 TS 22.125 [5] according to which the flight information needs to be reported from NG-RAN to CN for both Rel-18 UAV UEs and pre Rel-18 non-UAV devices – refer to the text highlighted in yellow below:

- Overall Description: SA1 thanks RAN3 for the LS on providing flight information from RAN to CN for R18 UAV UEs and pre-R18 non-UAV devices. RAN3 asked the following question to SA2: RAN3 would like to ask for the guidance from SA2 whether it is a requirement for NG-RAN node to provide flight information to CN for the following cases: 1. R18 UAV UEs; 2. Legacy devices (non-UAV device); SA1 would like to reply that there are no explicit service requirements for this function for these devices in TS 22.125. […]

While in [3], instead, SA2 replied that there is no requirement for the NG-RAN to report to the CN the flight status information associated to both Rel-18 UAV UEs (i.e., Case 1 in RAN3 LS) and legacy devices (non-UAV device, i.e., Case 2 in RAN3 LS) – refer to the text highlighted in green below:

1. Overall Description: SA2 would like to thank RAN3 regarding the LS on UAV regulation. SA2 provides following answers to RAN3 questions. RAN3 Question: During RAN3#125bis meeting, RAN3 has discussed whether to convey flight information e.g. altitude and velocity, from RAN to CN for control and management of the R18 UAV UEs and legacy devices (i.e. a Rel-15/16/17 device and non-UAV Rel-18 device) equipped in an aerial vehicle, based on operator’s contribution. RAN3 would like to ask for the guidance from SA2 whether it is a requirement for NG-RAN node to provide flight information to CN for the following cases: 1. R18 UAV UEs; 2. Legacy devices (non-UAV device); SA2 Reply: - On Case 2, SA2 has discussed and concluded that no such requirements nor solutions exist for non-UAV legacy devices. - On Case 1, SA2 does not have any requirement dealing with Rel-18 UAV UEs, but SA2 believes current SA2 work for Rel-19 UAS_Ph3 on an unrelated use case requires AMF to get flight information such as altitude and velocity. SA2 is specifying a new feature “Pre-flight Planning and In-flight Monitoring for UAVs” in clause 5.12, and USS changeover in clause 5.13 in TS 23.256. In case of in-flight monitoring for UAV, USS/UTM, may request for flight assistance information and that network notifies the USS if UAV deviates from the defined flight path including the UAV altitude and velocity. SA2 agrees that the UAV altitude and velocity can be received from NG-RAN, assuming to reuse the existing feature like eventH1/H2 reporting together with additional trigger from the core network as specified in Rel-19. Availability of such information is applicable to any UAV UE(s) that support the relevant procedures. […]

And, in our view, the above highlighted information from SA2 is in line with the information provided by SA1 in [4].

Observation 1: Both SA1 and SA2 confirmed that there is no requirement for the NG-RAN to provide flight status information to the CN for both Rel-18 UAV UEs and legacy devices.

However, SA2 provided further input to RAN3 for further discussion. To be specific, in their reply LS it is stated that, for legacy devices (Case 2 in RAN3 LS), SA2 concluded that not only requirements but also solutions do not exist for the NG-RAN to report flight status information to the CN:

[…] SA2 Reply: - On Case 2, SA2 has discussed and concluded that no such requirements nor solutions exist for non-UAV legacy devices. […]

while, for Rel-18 UAV UEs (Case 1 in RAN3 LS), SA2 indicated that, although no requirements exist, the flight status information reporting from NG-RAN to the CN could be supported based on the current specification work that SA2 is conducting on different use cases as part of the Rel-19 WI “Uncrewed Aerial System Phase 3” (UAS_Ph3) [6].

[…] SA2 Reply: […] - On Case 1, SA2 does not have any requirement dealing with Rel-18 UAV UEs, but SA2 believes current SA2 work for Rel-19 UAS_Ph3 on an unrelated use case requires AMF to get flight information such as altitude and velocity. […]

Observation 2: SA2 concluded that, although there are no requirements concerning the Rel-18 UAV UEs, the NG-RAN could be configured to provide flight status information such as altitude and velocity to the CN to support features defined in Rel-19 UAS_Ph3 WI.

More specifically, SA2 specified new features in Rel-19 termed as “NEF Assisted Pre-flight Planning”, “NEF Assisted In-flight Monitoring” as well as “USS changeover during a UAV flight”, detailed in clauses 5.12.2, 5.12.3 and 5.13.2 of TS 23.256 [7], respectively. For these features, SA2 also specified means to instruct the UAV UEs to perform altitude reporting in clause 5.16 of TS 23.256 (excerpted below for convenience), which can be done via application layer or via node-level signalling:

## 5.16 Altitude reporting for aerial UEs ### 5.16.1 General In order to assist a USS/UTM with aerial UE's flight planning and monitoring, the 5GS may enable configuration of aerial UEs (UAVs) to perform altitude reporting. This may include providing aerial UEs with threshold values for altitude reporting and/or request reporting periodicity (e.g. threshold-based, event-based, periodic). For that, the 5GS may determine altitude thresholds and periodicity with which an aerial UE should report its altitude. Altitude thresholds can be derived, for instance, based on the flight path information provided by the serving USS/UTM during a pre-flight planning request or a USS changeover request. Delivery of the altitude thresholds and reporting periodicity can be done via application layer or via node-level signalling. The 5GS should forward the altitude measurements results to the AMF and NEF/UAS NF. ### 5.16.2 Instructing aerial UEs to perform altitude reporting Procedure shown in Figure 5.16.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV's flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively. […]

In our view, the scenario described by the proponents of the network-based mechanism for UAV regulation in [1] can be regarded as an in-flight monitoring scenario specified in Rel-19 based on the node-level signalling solution, for which SA2 already foresaw the NG-RAN to be required to report flight status information to the CN/AMF which, in turn, will report such information to the UAV monitoring platform via the NEF. This seems to be also supported by SA2 according to what they stated in their reply LS [3], where the general assumption for a node-level signalling solution seems to be that the NG-RAN can rely on the existing mobility measurements in RRC to trigger a UE report when the altitude has reached a certain threshold, e.g., event H1/H2:

[…] SA2 Reply: […] - On Case 1, SA2 does not have any requirement dealing with Rel-18 UAV UEs, but SA2 believes current SA2 work for Rel-19 UAS_Ph3 on an unrelated use case requires AMF to get flight information such as altitude and velocity. SA2 is specifying a new feature “Pre-flight Planning and In-flight Monitoring for UAVs” in clause 5.12, and USS changeover in clause 5.13 in TS 23.256. In case of in-flight monitoring for UAV, USS/UTM, may request for flight assistance information and that network notifies the USS if UAV deviates from the defined flight path including the UAV altitude and velocity. SA2 agrees that the UAV altitude and velocity can be received from NG-RAN, assuming to reuse the existing feature like eventH1/H2 reporting together with additional trigger from the core network as specified in Rel-19. Availability of such information is applicable to any UAV UE(s) that support the relevant procedures. […]

Even if the “NEF Assisted In-flight Monitoring” has been specified by SA2 in Rel-19, SA2 also stated in their LS that the feature is applicable to any UAV UE that supports the relevant procedures, hence also including a Rel-18 UAV UE.

Proposal 1: RAN3 to consider a network-based mechanism, i.e., node-level signalling, for UAV regulation of Rel-18 UAV UEs relying on the “NEF Assisted In-flight Monitoring” feature specified by SA2 in TS 23.256.

## 2.1 Potential RAN3 impacts of flight status information reporting from NG-RAN to CN

In our understanding, any UAV UE supporting the relevant procedures for the “NEF Assisted In-flight Monitoring” can be currently configured to perform altitude reporting only, i.e., not velocity. The latest Rel-19 version of TS 23.256 (V19.1.0, 2024-12) [7], in fact, only includes specification of altitude reporting of UAV UEs.

But considering what SA2 already specified in the latest version of TS 23.256 [7], as previously stated, the altitude reporting can be via the application layer or via node-level signalling, and only the latter option has RAN3 impacts. Hence we will discuss only the node-level signalling option for altitude reporting.

In our understanding, it seems that RAN3 impacts for the node-level signalling option are limited to NGAP for:

- Measurement initiation: convey altitude thresholds and indication about reporting periodicity received by the AMF from the UAS NF/NEF for the UAV UE’s altitude reporting configuration, from AMF to NG-RAN over NG;

- Measurement report: transfer the report with the altitude information received by the UAV UE from the NG-RAN to the AMF over NG.

### 2.1.1 Measurement initiation

TS 23.256 [7], clause 5.16.2, specified the procedure for instructing a UAV UE to perform altitude reporting for in-flight monitoring (among other purposes), considering request from a USS/UTM (UAS Service Supplier / Uncrewed Aerial System Traffic Management) for assistance with UAV’s flight.

### 5.16.2 Instructing aerial UEs to perform altitude reporting Procedure shown in Figure 5.16.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV's flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively. Figure 5.16.2-1: Procedure for instructing aerial UEs for altitude reporting 1. UAS NF/NEF assists the USS/UTM with pre-fligh planning, in-flight monitoring and/or USS changeover based on the received request, which is either a Nnef_RetrieveInfoUAVFlight_Get or a Nnef_UAVFlightAssistance_Create, as specified in clauses 5.12 or 5.13. 2. Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point. The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events. NOTE 1: Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting. Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node). 3. Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for UE's altitude reporting to the aerial UE(s). Option A: Application layer. *** skip non-relevant text *** NOTE 2: Application layer reporting itself is outside of 3GPP scope. Option B: Node-level signalling. Editor's note: This requires RAN to provide further enhancement of the procedure under RAN responsibility. 3b. The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE's altitude reporting. Additionally, the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s). 4b. The AMF sends the corresponding N2 messages to all applicable NG-RAN node(s). 5b. The NG-RAN node(s) instructs the relevant UE(s) to perform altitude reporting as requested by the UAS NF/NEF (via AMF). NOTE 3: Aspects related to AMF informing NG-RAN regarding instructing UEs to perform altitude reporting are specified in TS 38.413 [24].

Our understanding is that the UAS NF/NEF derives, in Step 2., the UAV UE’s altitude thresholds and indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for the UE's altitude reporting.

Then, in case of node-level signalling, the UAS NF/NEF may send, in Step 3b., a subscription request to the AMF to get notified when the UAV UE’s assigned trajectory deviates unexpectedly. Within the request, the UAS NF/NEF may include the altitude thresholds and indication of reporting periodicity derived in Step 2. In Step 4., the AMF sends the corresponding N2 message (over NG) to all applicable NG-RAN node(s).

It is also mentioned, in Step 3b.,  that the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s). Since this scenario seems not to address thresholds, we could assume that measurement reporting should be event-based or periodic.

Based on the above, the measurement initiation over NG (AMF  NG-RAN) could be achieved with:

- one initiation using UE-associated signalling, including altitude thresholds and indication about reporting periodicity (i.e., threshold-based, event-based, or periodic), on a per UE basis;

- one initiation using non-UE associated signalling, including the indication about reporting periodicity (i.e., event-based, or periodic), on either per TA or per NG-RAN node basis.

In an effort to specify a single signalling solution covering both options above, RAN3 could consider developing a solution for measurement initiation using UE-associated signalling which includes altitude thresholds, indication about reporting periodicity (i.e., threshold-based, event-based, or periodic) and an area of interest (e.g., TA, NG-RAN node, cell).

Proposal 2: RAN3 to discuss a signalling solution for the measurement initiation over NG (AMF  NG-RAN) which uses UE-associated signalling and includes altitude thresholds, indication about reporting periodicity (i.e., threshold-based, event-based, or periodic) and an area of interest (e.g., TA, NG-RAN node, cell).

Proposal 3: RAN3 to agree to a signalling solution for measuremet initiation over NG as in Proposal 2, and check with SA2.

### 2.1.2 Measurement reporting

TS 23.256 [7], clause 5.16.3, specified the procedure for reporting the UAV UE’s altitude information to CN, following the configuration (i.e., altitude thresholds, indication about the reporting periodicity) as specified in clause 5.16.2 of the same TS.

### 5.16.3 Reporting UE's altitude information A procedure for reporting aerial UE's altitude information to the core network is shown in Figure 5.16.3-1. The reporting can be done by the aerial UE based on the received altitude reporting thresholds and indication about reporting periodicity as specified in clause 5.16.2. Figure 5.16.3-1: Procedure for reporting of UE's altitude information 0. An aerial UE receives request to perform its altitude information reporting as specified in clause 5.16.2. The request may include altitude reporting thresholds and, optionally, indication about reporting periodicity. 1. UE's flight altitude meets the previously provided altitude reporting conditions (i.e. altitude thresholds and, optionally, reporting periodicity). Option A: Application layer. *** skip non-relevant text *** Option B: Node-level signalling. 1a-2b. The UE may provide a report with the altitude information to the NG-RAN. The report may contain the altitude information and, for instance, a time stamp or a report ID indicating when the UE has measured the altitude information that is being reported. NOTE 2: Aspects related to a UE providing altitude information to an NG-RAN node are specified by RAN in TS 38.331 [23]. The NG-RAN node includes the received UE's altitude information in an N2 message to the AMF. 3b. The AMF provides the UAS NF/NEF the altitude information report inside a Namf_EventExposure_Notify request. 4. The UAS NF/NEF together with the USS determines the impact on UAV's flight path based on the received altitude information report; this aspect is outside the scope of the 3GPP.

According to TS 23.256 [7], in Step 1a., the UE may be configured to report altitude information to the NG-RAN along with a time stamp or a report ID indicating when the UE has measured the altitude information. The UE configuration is provided by NG-RAN according what is specified in TS 38.331 [8] and TS 38.300 [9] for the reporting events H1 and H2 as also mentioned in NOTE 1 of clause 5.16.2 in TS 23.256.

TS 38.300, clause 16.18.3, specifies that the UE includes its altitude and location information in the measurement report if configured with events H1 and H2:

### 16.18.3 Altitude-based Reporting for Aerial UE Communication An Aerial UE can be configured with altitude-dependent, event-based measurement reporting (i.e., eventH1 and eventH2 as defined in TS 38.331 [12]). An Aerial UE sends a measurement report when its altitude becomes higher or lower than configured threshold. The UE includes its altitude and location information in the measurement report if configured to do so by NG-RAN. RSRP/RSRQ/SINR measurement results are always reported when height reporting is configured. *** skip non-relevant text ***

Definitions of events H1 and H2 are in TS 38.331 (i.e., eventH1 and eventH2) and are included within ReportConfigNR IE along with the coarseLocationRequest and includeAltitudeUE IEs which indicate whether the UE has to report coarse location information (coarseLocationInfo in TS 38.331) and altitude information (altitudeUE in TS 38.331), respectively, as follows:

#### – ReportConfigNR The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event or of a CHO, CPA or CPC event or of an L2 U2N relay measurement reporting event. For events labelled AN with N equal to 1, 2 and so on, measurement reporting events and CHO, CPA or CPC events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS. *** skip non-relevant text *** The reporting events concerning Aerial UE altitude are labelled HN with N equal to 1 and 2. Additionally, the reporting events concerning Aerial UE altitude and the neighboring cell measurements simultaneously are labelled AMHN with M equal to 3, 4, 5 and N equal to 1, 2. Event H1: Aerial UE altitude becomes higher than a threshold; Event H2: Aerial UE altitude becomes lower than a threshold. *** skip non-relevant text ***

SA2 specified in Step 1a. that the UE report may contain a time stamp or a report ID along with the altitude information – it should also be noted that a definition of report ID is not available within the latest version of TS 23.256, and our understanding is that it may be used to distinguish a certain report from the UE in case the UE provides multiple reports, each of which could be triggered by different reporting configurations defined by the UAS NF/NEF.

According to the current TS 38.300 and 38.331, the current reporting mechanism does not contemplate that the UE includes neither the time stamp nor the report ID in the measurement report.

Proposal 4: RAN3 to agree to make SA2 aware that, based on current TS 38.331 (under RAN2 responsibility), the UAV UE may report altitude and coarse location information. Neither time stamp nor report ID can be reported.

Step 2b. in TS 23.256 [7], instead, concerns the actual reporting from NG-RAN to the AMF over NG. A potential solution could be based on re-using the framework of User Location Information in TS 38.413 [10], however, and in light of Proposal 4, the UAV UE can currently report only a subset of the information listed in Step 1a. of clause 5.16.3 of TS 23.256.

Proposal 5: RAN3 to inform SA2 about RAN3 agreements on the actual content of the reporting from NG-RAN to AMF over NG.

A draft reply LS to SA2 can be found in R3-250371.

# 3	Conclusion

This document proposes the following observation and proposals:

Observation 1: Both SA1 and SA2 confirmed that there is no requirement for the NG-RAN to provide flight status information to the CN for both Rel-18 UAV UEs and legacy devices.

Observation 2: SA2 concluded that, although there are no requirements concerning the Rel-18 UAV UEs, the NG-RAN could be configured to provide flight status information such as altitude and velocity to the CN to support features defined in Rel-19 UAS_Ph3 WI.

Proposal 1: RAN3 to consider a network-based mechanism, i.e., node-level signalling, for UAV regulation of Rel-18 UAV UEs relying on the “NEF Assisted In-flight Monitoring” feature specified by SA2 in TS 23.256.

Proposal 2: RAN3 to discuss a signalling solution for the measurement initiation over NG (AMF  NG-RAN) which uses UE-associated signalling and includes altitude thresholds, indication about reporting periodicity (i.e., threshold-based, event-based, or periodic) and an area of interest (e.g., TA, NG-RAN node, cell).

Proposal 3: RAN3 to agree to a signalling solution for measuremet initiation over NG as in Proposal 2, and check with SA2.

Proposal 4: RAN3 to agree to make SA2 aware that, based on current TS 38.331 (under RAN2 responsibility), the UAV UE may report altitude and coarse location information. Neither time stamp nor report ID can be reported.

Proposal 5: RAN3 to inform SA2 about RAN3 agreements on the actual content of the reporting from NG-RAN to AMF over NG.

A draft reply LS to SA2 can be found in R3-250371.

# 4	References

- R3-245527, Discussion on mechanism for UAV regulation, CMCC, ZTE, CATT, China Telecom, NTT DOCOMO

- R3-245815, LS on UAV regulation, CMCC

- R3-250022 (S2-2413011), Reply LS on UAV regulation, Samsung

- R3-250015 (S1-244759), Reply LS on UAV regulation, Huawei

- 3GPP TS 22.125 “Unmanned Aerial System (UAS) support in 3GPP”, Rel-19

- SP-230518, Uncrewed Aerial System Phase 3, Rel-19

- 3GPP TS 23.256 “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”, Rel-19

- 3GPP TS 38.331 “NR; Radio Resource Control (RRC); Protocol specification”, Rel-19

- 3GPP TS 38.300 “NR; NR and NG-RAN Overall description; Stage-2”, Rel-19

- 3GPP TS 38.413, “NG-RAN; NG Application Protocol (NGAP)”, Rel-19

- R3-250371, [Draft] Reply LS to SA2 on UAV regulation, Huawei