3GPP TSG-RAN WG3 Meeting #127	R3-250373

Athens, Greece, 17th – 21st February, 2025

Title: 	([Draft] LS to SA5 and TP for AI/ML BLCR to TS 38.423) Discussion on the AI/ML-based Network Slicing

Source: 	Huawei

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing.

This contribution analyzes further the technical enablers for the AI/ML-based Network Slicing use case to be introduced over the Xn interface, and provides proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting the following agreements were achieved [4]:

Stage3 signalling design: First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message. Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE. Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message. Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node. Slice UE performance: Companies have different understandings on if per slice per UE performance (e.g., throughput) is really supported by SA5 specification. Source node can request per UE per S-NSSAI granularity for UE performance feedback. There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

We will first discuss further the Stage 3 signalling details for the Xn interface and then we provide our views on the granularity of the UE performance feedback for the AI/ML-based Network Slicing use case.

## 2.1 XnAP Stage 3 signalling details

RAN3 agreed that the Data Collection Reporting Initiation and Data Collection Reporting procedures over Xn need to be enhanced to request and report the predicted radio resource status per slice and the predicted slice available capacity. Moreover, the Slice To Report List for Data Collection IE was agreed to be introduced in the DATA COLLECTION REQUEST message pertaining the Data Collection Reporting Initiation procedure to indicate the slice(s) for which the predicted radio resource status per slice and predicted slice available capacity need to be reported.

In the last meeting, instead, further agreements were reached concerning the encoding of the predicted radio resource status per slice (which reuses the Rel-18 Predicted Radio Resource Status IE) and the predicted slice available capacity (which is a new per cell IE in the DATA COLLECTION UPDATE message) – all the above agreements have already been reflected in the XnAP BLCR for enhancements on AI/ML for NG-RAN in [5].

In RAN3#125bis meeting the contribution in [6] proposed to introduce the predicted Composite Available Capacity (CAC) to be reported per cell together with the predicted slice available capacity. The legacy CAC information indicates the amount of resources per cell and per SSB area that are available relative to the total NG-RAN resources per cell, while the slice available capacity indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. And in legacy Resource Status Reporting over Xn, the slice available capacity is reported together with the CAC because CAC allows to correctly interpret the slice available capacity for cases where some slices share resources from the same resource pool. We think that, for the AI/ML-based Network Slicing, we should follow the legacy principle and hence discuss whether the predicted CAC should be introduced to be reported by the reporting node together with the predicted slice available capacity.

Proposal 1: RAN3 to discuss whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity.

If Proposal 1 above is agreed, then the tenth bit “Predicted Slice Available Capacity” in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message, as well as the “Predicted Slice Available Capacity” IE in the DATA COLLECTION UPDATE message need to be revised in order take the predicted composite available capacity into consideration. The TP for the XnAP BLCR in the Annex 2 addresses the needed revision by taking the Radio Resource Status Reporting (Initiation) procedures into consideration as baseline design.

Another aspect that still needs to be further discussed concerns the partial reporting in case the requesting node requests the predicted radio resource status per slice and the predicted slice available capacity but the reporting node can only report one of the two predictions (and indicates the failure in providing the other one via the DATA COLLECTION RESPONSE message). This open issue was briefly touched during the RAN3#125bis meeting (October 2024), with the corresponding FFS captured as follows [6]:

The failure of the predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node? Per cell per slice?

Based on the latest RAN3 agreements and Proposal 1 above into account, and considering that:

- the slice-specific information (predicted radio resource status per slice and predicted slice available capacity) are per cell metrics, and

- partial reporting of a certain prediction should be indicated per slice, as it could be reasonable to assume that a model may infer the requested slice-specific information according to the local conditions of each slice (e.g., slice load) that, in turn, could differ from one slice to another for a certain cell.

Therefore, we think that a new list Slice Measurements Initiation Result List IE should be introduced within the Cell Measurement Initiation Result List IE of the DATA COLLECTION RESPONSE message.

In this way, it could be possible to address the scenario where the requesting node configures the reporting node for providing the predicted radio resource status (the legacy Rel-18 prediction) and, by also including the Slice To Report List for Data Collection IE in the DATA COLLECTION REQUEST message, also the predicted radio resource status per slice but the reporting node can only provide one of the two predictions, either the legacy Rel-18 predicted radio resource status or the predicted radio resource status per slice. In our view there should be enough flexibility to allow the AI/ML model being used to predict the radio resource status to provide a prediction that is per SSB area of the cell (as for the legacy Rel-18 predicted radio resource status) while failing the same prediction (i.e., the radio resource status) per slice due to the above mentioned local conditions in the slice. It will anyway be possible that the reporting node fails both predictions because it is the AI/ML model which, for any reason, cannot predict the radio resource status regardless of the granularity for which predictions are requested.

Proposal 2: RAN3 to agree to introduce Slice Measurements Initiation Result List IE within the Cell Measurement Initiation Result List IE of the DATA COLLECTION RESPONSE message to address partial reporting of slice-specific predictions.

The TP for the XnAP BLCR in the Annex 2 reflects the Proposal 2 above.

## 2.2 Finer granularity of UE performance feedback

With regard to the feedback information, RAN3 agreed during the SI phase to collect from (neighbour) gNBs both the legacy per-UE UE performance feedback for those UEs handed over from the source gNB as well as the finer granularity UE performance feedback (including Average UE Throughput DL/UL, Average Packet Delay and Average Packet Loss DL) for those UEs handed over from the source gNB to determine UE performance for a certain slice in use by a certain UE. However, the exact level of finer granularity was left for further discussion during the normative phase.

So far, three possible options for the UE performance finer granularity have been proposed and discussed:

- Option 1: per PDU session per slice (S-NSSAI)

- Option 2: per slice (S-NSSAI)

- Option 3: per-QoS-flow-group

Among the three options above, Option 3 is the finest granularity, while Option 2 is the coarsest one. Therefore, it seems that Option 1 – per PDU session per slice granularity – is a compromise which should provide enough granular information for the requesting node to evaluate its AI/ML model. In fact, in RAN3#125bis meeting, the majority of companies preferred to have UE performance to be reported on a per PDU session per slice granularity, meaning that UE performance should be collected and reported for each PDU session belonging to a certain slice in use by the UE. This led to the following Working Assumption, with the possibility to further discuss about the feasibility of other granularities [6]:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. FFS if other finer granularity of UE performance is feasible.

However, concerns were raised in the last meeting on the actual feasibility of such granularity. Our understanding is that UE performance metrics are typically measured on a per DRB granularity as per TS 28.558 [9], and multiple DRBs can be mapped into the same PDU session. Therefore, we should ask SA5 how per PDU session measurement could be derived from the per DRB measurements (e.g., weighted averaging of per DRB measurements performed over all configured DRBs for the concerned UE) so to ensure that source and target gNBs apply the same methodology for deriving per PDU session per slice UE performance feedback before and after an AI/ML-based mobility action while the UE is using a certain slice, hence making the comparison between the two per PDU session measurements meaningful.

For Option 3 (per-QoS-flow-group UE performance), it is still unclear whether this could actually be implemented. In our understanding the “QoS-flow-group” represents the set of QoS flows that are mapped to a certain DRB. Therefore, in the source gNB side, the gNB can measure per DRB UE performance metrics (as per TS 28.558 [9]) and then implicitly derive the per-QoS-flow-group measurement (which corresponds exactly to the per DRB measurement). However, after the AI/ML-based mobility action for the UE using a certain slice, the target gNB may decide to map the set QoS flows (mapped into a single DRB in the source gNB) into multiple different DRBs. In other words, we think that only in case the target node decides to map the same set of QoS flows into a single DRB (as done by the source gNB) the comparison of UE performance before and after the AI/ML-based mobility action could be possible. However, it is kind of a corner case that this could occur, and for sure the target node cannot be forced to apply the same QoS-flows-to-DRB mapping as used by the source gNB.

When it comes to Option 2 (per slice UE performance), instead, it is worth to consider that, for the AI/ML-based Network Slicing, we focused during the SI on slice-level resource allocation and slice-based load balancing/mobility actions, therefore it would make sense to provide UE performance feedback on a per slice level as well.

In the last meeting RAN3 agreed that the source node can request per UE per slice UE performance considering that some companies (including us) think that per slice granularity of UE performance metrics is already supported by the standard as per SA5’s TS 28.558 [9], even if some companies do not think the same and hence they claim that UE performance cannot be measured by the target node on a per UE per slice granularity and reported back to the source node – all these considerations were also captured in the last meeting’s minutes [4]:

Companies have different understandings on if per slice per UE performance (e.g., throughput) is really supported by SA5 specification. Source node can request per UE per S-NSSAI granularity for UE performance feedback. There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

In order to progress with the discussion we should involve SA5 and get some feedback from them.

Proposal 3: RAN3 to agree to liaise SA5 to ask for a confirmation on the standard support in TS 28.558 of per UE per slice UE performance. A feasibility check of (per UE) per PDU session per slice UE performance could also be requested to SA5 before turning the WA from RAN3#125bis into an agreement.

A draft LS to SA5 can be found in Annex 1 of this paper.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

XnAP Stage 3 signalling details

Proposal 1: RAN3 to discuss whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity.

Proposal 2: RAN3 to agree to introduce Slice Measurements Initiation Result List IE within the Cell Measurement Initiation Result List IE of the DATA COLLECTION RESPONSE message to address partial reporting of slice-specific predictions.

The TP for the XnAP BLCR in the Annex 2 reflects the Proposals above.

Finer granularity of UE performance feedback

Proposal 3: RAN3 to agree to liaise SA5 to ask for a confirmation on the standard support in TS 28.558 of per UE per slice UE performance. A feasibility check of (per UE) per PDU session per slice UE performance could also be requested to SA5 before turning the WA from RAN3#125bis into an agreement.

A draft LS to SA5 can be found in Annex 1 of this paper.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- R3-247928, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio, Rakuten

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-245603, Extension of Data Collection Procedures for AI/ML Slicing, Nokia, Jio

- R3-245850, (BL CR to 38.423) Support of AI/ML assisted Network Slicing, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio, Rakuten

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-18

# 5. Annex 1 – [Draft] LS to SA5 on granularity of UE performance metrics for AI/ML-based Network Slicing

3GPP TSG-RAN3 Meeting #127	R3-25xxxx

Athens, Greece, 17th – 21st February, 2025

Title:	[Draft] LS on granularity of UE performance metrics for AI/ML Network Slicing

Response to:

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh

Source:	Huawei [to be RAN3]

To:	SA5

Cc:	RAN2

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of the Rel-19 Work Item on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 is currently discussing the granularity of UE performance metrics for a UE served by a certain slice to be reported from a target gNB to a source gNB over Xn after an AI/ML-based mobility action (i.e., handover) took place.

The UE performance metrics are Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss DL, already specified in TS 38.423 in Rel-18 on a per UE granularity.

RAN3 discussed about the feasibility of UE performance for a certain UE on a per slice granularity and RAN3 cannot currently converge on whether per slice (i.e., per S-NSSAI) UE performance metrics can already be  provided according to what is specified in TS 28.558 by means of subcounters per supported S-NSSAI.

Moreover, in RAN3#125bis meeting, the following Working Assumption was also reached:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

RAN3 understanding is that UE performance metrics can be measured for a certain UE on a per DRB granularity as per TS 28.558, and multiple DRBs can be mapped into the same PDU session of a certain slice. Therefore, RAN3 asks SA5 to indicate how per PDU session per slice performance metrics can be derived from the per DRB measurements.

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks confirmation from SA5 on whether UE performance for a certain UE can already be provided on a per slice (i.e., per S-NSSAI) granularity according to TS 28.558.

RAN3 respectfully asks SA5 to indicate how per PDU session per slice performance metrics can be derived from the per DRB measurements.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127bis			2025-04-07 – 2025-04-11				China, CN

RAN3#128			2025-05-19 – 2025-05-23				Malta, MT

# 6. Annex 2 – TP for AI/ML BLCR to TS 38.423 (based on R3-250042)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the slice-specific information reporting is requested, the Slice To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message. If NG-RAN node2 is capable of providing some but not all of the requested slice-specific information, it shall initiate the information reporting for the admitted requested slice-specific information and include the Slice Measurement Initiation Result List IE within the Cell Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Composite Available Capacity Group IE, if the tenth bit, "Predicted Composite Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if

- the Slice To Report List for Data Collection IE is included for the cell, and

- the measurement object is admitted by NG-RAN node2.

If the Cell Capacity Class Value IE is included within the Predicted Composite Available Capacity Group IE, this IE is used to assign weights to the predicted available capacity indicated in the Capacity Value IE. If the cell for which Predicted Composite Available Capacity Group IE is requested to be reported supports more than one SSB, the Predicted Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List for all SSB areas supported by the cell, providing the predicted SSB area capacity with respect to the Cell Capacity Class Value. If SSB To Report List IE is included for a cell, the Predicted Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE.

If the Slice To Report List for Data Collection IE is included for a cell for which the Predicted Composite Available Capacity Group IE is requested to be reported, the Predicted Slice Available Capacity IE for such cell shall include the requested Predicted Slice Available Capacity Value Downlink IE and Predicted Slice Available Capacity Value Uplink IE, providing the predicted slice capacity with respect to the Cell Capacity Class Value.

#### 8.4.13.3	Unsuccessful Operation

Figure 8.4.13.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, NG-RAN node2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.4.13.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the NG-RAN node1 may reinitiate the Data Collection Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the NG-RAN node2 Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the NG-RAN node2 shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going Data Collection reporting, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Composite Available Capacity Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  |  | 1 |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >> Slice Measurement Initiation Result List | O |  | 9.2.1.a |  |  |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally includes the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Composite Available Capacity Group | O |  | 9.2.2.xx |  |  |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.yy |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.a	Slice Measurement Initiation Result List

This IE indicated the list of measurement objects that failed to be initiated per slice.

| Slice Measurement Initiation Result List |  | 1..<maxnoofBPLMNs> |  |  |
| --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI Measurement Initiation Result List |  | 1 |  |  |
| >>S-NSSAI Measurement Initiation Result Item |  | 1..<maxnoofSliceItems |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>S-NSSAI Measurement Failure Cause List |  | 1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the slice. |
| >>>>S-NSSAI Measurement Failure Cause Item |  | 1..<maxFailedSliceMeasObjects> |  |  |
| >>>>>S-NSSAI Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Composite Available Capacity Other bits are ignored by the NG-RAN node1. |
| >>>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. PLMN Ids. Broadcast in a cell. Value is 12. |
| maxFailedSliceMeasObjects | Maximum no. of measurement objects that can fail per slice. Value is 124. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.xx	Predicted Composite Available Capacity Group

The Predicted Composite Available Capacity Group IE indicates the predicted overall available resource level per cell and per SSB area in the cell in Downlink, Uplink and Supplementary Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Composite Available Capacity Downlink | M |  | Composite Available Capacity 9.2.2.52 | For the Downlink | – |  |
| Predicted Composite Available Capacity Uplink | M |  | Composite Available Capacity 9.2.2.52 | For the Uplink, including both NUL and SUL (if available) | – |  |
| Predicted Composite Available Capacity Supplementary Uplink | O |  | Composite Available Capacity 9.2.2.52 | For the SUL | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.yy	Predicted Slice Available Capacity

The Predicted Slice Available Capacity IE indicates the amount of resources per network slice that are predicted to be available per cell relative to the total NG-RAN resources per cell. The Predicted Slice Available Capacity Value Downlink IE and the Predicted Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Predicted Composite Available Capacity Group IE, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted Slice Available Capacity |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >Predicted S-NSSAI Available Capacity List |  | 1 |  |  |
| >>Predicted S-NSSAI Available Capacity Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Predicted Slice Available Capacity Value Downlink | M |  | INTEGER (0..100) | Value 0 indicates no predicted available capacity, and 100 indicates predicted maximum available capacity. Predicted Slice Available Capacity Value Downlink should be measured on a linear scale. |
| >>>Predicted Slice Available Capacity Value Uplink | M |  | INTEGER (0..100) | Value 0 indicates no predicted available capacity, and 100 indicates predicted maximum available capacity. Predicted Slice Available Capacity Value Uplink should be measured on a linear scale. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids. broadcast in a cell. Value is 12. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

*** Unmodified Text Omitted ***

FROM XnAP-Containers

id-A2XPC5QoSParameters,

*** Unmodified Text Omitted ***

maxnoofSSBAreas

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

*** Unmodified Text Omitted ***

id-NRPPaPositioningInformation,

id-SliceToReportForDataCollection-List,

id-PredictedSliceAvailableCapacity,

id-PredictedCompositeAvailableCapacityGroup,

id-SliceMeasurementInitiationResult-List

maxEARFCN,

*** Unmodified Text Omitted ***

maxnoofRSPPQoSFlows

maxFailedSliceMeasObjects

*** Unmodified Text Omitted ***

#### -- C

CellToReportForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellToReportForDataCollection-Item

CellToReportForDataCollection-Item	::= SEQUENCE {

cell-ID								GlobalNG-RANCell-ID,

iE-Extensions						ProtocolExtensionContainer { { CellToReportForDataCollection-Item-ExtIEs} }	OPTIONAL,

...

}

CellToReportForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceToReportForDataCollection-List		CRITICALITY ignore	EXTENSION SliceToReportForDataCollection-List	PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

CellInfoResultForDataCollection-Item ::= SEQUENCE {

cellID											GlobalNG-RANCell-ID,

predictedRadioResourceStatus					RadioResourceStatus					OPTIONAL,

predictedNumberofActiveUEs						NumberofActiveUEs					OPTIONAL,

predictedRRCConnections							RRCConnections						OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {CellInfoResultForDataCollection-Item-ExtIEs} }	OPTIONAL,

...

}

CellInfoResultForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PredictedCompositeAvailableCapacityGroup		CRITICALITY ignore	EXTENSION PredictedCompositeAvailableCapacityGroup	PRESENCE optional},

ID id-PredictedSliceAvailableCapacity		CRITICALITY ignore	EXTENSION PredictedSliceAvailableCapacity	PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

CellMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellMeasurementInitiationResult-Item

CellMeasurementInitiationResult-Item ::= SEQUENCE {

cellID											GlobalNG-RANCell-ID,

cellMeasurementFailureCause-List				CellMeasurementFailureCause-List OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { CellMeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

CellMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceMeasurementInitiationResult-List	CRITICALITY ignore	EXTENSION	SliceMeasurementInitiationResult-list	PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- P

PredictedCompositeAvailableCapacityGroup ::= SEQUENCE {

predictedCompositeAvailableCapacityDownlink		CompositeAvailableCapacity,

predictedCompositeAvailableCapacityUplink		CompositeAvailableCapacity,

predictedCompositeAvailableCapacitySupplementaryUplink	CompositeAvailableCapacity		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { PredictedCompositeAvailableCapacityGroup-ExtIEs} }	OPTIONAL,

...

}

PredictedCompositeAvailableCapacityGroup-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PredictedSliceAvailableCapacity ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PredictedSliceAvailableCapacity-Item

PredictedSliceAvailableCapacity-Item	::= SEQUENCE {

pLMNIdentity						PLMN-Identity,

predictedSNSSAIAvailableCapacity-List		PredictedSNSSAIAvailableCapacity-List,

iE-Extensions						ProtocolExtensionContainer { { PredictedSliceAvailableCapacity-Item-ExtIEs} }	OPTIONAL,

...

}

PredictedSliceAvailableCapacity-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PredictedSNSSAIAvailableCapacity-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF PredictedSNSSAIAvailableCapacity-Item

PredictedSNSSAIAvailableCapacity-Item ::= SEQUENCE {

sNSSAI		S-NSSAI,

predictedSliceAvailableCapacityValueDownlink	INTEGER (0..100),

predictedSliceAvailableCapacityValueUplink	INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { PredictedSNSSAIAvailableCapacity-Item-ExtIEs } }	OPTIONAL

}

PredictedSNSSAIAvailableCapacity-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- S

SliceToReportForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceToReportForDataCollection-List-Item

SliceToReportForDataCollection-List-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions						ProtocolExtensionContainer { { SliceToReportForDataCollection-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceToReportForDataCollection-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceMeasurementInitiationResult-List-Item

SliceMeasurementInitiationResult-List-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIMeasurementInitiationResultlist					SNSSAIMeasurementInitiationResult-list,

iE-Extensions						ProtocolExtensionContainer { { SliceMeasurementInitiationResult-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceMeasurementInitiationResult-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIMeasurementInitiationResult-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIMeasurementInitiationResult-Item

SNSSAIMeasurementInitiationResult-Item ::= SEQUENCE {

sNSSAI		S-NSSAI,

sNSSAIMeasurementFailureCauselist					SNSSAIMeasurementFailureCause-list,

iE-Extensions				ProtocolExtensionContainer { { SNSSAIMeasurementInitiationResult-Item-ExtIEs } }	OPTIONAL

}

SNSSAIMeasurementInitiationResult-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIMeasurementFailureCause-list ::= SEQUENCE (SIZE(1.. maxFailedSliceMeasObjects)) OF SNSSAIMeasurementFailureCause-Item

SNSSAIMeasurementFailureCause-Item ::= SEQUENCE {

sNSSAIMeasurementFailedReportCharacteristics		BIT STRING(SIZE(32)),

cause										Cause,,

iE-Extensions				ProtocolExtensionContainer { { SNSSAIMeasurementFailureCause-Item-ExtIEs } }	OPTIONAL

}

SNSSAIMeasurementFailureCause-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

*** Unmodified Text Omitted ***

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxFailedSliceMeasObjects					INTEGER ::= 124

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

*** skip non-related text ***

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-Future-Coverage-Modification-List																ProtocolIE-ID ::= xx1

id-SliceToReportForDataCollection-List																ProtocolIE-ID ::= xx2

id-PredictedSliceAvailableCapacity																	ProtocolIE-ID ::= xx3

id-PredictedCompositeAvailableCapacityGroup															ProtocolIE-ID ::= xx4

id-SliceMeaurementInitiationResult-List																ProtocolIE-ID ::= xx5

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>