3GPP TSG-RAN WG3 Meeting #127	R3-250374

Athens, Greece, 17th – 21st February, 2025

Title: 	(TPs for AI/ML BLCR to TS 38.423 and AI/ML BLCR to TS 38.473) Discussion on the AI/ML-based Coverage and Capacity Optimization

Source: 	Huawei

Agenda item:	11.3

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based CCO (Coverage and Capacity Optimization).

This contribution analyzes further the technical enablers for the AI/ML-based CCO use case to be introduced over the Xn and F1 interfaces, and provides proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting the following agreements and open issues were achieved [4]:

Stage3 signalling design: Xn interface: Introducing new IE for Future Coverage Modification List over Xn. F1 interface: Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1. Introducing new IE for Future Coverage Modification Notification over F1. Introducing new IE for predicted CCO assistance information over F1. Further discussion on whether Value 0 needed for Future Cell Coverage State IE. Timing information: Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1. Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn. WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE. Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds. FFS on whether to send time information related to coverage modification cause over Xn.

Moreover, from RAN3#125bis meeting, there is another FFS aspect that was not tackled in the last meeting which is related to the feedback information for the AI/ML-based CCO use case [5]:

Information to validate the prediction is needed.

We further discuss here the FFS aspects above from previous RAN3 meetings.

## 2.1 F1AP Stage 3 signalling details

In the last RAN3 meeting it was agreed to enhance the existing NG-RAN node Configuration Update, gNB-CU Configuration Update and gNB-DU Configuration Update procedures to convey the AI/ML-specific information for CCO. In the BLCR to TS 38.473 for the AI/ML-based CCO in [6], a new Future Coverage Modification Notification IE has been introduced within the GNB-DU CONFIGURATION UPDATE, taking the existing Coverage Modification Notification IE as baseline, to transfer the future coverage configuration for cells and SSBs selected by the gNB-DU as a consequence of a CCO issue predicted by the gNB-CU. In the existing Coverage Modification Notification IE, for each Coverage Modification Item included in the Coverage Modification List IE, an associated Coverage Modification Cause IE is indicated. When comparing the existing Coverage Modification Notification IE with the newly introduced Future Coverage Modification Notification IE we noticed that the coverage modification cause in the new Future Coverage Modification Notification IE is missing, and we think that this is needed. Therefore, for each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE the associated Future Coverage Modification Notification Cause IE is proposed to be introduced.

Proposal 1: RAN3 to agree that each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE has a Future Coverage Modification Cause IE associated to it.

Proposal 2: RAN3 to agree that the Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only (same as for the predicted CCO issue).

Still concerning the F1AP aspects, there is a FFS from the last meeting related to the encoding of the Future Cell Coverage State IE within the Future Coverage Modification Notification IE.

In the latest BLCR to TS 38.473 for AI/ML-based CCO [6], the Future Cell Coverage State IE is an index encoded as an INTEGER in a range from 0 to 63 (with the possibility to further extend the range, if needed) and, in its semantics description, it is stated that ‘Value 0 indicates that the cell will be inactive’. There is however an Editor’s note in the BLCR to TS 38.473 according to which the value 0 for the Future Cell Coverage State IE is to be further discussed, reflecting the FFS as captured in [4]:

Further discussion on whether Value 0 needed for Future Cell Coverage State IE.

The FFS above was captured in the last meeting due to the concern from some companies about the actual meaning and implications of value 0, i.e., whether it could refer to the case of cell merging / cell splitting, for which RAN3 have never discussed in the context of the Rel-19 SI and hence should not be considered in the WI. In our understanding, value 0 covers both the cases of cell merging / cell splitting as well as cell being switched-off, in any case it is needed to indicate that a cell shall not serve UEs even if the cell is known by both the gNB-DU and gNB-CU as specified in TS 38.401 clause 8.5 [7]. Based on that, we think that value 0 should be kept as one of the possible values for the Future Cell Coverage State IE. There is also another negative impact that could be foreseen in case value 0 is excluded: it should be recalled that, at the time of the Rel-19 SI, it was agreed in RAN3#124 meeting that the future coverage state could be derived by the gNB-DU by either using an AI/ML model deployed in the gNB-DU or by using legacy means, up to implementation [8]:

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

And, in the legacy means, the gNB-DU is allowed to select value 0, therefore, by excluding value 0, it means that we are also posing restrictions to the legacy method that a gNB-DU implementation may use to derive the future coverage state as a consequence of a predicted CCO issue provided by the gNB-CU. Based on the above, we think that value 0 is needed and we should allow the gNB-DU to select it as one of the possible future coverage states for a cell.

Proposal 3: RAN3 to agree that value ‘0’ for the Future Cell Coverage State IE has the same meaning (cell inactive) as for the legacy Cell Coverage State IE within the legacy Coverage Modification Notification IE in the GNB-DU CONFIGURATION UPDATE message.

## 2.2 Timing information for AI/ML-based CCO

In RAN3#125bis meeting it was agreed to specify separate timing information for predicted CCO issue and future CCO state, and also definitions for these two pieces of information were agreed as follows [5]:

Timing information: Introducing separate time information for future coverage state and predicted CCO issue. Time for future coverage state: The point in time when the future coverage state will be applied. Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

while, in the last meeting, the discussion on the timing aspects progressed further and the following was achieved [4]:

Timing information: Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1. Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn. WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE. Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds. FFS on whether to send time information related to coverage modification cause over Xn.

We will start our discussion on timing aspects by focusing on the FFS from the last meeting first, then we will provide our views on the WA concerning the encoding aspects of the time information for both predicted CCO issue and future CCO state.

The timing information associated to the predicted CCO issue (i.e., Time for Predicted CCO Issue IE in the F1AP BLCR [6]) is transferred from the gNB-CU to the gNB-DU so that the gNB-DU is made aware of the fact that the new CCO state to cope with the predicted CCO issue is to be applied in the future and not immediately because the CCO issue did not occur yet. Conversely, the timing information of the future CCO state (i.e., Time for Future Coverage Modification IE in the F1AP BLCR [6]) is sent from the gNB-DU to the gNB-CU so that the gNB-CU will know when the future CCO state will be activated so to prevent (or limit at an early stage) the occurrence of the CCO issue.

Concerning the XnAP signalling, RAN3 agreed to transfer the timing information of the future CCO state (i.e., Time for Future Coverage Modification IE in the XnAP BLCR [9]) over Xn, within the NG-RAN CONFIGURATION UPDATE message. However, it needs to be further discussed whether also the time information of the coverage modification cause (which reflects the predicted CCO issue) needs to be transferred over Xn as well.

Let us assume that we have a split gNB1 (gNB-CU1 and gNB-DU1) where the CCO issue is predicted (in gNB-CU1), and a neighbor split gNB2 (gNB-CU2 and gNB-DU2) that is also impacted by this predicted CCO issue. gNB-CU1 sends over F1 the Predicted CCO issue and Time for Predicted CCO Issue IEs to gNB-DU1 which, in turn, generates the future CCO state together with the Time for Future Coverage Modification and sends these two pieces of information to the gNB-CU1.

According to what has been captured in TR 38.743 clause 4.2.2.5 for the impacts on Xn interface [3], the gNB-CU1 sends over Xn the future CCO state (of gNB1) with an associated coverage modification cause and “timing information”. To understand what “timing information” over Xn is, we shall consider what has been captured in the same clause 4.2.2.5 of TR 38.743 concerning the impacts on F1 interface from the gNB2 perspective. In order to allow the gNB2 to take proper counter-measures following the future CCO state derived at gNB1 side, and based on the F1 impacts captured in TR 38.743, gNB-CU2 shall send to gNB-DU2 a predicted CCO issue (including predicted affected cells and SSBs of gNB1) and Time for Predicted CCO Issue.

In our view the predicted CCO issue signalled by gNB-CU2 to gNB-DU2 typically is not a new issue but it is the same issue predicted by gNB-CU1 and signalled to gNB-CU2 as the coverage modification cause over Xn. This is as in the legacy CCO, where a CCO issue detected by gNB-CU1 is also signalled to gNB-CU2 as the coverage modification cause over Xn and, in turn, it is signalled to gNB-DU2 by gNB-CU2 over F1.

Proposal 4: RAN3 to agree that the predicted CCO issue signalled by gNB-CU2 to gNB-DU2 can be the same issue predicted by gNB-CU1 and signalled to gNB-CU2 as the coverage modification cause over Xn.

Therefore, gNB-CU2 needs to receive over Xn the predicted coverage modification cause (reflecting the predicted CCO issue at gNB1), the future CCO state and the Time for Future Coverage Modification both at gNB1, and the Time for Predicted CCO Issue (indicating when the CCO issue is predicted to occur). The first three pieces of information have already been captured in the XnAP BLCR in [9]. Moreover, the gNB-CU2 would benefit from knowing the Time for Predicted CCO Issue in order to understand the point in time after which it needs to start collecting metrics to be used as feedback information, as it is our understanding that neighbor nodes need to collect and report feedback information – the exact metrics are to be discussed in RAN3 – to be transferred back to the gNB-CU1 immediately after the predicted CCO issue was expected to occur.

Proposal 5: RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn also the Time for Predicted CCO Issue.

When it comes to the encoding aspects of the time information for both predicted CCO issue and future CCO state, RAN3 already agreed the following [4]:

WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE. Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds.

The above WA states that the Time for Future Coverage Modification and the Time for Predicted CCO Issue are encoded as the Requested Prediction Time IE, which is a configuration parameter that has been specified in Rel-18 within the DATA COLLECTION REQUEST message to indicate the point in time in the future for which a certain requested prediction refers to. It is encoded as an INTEGER from 1 to 60 seconds (extendable). The reason why RAN3 decided to limit the Requested Prediction Time IE to 60 seconds is because of the accuracy of the prediction which, in general, decreases if the prediction is requested for a time instant which is “too far in the future” with respect to time of receiving the DATA COLLECTION REQUEST message (carrying the Requested Prediction Time IE). For the two pieces of timing information discussed in the AI/ML-based CCO use case, these are not configuration parameters (as the Requested Prediction Time IE) used to derive the predicted CCO issue and future coverage state but, instead, they are provided along with these information.

Take the predicted CCO issue into account: first of all, it is not requested to be provided – there is no request/response mechanism for the gNB-DU to request the gNB-CU to provide the predicted CCO issue – but it is determined in the gNB-CU if there is a deployed AI/ML model in the gNB-CU and without any configuration parameter concerning the point in time to which the predicted CCO issue refers to. In our view, the Time for Predicted CCO Issue can be seen as an additional output of the AI/ML model which has predicted the CCO issue, and hence it has a different meaning than the Requested Prediction Time which, in turn, can be seen more as an input parameter for deriving a certain prediction.

Based on the above, we think that the encoding of timing information associated to predicted CCO issue and future coverage state should not be related to the encoding of the Requested Prediction Time IE, that is, it may span over the maximum value of 60 seconds. In our view, a reasonable time interval for both the Time for Predicted CCO Issue and the Time for Future Coverage Modification is up to two minutes (120 seconds). In this way, we should also ensure that the whole proactive CCO mechanism made of CU-DU interaction (for the DU to derive the future CCO state as a consequence of the predicted CCO issue from the CU) and source CU – neighbour node interaction (for the neighbour node to properly adapt to the new CCO configuration in the source CU) will be performed within a reasonable timeframe.

Proposal 6: RAN3 to agree that the Time for Predicted CCO Issue and the Time for Future Coverage Modification are encoded as INTERGER (1..120, …).

The TPs for AI/ML BLCR to TS 38.423 in Annex 1 and AI/ML BLCR to TS 38.473 in Annex 2 for the AI/ML-based CCO use case reflect the above proposals.

## 2.3 Feedback information AI/ML-based CCO

In RAN3#125bis meeting the following open point was in the meeting minutes [5]:

Information to validate the prediction is needed.

The above open issue is related to the discussion RAN3 had on the feedback information for the AI/ML-based CCO use case, in particular on UE performance feedback which is one of the items captured in TR 38.743 clause 4.2.2.4 along with measured radio resource status and SON reports [3]. Before discussing which kind of measurements (e.g., legacy per UE UE performance, radio-level UE measurements, network-level UE performance, etc.) should the neighbour node report to the node hosting the AI/ML model, it should be clarified in RAN3 what is intended to be validated by such report. As can be seen from the text captured as open point, such text is very generic as it refers to “prediction” without clarifying whether it is the “CCO prediction”, the “future CCO state” (if derived via AI/ML in gNB-DU, which is possible only by implementation) or a combination of the two.

In our view, we should base the discussion according to the definition of “feedback” as captured in TR 37.817 [10]:

## 4.2 Functional Framework Figure 4.2-1. Functional Framework for RAN Intelligence This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1. For the functions and data/information flows shown in the Figure 4.2-1, whether there is any standardization impact and what is the standardization impact are discussed in clause 5. **** Skip non-relevant text **** - Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself. - Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

Based on the above it is clear that the feedback information is strictly related to the AI/ML model, and for the AI/ML-based CCO the only AI/ML model which is supported by the standard is the one hosted by the gNB(-CU) which predicts the CCO issue. As a consequence, the feedback information that should be discussed in RAN3 should be for evaluating the CCO issue prediction.

Proposal 7: RAN3 to agree that feedback information is needed to evaluate the CCO issue prediction.

With reference to the specific information that should be reported at the node hosting the model which predicted the CCO issue, we think it is not enough for the AI/ML model to obtain legacy (Rel-18) per UE UE performance metrics as feedback information. We believe that the local node should also receive node-/cell-/SSB-level UE performance metrics from the neighbour node, since this information may further help to monitor the AI/ML model.

Proposal 8: RAN3 to agree to introduce node-/cell-/SSB-level UE performance feedback over the Xn interface for the AI/ML-based CCO use case.

Moreover, we think it would be beneficial for AI/ML model’s monitoring purposes that the local node could receive from the neighbour node information related to its current CCO state just after the Time for Predicted CCO Issue, that is, after the previously predicted CCO issue was expected to occur and the corresponding future CCO state was actually applied by the local node at the Time for Future Coverage Modification to avoid such predicted CCO issue. This would allow the model in the gNB(-CU) to have a clearer view of the AI/ML-based CCO mechanism and to fine-tune the model accordingly.

Proposal 9: RAN3 to agree to transfer the current CCO state of the neighbour gNB to the local node over the Xn interface as a feedback information.

## 2.4 On recommended future CCO state from gNB-CU to gNB-DU

TR 38.743, clause 4.2.2.5, contains also a note (NOTE 4) related to the possibility for the gNB-CU to generate a recommended/suggested future CCO state and send it to the gNB-DU as assistance information, so that the gNB-DU could take this information into account in order to determine the actual future CCO state in response of the predicted CCO issue. Below the note mentioned above is excerpted from in the TR [3]:

#### 4.2.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18 based on the CCO framework described in TS 38.300 [2] and TS 38.401 [3]. **** Skip non-relevant text **** F1 interface: **** Skip non-relevant text **** NOTE 4: Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion. **** Skip non-relevant text ****

In our previous paper in [11] we already provided our view on such scenario. From the legacy CCO mechanism, it could be noted that the gNB-CU actually has visibility of the CCO state and corresponding coverage modification cause information from the gNB-DU (i.e., Coverage Modification Notification IE within the GNB-DU CONFIGURATION UPDATE message) and, if available, from the neighbour gNBs (i.e., Coverage Modification List IE within the NG-RAN NODE CONFIGURATION UPDATE message) related to either another CCO issue detected in the neighbour gNB itself or as a consequence of a coverage configuration change in the concerned gNB-CU. In our understanding, this makes it possible for the AI/ML model in the gNB-CU to infer not only the predicted CCO issue but also the corresponding future CCO state based on historical information such as the previous CCO state from the gNB-DU and neighbour gNB. In this way, the gNB-DU could obtain richer and more meaningful information from the gNB-CU (i.e., not only the predicted CCO issue but also an indication of future CCO state) to solve or mitigate the predicted CCO issue in advance. It should be noted that we are not excluding the possibility that the gNB-DU determines the actual future CCO state for its own cells/SSBs, that is, the gNB-DU could either directly consider the recommended future CCO state as determined by the gNB-CU or determine the actual future CCO state on its own based on the predicted CCO issue from the gNB-CU and by also taking the recommended future coverage state from the gNB-CU as a reference.

Proposal 10: RAN3 to agree that the gNB-CU could provide to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation: the gNB-DU could either confirm the recommendation from the gNB-CU or use it as a reference in determining the actual future CCO state.

# 3. Conclusion

Based on the above discussion, we have the following proposals:

On F1AP Stage 3 signalling details

Proposal 1: RAN3 to agree that each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE has a Future Coverage Modification Cause IE associated to it.

Proposal 2: RAN3 to agree that the Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only (same as for the predicted CCO issue).

Proposal 3: RAN3 to agree that value ‘0’ for the Future Cell Coverage State IE has the same meaning (cell inactive) as for the legacy Cell Coverage State IE within the legacy Coverage Modification Notification IE in the GNB-DU CONFIGURATION UPDATE message.

On timing information

Proposal 4: RAN3 to agree that the predicted CCO issue signalled by gNB-CU2 to gNB-DU2 can be the same issue predicted by gNB-CU1 and signalled to gNB-CU2 as the coverage modification cause over Xn.

Proposal 5: RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn also the Time for Predicted CCO Issue.

Proposal 6: RAN3 to agree that the Time for Predicted CCO Issue and the Time for Future Coverage Modification are encoded as INTERGER (1..120, …).

The TPs for AI/ML BLCR to TS 38.423 in Annex 1 and AI/ML BLCR to TS 38.473 in Annex 2 for the AI/ML-based CCO use case reflect the above proposals.

On feedback information

Proposal 7: RAN3 to agree that feedback information is needed to evaluate the CCO issue prediction.

Proposal 8: RAN3 to agree to introduce node-/cell-/SSB-level UE performance feedback over the Xn interface for the AI/ML-based CCO use case.

Proposal 9: RAN3 to agree to transfer the current CCO state of the neighbour gNB to the local node over the Xn interface as a feedback information.

On recommended future CCO state from gNB-CU to gNB-DU

Proposal 10: RAN3 to agree that the gNB-CU could provide to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation: the gNB-DU could either confirm the recommendation from the gNB-CU or use it as a reference in determining the actual future CCO state.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-247929, (BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN, Ericsson

- 3GPP TS 38.401 “NG-RAN; Architecture description”, Rel-19

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_EOM1.doc)

- R3-247928, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE

- 3GPP TR 37.817 “Study on enhancement for Data Collection for NR and EN-DC”, Rel-17

- R3-243467, (TP to TR 38.743) AI/ML-based Coverage and Capacity Optimization, Huawei

# 5. Annex 1 – TP for AI/ML BLCR to TS 38.423 (based on R3-250042)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Update of Cell Coverage:

If the Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information in the Cell Coverage State IE to identify the cell deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

-	If the Cell Deployment Status Indicator IE is present in the Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the cell to be modified as the next planned configuration and shall remove any planned configuration stored for this cell.

-	If the Cell Deployment Status Indicator IE is present and the Cell Replacing Info IE contains non-empty cell list, the NG-RAN node2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the cells in the list as possible alternative handover targets.

-	If the Cell Deployment Status Indicator IE is not present, the NG-RAN node2 shall consider the cell deployment configuration of cell to be modified as activated and replace any previous configuration for the cells indicated in the Coverage Modification List IE.

If the SSB Coverage Modification List IE is present in the Coverage Modification List IE, the NG-RAN node2 may use the information in the SSB Coverage State IE to identify the SSB beam deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue detected at NG-RAN node1 and for configuring coverage state of its served cell(s).

If the Coverage Modification Cause IE set to "network energy saving" is present and the SSB Coverage State IE is zero for a set of SSB beams in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to decide the SSB beam activation for the concerned beams when necessary.

Update of Future Cell Coverage:

If the Future Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to identify the future cell deployment configuration to be enabled by the NG-RAN node1 at a time indicated by Time for Future Coverage State IE and for configuring mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

If the Future SSB Modification List IE is present in the Future Coverage Modification List IE, the NG-RAN node2 may use the information to identify the future SSB beam deployment configuration to be enabled by the NG-RAN node1 at a time indicated by Time for Future Coverage State IE and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Predicted Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue predicted in NG-RAN node1 and for configuring coverage state of its served cell(s).

Interactions with other procedures:

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier within the Neighbour NG-RAN Node List IE identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |
| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| >Future Coverage Modification Item |  | 0..<maxnoofCellsinNG-RAN node> |  |  | - |  |
| >>Global NG-RAN Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell whose coverage will be modified. | - |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicates that the cell will be active and also indicates the future coverage configuration of the concerned cell. | - |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams whose coverage will be modified. | - |  |
| >>>Future SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB beam whose coverage will be modified. | - |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also Indicates the future coverage configuration of the concerned SSB beams. | - |  |
| >>Predicted Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the reason for the predicted coverage modification in NG-RAN node1. | - |  |
| >>Time for Future Coverage State | O |  | INTEGER (1..120, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. | - |  |
| >>Time for Predicted Coverage Modification Cause | O |  | INTEGER (1..120, …) | Indicates the time when the Predicted Coverage Modification Cause will happen from the time of receiving this information, in seconds. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

*** Unmodified text omitted ***

NRPPaPositioningInformation,

Future-Coverage-Modification-List

FROM XnAP-IEs

PrivateIE-Container{},

*** Unmodified text omitted ***

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-A2XPC5QoSParameters,

*** Unmodified text omitted ***

id-NRPPaPositioningInformation,

id-Future-Coverage-Modification-List

maxnoofCellsinNG-RANnode,

*** Unmodified text omitted ***

maxnoofSSBAreas

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- NG-RAN NODE CONFIGURATION UPDATE

--

-- **************************************************************

NGRANNodeConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ NGRANNodeConfigurationUpdate-IEs}},

...

}

NGRANNodeConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TAISupport-list							CRITICALITY reject	TYPE TAISupport-List									PRESENCE optional }|

{ ID id-ConfigurationUpdateInitiatingNodeChoice	CRITICALITY ignore	TYPE ConfigurationUpdateInitiatingNodeChoice	PRESENCE mandatory}|

{ ID id-TNLA-To-Add-List						CRITICALITY ignore	TYPE TNLA-To-Add-List									PRESENCE optional }|

{ ID id-TNLA-To-Remove-List						CRITICALITY ignore	TYPE TNLA-To-Remove-List								PRESENCE optional }|

{ ID id-TNLA-To-Update-List						CRITICALITY ignore	TYPE TNLA-To-Update-List								PRESENCE optional }|

{ ID id-GlobalNG-RAN-node-ID					CRITICALITY reject	TYPE GlobalNG-RANNode-ID								PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Add			CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-AMF-Region-Information-To-Delete		CRITICALITY reject	TYPE AMF-Region-Information							PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication					PRESENCE optional }|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo								PRESENCE optional }|

{ ID id-Coverage-Modification-List				CRITICALITY reject	TYPE Coverage-Modification-List						PRESENCE optional }|

{ ID id-Local-NG-RAN-Node-Identifier			CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier					PRESENCE optional }|

{ ID id-Neighbour-NG-RAN-Node-List				CRITICALITY ignore	TYPE Neighbour-NG-RAN-Node-List						PRESENCE optional }|

{ ID id-Local-NG-RAN-Node-Identifier-Removal	CRITICALITY ignore	TYPE Local-NG-RAN-Node-Identifier					PRESENCE optional }|

{ ID id-Future-Coverage-Modification-List		CRITICALITY ignore	TYPE Future-Coverage-Modification-List			PRESENCE optional },

...

}

ConfigurationUpdateInitiatingNodeChoice ::= CHOICE {

gNB									ProtocolIE-Container	{ {ConfigurationUpdate-gNB} },

ng-eNB								ProtocolIE-Container	{ {ConfigurationUpdate-ng-eNB} },

choice-extension					ProtocolIE-Single-Container { {ServedCellsToUpdateInitiatingNodeChoice-ExtIEs} }

}

ServedCellsToUpdateInitiatingNodeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ConfigurationUpdate-gNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-NR			CRITICALITY ignore TYPE	ServedCellsToUpdate-NR							PRESENCE optional }|

{ ID id-cellAssistanceInfo-NR			CRITICALITY ignore TYPE	CellAssistanceInfo-NR							PRESENCE optional }|

{ ID id-cellAssistanceInfo-EUTRA		CRITICALITY ignore TYPE	CellAssistanceInfo-EUTRA						PRESENCE optional }|

{ ID id-ServedCellSpecificInfoReq-NR	CRITICALITY ignore TYPE	ServedCellSpecificInfoReq-NR					PRESENCE optional},

...

}

ConfigurationUpdate-ng-eNB XNAP-PROTOCOL-IES ::= {

{ ID id-servedCellsToUpdate-E-UTRA			CRITICALITY ignore TYPE	ServedCellsToUpdate-E-UTRA				PRESENCE optional }|

{ ID id-cellAssistanceInfo-NR				CRITICALITY ignore TYPE	CellAssistanceInfo-NR						PRESENCE optional }|

{ ID id-cellAssistanceInfo-EUTRA			CRITICALITY ignore TYPE	CellAssistanceInfo-EUTRA					PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

*** Unmodified text omitted ***

id-NRPPaPositioningInformation,

id-SliceToReportForDataCollection-List,

id-PredictedSliceAvailableCapacity,

maxEARFCN,

*** Unmodified text omitted ***

maxnoofRSPPQoSFlows

*** Unmodified text omitted ***

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- F

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofCellsinNG-RANnode)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

globalNG-RANCell-ID						GlobalCell-ID,

future-cellCoverageState				INTEGER (0..63, ...),

future-SSB-Coverage-Modification-List	Future-SSB-Coverage-Modification-List		OPTIONAL,

predicted-Coverage-Modification-Cause	Predicted-CoverageModificationCause			OPTIONAL,

timeForFutureCoverageState				INTEGER (1..120,...)											OPTIONAL,

timeForPredictedCoverageModificationCause	INTEGER (1..120,...)					OPTIONAL

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Future-SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofSSBAreas)) OF Future-SSB-Coverage-Modification-Item

Future-SSB-Coverage-Modification-Item ::= SEQUENCE {

sSBIndex						INTEGER(0..63),

future-SSBCoverageState			INTEGER (0..15, ...),

iE-Extension					ProtocolExtensionContainer { { Future-SSB-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-SSB-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 6. Annex 2 – TP for AI/ML BLCR to TS 38.473 (based on R3-250043)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization and network energy saving. If the Coverage Modification Cause IE is set to the "network energy saving", gNB-CU may consider those deactivated SSB beams are due to network energy saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the RRC Terminating IAB-Donor Related Info IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the BAP address indicated by the Mobile IAB-MT BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated  by the RRC Terminating IAB-Donor gNB-ID IE, and it shall use this  BAP address and gNB ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node as needed, as specified in TS 38.423 [28].

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for proactive Coverage and Capacity Optimization.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.If the CCO issue detection IE set to "network energy saving" is included in the CCO Assistance Information IE, the gNB-DU may consider the indicated SSB beams by the Affected Cells and Beam IE are deactivated due to network energy saving.

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a future cell and/or beam configuration.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust the coverage of its cells. 
Editor´s note: to be further discussed whether any more details need to be added to the above paragraph.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | ignore |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-DU. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| RRC Terminating IAB-Donor Related Info | O |  | 9.3.1.306 | Indicates the information related to a mobile IAB-node’s RRC-terminating IAB-donor. | YES | reject |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Future Coverage Modification Notification | O |  | 9.3.1.B |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Predicted CCO Assistance Information | O |  | 9.3.1.A | Indicates predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.212	Affected Cells and Beams

This IE includes a list of cells and/or SS/PBCH block indexes affected by the detected CCO issue or predicted to be affected by the predicted CCO issue.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Affected Cell List |  | 1 .. < maxAffectedCells> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| >Affected SSB List |  | 0..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxAffectedCells | Maximum numbers of cells affected by a CCO issue. Value is 32. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO Issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..120, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..120, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

*** unmodified text omitted ***

F1U-PathFailure,

Future-Coverage-Modification-Notification,

Predicted-CCO-Assistance-Information

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-AssociatedSessionID,

*** unmodified text omitted ***

id-F1U-PathFailure,

id-Future-Coverage-Modification-Notification,

id-Predicted-CCO-Assistance-Information,

maxCellingNBDU,

*** unmodified text omitted ***

maxnoofServingCellMOs

### 9.4.4	PDU Definitions

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- GNB-DU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE

--

-- **************************************************************

GNBDUConfigurationUpdate::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateIEs} },

...

}

GNBDUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID												PRESENCE mandatory	}|

{ ID id-Served-Cells-To-Add-List				CRITICALITY reject	TYPE Served-Cells-To-Add-List								PRESENCE optional	}|

{ ID id-Served-Cells-To-Modify-List				CRITICALITY reject	TYPE Served-Cells-To-Modify-List							PRESENCE optional	}|

{ ID id-Served-Cells-To-Delete-List				CRITICALITY reject	TYPE Served-Cells-To-Delete-List							PRESENCE optional	}|

{ ID id-Cells-Status-List						CRITICALITY reject	TYPE Cells-Status-List											PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List		CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List					PRESENCE optional	}|

{ ID id-gNB-DU-ID								CRITICALITY reject	TYPE GNB-DU-ID													PRESENCE optional	}|

{ ID id-GNB-DU-TNL-Association-To-Remove-List	CRITICALITY reject	TYPE GNB-DU-TNL-Association-To-Remove-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE Transport-Layer-Address-Info							PRESENCE optional	}|

{ ID id-Coverage-Modification-Notification		CRITICALITY ignore	TYPE Coverage-Modification-Notification					PRESENCE optional	}|

{ ID id-gNB-DU-Name								CRITICALITY ignore	TYPE GNB-DU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name					CRITICALITY ignore	TYPE Extended-GNB-DU-Name										PRESENCE optional	}|

{ ID id-RRC-Terminating-IAB-Donor-Related-Info	CRITICALITY reject	TYPE RRC-Terminating-IAB-Donor-Related-Info	         PRESENCE optional }|

{ ID id-Mobile-IAB-MTUserLocationInformation    CRITICALITY ignore	TYPE Mobile-IAB-MTUserLocationInformation 		      PRESENCE optional	}|

{ ID id-Future-Coverage-Modification-Notification		CRITICALITY ignore	TYPE Future-Coverage-Modification-Notification					PRESENCE optional	}},

...

}

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- GNB-CU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	}|

{ ID id-Cells-Allowed-to-be-Deactivated-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List		PRESENCE optional	}|

{ ID id-Predicted-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	Predicted-CCO-Assistance-Information							PRESENCE optional	},

...

}

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

*** unmodified text omitted ***

maxnoAggregatedPosSRSCombinations

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- F

Future-Coverage-Modification-Notification ::= SEQUENCE {

future-coverage-Modification-List			Future-Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Future-Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxCellingNBDU)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

futurecellCoverageState				FutureCellCoverageState,

futureSSBCoverageModificationList	FutureSSBCoverageModification-List	OPTIONAL,

timeforFutureCoverageModification	TimeforFutureCoverageModification	OPTIONAL,

futureCoverageModificationCause		Predicted-CCO-issue-detection		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FutureCellCoverageState ::= INTEGER (1..63, ...)

FutureSSBCoverageModification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF FutureSSBCoverageModification-Item

FutureSSBCoverageModification-Item::= SEQUENCE {

sSBIndex						INTEGER(0..63),

futureSSBCoverageState			FutureSSBCoverageState,

iE-Extensions		ProtocolExtensionContainer { { FutureSSBCoverageModification-Item-ExtIEs} }	OPTIONAL,

...

}

FutureSSBCoverageModification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FutureSSBCoverageState ::= INTEGER (1..15, ...)

#### -- T

TimeforFutureCoverageModification ::= INTERGER (1..120, ...)

TimeforPredictedCCOIssue ::= INTEGER (1..120, ...)

#### -- P

Predicted-CCO-Assistance-Information ::= SEQUENCE {

predicted-CCO-issue-detection	Predicted-CCO-issue-detection	OPTIONAL,

predictedAffectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

timeforPredictedCCOIssue		TimeforPredictedCCOIssue,	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Predicted-CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

Predicted-CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Predicted-CCO-issue-detection	::=	ENUMERATED {coverage, cell-edge-capacity, ...}

<<<<<<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

*** unmodified text omitted ***

id-TagIDPointer										ProtocolIE-ID ::= 853

id-Future-Coverage-Modification-Notification		ProtocolIE-ID ::= xxx

id-Predicted-CCO-Assistance-Information				ProtocolIE-ID ::= yyy

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>