3GPP TSG-RAN WG3 Meeting #127	R3-250375

Athens, Greece, 17th – 21st February, 2025

Title: 	(TP for AI/ML BLCR to TS 38.423) AI/ML-based Mobility Optimization in NR-DC

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to the mobility optimization for NR-DC.

This contribution analyzes the technical enablers for specifying support over the Xn interface for mobility optimization in NR-DC, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

## 2.1 UE performance collection and reporting in NR-DC

During the Rel-19 SI, for mobility optimization in NR-DC, the use case description and standard impacts have been investigated and reported in TR 38.743 clause 5.1 as follows, where the standard impacts are mainly on enabling the collection and reporting of UE performance from the Secondary Node (SN) to the Master Node (MN) over the Xn [3]:

## 5.1 Mobility optimization for NR-DC ### 5.1.1 Use case description Mobility in NR-DC can be optimized by means of AI/ML. Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only and Conditional Dual Connectivity procedures are out of scope. ### 5.1.2 Potential Standard impacts The Dual Connectivity procedures (e.g., SN Addition, MN-initiated SN Change) are enhanced to trigger the collection of measured UE performance.

In the last meeting RAN3 agreed to enhance the existing SN ADDITION REQUEST message in XnAP specification by introducing the Data Collection ID IE identifying a Data Collection Reporting context, that is, a Data Collection Reporting Initiation procedure to configure the collection of the UE performance at the SN side. The exact agreements are as follows [4]:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side. Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

The above agreements follow the outcome of RAN3#125bis meeting, during which RAN3 discussed about the scenarios to be considered for the collection and reporting of UE performance and reached the following agreement [5]:

As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance.

Based on the above agreement, the “basic” scenarios agreed to be considered for triggering the collection of UE performance from SN to MN are related to the successful completion of SN Addition and MN-initiated SN Change. Moreover, it is also part of the agreement that the involved gNBs (that is, MN and SN) need to have an existing Data Collection Reporting Initiation procedure already in place between the two, which is needed for configuring the collection of the UE performance by means of the UE Performance Collection Configuration IE within the DATA COLLECTION REQUEST message. To “link” the SN Addition and MN-initiated SN Change procedures with the Data Collection Reporting Initiation procedure, the Data Collection ID IE is used.

The actual reporting of UE performance from the SN to the MN can be performed via the Data Collection Reporting procedure (namely, within the DATA COLLECTION UPDATE message) already in place between MN and SN, based on the configuration from the MN included in the Data Collection Reporting Initiation procedure (identified by the Data Collection ID IE).

Proposal 1: RAN3 to agree that the reporting of UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message) already in place between MN and SN. No specification impact is foreseen.

In RAN3#125bis meeting the following open issue was captured as an aspect that needs further discussion [5]:

Whether to use the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side or whether to reuse existing DC procedure to configure the collection of UE performance at SN side and a new procedure for the reporting of UE performance needs further discussion.

Even if it is not explicitly mentioned, the above open issue is related to a scenario being different from the ones addressed before (i.e., SN Addition and MN-initiated SN Change) and covered by the agreement reached in the last RAN3 meeting. This additional scenario has been described in the contribution [6] submitted in RAN3#125bis meeting, where a UE initially single-connected to a source gNB (say gNB1) is handed over to a target gNB (say gNB2) which, in turn, decides to set-up a NR-DC with a third gNB (say gNB3). In such scenario, the target gNB (gNB2) becomes the target MN while the third gNB (gNB3) becomes the target SN, and UE performance collection is configured from the source gNB (gNB1) to be reported by the target MN (gNB2) by means of the Data Collection Reporting Initiation procedure existing between gNB1 and gNB2. There is however the need to discuss which is the impact of the target SN (gNB3) on the UE performance reported from the target MN to the source gNB, as well as how to configure the collection of UE performance at the target SN side and reporting of such UE performance back to the target MN. The target MN should report to the source gNB – via the Data Collection Reporting procedure – either the aggregated UE performance collected at both target MN and target SN or UE performance measured at MN and SN separately. Moreover, it is our understanding that UE performance are typically collected immediately after the handover (source gNB gNB1 to a target gNB gNB2 in the example above) and for a short time window (configured by the source gNB), so it is unclear whether there will be enough time for the target gNB to establish NR-DC with the third gNB (gNB3). Therefore, considering that Rel-19 is the first release to discuss AI/ML-based mobility optimization in NR-DC, new scenarios other than the “basic” SN Addition and MN-initiated SN Change should be deprioritized and, if time allows, their actual feasibility can be discussed already in Rel-19 or in a future release.

Proposal 2: RAN3 to agree that scenarios other than “basic” SN Addition and MN-initiated SN Change are deprioritized. Their actual feasibility could be discussed in Rel-19 only if time allows.

## 2.2 On UE performance metrics in NR-DC

The UE performance metrics to be reported as part of the feedback information after an AI/ML-based action as well as the UE performance collection configuration have been specified in Rel-18 for XnAP as follows [7]:

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11. **** Skip non-relevant text **** #### 9.2.3.186 UE Performance Collection Configuration This IE indicates the configuration for UE performance measurement collection. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Collection Time Duration for UE Performance; M; INTEGER(1..5000, ...); Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond. As

It is obvious that the UE performance metrics are only for the basic single connectivity case, where the collection and reporting is done at the target node side towards the source node. For the case of NR-DC, RAN3 needs to investigate which UE performance metrics are triggered to be collected by the SN after successful SN Addition / Change (MN-initiated) for the concerned UE. It should be recalled that the Rel-18 UE performance metrics are on a per-UE granularity. Let’s first provide an insight on how these UE performance metrics are specified:

- Average UE throughput DL/UL. As specified in TS 28.558 [8], this measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. Since the data volume is counted on the RLC SDU level, it is the RLC layer which is in charge of calculating the per-UE Average UE throughput DL/UL. Note that RAN3 is waiting for SA5 feedback on whether the Average UE throughput DL/UL measurements defined in TS 28.558 are actually measured per-UE (refer to the LS sent by RAN3 in [9]).

- Average packet delay. As specified in TS 38.314 [10], the packet delay includes RAN part of delay and CN part of delay, and the RAN part delay is separated into multiple components defined for both UL and DL, where also the granularity of such measurements is indicated (i.e., per-DRB per-UE).

- Average packet loss DL. As specified in TS 38.314 [10], this metric is to measure packets that are lost at Uu transmission. And each packet is corresponding to one RLC SDU. Thus, this metric is also calculated at RLC level, and it is per-DRB per-UE.

Below we provide in Table 1 a summary indicating the measured UE performance information that need to be collected from SN and reported to MN, as a function of the bearer type.

Table 1 - UE performance components in the SN side as a function of the bearer type.

| Bearer types | Average UE throughput DL/UL | Average Packet Delay DL | Average Packet Delay UL | Average Packet Loss DL |
| --- | --- | --- | --- | --- |
| MN-terminated split bearer | RLC level THP for SCG part. | D1, D2, measured in SCG. | D2.1, D2.2, measured in SCG. And potentially D1(if UE reports to SN). | RLC level packet loss for SCG part. |
| SN-terminated split bearer | RLC level THP for SCG part. | D1, D2, D4 measured in SCG. | D2.1, D2.2 and D2.4 measured in SN. And potentially D1(if UE reports to SN). | RLC level packet loss for SCG part. |
| MN-terminated SCG bearer | RLC level THP in SCG. | D1, D2, measured in SCG. | D2.1, D2.2, measured in SCG. And potentially D1(if UE reports to SN). | RLC level packet loss in SCG. |
| SN-terminated MCG bearer | No need to provide. | D4 measured in SN. | D2.4 measured in SN. And potentially D1(if UE reports to SN). | No need to provide. |
| SN-terminated SCG bearer | RLC level THP in SCG. | Average packet delay DL calculated by SN. | Average packet delay UL calculated by SN. | RLC level packet loss in SCG. |

We need to note that though DL/UL delay calculation per split-DRB per UE is already defined in TS 38.314 [10] clause 4.2.1.9, the above-mentioned information exchange over Xn interface from SN to MN is to help the MN to obtain a full understanding of UE performance in SN side. Hence, we suggest using the above table as a start point for analysing the UE performance feedback in NR-DC.

Proposal 3: RAN3 to take Table 1 as the start point for analysing UE performance metrics in NR-DC so that SN-relevant UE performance metrics components can be collected by the SN and reported to the MN.

Note that, as captured in RAN3#125bis meeting minutes [5], Table 1 above was already proposed to be considered as the starting point for analysing the UE performance metrics in NR-DC.

An important issue to discuss is the granularity of the UE performance metrics that the SN needs to report. As mentioned above, basically almost all UE performance metrics are measured on a per-DRB per-UE granularity, apart from the Average UE throughput DL/UL which should already be measured per-UE (pending SA5 confirmation) . However, the UE performance metrics in clause 9.2.3.179 of TS 38.423 [7] are intended to be reported on a per-UE granularity. Thus, we suggest RAN3 further discussing this issue and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios.

Proposal 4: RAN3 to further discuss and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios (e.g., per-UE as in Rel-18 or a finer granularity).

As a starting point, we can consider UE performance to be reported on a per-UE granularity as done in Rel-18. This because the intended use of such UE performance should be to evaluate whether the AI/ML model decision to setup NR-DC with a certain SN for the UE was beneficial with respect to keeping the concerned UE in single connectivity. And single connectivity is addressed in Rel-18 where only per-UE UE performance are reported. To derive per-UE UE performance metrics starting from the corresponding per-DRB per-UE measurements the RAN3 common understanding from RAN3#124 meeting can be considered, that is, per-UE UE performance is derived as the average value of per-DRB measurements of the UE performance metrics [11]:

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

Proposal 5: RAN3 to agree that, as a starting point, UE performance is reported from the SN to the MN on a per-UE granularity.

## 2.3 Assistance information for SN configuration

In the last meeting the following FFSs have been captured in the meeting minutes [4], which are related to the specification of assistance information that could be sent over Xn from MN to SN for subsequent SN configuration:

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information? Other information, e.g., predicted SCG bearer?

There were some contributions submitted in the last meeting, e.g. [12][13], proposing to indicate the PSCell ID predicted (or assumed, as labelled in [13]) by the MN to help the SN in choosing and configuring the Secondary Cell Group (SCG). First of all, it should be recalled that RAN3 had a discussion on providing predicted information from the MN to the SN during the SI, and discussed in particular the predicted UE trajectory in the SN (across PSCells). Back then, there was no consensus in RAN3 on transferring such prediction over the Xn interface as captured in RAN3#125 meeting [14]:

There is no consensus to transfer the predicted/measured UE trajectory over NR-DC. Check online whether to support it during normative phase.

and in our view the predicted PSCell ID is a particular case where the predicted UE trajectory in the SN is only limited to a single PSCell. Moreover, in our understanding, in the legacy NR-DC the MN does not provide any information to the SN to help it in selecting the PSCell of the SCG, and we see no technical reasons to deviate from this behaviour, also considering that among proposing companies there is no clear view on whether the PSCell ID information to be transferred to the SN is either a prediction or an assumption, and what is the difference between the two.

Our understanding is that the predicted PSCell ID (as proposed in [12]) is an output of the AI/ML model used in the MN – which, by the way, has not been agreed to be included in TR 38.743 [3] in clause 5.1.2 for the potential standard impact – and hence this will require RAN3 to also discuss the feedback information which is needed to validate such prediction. Proponent of [13], instead, refers to the PSCell ID to be transferred to the SN as an assumption made by the MN. Still based on our understanding, this information should be derived according to actual PSCell serving the UE that contribution [13] proposes in Proposal 4 to be transferred to the MN within the existing Rel-18 UE Performance IE. Hence, if a UE was served well by an SN on a certain PSCell in the past, the AI/ML model would likely predict that specific SN as the best option for the UE. It is however not clear how the MN would use past information for the inference process in order to predict the best SN to setup NR-DC with. And RAN3 had no discussions about this aspect during the SI.

Proposal 6: RAN3 to agree to not transfer predicted/assumed PSCell ID from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

In a similar way, [13] also proposes to transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers. This is not the current NR-DC behaviour, where an SN may decide that all the QoS flows for which the MN requests SN-terminated bearers will be served via the Master Cell Group (MCG) as MCG bearers, and informs the MN about this decision – refer to QoS Flows Not Admitted List IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message in TS 38.423 [7]. Differently from the assumed PSCell ID (where the PSCell actually selected by the SN after the Addition procedure is not known), the MN may have historical information from the previous SN Addition procedures for the UE from which it may deduce whether a certain SN was a good choice or not by checking the above mentioned QoS Flows Not Admitted List IE. And this information may be used during the inference process in the MN to determine the best SN to add as an input information (if available) locally available. In this way the MN may already be able to predict the best SN to add by “simply” discarding those SNs which, in the past, had less chances to serve SN-terminated bearers via the SCG. Therefore, we think there is no need for the MN to send to the SN the list of QoS flows that are assumed to be served by the SN as SCG bearers.

Proposal 7: RAN3 to agree to not transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

## 2.4 Exit conditions to stop the collection of UE performance at SN side

Current TS 38.423 clause 8.4.13.2 captures, for the case of UE in single connectivity, the exit conditions to be considered by the target node in order to evaluate when to stop the collection of UE performance [7]:

### 8.4.13 Data Collection Reporting Initiation ### **** Skip non-relevant text **** #### 8.4.13.2 Successful Operation **** Skip non-relevant text **** If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell. The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message. **** Skip non-relevant text ****

As already specified in Rel-18 for the AI/ML mobility optimization, RAN3 should also discuss about the exit conditions to be evaluated by the SN in order to stop the collection of UE performance at the SN side. In light of what we proposed in Section 2.3, for the AI/ML-based NR-DC we think that at least the following conditions should be introduced so that the SN can understand when to stop the collection of the UE performance:

- the time since NR-DC was successfully setup for the UE is equal to the value configured to the SN as part of the preceding Data Collection Reporting Initiation procedure between MN and SN;

- the NR-DC for the concerned UE is released (in both cases of SN-initiated and MN-initiated);

- the SN Change procedure is performed.

Proposal 8: RAN3 to agree that exit conditions to be evaluated by the SN to stop the collection of UE performance at the SN side need to be specified.

Proposal 9: RAN3 to agree the following list of exit conditions to stop the collection of UE performance at the SN side:

- the time since NR-DC was successfully setup for the UE is equal to the value configured to the SN as part of the preceding Data Collection Reporting Initiation procedure between MN and SN;

- the NR-DC for the concerned UE is released (in both cases of SN-initiated and MN-initiated);

- the SN Change procedure is performed.

## 2.4 UE trajectory feedback in NR-DC

In the Rel-18 mobility optimization use case, it is the source gNB to predict the cell-based UE trajectory and, based on that information, to choose the next hop target gNB. And as in the current Rel-18 TS 38.423 [7], the source gNB can configure the target gNB to collect and report the measured UE trajectory via the Data Collection Reporting (Initiation) procedures, to help the source gNB evaluating the AI/ML-driven mobility decision. The source gNB configures the collection time duration and number of visited cells as triggers to stop the collection of measured UE trajectory information. And the target gNB will feedback the measured UE trajectory, including the list of cells that the UE visited and the corresponding time spent in each cell.

#### 9.2.3.182 Measured UE Trajectory The Measured UE Trajectory IE contains information related to NG-RAN cells where the UE connected after being handed over to the target NG-RAN node. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Measured UE Trajectory; 1; List of cells where the UE connected, in chronological order. Most recent information is added to the top of this list.. >Measured UE Trajectory Item; 1..<maxnoofCellsTrajectory>. >>Measured Trajectory Cell Information; M; 9.2.3.183. Range bound; Explanation. maxnoofCellsTrajectory; Maximum number of cells that can be reported for UE trajectory. Value is 16.. - 9.2.3.183 Measured Trajectory Cell Information The Measured Trajectory Cell Information contains information related to the NG-RAN cells where a UE connected after being handed over to the target NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Measured Trajectory Cell Information; M. >NG-RAN Cell. >>Global NG-RAN Cell Identity; M; 9.2.2.27; Indicates an NR Cell Identity.. >>Time UE Stayed in Cell; M; INTEGER (0..4095); The duration of time the UE stayed in the cell If the duration is more than 4095s, this IE is set to 4095.. **** Skip non-relevant text **** #### 9.2.3.185 UE Trajectory Collection Configuration This IE contains configurations for UE trajectory collection after successful handover. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Collection Time Duration for UE Trajectory; M; INTEGER (1..4096, …); Time duration starting at successful handover within which UE Trajectory Information is collected. Unit: second. Number of Visited Cells; O; INTEGER (1..16, …); Maximum number of intra-node visited cells.. As

In the NR-DC, the MN may predict the SN for DC, but the PSCell and corresponding change strategies are decided by the SN. This means that the MN is not aware of which PSCell the SN will choose for the UE as well as possible PSCell changes. With the measured UE trajectory among PSCells, the MN may evaluate whether the previous decision on the SN selection and addition via AI/ML was accurate or AI/ML model’s optimization is needed.

In our opinion, the current measured UE trajectory collection configuration related signalling can be the baseline for further enhancements aimed at including the configuration for reporting information about the measured UE trajectory in PSCells.

Proposal 10: RAN3 to agree that the current measured UE trajectory collection configuration related signalling is further enhanced to include the configuration for reporting information about UE trajectory in PSCells.

The TP for the XnAP BLCR for AI/ML for NG-RAN in the Annex reflects the above proposals.

# 3. Conclusion

Based on the above discussion, we have the following proposals:

On configuration of UE performance collection in NR-DC

Proposal 1: RAN3 to agree that the reporting of UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message) already in place between MN and SN. No specification impact is foreseen.

Proposal 2: RAN3 to agree that scenarios other than “basic” SN Addition and MN-initiated SN Change are deprioritized. Their actual feasibility could be discussed in Rel-19 only if time allows.

On UE performance metrics in NR-DC

Proposal 3: RAN3 to take Table 1 as the start point for analysing UE performance metrics in NR-DC so that SN-relevant UE performance metrics components can be collected by the SN and reported to the MN.

Proposal 4: RAN3 to further discuss and choose the granularity of the UE performance metrics to be reported in NR-DC scenarios (e.g., per-UE as in Rel-18 or a finer granularity).

Proposal 5: RAN3 to agree that, as a starting point, UE performance is reported from the SN to the MN on a per-UE granularity.

Proposal 6: RAN3 to agree to not transfer predicted/assumed PSCell ID from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

Proposal 7: RAN3 to agree to not transfer the list of QoS flows that are assumed to be served by the SN as SCG bearers from MN to SN as part of NR-DC procedures (SN Addition and MN-initiated SN change).

Proposal 8: RAN3 to agree that exit conditions to be evaluated by the SN to stop the collection of UE performance at the SN side need to be specified.

Proposal 9: RAN3 to agree the following list of exit conditions to stop the collection of UE performance at the SN side:

- the time since NR-DC was successfully setup for the UE is equal to the value configured to the SN as part of the preceding Data Collection Reporting Initiation procedure between MN and SN;

- the NR-DC for the concerned UE is released (in both cases of SN-initiated and MN-initiated);

- the SN Change procedure is performed.

Other aspects

Proposal 10: RAN3 to agree that the current measured UE trajectory collection configuration related signalling is further enhanced to include the configuration for reporting information about UE trajectory in PSCells.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-245608, Procedures for AI/ML Mobility Optimization in NR-DC, Nokia, Jio

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), V18.4.0 (2024-12)

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, V19.2.0 (2025-01)

- R3-247775, LS on per-UE UE performance metrics, Huawei

- 3GPP TS 38.314, NR; Layer 2 measurements, V18.0.0 (2024-04)

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)

- R3-247140, (TP to 38.423) Introduction of predicted PSCell for Mobility Optimization in NR-DC, ZTE, CATT, Samsung

- R3-247535, AI/ML support for NR-DC, Ericsson

- 3GPP RAN3#125 Chairman’s notes (RAN3_125_agenda_20240823EOM1.doc)

# 5. Annex – TP for BLCR to TS 38.423 for AI/ML NR-DC (based on R3-250042)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the slice-specific information reporting is requested, the Slice To Report for Data Collection List IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	the time since NR-DC was successfully setup with the NG-RAN node2 (acting as S-NG-RAN node) for the UE is equal to the value of the Collection Time Duration for UE Performance IE;

-	the NR-DC with the NG-RAN node2 (acting as S-NG-RAN node) for the UE is released.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Slice Available Capacity IE, if the tenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if

- the Slice To Report List for Data Collection IE is included for the cell, and

- the measurement object is admitted by NG-RAN node2.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>