3GPP TSG-RAN WG3 Meeting #127	R3-250376

Athens, Greece, 17th – 21st February, 2025

Title: 	(TP for AI/ML BLCR to TS 38.473 and TP for AI/ML BLCR to TS 37.483) Split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to split architecture support for Rel-18 use cases.

This contribution analyzes the technical enablers for specifying support of split architecture for Rel-18 use cases, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

## 2.1 On gNB-DU Energy Cost (EC) reporting over F1

In RAN3#125bis meeting, as part of the Rel-18 leftover related to split architecture support for Rel-18 use cases, RAN3 agreed to introduce the Data Collection Reporting Initiation and Data Collection Reporting procedures over F1, as done in Rel-18 for the Xn interface, to enable the reporting of the measured Energy Cost (EC) from the DU to the CU [4]:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In the last meeting, further agreements for enabling the reporting of the gNB-DU’s EC to the gNB-CU, and for the intended usage of the reported gNB-DU’s EC have been reached [5]:

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs. Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

Moreover, due to the support needed from the OAM side for configuring the unified “energy consumption – to – energy cost” mapping rule to the gNB-DU, RAN3 agreed to send a LS to SA5 in [6].

Similar to the procedure over Xn, the Data Collection Reporting Initiation procedure over F1 is used by the gNB-CU to request from the gNB-DU the reporting of information for, e.g., AI/ML in NG-RAN, by using non UE-associated signalling.

The gNB-CU sends the DATA COLLECTION REQUEST message to the gNB-DU to start (or stop) the information reporting and, following the reception of this message, the gNB-DU shall initiate (or stop) the requested information reporting. To enable this mechanism the Registration Request for Data Collection IE needs to be included in the DATA COLLECTION REQUEST message.

Proposal 1: RAN3 to agree that the DATA COLLECTION REQUEST message includes the Registration Request for Data Collection IE to start and stop the requested information reporting by the gNB-DU.

Proposal 2: The Registration Request for Data Collection IE is an extensible ENUMERATED type with “start” and “stop” codepoints.

To indicate the pieces of information that need to be reported by the gNB-DU to the gNB-CU, the Report Characteristics for Data Collection IE needs to be included in the DATA COLLECTION REQUEST message as well.

Proposal 3: RAN3 to agree that the DATA COLLECTION REQUEST message includes the Report Characteristics for Data Collection IE to indicate the information that the gNB-DU reports to the gNB-CU.

In the last meeting it was agreed that the gNB-DU reports to the gNB-CU its own measured EC, which is a node-level measurement. For the Data Collection Reporting Initiation procedure over Xn the partial reporting mechanism was specified. This allows the reporting node to indicate in the DATA COLLECTION RESPONSE message the list of measurements that the reporting node cannot initiate, along with an appropriate cause value. Considering that RAN3 will discuss the reporting of UE performance measurements over F1, and hence multiple measurements could be requested to be reported by the gNB-DU other than the measured EC, we think that partial reporting should also be supported for the Data Collection Reporting Initiation over F1 as well.

Proposal 4: RAN3 to agree that partial reporting is supported in the Data Collection Reporting Initiation procedure over F1.

If Proposal 4 above is agreed, we need to discuss the structure of the DATA COLLECTION RESPONSE message (still part of the Successful Operation). By recalling that the gNB-DU’s measured EC is a node-level measurement, and following the design of the DATA COLLECTION RESPONSE message over Xn, the DATA COLLECTION RESPONSE message sent by the gNB-DU to the gNB-CU should include the Node Measurement Initiation Result List IE indicating the list of measurements that failed to be initiated by the gNB-DU (with an appropriate cause value).

Proposal 5: RAN3 to agree to introduce the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

Proposal 6: RAN3 to agree to indicate the ‘Energy Cost’ as the first bit of the Node Measurement Failed Report Characteristics bitmap (within the Node Measurement Initiation Result List IE).

Still based on the design of the Data Collection Reporting Initiation procedure over Xn, the DATA COLLECTION REQUEST message allows to configure both one-time and periodic reporting of information, by means of the Reporting Periodicity for Data Collection IE. We think that similar design should also be followed for F1, to allow periodic reporting of the gNB-DU’s EC as well as of UE performance.

Proposal 7: RAN3 to agree to introduce the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message.

When it comes to the Data Collection Reporting procedure, instead, the DATA COLLECTION UPDATE message is used. In XnAP this message includes both cell-level and node-level measurements that could be reported based on the configuration in the associated DATA COLLECTION REQUEST message. For F1, and considering that RAN3 so far only agreed to report the gNB-DU’s measured EC (i.e., a node-level measurement), we think that at least the Node Associated Info Result IE should be included in the DATA COLLECTION UPDATE message over F1.

Proposal 8: RAN3 to agree to introduce the Node Associated Info Result IE in the DATA COLLECTION UPDATE message over F1.

Proposal 9: The Node Associated Info Result IE includes the measured Energy Cost of the gNB-DU.

It is well known that the Data Collection Reporting Initiation and Data Collection Reporting over Xn originate from the Resource Status Reporting Initiation and Resource Status Reporting procedures, respectively. For F1 it has been agreed in RAN3#125bis meeting to basically mirror the Data Collection Reporting Initiation and Data Collection Reporting procedures specified over Xn. However, by checking F1AP non UE-associated procedures, e.g., the Resource Status Reporting Initiation and Resource Status Reporting, it can be noticed that messages used for these procedures include a mandatory IE which is termed as Transaction ID. This IE is determined by the initiating peer of a procedure and uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer; moreover, messages belonging to the same procedure shall use the same Transaction ID IE. If the Transaction ID IE is missing from a certain non UE-associated procedure, this may cause problems in protocol error handling (refer to TS 38.473 clause 10). Based on that, we think that the Transaction ID IE should be introduced in messages used for Data Collection Reporting Initiation and Data Collection Reporting procedures over F1.

Proposal 10: RAN3 to agree to introduce the mandatory Transaction ID IE in DATA COLLECTION REQUEST, DATA COLLECTION RESPONSE, DATA COLLECTION FAILURE and DATA COLLECTION UPDATE messages.

The TP for the AI/ML BLCR to TS 38.473 in the Annex 1 reflects the above proposals.

## 2.2 On reporting measured UE performance

During the Rel-19 SI, for split architecture support, the use case description and potential standard impacts have been investigated and reported in TR 38.743 clause 5.2 as follows [3]:

## 5.2 Split architecture support for Rel-18 use cases ### 5.2.1 Use case description The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization. In case of CU-DU architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP); - AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP). ### 5.2.2 Potential Standard impacts The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18: - The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase. - Measured Energy Cost from gNB-DU to gNB-CU.

TR 38.743 captures that signalling details for transferring measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need to be discussed during the Rel-19 WI phase.

The scenario we propose to consider relates to UE performance metrics collection at the gNB-CU-CP of a split target gNB from the gNB-CU-UP(s) and/or gNB-DU(s) controlled by the concerned gNB-CU-CP – such measurement collection is triggered e.g. after an AI/ML-based mobility decision (i.e., handover) taken by the source gNB is successfully executed. This scenario is illustrated in Figure 1.

Figure 1 - Scenario for reporting UE performance from split target gNB to source gNB.

RAN3 introduced in Rel-18 the Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss as the per-UE metrics representing the UE performance to be reported from target gNB to source gNB over Xn. In case the target gNB is a split node, we think it is important that the target gNB-CU-CP is able to collect UE performance information, so that the target gNB-CU-CP can report the aggregated UE performance to the source gNB for, e.g., AI/ML model monitoring in the source node.

As per TS 28.558 [7], the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss are measured in the gNB-DU.

While, for the Average Packet Delay, TS 28.558 specified the delay components for both DL and UL – refer to Figure 2 and Figure 3, respectively – some of which are measured by the gNB-DU and others by the gNB-CU-UP.

Figure 2 - RAN part of DL packet delay measurement.

Figure 3 - RAN part (including UE) of UL packet delay measurement.

Table 1 – UE performance metrics, where they are measured.

|  | gNB-DU | gNB-CU-UP |
| --- | --- | --- |
| Average UE Throughput DL | X |  |
| Average UE Throughput UL | X |  |
| Average Packet Loss | X |  |
| DL packet delay components | D1, D2 | D3, D4 |
| UL packet delay components | D2.1, D2.2 | D2.3, D2.4 |

The above Table 1 shows where each of the UE performance metrics is measured, and it is clear that Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss should be reported to the gNB-CU-CP via F1 signalling.

- Proposal 11: RAN3 to agree on exchanging over F1 (gNB-DU to gNB-CU-CP) the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss.

The exchange of Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss can be performed via the new F1AP Data Collection Reporting Initiation and Data Collection Reporting procedures already agreed for the reporting of the measured gNB-DU’s EC.

For the Average Packet Delay, instead, since there are multiple components, some of them measured at gNB-DU level and others at gNB-CU-UP level, we think that the gNB-CU-CP could be made aware of the overall Average Packet Delay via one of the following options:

- Option 1: gNB-CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to gNB-CU-CP;

- Option 2: gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively; it is then up to gNB-CU-CP to aggregate the received measurements to derive the overall Average Packet Delay measurement;

- Option 3: gNB-DU sends its own measurements of the Average Packet Delay components to gNB-CU-UP over F1 and gNB-CU-UP forwards such measurements to gNB-CU-CP transparently over E1; with respect to Option 1, in this way it could still be possible for the gNB-CU-CP to have visibility of the different delay components in gNB-DU and gNB-CU-UP (same as in Option 2).

Proposal 12: RAN3 to decide which option to select for the Average Packet Delay measurement in split architecture; possible options are:

- Option 1: gNB-CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to gNB-CU-CP;

- Option 2: gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively, and gNB-CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement;

- Option 3: gNB-DU sends its own measurements of the Average Packet Delay components to gNB-CU-UP over F1 and gNB-CU-UP forwards such measurements to gNB-CU-CP transparently over E1.

To support the finer-grained measurement transfer, Option 2 and 3 can be adopted to inform the gNB-CU-CP about the specific component of the Average Packet Delay. Between them, Option 2 is more reasonable and efficient since the components are directly sent to gNB-CU-CP from the network entity where such components are measured.

Proposal 13: RAN3 to agree to select Option 2, i.e., gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively, and gNB-CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement.

If Option 2 is agreed, the gNB-DU-measured Average Packet Delay components can be sent directly from gNB-DU to gNB-CU-CP by means of the new F1AP Data Collection Reporting Initiation and Data Collection Reporting procedures agreed in the last RAN3 meeting. While for the gNB-CU-UP-measured Average Packet Delay components a new procedure for E1 – also mirroring the XnAP’s Data Collection Reporting (Initiation) procedures – can be introduced.

Proposal 14: RAN3 to agree to introduce new E1AP procedures similar to the XnAP’s Data Collection Reporting Initiation and Data Collection Reporting.

In the last RAN3 meetings there were a lot of discussions related to the granularity of UE performance. For Rel-18, the granularity has been specified to be per-UE but RAN3 agreed to send a LS to SA5 in [8] asking confirmation on whether the Average UE Throughput DL and Average UE Throughput UL are already defined with per-UE granularity in TS 28.558 [7]. Our understanding is that Average UE Throughput DL and Average UE Throughput UL are already defined in TS 28.558 with a per-UE granularity, while Average Packet Loss and the multiple components of the Average Packet Delay are defined in TS 28.558 with a per-DRB granularity and, to derive the corresponding per-UE UE performance metrics, the RAN3 common understanding from RAN3#124 meeting can be considered, that is, per-UE UE performance is derived as the average value of per-DRB measurements of the UE performance metrics [9]:

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

Still referring to Figure 1, the focus of the discussion is on collecting at gNB-CU-CP of the target split gNB the UE performance metrics components reported by target gNB-DU and target gNB-CU-UP. In this case we think that Average UE Throughput DL and Average UE Throughput UL can already be reported by the gNB-DU with a per-UE granularity, while we see feasible to report Average Packet Loss and the multiple components of the Average Packet Delay to the gNB-CU-CP on a per-DRB granularity as per current specification. For those measurements collected by the gNB-CU-CP of the target split gNB on a per-DRB granularity, i.e., Average Packet Loss and the multiple components of the Average Packet Delay, the gNB-CU-CP will derive the corresponding per-UE measurements following the RAN3 common understanding from RAN3#124 meeting and, together with the per-UE Average UE Throughput DL and Average UE Throughput UL already collected by the gNB-CU-CP as per-UE measurement, they will be reported over Xn to the source gNB.

Proposal 15: RAN3 to agree that UE performance metrics components that are defined per-DRB are reported by the gNB-DU and gNB-CU-UP to the gNB-CU-CP with per-DRB granularity. It will be up to the gNB-CU-CP to derive the corresponding per-UE measurements.

The TPs for the AI/ML BLCR to TS 38.473 and AI/ML BLCR to TS 37.843 in the Annex 1 and Annex 2, respectively, reflect the above proposals.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

On F1 signalling design for reporting measured EC from gNB-DU to gNB-CU

Proposal 1: RAN3 to agree that the DATA COLLECTION REQUEST message includes the Registration Request for Data Collection IE to start and stop the requested information reporting by the gNB-DU.

Proposal 2: The Registration Request for Data Collection IE is an extensible ENUMERATED type with “start” and “stop” codepoints.

Proposal 3: RAN3 to agree that the DATA COLLECTION REQUEST message includes the Report Characteristics for Data Collection IE to indicate the information that the gNB-DU reports to the gNB-CU.

Proposal 4: RAN3 to agree that partial reporting is supported in the Data Collection Reporting Initiation procedure over F1.

Proposal 5: RAN3 to agree to introduce the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

Proposal 6: RAN3 to agree to indicate the ‘Energy Cost’ as the first bit of the Node Measurement Failed Report Characteristics bitmap (within the Node Measurement Initiation Result List IE).

Proposal 7: RAN3 to agree to introduce the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message.

Proposal 8: RAN3 to agree to introduce the Node Associated Info Result IE in the DATA COLLECTION UPDATE message over F1.

Proposal 9: The Node Associated Info Result IE includes the measured Energy Cost of the gNB-DU.

Proposal 10: RAN3 to agree to introduce the mandatory Transaction ID IE in DATA COLLECTION REQUEST, DATA COLLECTION RESPONSE, DATA COLLECTION FAILURE and DATA COLLECTION UPDATE messages.

The TP for the AI/ML BLCR to TS 38.473 in the Annex 1 reflects the above proposals.

On reporting measured UE performance over F1 and E1

- Proposal 11: RAN3 to agree on exchanging over F1 (gNB-DU to gNB-CU-CP) the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss.

Proposal 12: RAN3 to decide which option to select for the Average Packet Delay measurement in split architecture; possible options are:

- Option 1: gNB-CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to gNB-CU-CP;

- Option 2: gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively, and gNB-CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement;

- Option 3: gNB-DU sends its own measurements of the Average Packet Delay components to gNB-CU-UP over F1 and gNB-CU-UP forwards such measurements to gNB-CU-CP transparently over E1.

Proposal 13: RAN3 to agree to select Option 2, i.e., gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively, and gNB-CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement.

Proposal 14: RAN3 to agree to introduce new E1AP procedures similar to the XnAP’s Data Collection Reporting Initiation and Data Collection Reporting.

Proposal 15: RAN3 to agree that UE performance metrics components that are defined per-DRB are reported by the gNB-DU and gNB-CU-UP to the gNB-CU-CP with per-DRB granularity. It will be up to the gNB-CU-CP to derive the corresponding per-UE measurements.

The TPs for the AI/ML BLCR to TS 38.473 and AI/ML BLCR to TS 37.483 in the Annex 1 and Annex 2, respectively, reflect the above proposals.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- R3-247907, LS on energy saving for split gNB, CATT

- 3GPP TS 28.558 “Management and orchestration; UE level measurements for 5G system”, Rel-19

- R3-247775, LS on per-UE UE performance metrics, Huawei

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)

# 5. Annex 1 - TP for AI/ML BLCR to TS 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| *** skip unmodified text *** |  |  |  |
| Multicast Common Configuration | MULTICAST COMMON CONFIGURATION REQUEST | MULTICAST COMMON CONFIGURATION RESPONSE | MULTICAST COMMON CONFIGURATION REFUSE |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| *** skip unmodified text *** |  |
| DU-CU Access And Mobility Indication | DU-CU ACCESS AND MOBILITY INDICATION |
| Data Collection Reporting | DATA COLLECTION UPDATE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.X	Data Collection Reporting procedures

### 8.X.1	Data Collection Reporting Initiation

#### 8.X.1.1	General

This procedure is used by the gNB-CU to request from the gNB-DU node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2-1: Data Collection Reporting Initiation, successful operation

The gNB-CU initiates the procedure by sending the DATA COLLECTION REQUEST message to the gNB-DU to start information reporting or to stop information reporting. Upon receipt, the gNB-DU:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the gNB-DU is capable of providing all of the requested information, it shall initiate the information reporting as requested by the gNB-CU and respond with the DATA COLLECTION RESPONSE message.

If the gNB-DU is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The gNB-DU shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the gNB-DU shall take it into account for the configuration of UE performance collection and reporting. The gNB-DU shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects the gNB-DU performs measurements on. The gNB-DU shall include in the DATA COLLECTION UPDATE message:

-	the Energy Cost IE, if the first bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average UE Throughput DL IE, if the second bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average UE Throughput UL IE, if the third bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average Packet Loss DL IE, if the fourth bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average Delay DL Air Interface IE, if the fifth bit, "Average Delay DL Air Interface" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average Delay DL in gNB-DU IE, if the sixth bit, "Average Delay DL in gNB-DU" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average Delay UL Air Interface IE, if the seventh bit, "Average Delay UL Air Interface" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

-	the Average RLC Packet Delay UL IE, if the eighth bit, "Average RLC Packet Delay UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

#### 8.X.1.3	Unsuccessful Operation

Figure 8.X.1.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, the gNB-DU shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.X.1.4	Abnormal Conditions

If the initiating gNB-CU does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message with the same Transaction ID.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the gNB-DU Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-DU shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then the gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going Data Collection reporting, for which a different Transaction ID is used, then the gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.X.2	Data Collection Reporting

#### 8.X.2.1	General

This procedure is initiated by the gNB-DU to report information accepted by the gNB-DU following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Data Collection Reporting, successful operation

The gNB-DU shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.X.2.3	Unsuccessful Operation

Not applicable.

#### 8.X.2.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.X	Data Collection Reporting Messages

#### 9.2.X.1	DATA COLLECTION REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-DU is requested to report. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Loss DL, Fifth Bit = Average Delay DL Air Interface, Sixth Bit = Average Delay DL in gNB-DU, Seventh Bit = Average Delay UL Air Interface, Eighth Bit = Average RLC Packet Delay UL. Other bits are ignored by the gNB-DU. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.3.1.a |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

#### 9.2.X.2	DATA COLLECTION RESPONSE

This message is sent by gNB-DU to gNB-CU to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the gNB-DU. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Loss, Fifth Bit = Average Delay DL Air Interface, Sixth Bit = Average Delay DL In gNB-DU, Seventh Bit = Average Delay UL Air Interface, Eighth Bit = Average RLC Packet Delay UL. Other bits are ignored by the gNB-CU. | – |  |
| >>Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.2.X.3	DATA COLLECTION FAILURE

This message is sent by gNB-DU to the gNB-CU to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.X.4	DATA COLLECTION UPDATE

This message is sent by gNB-DU to the gNB-CU to report the requested information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | gNB-CU UE F1AP ID 9.3.1.4 | gNB-CU UE F1AP ID allocated by gNB-CU. | – |  |
| >>UE Performance | O |  | 9.3.1.b |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.a	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.b	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.3.1.22 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.3.1.22 |  |
| DRB Measurement Result List |  | 0..1 |  | Per-DRB UE performance metrics. |
| >DRB Measurement Result Item |  | 1 .. <maxnoofDRBs> |  |  |
| >>DRB ID | M |  | 9.3.1.8 |  |
| >>Average Packet Loss DL | O |  | Maximum Packet Loss Rate 9.3.1.50 |  |
| >>Average Delay DL Air Interface | O |  | Average Packet Delay 9.3.1.c |  |
| >>Average Delay DL in gNB-DU | O |  | Average Packet Delay 9.3.1.c |  |
| >>Average Delay UL Air Interface | O |  | Average Packet Delay 9.3.1.c |  |
| >>Average RLC Packet Delay UL | O |  | Average Packet Delay 9.3.1.c |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.c	Average Packet Delay

This IE indicates a component of the RAN average packet delay in the UL and DL directions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay Value | M |  | INTEGER (0.. 10000, …) | Defined in TS 38.314 [32]. Unit: 0.1 millisecond |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

*** skip unmodified text ***

SRSInformationReservationNotification,

DataCollectionRequest,

DataCollectionResponse,

DataCollectionFailure,

DataCollectionUpdate

FROM F1AP-PDU-Contents

id-Reset,

*** skip unmodified text ***

id-SRSInformationReservationNotification

id-dataCollectionReportingInitiation

id-dataCollectionReporting

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURES F1AP-ELEMENTARY-PROCEDURE ::= {

F1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

F1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-1 F1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

*** skip unmodified text ***

multicastCommonConfiguration	|

dataCollectionReportingInitiation,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

*** skip unmodified text ***

sRSInformationReservationNotification	|

dataCollectionReporting,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

reset F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-Reset

CRITICALITY				reject

}

*** skip unmodified text ***

sRSInformationReservationNotification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SRSInformationReservationNotification

PROCEDURE CODE			id-SRSInformationReservationNotification

CRITICALITY				reject

}

dataCollectionReportingInitiation	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				DataCollectionRequest

SUCCESSFUL OUTCOME				DataCollectionResponse

UNSUCCESSFUL OUTCOME			DataCollectionFailure

PROCEDURE CODE					id-dataCollectionReportingInitiation

CRITICALITY						reject

}

dataCollectionReporting	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataCollectionUpdate

PROCEDURE CODE			id-dataCollectionReporting

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

*** skip unmodified text ***

LTMResetInformation,

RegistrationRequestForDataCollection,

ReportCharacteristicsForDataCollection,

ReportingPeriodicityForDataCollection,

UEPerformanceCollectionConfiguration,

NodeMeasurementInitiationResult-List,

NodeAssociatedInfoResult,

UEAssociatedInfoResult-List

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-AssociatedSessionID,

*** skip unmodified text ***

id-PreconfiguredSRSInformation,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-UEPerformanceCollectionConfiguration,

id-NodeMeasurementInitiationResult-List,

id-NodeAssociatedInfoResult,

id-UEAssociatedInfoResult-List,

maxCellingNBDU,

*** skip unmodified text ***

maxnoofServingCellMOs,

maxFailedMeasPerNode,

maxnoofUEReports

FROM F1AP-Constants;

-- **************************************************************

--

#### -- RESET ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

*** skip unmodified text ***

DUCUAccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-DLLBTFailureInformationList				CRITICALITY ignore	TYPE DLLBTFailureInformationList	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION REQUEST

--

-- **************************************************************

DataCollectionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionRequest-IEs}},

...

}

DataCollectionRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID										PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID							CRITICALITY reject	TYPE GNBCUMeasurementID								PRESENCE mandatory}|

{ ID id-gNBDUMeasurementID							CRITICALITY ignore	TYPE GNBDUMeasurementID								PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "stop".

{ ID id-RegistrationRequestForDataCollection		CRITICALITY reject	TYPE RegistrationRequestForDataCollection		PRESENCE mandatory}|

{ ID id-ReportCharacteristicsForDataCollection		CRITICALITY reject	TYPE ReportCharacteristicsForDataCollection	PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "start".

{ ID id-ReportingPeriodicityForDataCollection		CRITICALITY ignore	TYPE ReportingPeriodicityForDataCollection		PRESENCE optional}|

{ ID id-UEPerformanceCollectionConfiguration		CRITICALITY ignore	TYPE UEPerformanceCollectionConfiguration		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION RESPONSE

--

-- **************************************************************

DataCollectionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionResponse-IEs}},

...

}

DataCollectionResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID										PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID							CRITICALITY reject	TYPE GNBCUMeasurementID								PRESENCE mandatory}|

{ ID id-gNBDUMeasurementID							CRITICALITY ignore	TYPE GNBDUMeasurementID								PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "stop".

{ ID id-NodeMeasurementInitiationResult-List		CRITICALITY reject	TYPE NodeMeasurementInitiationResult-List		PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION FAILURE

--

-- **************************************************************

DataCollectionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionFailure-IEs}},

...

}

DataCollectionFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID						CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID						CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID							CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID							CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-NodeAssociatedInfoResult					CRITICALITY ignore	TYPE NodeAssociatedInfoResult								PRESENCE optional }|

{ ID id-UEAssociatedInfoResult-List					CRITICALITY ignore	TYPE UEAssociatedInfoResult-List							PRESENCE optional },

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

*** skip unmodified text ***

#### -- A

AbortTransmission ::= CHOICE {

sRSResourceSetID		SRSResourceSetID,

releaseALL				NULL,

choice-extension		ProtocolIE-SingleContainer { { AbortTransmission-ExtIEs } }

}

*** skip unmodified text ***

ARPLocationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

AveragePacketDelayValue ::= INTEGER (0..10000,...)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- D

DAPS-HO-Status::= ENUMERATED{initiation,... }

*** skip unmodified text ***

DLLBTFailureInformationList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBMeasurementResult-List ::= SEQUENCE (SIZE(1..maxnoofDRBs) OF DRBMeasurementResult-Item

DRBMeasurementResult-Item ::= SEQUENCE {

dRBID										DRBID,

uE-AveragePacketLossDL						MaxPacketLossRate		OPTIONAL,

uE-AveragePacketDelayDLAirInterface			AveragePacketDelayValue		OPTIONAL,

uE-AveragePacketDelayDLInGNB-DU				AveragePacketDelayValue		OPTIONAL,

uE-AveragePacketDelayULAirInterface			AveragePacketDelayValue		OPTIONAL,

uE-AverageRLCPacketDelayUL					AveragePacketDelayValue		OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRBMeasurementResult-Item-ExtIEs} } OPTIONAL,

...

}

DRBMeasurementResult-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- N

NRA2XServicesAuthorized ::= SEQUENCE {

aerialUE			AerialUE														OPTIONAL,

controllerUE		ControllerUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NRA2XServicesAuthorized-ExtIEs} }	OPTIONAL

}

*** skip unmodified text ***

N6JitterInformationExtIEs   F1AP-PROTOCOL-EXTENSION ::= {

...

}

NodeMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxFailedMeasPerNode)) OF NodeMeasurementInitiationResult-Item

NodeMeasurementInitiationResult-Item ::= SEQUENCE {

NodeMeasurementFailedReportCharacteristics		BIT STRING(SIZE(32)),

cause											Cause,

iE-Extensions									ProtocolExtensionContainer { { NodeMeasurementInitiationResult-Item-ExtIEs} } OPTIONAL,

...

}

NodeMeasurementInitiationResult-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NodeAssociatedInfoResult ::= SEQUENCE {

energyCost				EnergyCost			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { NodeAssociatedInfoResult-ExtIEs} } OPTIONAL,

...

}

NodeAssociatedInfoResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- R

RACH-Config-Common	::= OCTET STRING

*** skip unmodified text ***

ReportingIntervalIMs  ::= INTEGER (1.. 999)

RegistrationRequestForDataCollection ::= ENUMERATED {start, stop, ... }

ReportCharacteristicsForDataCollection ::= BIT STRING(SIZE(32))

ReportingPeriodicityForDataCollection ::= ENUMERATED {

half-thousand-ms,

one-thousand-ms,

two-thousand-ms,

five-thousand-ms,

ten-thousand-ms,

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- U

UAC-Assistance-Info ::= SEQUENCE {

uACPLMN-List		UACPLMN-List,

iE-Extensions		ProtocolExtensionContainer { { UAC-Assistance-InfoExtIEs} } OPTIONAL

}

*** skip unmodified text ***

UuRLCChannelRequiredToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UEPerformanceCollectionConfiguration ::= SEQUENCE {

collectionTimeDurationForUEPerformance				INTEGER(1..5000, ...),

iE-Extensions								ProtocolExtensionContainer { { UEPerformanceCollectionConfiguration-ExtIEs} } OPTIONAL,

...

}

UEPerformanceCollectionConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						GNB-CU-UE-F1AP-ID,

uEPerformance								UEPerformance			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UEPerformance ::= SEQUENCE {

dL-UE-AverageThroughput						BitRate					OPTIONAL,

uL-UE-AverageThroughput						BitRate					OPTIONAL,

dRBMeasurementResult-List					DRBMeasurementResult-List		OPTIONAL,

...

}

UEPerformance-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

*** skip unmodified text ***

id-SRSInformationReservationNotification	ProcedureCode ::= 96

id-dataCollectionReportingInitiation		ProcedureCode ::= aa

id-dataCollectionReporting					ProcedureCode ::= bb

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

*** skip unmodified text ***

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

maxFailedMeasPerNode					INTEGER ::= 124

maxnoofUEReports						INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

*** skip unmodified text ***

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-RegistrationRequestForDataCollection				ProtocolIE-ID ::= aaa

id-ReportCharacteristicsForDataCollection			ProtocolIE-ID ::= bbb

id-ReportingPeriodicityForDataCollection			ProtocolIE-ID ::= ccc

id-UEPerformanceCollectionConfiguration				ProtocolIE-ID ::= ddd

id-NodeMeasurementInitiationResult-List				ProtocolIE-ID ::= eee

id-NodeAssociatedInfoResult							ProtocolIE-ID ::= fff

id-UEAssociatedInfoResult-List						ProtocolIE-ID ::= ggg

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 6. Annex 2 - TP for AI/ML BLCR to TS 37.483

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

*** skip unmodified text ***

[36]	3GPP TS 23.527: "5G System; Restoration procedures”.

[XX]	3GPP TS 38.314: “NR; Layer 2 measurements”.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

# 8	E1AP procedures

NOTE:	The procedures listed in this section should also be applied to CP/UP separation for eNB and ng-eNB, except for the IAB UP TNL Address Update procedure, if not stated otherwise. With this understanding, in this section each instance of gNB-CU-CP could be treated as eNB-CP or ng-eNB-CU-CP, and each gNB-CU-UP could be treated as eNB-UP or ng-eNB-CU-UP, for eNB or ng-eNB CP/UP separation respectively.

## 8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| *** skip unmodified text *** |  |  |  |
| MC Bearer Context Release (gNB-CU-CP initiated) | MC BEARER CONTEXT RELEASE COMMAND | MC BEARER CONTEXT RELEASE COMPLETE |  |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| *** skip unmodified text *** |  |
| MC Bearer Notification | MC BEARER NOTIFICATION |
| Data Collection Reporting | DATA COLLECTION UPDATE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.X	Data Collection Reporting procedures

### 8.X.1	Data Collection Reporting Initiation

#### 8.X.1.1	General

This procedure is used by the gNB-CU-CP to request from the gNB-CU-UP the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2-1: Data Collection Reporting Initiation, successful operation

The gNB-CU-CP initiates the procedure by sending the DATA COLLECTION REQUEST message to the gNB-CU-UP to start information reporting or to stop information reporting. Upon receipt, the gNB-CU-UP:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the gNB-CU-UP is capable of providing all of the requested information, it shall initiate the information reporting as requested by the gNB-CU-CP and respond with the DATA COLLECTION RESPONSE message.

If the gNB-CU-UP is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the gNB-CU-UP shall take it into account for the configuration of UE performance collection and reporting. The gNB-CU-UP shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects the gNB-CU-UP performs measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average Delay DL on F1-U IE, if the first bit, "Average Delay DL on F1-U" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-CU-UP.

-	the Average Delay DL in CU-UP IE, if the second bit, "Average Delay DL in CU-UP" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-CU-UP.

-	the Average Delay UL on F1-U IE, if the third bit, "Average Delay UL on F1-U" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-CU-UP.

-	the Average PDCP Re-ordering Delay UL IE, if the fourth bit, "Average PDCP Re-ordering Delay UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-CU-UP.

#### 8.X.1.3	Unsuccessful Operation

Figure 8.X.1.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, the gNB-CU-UP shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.X.1.4	Abnormal Conditions

If the initiating gNB-CU-CP does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU-CP may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-CU-UP, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message with the same Transaction ID.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the gNB-CU-UP Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-CU-UP shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then the gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going Data Collection reporting, for which a different Transaction ID is used, then the gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.X.2	Data Collection Reporting

#### 8.X.2.1	General

This procedure is initiated by the gNB-CU-UP to report information accepted by the gNB-CU-UP following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Data Collection Reporting, successful operation

The gNB-CU-UP shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.X.2.3	Unsuccessful Operation

Not applicable.

#### 8.X.2.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.X	Data Collection Reporting Messages

#### 9.2.X.1	DATA COLLECTION REQUEST

This message is sent by gNB-CU-CP to gNB-CU-UP to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average Delay DL on F1-U, Second Bit = Average Delay DL in CU-UP, Third Bit = Average Delay UL on F1-U, Fourth Bit = Average PDCP Re-ordering Delay UL. Other bits are ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.3.1.a |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

#### 9.2.X.2	DATA COLLECTION RESPONSE

This message is sent by gNB-CU-UP to gNB-CU-CP to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the gNB-CU-UP. First Bit = Average Delay DL on F1-U, Second Bit = Average Delay DL in CU-UP, Third Bit = Average Delay UL on F1-U, Fourth Bit = Average PDCP Re-ordering Delay UL. Other bits are ignored by the gNB-CU-CP. | – |  |
| >>Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.2.X.3	DATA COLLECTION FAILURE

This message is sent by gNB-CU-UP to the gNB-CU-CP to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU-UP | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.X.4	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to the gNB-CU-CP to report the requested information.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | gNB-CU-CP UE E1AP ID 9.3.1.4 | gNB-CU-CP UE E1AP ID allocated by gNB-CU-CP. | – |  |
| >>UE Performance | O |  | 9.3.1.b |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.a	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.b	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| DRB Measurement Result List |  | 0..1 |  | Per-DRB UE performance metrics. |
| >DRB Measurement Result Item |  | 1 .. <maxnoofDRBs> |  |  |
| >>DRB ID | M |  | 9.3.1.16 |  |
| >>Average Delay DL on F1-U | O |  | Average Packet Delay 9.3.1.c |  |
| >> Average Delay DL in CU-UP | O |  | Average Packet Delay 9.3.1.c |  |
| >> Average Delay UL on F1-U | O |  | Average Packet Delay 9.3.1.c |  |
| >> Average PDCP Re-ordering Delay UL | O |  | Average Packet Delay 9.3.1.c |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.c	Average Packet Delay

This IE indicates a component of the RAN average packet delay in the UL and DL directions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay Value | M |  | INTEGER (0.. 10000, …) | Defined in TS 38.314 [XX]. Unit: 0.1 millisecond |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

E1AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM E1AP-CommonDataTypes

Reset,

*** skip unmodified text ***

IABPSKNotification,

DataCollectionRequest,

DataCollectionResponse,

DataCollctionFailure,

DataCollectionUpdate

FROM E1AP-PDU-Contents

id-reset,

*** skip unmodified text ***

id-MCBearerContextReleaseRequest

id-dataCollectionReportingInitiation

id-dataCollectionReporting

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURES E1AP-ELEMENTARY-PROCEDURE ::= {

E1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

E1AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-1 E1AP-ELEMENTARY-PROCEDURE ::= {

reset										|

*** skip unmodified text ***

mCBearerContextRelease 						|

dataCollectionReportingInitiation			,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-2 E1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

*** skip unmodified text ***

mCBearerNotification					|

dataCollectionReporting					,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

reset E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

*** skip unmodified text ***

mCBearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerContextReleaseRequest

PROCEDURE CODE			id-MCBearerContextReleaseRequest

CRITICALITY				reject

}

dataCollectionReportingInitiation	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				DataCollectionRequest

SUCCESSFUL OUTCOME				DataCollectionResponse

UNSUCCESSFUL OUTCOME			DataCollectionFailure

PROCEDURE CODE					id-dataCollectionReportingInitiation

CRITICALITY						reject

}

dataCollectionReporting	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataCollectionUpdate

PROCEDURE CODE			id-dataCollectionReporting

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

*** skip unmodified text ***

SDT-data-size-threshold-Crossed,

RegistrationRequestForDataCollection,

ReportCharacteristicsForDataCollection,

ReportingPeriodicityForDataCollection,

UEPerformanceCollectionConfiguration,

NodeMeasurementInitiationResult-List,

UEAssociatedInfoResult-List

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-AssociatedSessionID,

*** skip unmodified text ***

id-SDT-data-size-threshold-Crossed,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-UEPerformanceCollectionConfiguration,

id-NodeMeasurementInitiationResult-List,

id-UEAssociatedInfoResult-List,

maxnoofErrors,

*** skip unmodified text ***

maxnoofPSKs,

maxFailedMeasPerNode,

maxnoofUEReports

FROM E1AP-Constants;

-- **************************************************************

--

#### -- RESET

--

-- **************************************************************

-- **************************************************************

--

-- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

*** skip unmodified text ***

MCBearerNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-MBSSessionResourceNotification	CRITICALITY ignore	TYPE	MBSSessionResourceNotification	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- DATA COLLECTION REQUEST

--

-- **************************************************************

DataCollectionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionRequest-IEs}},

...

}

DataCollectionRequest-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID										PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Measurement-ID					CRITICALITY reject	TYPE INTEGER (1..4095, ...)							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID					CRITICALITY ignore	TYPE INTEGER (1..4095, ...)							PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "stop".

{ ID id-RegistrationRequestForDataCollection		CRITICALITY reject	TYPE RegistrationRequestForDataCollection		PRESENCE mandatory}|

{ ID id-ReportCharacteristicsForDataCollection		CRITICALITY reject	TYPE ReportCharacteristicsForDataCollection	PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "start".

{ ID id-ReportingPeriodicityForDataCollection		CRITICALITY ignore	TYPE ReportingPeriodicityForDataCollection		PRESENCE optional}|

{ ID id-UEPerformanceCollectionConfiguration		CRITICALITY ignore	TYPE UEPerformanceCollectionConfiguration		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION RESPONSE

--

-- **************************************************************

DataCollectionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionResponse-IEs}},

...

}

DataCollectionResponse-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID										PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Measurement-ID					CRITICALITY reject	TYPE INTEGER (1..4095,...)							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID					CRITICALITY ignore	TYPE INTEGER (1..4095,...)							PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "stop".

{ ID id-NodeMeasurementInitiationResult-List		CRITICALITY reject	TYPE NodeMeasurementInitiationResult-List		PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION FAILURE

--

-- **************************************************************

DataCollectionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionFailure-IEs}},

...

}

DataCollectionFailure-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Measurement-ID				CRITICALITY reject	TYPE INTEGER (1..4095,...)	PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Measurement-ID				CRITICALITY ignore	TYPE INTEGER (1..4095,...)	PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-CPMeasurement-ID						CRITICALITY reject	TYPE INTEGER (1..4095,...)			PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID							CRITICALITY ignore	TYPE INTEGER (1..4095,...)			PRESENCE mandatory	}|

{ ID id-UEAssociatedInfoResult-List					CRITICALITY ignore	TYPE UEAssociatedInfoResult-List	PRESENCE optional }|,

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

*** skip unmodified text ***

#### -- A

ActivityInformation	::=	CHOICE	{

dRB-Activity-List						DRB-Activity-List,

pDU-Session-Resource-Activity-List		PDU-Session-Resource-Activity-List,

uE-Activity								UE-Activity,

choice-extension						ProtocolIE-SingleContainer	{{ActivityInformation-ExtIEs}}

}

*** skip unmodified text ***

AssociatedSessionID ::= OCTET STRING

AveragePacketDelayValue ::= INTEGER (0..10000,...)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- D

DAPSRequestInfo ::= SEQUENCE {

dapsIndicator				ENUMERATED {daps-HO-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,

...

}

*** skip unmodified text ***

DataDiscardRequired	::= 	ENUMERATED {

required,

...

}

DRBMeasurementResult-List ::= SEQUENCE (SIZE(1..maxnoofDRBs) OF DRBMeasurementResult-Item

DRBMeasurementResult-Item ::= SEQUENCE {

dRBID										DRBID,

uE-AveragePacketDelayDLonF1U				AveragePacketDelayValue		OPTIONAL,

uE-AveragePacketDelayDLInGNB-CU-UP			AveragePacketDelayValue		OPTIONAL,

uE-AveragePacketDelayULonF1U				AveragePacketDelayValue		OPTIONAL,

uE-AveragePDCPReOrderingDelayUL				AveragePacketDelayValue		OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRBMeasurementResult-Item-ExtIEs} } OPTIONAL,

...

}

DRBMeasurementResult-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- N

NetworkInstance ::= INTEGER (1..256, ...)

*** skip unmodified text ***

N6JitterInformationExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

NodeMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxFailedMeasPerNode)) OF NodeMeasurementInitiationResult-Item

NodeMeasurementInitiationResult-Item ::= SEQUENCE {

NodeMeasurementFailedReportCharacteristics		BIT STRING(SIZE(32)),

cause											Cause,

iE-Extensions									ProtocolExtensionContainer { { NodeMeasurementInitiationResult-Item-ExtIEs} } OPTIONAL,

...

}

NodeMeasurementInitiationResult-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- R

RANUEID ::= OCTET STRING (SIZE (8))

*** skip unmodified text ***

F1UTunnelNotEstablished::= ENUMERATED {

true,

...

}

RegistrationRequestForDataCollection ::= ENUMERATED {start, stop, ... }

ReportCharacteristicsForDataCollection ::= BIT STRING(SIZE(32))

ReportingPeriodicityForDataCollection ::= ENUMERATED {

half-thousand-ms,

one-thousand-ms,

two-thousand-ms,

five-thousand-ms,

ten-thousand-ms,

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- U

UDC-Parameters	::= SEQUENCE {

bufferSize					BufferSize,

dictionary					Dictionary											OPTIONAL,

continueUDC					ENUMERATED {true, ...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UDC-Parameters-ExtIEs } } 		OPTIONAL

}

*** skip unmodified text ***

UserPlaneFailureType ::= ENUMERATED {

gtp-u-error-indication-received,

up-path-failure,

...

}

UEPerformanceCollectionConfiguration ::= SEQUENCE {

collectionTimeDurationForUEPerformance				INTEGER(1..5000, ...),

iE-Extensions								ProtocolExtensionContainer { { UEPerformanceCollectionConfiguration-ExtIEs} } OPTIONAL,

...

}

UEPerformanceCollectionConfiguration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						GNB-CU-CP-UE-E1AP-ID,

uEPerformance								UEPerformance			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UEPerformance ::= SEQUENCE {

dRBMeasurementResult-List					DRBMeasurementResult-List		OPTIONAL,

...

}

UEPerformance-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

*** skip unmodified text ***

id-MCBearerNotification											ProcedureCode ::= 39

id-dataCollectionReportingInitiation							ProcedureCode ::= aa

id-dataCollectionReporting										ProcedureCode ::= bb

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

*** skip unmodified text ***

maxnoofDUs									INTEGER ::= 512

maxnoofUEReports							INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

*** skip unmodified text ***

id-UserPlaneFailureIndication									ProtocolIE-ID ::= 219

id-RegistrationRequestForDataCollection							ProtocolIE-ID ::= aaa

id-ReportCharacteristicsForDataCollection						ProtocolIE-ID ::= bbb

id-ReportingPeriodicityForDataCollection						ProtocolIE-ID ::= ccc

id-UEPerformanceCollectionConfiguration							ProtocolIE-ID ::= ddd

id-NodeMeasurementInitiationResult-List							ProtocolIE-ID ::= eee

id-UEAssociatedInfoResult-List									ProtocolIE-ID ::= fff

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>