3GPP TSG-RAN WG3 Meeting #127	R3-250377

Athens, Greece, 17th – 21st February, 2025

Title: 	([Draft LS to SA5]) On Continuous MDT for AI/ML

Source: 	Huawei, FiberCop

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to Continuous MDT collection targeting the same UE across RRC states.

This contribution discusses the Continuous MDT feature based on the outcome of the last RAN3 meeting.

# 2. Background

The main motivation for introducing Continuous MDT arose during the Rel-18 WI “AI/ML for NG-RAN” [4], where the issue on how to collect measurements from UE(s) to be used as datasets for AI/ML models’ (re-)training was initially discussed. Back then it was argued that, for the AI/ML model (re-)training to be successful, the MDT data collected from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. It was claimed that, when the UE enters to connected state from idle/inactive or when the UE is subject to connected mode mobility (handover), consecutive (without time gaps) information would not be collected, with the consequence that the data gathered from the UE would not constitute a valid training dataset for AI/ML models’ (re-)training. By means of the Continuous MDT, that is, an enhancement of the current MDT mechanism suited for AI/ML models’ (re-)training, the continuous collection of MDT data from the same UE during the same RRC state as well as across RRC states can be enabled.

During the Rel-19 SI, for Continuous MDT, the use case description and problems to be solved have been acknowledged and captured in TR 38.743 clause 5.4 as follows [3]:

## 5.4 Continuous MDT collection targeting the same UE across RRC states ### 5.4.1 Use Case Description The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection. The Data Collection continuity in this scenario can be split into two tasks as below: - Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states. - Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE. ### 5.4.2 Potential Standard impacts Potential solutions addressing the problems above can be discussed during normative phase.

In the last meeting RAN3 made an initial attempt to down-select solutions for Continuous MDT and, based on Operators’ input in [5], RAN3 agreed that a solution for Continuous MDT needs to be applicable not only to Rel-19 UEs but also to legacy UEs [6], meaning that UE-based solutions will not be considered in future RAN3 discussions. Moreover, RAN3 agreed to send a LS to SA5 aiming at explaining which is the problem to be solved by this feature, which are the solutions’ requirements and a description of the solutions themselves in order for RAN3 to then select a solution also based on SA5 feedback:

In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs. Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS. Identification of requirements and description of problem statement, followed by LS drafting to be continued.

Asking SA5 feedback on the solutions is in our view needed due to the following agreement from RAN3#125bis [7]:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

The above agreement reflects the fact that data collection serves the purpose of providing sufficient amount of data as input for AI/ML model (re-)training, meaning that data needs to be collected from as many UEs as possible, under different radio environments and RRC states. Fulfilling such requirements for proper data collection from UEs can be possible via management-based MDT. Not to mention that TR 38.743, clause 5.4.1, explicitly refers to management-based MDT for the Continuous MDT problem statement [3].

In the next Section 3 we provide a summary of the solutions brought up by companies in RAN3, and a draft LS to SA5 is also provided in the Annex of this paper.

# 3. Discussion

As previously recalled, for the problems to be solved by Continuous MDT, i.e., measurement continuity and trace correlation, the RAN3 TR 38.743 can be referenced. While, for solution’s requirements, the content of the SoD in [8] from the last meeting can be considered, which is based on the agreements that RAN3 achieved so far:

The agreement above implies that solutions for continuous MDT should: - Reuse the Management Based MDT framework - Rely on the OAM to configure the RAN with a Management Based MDT configuration to be used for Continuous MDT - Be network based and avoid impacts on legacy UEs.

In the following Table 1 a summary of the solutions is provided:

Table 1 - Network-based solutions for Continuous MDT.

| Solution ID | Description | Measurement Continuity | Trace correlation | Pros. & Cons. |
| --- | --- | --- | --- | --- |
| Solution 1 (R3-247537) | - OAM configures Management Based MDT to the gNBs taking part in Continuous MDT, including a TRs dedicated to Continuous MDT; - Serving gNB selects a UE and configures TRSR which is reassigned to the UE across RRC states and during connected mobility. Involved domains: RAN and OAM | - explicit indication in handover signalling that UE is selected for Continuous MDT (e.g., assigned TR/TRSR); - presence of TR/TRSR in the Logged MDT report sent to the serving gNB after idle-to-connected transition. | Reusing part/all of the identifiers in the Trace Record header, e.g. the TRSR, for the whole duration of the Continuous MDT trace. | Pros: only RAN and OAM involved. Cons: - Dedicated TRs for Continuous MDT; - gNB coordination in TRSR assignment to enable trace correlation. |
| Solution 2 (R3-247623) | - OAM configures Management Based MDT at a gNB; - At connected-to-idle transition the MDT configuration is stored by the serving gNB in 5GC/AMF; - Upon idle-to-connected transition the 5GC/AMF indicates to the (new) serving gNB the MDT configuration of the UE along with an identifier that the UE needs to be selected again for management-based immediate MDT; - During handover, the source gNB sends the Immediate MDT configuration to the target gNB as part of the handover preparation. Involved domains: RAN, OAM and 5GC/AMF | - target gNB receives at handover the UE’s MDT configuration used in the source gNB; - (new) serving gNB receives the UE’s MDT configuration previously stored in the 5GC/AMF. | Ensured by the identifier assigned and maintained by the 5GC/AMF; the identifier is assigned to the UE as part of the MDT configuration, and the serving gNB includes this identifier in the trace files sent to the TCE. | Pros: no need for gNB coordination in TRSR assignment. Cons: - AMF is impacted as it needs to define, assign and maintain an ID for the UE; - security issues due to the ID for the UE. |
| Solution 3 (R3-247419) | - OAM configures Management Based MDT with a new Job Type “Immediate MDT and Logged MDT” and assigns a unique TR to this MDT session (valid for all UEs in the area of gNBs where this configuration is provided); - The gNB selects a UE and assigns a TRSR which is UE-specific; - At connected-to-idle transition the MDT configuration (at least Job Type and TRSR) is sent by the serving gNB to the 5GC/AMF which stores this information; when the UE resumes the (new) serving gNB receives from 5GC/AMF the stored MDT configuration; the presence of the MDT configuration is an indication for the (new) serving gNB that the UE needs to be reselected for Continuous MDT - At handover, the source gNB sends the MDT configuration (at least Job Type and TRSR) to the target gNB; the presence of the MDT configuration is an indication for the target gNB that the UE needs to be reselected for Continuous MDT. Involved domains: RAN, OAM and 5GC/AMF | - source gNB provides the target gNB with MDT configuration which is an indication for the target gNB to re-select the UE for Continuous MDT; - serving gNB receiving from 5GC/AMF the MDT configuration previously stored in the 5GC/AMF by the old serving gNB, so that the same UE can be re-selected. | Ensured by correlating at TCE the MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE. | Pros: no need for gNB coordination in TRSR assignment. TRSR conflicts at the target gNB (or new serving gNB) are solved by keeping the mapping relationship of TRSRs allocated to the UE (and sent to the TCE). Cons: OAM impacted due to the need to define a new Job Type. |
| Solution 4 (R3-247393) | - OAM configures gNBs in an area with same immediate MDT configuration (TR1) and logged MDT configuration (TR2); - A gNB chooses UE and configures the UE for Immediate MDT and Logged MDT separately (with separate TRSRs, i.e. TRSR1 and TRSR2); - At connected-to-idle transition the serving gNB sends the Immediate MDT report (TR1/TRSR1) to TCE by also including the TR2/TRSR2 assigned for the Logged MDT; - At idle-to-connected transition the (new) serving gNB sends the Logged MDT report (TR2/TRSR2) to TCE by also including the TR1/TRSR3 assigned for the Immediate MDT. Involved domains: RAN and OAM | Same as in Solution 2 | Ensured by correlating at TCE the Immediate MDT report (TR1/TRSR1) which includes also TR2/TRSR2 of the Logged MDT session with the Logged MDT report (TR2/TRSR2). | Pros: no need for gNB coordination in TRSR assignment. Cons: Multiple TRSRs are assigned to the same UE even if there are no TRSR conflicts. |

We think that the above Table 1 could be included in the LS to SA5 as a way to briefly describe each solution for Continuous MDT, so that SA5 could discuss based on it and provide their feedback accordingly. Based on the SA5 feedback RAN3 should be able to select a solution for Continuous MDT.

Proposal: RAN3 to agree to send the LS to SA5 as in the Annex of this paper (R3-250377).

# 4. Conclusion

Proposal: RAN3 to agree to send the LS to SA5 as drafted in the Annex of this paper (R3-250377).

# 5. Reference

- RP-242385, “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- R3-247144, Discussion on requirements for Continuous MDT solution, Telecom Italia, Jio, BT, Deutsche Telekom

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-247790, SoD on CB: # AIRAN5_ContinuousMDT, Ericsson (Moderator)

# 6. Annex – Draft LS to SA5

3GPP TSG-RAN3 Meeting #127	R3-25abcd

Athens, Greece, 17th – 21st February, 2025

Title:	[Draft] LS on Continuous MDT for AI/ML model training in OAM

Response to:

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-core

Source:	Huawei [to be RAN3]

To:	SA5

Cc:	RAN2

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of the Rel-19 Work Item on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 agreed to specify an enhancement of the MDT framework termed as Continuous MDT in order to continuously collect measurements from the same UE during the same RRC state (i.e., during connected mode mobility) and across RRC states (i.e., after idle/inactive to connected mode transition) that could be used for the AI/ML model (re-)training in OAM.

In order for the AI/ML model (re-)training to be successful the collected data from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. RAN3 discussed solutions to perform the following tasks as captured in TR 38.743:

- measurement continuity: how to ensure that the same UE keeps on collecting MDT measurements during the same RRC state and across different RRC states;

- trace correlation: how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

RAN3 also identified requirements to be taken into account for developing a Continuous MDT solution:

- reuse the Management Based MDT framework;

- rely on the OAM to configure the RAN with a Management Based MDT configuration for Continuous MDT;

- be network-based and avoid impacts on legacy UEs.

In the following Table 1, a summary of solutions developed to solve the above problems and, at the same time, fulfilling the Continuous MDT requirements is provided.

Since solutions in Table 1 may impact the MDT functional aspects for which SA5 is responsible, RAN3 would appreciate a feedback from SA5 on their feasibility that would help RAN3 in the down-selection process.

Table 1 - Network-based solutions for Continuous MDT.

| Solution ID | Description | Measurement Continuity | Trace correlation | Pros. & Cons. |
| --- | --- | --- | --- | --- |
| Solution 1 (R3-247537) | - OAM configures Management Based MDT to the gNBs taking part in Continuous MDT, including a TRs dedicated to Continuous MDT; - Serving gNB selects a UE and configures TRSR which is reassigned to the UE across RRC states and during connected mobility. Involved domains: RAN and OAM | - explicit indication in handover signalling that UE is selected for Continuous MDT (e.g., assigned TR/TRSR); - presence of TR/TRSR in the Logged MDT report sent to the serving gNB after idle-to-connected transition. | Reusing part/all of the identifiers in the Trace Record header, e.g. the TRSR, for the whole duration of the Continuous MDT trace. | Pros: only RAN and OAM involved. Cons: - Dedicated TRs for Continuous MDT; - gNB coordination in TRSR assignment to enable trace correlation. |
| Solution 2 (R3-247623) | - OAM configures Management Based MDT at a gNB; - At connected-to-idle transition the MDT configuration is stored by the serving gNB in 5GC/AMF; - Upon idle-to-connected transition the 5GC/AMF indicates to the (new) serving gNB the MDT configuration of the UE along with an identifier that the UE needs to be selected again for management-based immediate MDT; - During handover, the source gNB sends the Immediate MDT configuration to the target gNB as part of the handover preparation. Involved domains: RAN, OAM and 5GC/AMF | - target gNB receives at handover the UE’s MDT configuration used in the source gNB; - (new) serving gNB receives the UE’s MDT configuration previously stored in the 5GC/AMF. | Ensured by the identifier assigned and maintained by the 5GC/AMF; the identifier is assigned to the UE as part of the MDT configuration, and the serving gNB includes this identifier in the trace files sent to the TCE. | Pros: no need for gNB coordination in TRSR assignment. Cons: - AMF is impacted as it needs to define, assign and maintain an ID for the UE; - security issues due to the ID for the UE. |
| Solution 3 (R3-247419) | - OAM configures Management Based MDT with a new Job Type “Immediate MDT and Logged MDT” and assigns a unique TR to this MDT session (valid for all UEs in the area of gNBs where this configuration is provided); - The gNB selects a UE and assigns a TRSR which is UE-specific; - At connected-to-idle transition the MDT configuration (at least Job Type and TRSR) is sent by the serving gNB to the 5GC/AMF which stores this information; when the UE resumes the (new) serving gNB receives from 5GC/AMF the stored MDT configuration; the presence of the MDT configuration is an indication for the (new) serving gNB that the UE needs to be reselected for Continuous MDT - At handover, the source gNB sends the MDT configuration (at least Job Type and TRSR) to the target gNB; the presence of the MDT configuration is an indication for the target gNB that the UE needs to be reselected for Continuous MDT. Involved domains: RAN, OAM and 5GC/AMF | - source gNB provides the target gNB with MDT configuration which is an indication for the target gNB to re-select the UE for Continuous MDT; - serving gNB receiving from 5GC/AMF the MDT configuration previously stored in the 5GC/AMF by the old serving gNB, so that the same UE can be re-selected. | Ensured by correlating at TCE the MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE. | Pros: no need for gNB coordination in TRSR assignment. TRSR conflicts at the target gNB (or new serving gNB) are solved by keeping the mapping relationship of TRSRs allocated to the UE (and sent to the TCE). Cons: OAM impacted due to the need to define a new Job Type. |
| Solution 4 (R3-247393) | - OAM configures gNBs in an area with same immediate MDT configuration (TR1) and logged MDT configuration (TR2); - A gNB chooses UE and configures the UE for Immediate MDT and Logged MDT separately (with separate TRSRs, i.e. TRSR1 and TRSR2); - At connected-to-idle transition the serving gNB sends the Immediate MDT report (TR1/TRSR1) to TCE by also including the TR2/TRSR2 assigned for the Logged MDT; - At idle-to-connected transition the (new) serving gNB sends the Logged MDT report (TR2/TRSR2) to TCE by also including the TR1/TRSR3 assigned for the Immediate MDT. Involved domains: RAN and OAM | Same as in Solution 2 | Ensured by correlating at TCE the Immediate MDT report (TR1/TRSR1) which includes also TR2/TRSR2 of the Logged MDT session with the Logged MDT report (TR2/TRSR2). | Pros: no need for gNB coordination in TRSR assignment. Cons: Multiple TRSRs are assigned to the same UE even if there are no TRSR conflicts. |

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to provide feedback on the feasibility of the solutions in Table 1.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127bis			2025-04-07 – 2025-04-11				China, CN

RAN3#128				2025-05-19 – 2025-05-23				Malta, MT