3GPP TSG-RAN WG3 Meeting #127									    	R3-250378

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	20.2

Source:	Huawei

Title:	(TP for AI/ML BLCR to TS 38.455 for Case 3a and TP for AI/ML BLCR to TS 38.455 for Case 3b) Discussion on RAN3 impacts for AI/ML positioning

Document for:	Discussion and decision

# Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” – latest WID in [1]. To be specific, the discussion was on the specification support for AI/ML-based positioning accuracy enhancements.

This contribution focuses on AI/ML assisted positioning Case 3a and Case 3b – being the only cases of AI/ML positioning that have RAN3 impacts – and provides proposals based on the latest progress in RAN1 and in RAN3.

# Discussion

In the following we further discuss aspects related to model training, model inference and model monitoring for AI/ML positioning Case 3a (i.e., NG-RAN node-assisted positioning with a gNB-side model) and Case 3b (i.e., NG-RAN node assisted positioning with LMF-side model).

In the last meeting RAN3 discussed Case 3a only, and the following agreements and open issues were captured in the meeting minutes [2]:

Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated.

while, for Case 3b, the following agreements and open issues captured in RAN3#125bis meeting minutes are referenced [3]:

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. For case 3b: - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

For AI/ML positioning, the preliminary Rel-18 study phase – whose outcome is summarized in TR 38.843 [4] – identified that, for Case 3a, the output of the AI/ML model hosted in the gNB is an intermediate measurement to be used for successive determination of the UE location at the Location Management Function (LMF). For Case 3b, instead, the output of the AI/ML model hosted in the LMF is an estimate (prediction) of the UE location which is inferred by means of measurements provided by the NG-RAN.

In order to assess the RAN3 impacts for supporting AI/ML positioning we will discuss model training, model inference and model monitoring aspects, for both Case 3a and Case 3b.

## 2.1 Model training considerations

### Case 3a

Based on the agreements from RAN3#125bis meeting, RAN3 already agreed that for Case 3a inference and training functions are located in the gNB [3]:

The AI/ML model (inference and training functions) is at the gNB.

However, in our understanding, the AI/ML model training function can also be located in the OAM. This is based on what has been reported in TR 38.843 clause 7.2.4, where the training data generation and termination points are identified for the gNB-side model as follows [4]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. **** Skip non-relevant text ****

Both possibilities concerning the termination point for training data – gNB or OAM – are aligned with the deployment scenarios also specified by RAN3 as part of the “AI/ML for NG-RAN” WI in Rel-18, as excerpted below from TS 38.300 clause 16.20.2 (for the non-split architecture) [5]:

### 16.20.2 Principles **** Skip non-relevant text **** For the deployment of AI/ML for NG-RAN the following scenarios may be supported: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node; - AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node. **** Skip non-relevant text ****

Therefore, we have the following proposal:

Proposal 1: For AI/ML assisted positioning Case 3a the AI/ML model training can also be located in the OAM other than in the gNB.

As per previous RAN1 agreements, a training data sample is composed of two parts, a “measurement” (Part A in RAN1 discussions) and a “ground-truth label” (Part B in RAN1 discussions). For Part A RAN1 agreed that it is generated by the TRP/gNB (for both Case 3a and Case 3b), that is, NG-RAN; while Part B can be generated at least by the LMF for Case 3a. RAN1 also agreed on the components that could be included in Part A and Part B of the training data sample (valid for both Case 3a and Case 3b),.

******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

In RAN1#118bis meeting, RAN1 discussed further details of Part A’s content of the training data sample as follows:

******** RAN1#118bis ******** Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information

For Case 3a, for both Part A and Part B, RAN3 also reached the following agreements in RAN3#125bis meeting [3]:

Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1.

Based on the above, RAN3 had a discussion on how the gNB can request the LMF to provide Part B information, and whether this request will also trigger a UE positioning session for the UE for which Part B information is provided, but no conclusion was reached. Therefore RAN3 needs to discuss further, by considering the Working Assumption based on which it is the LMF that initiates a NRPPa transaction (as per legacy positioning) and that it is the gNB which determines the need for data collection at the gNB side. These aspects have been captured in the last meeting minutes as follows [2]:

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

A possible solution allowing the gNB to request the LMF to provide Part B information for model training is shown in Fig. 1.

Fig. 1: Model training for Case 3a.

Step 1: LMF indicates to the gNB that Part B information can be made available at the LMF. The TRP 			INFORMATION REQUEST message could be used for this purpose.

Step 2: gNB sends data collection request to the LMF. The TRP INFORMATION RESPONSE 				message could be used for this purpose.

Step 3: Legacy positioning procedures are performed to obtain UE’s location.

Step 4: LMF sends NRPPa MEASUREMENT  REQUEST to the gNB to trigger the collection of Part 			A information, and includes the ground truth label info.

Step 5: The gNB obtains the Part A information.

Step 6: The gNB notifies the LMF about Part A data collection.

Proposal 2: RAN3 to consider following procedure for AI/ML model training procedure for Case 3a:

Step 1: LMF indicates to the gNB that Part B information can be made available at the 				LMF. The TRP INFORMATION REQUEST message could be used for this purpose.

Step 2: gNB sends data collection request to the LMF. The TRP INFORMATION RESPONSE 			message could be used for this purpose.

Step 3: Legacy positioning procedures are performed to obtain UE’s location.

Step 4: LMF sends NRPPa MEASUREMENT REQUEST to the gNB to trigger the collection of 			Part A information, and includes the ground truth label info.

Step 5: The gNB obtains the Part A information.

Step 6: The gNB notifies the LMF about Part A data collection.

For Part A, instead, since it is generated by the TRP/gNB, there is no RAN3 impact in making this information available in the entity performing AI/ML model training, i.e., the NG-RAN (if the AI/ML model training is located in the NG-RAN). Part A’s content for Case 3a is still under discussion in RAN1 and our understanding is that it reflects the content of the measurement report provided by the gNB to the LMF which includes the inference results (discussed in Section 2.2).

Even though RAN1 didn’t agree on a specific indication concerning the user data privacy requirements for the provisioning of Part B from the LMF to the NG-RAN for Case 3a, we think that a similar note from RAN1 about the user data privacy as included in the description of Option A for the AI/ML model monitoring options for Case 3a should also be considered for the model training as well – note that the model monitoring considerations for Case 3a can be found in Section 2.3 of this contribution.

******** RAN1#118 ******** Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

In addition, RAN1 agreed that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18, for which the following was specified in TS 38.300 clause 16.20.2 [5]:

### 16.20.2 Principles **** Skip non-relevant text **** Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

Therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting Case 3a. Based on the above, we have the following proposal:

Proposal 3: For Case 3a, the signalling procedure from LMF to gNB for transferring Part B information of the training data sample shall fulfil the following:

- Provisioning of Part B ensures and preserves the corresponding user data privacy requirements.

- Purpose, such as “training data collection”, will not necessarily be specified in RAN3 specification.

Proposal 4: For Case 3a, there is no RAN3 impact for Part A of the training data sample to be made available in the NG-RAN (if AI/ML model training is located in the NG-RAN).

### Case 3b

For Case 3b the AI/ML model training is located at the LMF, which is the termination point of training data, as reflected in TR 38.843 clause 7.2.4 [4]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** o For LMF-side model, the LMF is the termination point for training data.

As previously said, for Part A of the training data sample, RAN1 agreed that it is generated by the TRP/gNB (for both Case 3a and Case 3b), that is, NG-RAN; while Part B for Case 3b can be generated by PRU, non-PRU UE with estimated location or LMF.

******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. ******** RAN1#117 ******** Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. ******** RAN1#118bis ******** Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; FFS: details on how to associate quality indicator to timing information

For Case 3b, there is RAN3 impact in providing Part A of the training data sample from NG-RAN to LMF, and this is also reflected in the agreement that RAN3 reached in RAN3#125bis meeting [3]:

For case 3b: - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

In Case 3b, for Part B of the training data sample, if it is generated by PRU/Non-PRU UE with estimated location, there is no RAN3 impact since this information can be provided via LPP protocol to the LMF. If Part B is generated by the LMF, there is as well no RAN3 impact since this information is already available in the entity performing AI/ML model training, i.e., LMF.

In RAN3#125bis meeting the open point on the details of the measurements (Part A) being pending and subject to further discussion in RAN1 was captured in the meeting minutes for both Case 3a and Case 3b [3]. Therefore, RAN3 expects to receive further inputs from RAN1 in order to discuss the details of Part A’s content which, for Case 3b, needs to be made available at the LMF to perform AI/ML model training. Part A’s content for Case 3b is still under discussion in RAN1 and our understanding is that it reflects the content of the measurement report provided by the gNB to the LMF for successive AI/ML model inference as per the agreement from RAN1#118bis meeting (discussed in Section 2.2).

Still based on RAN1 agreements, for Case 3b, Part A of the training data sample includes a timestamp of the channel measurement. For this piece of information RAN1 also agreed that the existing IE “Time Stamp” in TS 38.455 (NRPPa protocol) can be reused from RAN1 perspective:

******** RAN1#118 ******** Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications.

Moreover, within Part A of a training data sample for Case 3b, the quality indicator of the channel measurement (where the channel measurement could be either a timing information or a paired timing information and power information, as per RAN1#116 meeting) is also included. And in RAN1#118bis meeting RAN1 agreed that the quality indicator in Part A reuses the existing IE “Timing Measurement Quality” in TS 38.455. Therefore, for Case 3b, there is no additional RAN3 impacts on how to transfer the time stamp and quality indicator both associated to the channel measurement to the LMF.

Proposal 5: For Case 3b, there is no RAN3 impact in providing the LMF with the time stamp and quality indicator of the channel measurement included in Part A of the training data sample.

The above Proposal 5 is also in line with the agreement reached in RAN3#125bis meeting concerning the actual reporting of measurements to the LMF for Case 3b, which will be performed by re-using the existing measurement procedure (and hence messages) already specified in NRPPa [6]:

Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

In addition, RAN1 agreed in RAN1#118 meeting a Note that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18 as captured in TS 38.300 clause 16.20.2 [5]. Therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting Case 3b.

Proposal 6: For the discussion of RAN3 impacts of Case 3b, RAN3 to agree that the purpose, such as “training data collection”, or “monitoring”, or “inference” will not necessarily be specified in RAN3 specification when providing Part A of the training data sample to the LMF.

## 2.2 Model inference considerations

### Case 3a

For Case 3a, to enable the AI/ML model in the gNB to generate measurements for successive determination at LMF side of the UE location, some input data needs to be provided to the AI/ML model itself which will then generate a proper output. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. ******** RAN1#118 ******** Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

Based on the above agreements, our understanding is that the AI/ML model in the gNB uses UL SRS-based measurements as model input for inference purposes (other than for other procedures such as training and monitoring). And, as per Proposal 3 above and based on RAN1 agreements, the purpose for which information are provided and used shall not be indicated in RAN3 specifications.  RAN1 made a conclusion in RAN1#118 meeting that the existing procedures can be reused for the SRS configuration, even though the details of the existing parameters (e.g., values) can be further discussed.

Proposal 7: The AI/ML model in the gNB for Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

With regards to the AI/ML model output for Case 3a, i.e., information that could be reported from the NG-RAN to the LMF, RAN1 agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting.

******** RAN1#116 ******** Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

For both pieces of information RAN1 also considered the existing methods already specified, that is, soft/hard indicator for the LOS/NLOS indicator, and UL RTOA (Uplink Relative Time of Arrival) or gNB Rx-Tx time difference for the timing information.

In the last two meetings RAN1 agreed further aspects related to the content of the measurement report from gNB to LMF for Case 3a, as follows:

******** RAN1#118bis ******** Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. ******** RAN1#119 ******** Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements

The first agreement above from RAN1 is related to the presence (mandatory/optional) of components (Information Elements, IEs) that constitute the measurement report from gNB to LMF in case timing information (either UL RTOA or gNB Rx-Tx difference) is reported as part of the AI/ML model output for Case 3a. In the RAN1 agreement there is also a Note stating that the final decision of “mandatory” or “optional” presence of each field is up to RAN3.

By checking the latest NRPPa specification [6], the following can be excerpted:

#### 9.1.4.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |

#### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

Based on the current NRPPa specification, in our understanding the presence of IEs (timing information, quality of the timing information, time stamp) in the measurement report from gNB to LMF in case timing information (either UL RTOA or gNB Rx-Tx difference) is reported is already aligned with the presence as suggested in the RAN1 agreement. Therefore, we think RAN3 should confirm what RAN1 already indicated for the presence of IEs in the measurement report for Case 3a, which has no additional standard impact on NRPPa. This is also motivated by the following agreement and common understanding from the last RAN3 meeting [2]:

Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

Proposal 8: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement report from gNB to LMF is as in legacy NRPPa, that is:

- Timing information: mandatory IE

- Quality of the timing information: optional IE

- Time stamp: mandatory IE.

Another FFS from the last meeting is related on whether the LMF should be made aware of the fact that measurements reported by the gNB are generated by an AI/ML model or they are generated as in legacy positioning [2]:

FFS if LMF should be aware that the reported measurements are AI/ML generated.

In light of the approach used so far by RAN3 to strive to reuse the existing procedures and IEs for positioning measurements, we believe that it would be useful for the LMF to be able to distinguish whether the reporting is related to predicted information. We think that a simple flag “predicted” introduced in the existing TRP Measurement Result IE could serve the purpose, as it is not only a simple solution but also more flexible in case RAN1 decides in the future to include additional predicted information in the reporting to the LMF.

Proposal 9: RAN3 to agree to introduce a flag in the existing TRP Measurement Result IE to allow the LMF to distinguish whether the measurement report from the gNB is related to predicted information. The flag can be an ENUMERATED IE type with codepoint “true”.

### Case 3b

For Case 3b, to enable the AI/ML model in the LMF to directly infer the UE location, some input data needs to be provided to the AI/ML model itself. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. ******** RAN1#116bis ******** Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT. ******** RAN1#117 ******** Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). ******** RAN1#118 ******** Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples, - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples. - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. ******** RAN1#118bis ******** Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

By specifically referring to the RAN1 agreement from RAN1#118bis concerning the content of the measurement report from gNB to LMF for Case 3b, we noticed that also for Case 3b RAN1 discussed about the presence (mandatory/optional) of components (IEs) that constitute the measurement report. Similar discussion occurred in RAN1#118bis meeting also for Case 3a as previously described, where a Note stating that the final decision of “mandatory” or “optional” presence of each field is up to RAN3 was captured in RAN1 meeting minutes.

In our understanding such Note also applies for the agreement from RAN1#118bis above mentioned for Case 3b, even if this has not been explicitly captured by RAN1 – the motivation of that is because the agreements about the presence of components that constitute the measurement report for Case 3b and Case 3a have been taken in different times during the meeting week, which led RAN1 in including the Note only for one of the two AI/ML positioning cases having RAN3 impacts.

Still based on the current NRPPa specification[6], in our understanding the presence of IEs (channel measurement, quality of the channel measurement, time stamp) in the measurement report from gNB to LMF for Case 3b is already aligned with the presence as suggested in the RAN1 agreement. Therefore, we think RAN3 should confirm what RAN1 already indicated for the presence of IEs in the measurement report, which has no additional standard impact on NRPPa.

Proposal 10: RAN3 to confirm the RAN1 agreement that the presence of IEs in the measurement report from gNB to LMF for Case 3b is as in legacy NRPPa, that is:

- Channel measurement: mandatory IE

- Quality of the channel measurement: optional IE

- Time stamp of the channel measurement: mandatory IE.

For Case 3b, RAN1 agreed that channel measurements supported for reporting can be either timing information or paired timing information and power information (RAN1#116).

In our understanding the power information associated to channel measurements that are provided from NG-RAN to the LMF are based on the existing UL SRS (Uplink Sounding Reference Signal), that is, UL SRS Reference Signal Received Path Power (UL SRS-RSRPP) measurement. Therefore, we think there is no standard impact in reporting such information to from NG-RAN to LMF for Case 3b.

Proposal 11: RAN3 to confirm that for Case 3b the power information associated to channel measurements to be provided from NG-RAN to LMF which can be used as model input are based on the existing UL SRS. There is no standard impact for reporting the power information (UL SRS-RSRPP) from NG-RAN to LMF.

- The purpose of the report such as “model input” will not necessarily be specified in RAN3 specification.

Moreover, still for Case 3b, for the timing information of channel measurements, RAN1 agreed that the timing information is represented relative to a reference time which is the existing UL RTOA (Uplink Relative Time of Arrival) reference time T0+tSRS as defined in TS 38.215 (RAN1#116bis). Hence in our view there is no RAN3 impact in reporting timing information of channel measurements from NG-RAN to LMF for Case 3b.

Proposal 12: RAN3 to confirm that for Case 3b there is no standard impact for reporting the timing information associated to channel measurements from NG-RAN to LMF.

Regarding the time domain channel measurements for Case 3b, RAN1 is also investigating two alternatives:

- Sample-based measurements, where the timing information is an integer multiple of sampling periods.

- Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods, and for which RAN1 also agreed that it refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

For the sample-based measurements RAN1 agreed some aspects such as its definition, the number of samples of the estimated channel response in time domain and the time granularity for the reporting already in previous meetings, but only in the last meeting (RAN1#119) RAN1 agreed to support also sample-based measurements in Rel-19 as part of the following agreements:

******** RAN1#119 ******** Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion. Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

In the last meeting RAN1 agreed that sample-based measurements are actually an enhancement of the path-based measurements and, in order to support such enhancement, there is the need to signal from LMF to gNB a set of configuration parameters (Nt, Nt', k as agreed by RAN1), but as can be noticed above, RAN1 needs further discussion on the actual values of k and Nt'. Anyway RAN3 could still start discussing about the signalling details, e.g., which procedure over NRPPa to use. In our view, and in light of the RAN3 agreements from RAN3#125bis that the Measurement procedure defined in NRPPa needs to be reused to support requesting and reporting the related measurements to LMF for Case 3b, we think that the MEASUREMENT REQUEST message (LMF  NG-RAN node) could be enhanced so that the LMF can signal Nt, Nt', k to the gNB.

Proposal 13: RAN3 to agree that, in order to support the enhancement of the path-based channel measurements for Case 3b, the MEASUREMENT REQUEST message in NRPPa is enhanced to signal the related configuration parameters (Nt, Nt', k as agreed by RAN1) from LMF to gNB.

## 2.3 Model monitoring considerations

Concerning the AI/ML model monitoring, in RAN3#125bis a Working Assumption (WA) based on what RAN1 identified as Option A for the model monitoring metric calculation was reached [3]:

Case 3a agreements: - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For model monitoring in Case 3a, RAN1 identified two options:

- Option A, where the NG-RAN (hosting the AI/ML model) performs the monitoring metric calculation – this is the option covered in the WA from the last RAN3 meeting;

- Option B, where the LMF performs the monitoring metric calculation for the model hosted in NG-RAN.

In the last meeting RAN3 concluded this discussion by agreeing to turn the above reported WA into an agreement [2]:

Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

On top of that, we think RAN3 should agree on a general principle concerning the location of the model monitoring function with respect to the entity which is in charge of the AI/ML model inference or, in general, of taking a proper “monitoring decision” for the model based on inference results (e.g., (de-)activation, fallback to legacy positioning methods, etc.). In our view, it would be better that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation, which is also the entity which has knowledge of the model being used. This because in our understanding the “monitoring decision” should be based on the model-specific monitoring metric calculated by considering two main components, that is, the inference results (i.e., model output) and the ground-truth labels. And for Case 3b in particular, this is also motivated by the following RAN1 agreement from RAN1#116, where it can be deduced that (model) performance monitoring is performed in LMF (i.e., the entity hosting the AI/ML model):

******** RAN1#116 ******** Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Proposal 14: For both Case 3a and Case 3b RAN3 to agree the general principle that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation and takes the corresponding monitoring decision.

Still based on the above mentioned agreement from RAN1#116 on Case 3b, our understanding is that RAN1 is still studying whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b. Therefore, it is suggested waiting for further RAN1 progress before starting the discussion in RAN3.

Proposal 15: RAN3 to wait further RAN1 progress on whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b.

A TP for the AI/ML BLCR to TS 38.455 covering Case 3a is in the Annex 1, while a TP for the AI/ML BLCR to TS 38.455 covering Case 3b is in the Annex 2.

# Conclusions

Model training considerations

Proposal 1: For AI/ML assisted positioning Case 3a the AI/ML model training can also be located in the OAM other than in the gNB.

Proposal 2: RAN3 to consider following procedure for AI/ML model training procedure for Case 3a:

Step 1: LMF indicates to the gNB that Part B information can be made available at the 				LMF. The TRP INFORMATION REQUEST message could be used for this purpose.

Step 2: gNB sends data collection request to the LMF. The TRP INFORMATION RESPONSE 			message could be used for this purpose.

Step 3: Legacy positioning procedures are performed to obtain UE’s location.

Step 4: LMF sends NRPPa MEASUREMENT REQUEST to the gNB to trigger the collection of 			Part A information, and includes the ground truth label info.

Step 5: The gNB obtains the Part A information.

Step 6: The gNB notifies the LMF about Part A data collection.

Proposal 3: For Case 3a, the signalling procedure from LMF to gNB for transferring Part B information of the training data sample shall fulfil the following:

- Provisioning of Part B ensures and preserves the corresponding user data privacy requirements.

- Purpose, such as “training data collection”, will not necessarily be specified in RAN3 specification.

Proposal 4: For Case 3a, there is no RAN3 impact for Part A of the training data sample to be made available in the NG-RAN (if AI/ML model training is located in the NG-RAN).

Proposal 5: For Case 3b, there is no RAN3 impact in providing the LMF with the time stamp and quality indicator of the channel measurement included in Part A of the training data sample.

Proposal 6: For the discussion of RAN3 impacts of Case 3b, RAN3 to agree that the purpose, such as “training data collection”, or “monitoring”, or “inference” will not necessarily be specified in RAN3 specification when providing Part A of the training data sample to the LMF.

Model inference considerations

Proposal 7: The AI/ML model in the gNB for Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

Proposal 8: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement report from gNB to LMF is as in legacy NRPPa, that is:

- Timing information: mandatory IE

- Quality of the timing information: optional IE

- Time stamp: mandatory IE.

Proposal 9: RAN3 to agree to introduce a flag in the existing TRP Measurement Result IE to allow the LMF to distinguish whether the measurement report from the gNB is related to predicted information. The flag can be an ENUMERATED IE type with codepoint “true”.

Proposal 10: RAN3 to confirm the RAN1 agreement that the presence of IEs in the measurement report from gNB to LMF for Case 3b is as in legacy NRPPa, that is:

- Channel measurement: mandatory IE

- Quality of the channel measurement: optional IE

- Time stamp of the channel measurement: mandatory IE.

Proposal 11: RAN3 to confirm that for Case 3b the power information associated to channel measurements to be provided from NG-RAN to LMF which can be used as model input are based on the existing UL SRS. There is no standard impact for reporting the power information (UL SRS-RSRPP) from NG-RAN to LMF.

- The purpose of the report such as “model input” will not necessarily be specified in RAN3 specification.

Proposal 12: RAN3 to confirm that for Case 3b there is no standard impact for reporting the timing information associated to channel measurements from NG-RAN to LMF.

Proposal 13: RAN3 to agree that, in order to support the enhancement of the path-based channel measurements for Case 3b, the MEASUREMENT REQUEST message in NRPPa is enhanced to signal the related configuration parameters (Nt, Nt', k as agreed by RAN1) from LMF to gNB.

Model monitoring considerations

Proposal 14: For both Case 3a and Case 3b RAN3 to agree the general principle that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation and takes the corresponding monitoring decision.

Proposal 15: RAN3 to wait further RAN1 progress on whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b.

A TP for the AI/ML BLCR to TS 38.455 covering Case 3a is in the Annex 1, while a TP for the AI/ML BLCR to TS 38.455 covering Case 3b is in the Annex 2.

# References

- RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Qualcomm, RAN#106

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (v18.0.0), Rel-18

- 3GPP TS 38.300, NR and NG-RAN Overall Description; Stage 2, Rel-18

- 3GPP TS 38.455, NG-RAN; NR Positioning Protocol A (NRPPa), Rel-18

# Annex 1 – TP for the AI/ML BLCR to TS 38.455 – Case 3a

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

### 8.2.8	TRP Information Exchange

#### 8.2.8.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.8.2	Successful Operation

Figure 8.2.8.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the TRP List IE is included in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message.

If the PRS Muting IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the muting information for the given PRS resource set.

If the QCL Info IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account for the given PRS resource list.

If the Aggregated PRS Resource Set List IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the PRS resource sets linked for PRS bandwidth aggregation.

If the DL-PRS Resource Coordinates IE is included in the Geographical Coordinates IE in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the DL PRS Resource Coordinates relative to the TRP coordinate.

If the Mobile IAB-MT UE ID IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF shall, if supported, use this information to determine an updated location of the Mobile TRP as specified in TS 23.273 [20].

If the TRP Information Type Item IE is set to 'mobile trp location info', the NG-RAN node shall, if supported, derive the location of the Mobile TRP as specified in TS 23.273 [20] and include the Mobile TRP Location Information in the TRP INFORMATION RESPONSE message.

If the Data Collection Available (NAME FFS) IE is included in the TRP List IE, the NG-RAN node may take it into account for subsequent data collection request and include the Data Collection Request (NAME FFS) IE in the TRP Information List IE of the TRP INFORMATION RESPONSE message.

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider this as the time window(s) within which to perform the positioning measurements, as specified in TS 38.305 [18].

If the Ground Truth Label Information (NAME FFS) IE in the TRP Measurement Quantities IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take into account for triggering data collection of SRS measurements.

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 8.5.3	Measurement Update

#### 8.5.3.1	General

The Measurement Update Procedure allows the LMF to notify the NG-RAN node of a change in a previously configured measurement. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.3.2	Successful Operation

Figure 8.5.3.2.1: Measurement Update: Successful Operation.

The LMF initiates the procedure by sending a MEASUREMENT UPDATE message.

If the SRS Configuration IE is included in the MEASUREMENT UPDATE message, the NG-RAN node shall overwrite the previously stored SRS configuration.

If the AoA Search Window Information IE is included in the TRP Measurement Update List IE in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored AoA search window information and store the newly received information.

If the Number of TRP Rx TEGs IE is included in the TRP Measurement Update List IE in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

If the Number of TRP RxTx TEGs IE is included in the TRP Measurement Update List IE in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

If the Measurement Time Occasion IE is included in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

If the Ground Truth Label Information (NAME FFS) IE is included in the TRP Measurement Update List IE in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### 9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Data Collection Available (NAME FFS) | O |  | ENUMERATED(true, …) |  | YES | ignore |
| TRP Information Type List |  | 1 |  |  | YES | reject |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, on-demand prs, trp tx teg, beam antenna info, mobile trp location info, common ta) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

#### 9.1.1.15	TRP INFORMATION RESPONSE

This message is sent by an NG-RAN node to convey TRP information to an LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Information List |  | 1 |  |  | YES | ignore |
| >TRP Information Item | M | 1 .. <maxnoTRPs> |  |  | - |  |
| >>TRP Information | M |  | 9.2.25 |  | - |  |
| >>Data Collection Request (NAME FFS) | O |  | ENUMERATED(true,…) |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Ground Truth Label Information (NAME FFS) | O |  | FFS | FFS | YES | ignore |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### 9.1.4.5	MEASUREMENT UPDATE

This message is sent by the LMF to update a previously configured measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| TRP Measurement Update List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Update Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Ground Truth Label Information (NAME FFS) | O |  | FFS | FFS | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | Ignore |
| >Predicted Indication (NAME FFS) | O |  | ENUMERATED(true,…) | If present, it indicates predicted TRP Measurement Result | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

*** skip unmodified text ***

id-SRSPosPeriodicConfigHyperSFNIndex

id-DataCollectionAvailable(NAME-FFS)

id-DataCollectionRequest(NAME-FFS)

id-GroundTruthLabelInformation(NAME-FFS)

id-PredictedIndication(NAME-FFS)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- D

DL-Bandwidth-EUTRA ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

*** skip unmodified text ***

DL-reference-signal-UERxTx-TD ::= ENUMERATED {csirs, prs, ...}

DataCollectionAvailable(NAME-FFS) ::= ENUMERATED (true, ...)

DataCollectionRequest(NAME-FFS) ::= ENUMERATED (true, ...)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- G

GeographicalCoordinates ::= SEQUENCE {

tRPPositionDefinitionType	TRPPositionDefinitionType,

dLPRSResourceCoordinates	DLPRSResourceCoordinates	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { GeographicalCoordinates-ExtIEs } } OPTIONAL,

...

}

*** skip unmodified text ***

GNBRxTxTimeDiffMeas-ExtIEs		NRPPA-PROTOCOL-IES ::= {

{ID id-ReportingGranularitykminus1 	CRITICALITY ignore TYPE ReportingGranularitykminus1 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus2 	CRITICALITY ignore TYPE ReportingGranularitykminus2 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus3 	CRITICALITY ignore TYPE ReportingGranularitykminus3 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus4 	CRITICALITY ignore TYPE ReportingGranularitykminus4 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus5 	CRITICALITY ignore TYPE ReportingGranularitykminus5 PRESENCE mandatory}|

{ID id-ReportingGranularitykminus6 	CRITICALITY ignore TYPE ReportingGranularitykminus6 PRESENCE mandatory},

...

}

GroundTruthLabelInformation(NAME-FFS) ::= FFS

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- P

PathlossReferenceInformation ::= SEQUENCE {

pathlossReferenceSignal					PathlossReferenceSignal,

iE-Extensions					ProtocolExtensionContainer { { PathlossReferenceInformation-ExtIEs } } OPTIONAL,

...

}

*** skip unmodified text ***

PointA  ::= INTEGER (0..3279165)

PredictedIndication(NAME-FFS) ::= ENUMERATED (true,...)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- T

TAC ::= OCTET STRING (SIZE(3))

*** skip unmodified text ***

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional},|

{ID id-GroundTruthLabelInformation(NAME-FFS)	CRITICALITY ignore EXTENSION GroundTruthLabelInformation(NAME-FFS)	   PRESENCE optional },

...

}

*** skip unmodified text ***

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSResourcetype	CRITICALITY ignore EXTENSION SRSResourcetype PRESENCE optional}|

{ ID id-ARP-ID		CRITICALITY ignore EXTENSION ARP-ID 	PRESENCE optional}|

{ ID id-LoS-NLoSInformation		CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional }|

{ ID id-Mobile-TRP-LocationInformation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE optional }|

{ ID id-MeasuredFrequencyHops			CRITICALITY ignore EXTENSION MeasuredFrequencyHops					PRESENCE optional }|

{ ID id-AggregatedPosSRSResourceID-List	CRITICALITY ignore EXTENSION AggregatedPosSRSResourceID-List	PRESENCE optional }|

{ ID id-MeasBasedOnAggregatedResources			CRITICALITY ignore EXTENSION MeasBasedOnAggregatedResources PRESENCE optional }|

{ ID id-PredictedIndication(NAME-FFS)	CRITICALITY ignore EXTENSION PredictedIndication(NAME-FFS)	PRESENCE optional },

...

}

*** skip unmodified text ***

TRP-MeasurementUpdateList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementUpdateItem

TRP-MeasurementUpdateItem ::= SEQUENCE {

tRP-ID						TRP-ID,

aoA-window-information		AoA-AssistanceInfo	OPTIONAL,

iE-extensions					ProtocolExtensionContainer { { TRP-MeasurementUpdateItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementUpdateItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-NumberOfTRPRxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTEG		PRESENCE optional }|

{ ID id-NumberOfTRPRxTxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTxTEG		PRESENCE optional }|

{ ID id-GroundTruthLabelInformation(NAME-FFS)	CRITICALITY ignore EXTENSION GroundTruthLabelInformation(NAME-FFS)	   PRESENCE optional },

...

}

*** skip unmodified text ***

TRPInformationListTRPResp ::= SEQUENCE (SIZE (1.. maxnoTRPs)) OF SEQUENCE {

tRPInformation					TRPInformation,

iE-Extensions					ProtocolExtensionContainer { {TRPInformationTRPResp-ExtIEs} } OPTIONAL,

...

}

TRPInformationTRPResp-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-DataCollectionAvailable(NAME-FFS)		CRITICALITY ignore	EXTENSION DataCollectionAvailable(NAME-FFS)		PRESENCE optional },		
	...

}

*** skip unmodified text ***

TRPList ::= SEQUENCE (SIZE(1.. maxnoTRPs)) OF TRPItem

TRPItem ::= SEQUENCE {

tRP-ID		TRP-ID,

iE-Extensions	ProtocolExtensionContainer { {TRPItem-ExtIEs} } OPTIONAL,

...

}

TRPItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-DataCollectionRequest(NAME-FFS)		CRITICALITY ignore	EXTENSION DataCollectionRequest(NAME-FFS)		PRESENCE optional },		
	...

}

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

*** skip unmodified text ***

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-SRSPosPeriodicConfigHyperSFNIndex							ProtocolIE-ID ::= 165

id-DataCollectionAvailable(NAME-FFS)							ProtocolIE-ID ::= aaa

id-DataCollectionRequest(NAME-FFS)								ProtocolIE-ID ::= bbb

id-GroundTruthLabelInformation(NAME-FFS)						ProtocolIE-ID ::= ccc

id-PredictedIndication(NAME-FFS)								ProtocolIE-ID ::= ddd

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>

# Annex 2 – TP for the AI/ML BLCR to TS 38.455 – Case 3b

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

### 9.1.4	Messages for Measurement Information Transfer Procedures

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | Ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Number of Consecutive Measurements (NAME FFS) | O |  | ENUMERATED(32, 64, 128, …) | It indicates the number of consecutive channel response values in time domain (Nt) |  |  |
| >>Number of Selected Measurements (NAME FFS) | O |  | FFS | It indicates the number of selected channel response values in time domain (Nt') over the number of consecutive channel response values (Nt) |  |  |
| >>Enhanced Timing Reporting Granularity Factor (NAME FFS) | O |  | FFS | It indicates the timing granularity of the selected channel response values in time domain (Nt') |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | Ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

*** skip unmodified text ***

id-SRSPosPeriodicConfigHyperSFNIndex,

id-NumberOfConsecutiveMeasurements(NAME-FFS)

id-NumberOfSelectedMeasurements(NAME-FFS)

id-EnhancedTimingReportingGranularityFactor(NAME-FFS)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					NG-RAN-CGI,

servingCellTAC					TAC,

nG-RANAccessPointPosition		NG-RANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL,

...

}

*** skip unmodified text ***

Expected-Value-ZoA ::= INTEGER (0..1799)

EnhancedTimingReportingGranularityFactor(NAME-FFS) ::= FFS

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- N

NarrowBandIndex ::= INTEGER (0..15,...)

*** skip unmodified text ***

NZP-CSI-RS-ResourceID::= INTEGER  (0..191)

NumberOfConsecutiveMeasurements(NAME-FFS) ::= ENUMERATED (32,64,128,...)

NumberOfSelectedMeasurements(NAME-FFS) ::= FFS

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- T

TAC ::= OCTET STRING (SIZE(3))

*** skip unmodified text ***

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional}|

{ID id-NumberOfConsecutiveMeasurements(NAME-FFS)	CRITICALITY ignore EXTENSION NumberOfConsecutiveMeasurements(NAME-FFS) PRESENCE	optional}|

{ID id-NumberOfSelectedMeasurements(NAME-FFS)	CRITICALITY ignore EXTENSION NumberOfSelectedMeasurements(NAME-FFS) PRESENCE	optional}|

{ID id-EnhancedTimingReportingGranularityFactor(NAME-FFS)	CRITICALITY ignore EXTENSION EnhancedTimingReportingGranularityFactor(NAME-FFS) PRESENCE	optional},

...

}

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

*** skip unmodified text ***

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

*** skip unmodified text ***

id-SRSPosPeriodicConfigHyperSFNIndex							ProtocolIE-ID ::= 165

id-NumberOfConsecutiveMeasurements(NAME-FFS)					ProtocolIE-ID ::= aaa

id-NumberOfSelectedMeasurements(NAME-FFS)						ProtocolIE-ID ::= bbb

id-EnhancedTimingReportingGranularityFactor(NAME-FFS)			ProtocolIE-ID ::= ccc

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>