3GPP TSG-RAN WG3 Meeting #127	R3-250395

Athens, Greece, 17-21 Feb 2025

Agenda Item:	32.1

Source:	Ericsson

Title:	Enhancements for Data Collection Reporting

Document for:	Discussion, Agreement

# 1.	Introduction

In Rel-18, the Data Collection Reporting Initiation and Data Collection Reporting procedures were introduced. In this contribution, we describe some limitations of the Rel-18 framework and present some possible enhancements to the procedures.

# 2.	Discussion

### 2.1	Invalidation of previously reported prediction

The XnAP Data Collection Reporting Initiation procedure allows a gNB1 to request predicted information from a neighbour gNB2 (e.g., predicted Radio Resource Status and predicted number of RRC connections). If the request from gNB1 (using the DATA COLLECTION REQUEST message) specifies a reporting periodicity (in the Reporting Periodicity for Data Collection IE), the gNB1 expects that gNB2 provides a new prediction to gNB1 in the DATA COLLECTION UPDATE message on every occasion of the Reporting Periodicity for Data Collection.

The DATA COLLECTION REQUEST message also specifies the Requested Prediction Time IE. According to the semantics description of this IE;

“For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity.”

With the current specification, we identify issues in case a prediction needs to be cancelled or updated.

For example, assume that, at t1, gNB2 sends a first DATA COLLECTION UPDATE with a Predicted Radio Resource Status “X” which refers to t1+30 seconds. If the Reporting Periodicity for Data Collection is 5 seconds, the gNB2 sends a second prediction (at t1+5 seconds) which refers to t1+30+5 seconds.

However, in the time interval between t1 and t1+5s, events may happen that can invalidate the prediction signalled at time t1. For example, if gNB2 predicted the Predicted Radio Resource Status IE for a given cell, it might happen that some neighbour cell is deactivated after the prediction is generated (and assumed to still be inactive at t1+30s), hence changing the conditions on the basis of which the prediction on cell load was calculated. In this case, the prediction signalled by gNB2 at time t1 is invalid, but the receiving node does not know that, and it operates believing that the prediction is still good. 
As an example, the future load on gNB2 cell could be much higher than what stated by the prediction generated at time t1 and for prediction time t1+30s. This may result in wrong mobility actions towards the cell with consequent performance degradation and possible failures.

- A node which reports predictions cannot signal when the conditions, on the basis of which a previous prediction was made, have changed and potentially invalidate the previous prediction. This can lead to bad decisions in the node(s) receiving the predictions.

One possible enhancement to help gNB1 to take better decisions, particularly decisions that mainly affect the time interval (Requested Prediction Time, Requested Prediction Time + Reporting Periodicity for Data Collection), is to indicate if the prediction for a specific prediction time is cancelled.

A simple way to achieve this is that the DATA COLLECTION UPDATE message is enhanced with an IE that informs gNB1 whether the content of the previously sent DATA COLLECTION UPDATE message is still good to use or not.

Therefore, we propose to enhance the DATA COLLECTION UPDATE message with an IE (e.g., Previous Prediction Cancel) that indicates to the receiver whether any of the predictions included in the previous DATA COLLECTION UPDATE message is cancelled.

- Enhance the DATA COLLECTION UPDATE message with an IE (e.g., Previous Prediction Cancel) that indicates whether any of the predictions included in the previous DATA COLLECTION UPDATE message is cancelled.

### 2.2	Enhanced UE feedback reporting

The Data Collection Reporting Initiation and the Data Collection Reporting XnAP procedures also allow a gNB (e.g., gNB1) to request UE feedback from a neighboring gNB (e.g., gNB2) in relation to handed-over UEs. The exit conditions under which gNB2 terminates the collection of UE feedback and reports it back to gNB1 are described in clause 8.4.13.2 in TS 38.423.

In the case of measured UE performance, gNB2 shall terminate the collection when at least one of the following conditions is fulfilled:

- the time since UE was successfully handed over to gNB2 is equal to the value of the Collection Time Duration for UE Performance IE included in the DATA COLLECTION REQUEST message;

- the UE moves to RRC_INACTIVE or RRC_IDLE state; or,

- the UE is handed over to another cell.

Except for the case where the exit condition is based on the expiration of the Collection Time Duration for UE Performance IE, if gNB2 terminates the collection due to one of the other conditions, gNB1 is not aware of the amount of time during which gNB2 collected the data. So, if we assume that gNB1 is interested in collecting measured UE performance during a 5-second interval, the UE can move to RRC_INACTIVE, or to RRC_IDLE, or can be handed over to another cell at any time between the moment the collection starts and until expiration of the Collection Time Duration for UE Performance. It is quite evident that the usefulness of the measured UE performance is not the same if the exit condition (UE transitions to RRC_INACTIVE or to RRC_IDLE, or UE is handed over to another cell) manifests itself after 200ms or after 4800ms. In other words, the amount of time during which the UE performance was collected is an important piece of information that makes the UE Performance feedback more or less relevant/accurate for the requesting node. However, the requesting node (gNB1) is totally unaware of the time duration for which the UE Performance was collected and because of that it cannot determine if some reported UE Performance metrics are valid and accurate or if they should be filtered out and not considered.

- The amount of time during which the UE performance is collected is a useful indication of the goodness of the collected UE performance.

In the present framework, the node receiving the measured UE performance does not know the conditions under which the UE performance was collected. Following the previous example, if the collection of measured UE performance for a particular UE was terminated after 200ms, it is most likely due to the UE being handed over because the original target cell was not the best cell or due to the UE moving to RRC_IDLE after an RLF. On the other hand, if the collection was terminated after 4800ms, the UE may have changed cells due to normal UE mobility or the UE may have moved to RRC_IDLE after finishing a connection.

For example, the node receiving the measured UE performance could decide to filter out any information for which the time the UE stayed in the target cell was below a certain threshold; or, the receiving node could treat differently the UE feedback depending on the time duration for which the feedback was collected. The present framework, however, does not allow this and forces the receiving node to consider all collected data as equal. This leads to a sub-optimal use of the data, where “noisy” measured UE performance (e.g., data collected during a very short time) can degrade the overall performance of AI/ML processes using this data.

- Without knowledge of the amount of time during which the UE performance was collected, the receiving node is forced to treat all the collected data as equal, which can degrade the overall performance of AI/ML processes.

Therefore, we propose to enhance the UE Associated Info Result Item IE in the DATA COLLECTION UPDATE message with an IE (e.g., Feedback Collection Duration) that indicates the amount of time during which the UE performance was collected. This will help the receiving node to take better decisions and will lead to a more intelligent use of the received data.

- Enhance the UE Associated Info Result Item IE in the DATA COLLECTION UPDATE message with an IE (e.g., Feedback Collection Duration) that indicates the amount of time during which the UE performance was collected.

- Agree to the CR to TS 38.423 in Appendix, which reflects the above proposals.

# 3.	Conclusions and Proposals

In the previous sections we made the following observations and proposals:

Observation 1	A node which reports predictions cannot signal when the conditions, on the basis of which a previous prediction was made, have changed and potentially invalidate the previous prediction. This can lead to bad decisions in the node(s) receiving the predictions.

Observation 2	The amount of time during which the UE performance is collected is a useful indication of the goodness of the collected UE performance.

Observation 3	Without knowledge of the amount of time during which the UE performance was collected, the receiving node is forced to treat all the collected data as equal, which can degrade the overall performance of AI/ML processes.

Proposal 1	Enhance the DATA COLLECTION UPDATE message with an IE (e.g., Previous Prediction Cancel) that indicates whether any of the predictions included in the previous DATA COLLECTION UPDATE message is cancelled.

Proposal 2	Enhance the UE Associated Info Result Item IE in the DATA COLLECTION UPDATE message with an IE (e.g., Feedback Collection Duration) that indicates the amount of time during which the UE performance was collected.

Proposal 3	Agree to the CR to TS 38.423 in Appendix, which reflects the above proposals.

# Appendix - CR to XnAP – Data Collection Reporting

3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, Greece, 17-21 Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement for Data Collection Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Limitations of the Rel-18 framework for reporting of information to support, e.g., AI/ML in NG-RAN: - An NG-RAN node1 receiving periodic predictions from NG-RAN node2 is not informed when a previous prediction is not valid anymore (e.g., due to different present or future network conditions), and it may still use the invalid prediction to take potentially wrong decisions. If the previous prediction is not valid anymore, NG-RAN node1 should avoid using it. - An NG-RAN node1 receiving measured UE performance from NG-RAN node2 is not aware of the conditions under which the UE performance was collected. If NG-RAN node1 knows the amount of time during which the UE performance was collected, it can filter out “noisy” data and improve the overall performance of AI/ML processes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new Previous Prediction Cancel IE in the DATA COLLECTION UPDATE message to inform that a prediction for certain objects included in the previous DATA COLLECTION UPDATE message is cancelled. - Add a new Feedback Collection Duration IE in the UE Associated Info Result Item IE in the DATA COLLECTION UPDATE message to indicate the amount of time during which the UE performance was collected. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the Data Collection Reporting procedure. This CR is BC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear for NG-RAN node1, receiving periodic predictions from NG-RAN node2, how to recognize potentially invalid predictions. It is also unclear for NG-RAN node1, receiving measured UE performance from NG-RAN node2, how understand the relevance of the collected UE performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.14.2, 9.1.3.29, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------ CHANGES START HERE ------------------------------------------

### 8.4.14	Data Collection Reporting

#### 8.4.14.1	General

This procedure is initiated by an NG-RAN node to report information accepted by the NG-RAN node following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.14.2	Successful Operation

Figure 8.4.14.2-1: Data Collection Reporting, successful operation

NG-RAN node2 shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

If the Previous Prediction Cancel IE is present in the DATA COLLECTION UPDATE message, the NG-RAN node1 shall, if supported, consider that predictions received in the previous DATA COLLECTION UPDATE message are cancelled as follows:

-	if the first bit “Predicted Radio Resource Status” of the Previous Prediction Cancel IE is set to 1, the Predicted Radio Resource Status IE received by the NG-RAN node1 in the previous DATA COLLECTION REQUEST message is cancelled.

-	if the second bit “Predicted Number of Active UEs” of the Previous Prediction Cancel IE is set to 1, the Predicted Number of Active UEs IE received by the NG-RAN node1 in the previous DATA COLLECTION UPDATE message is cancelled.

-	if the third bit “Predicted RRC Connections” of the Previous Prediction Cancel IE is set to 1, the Predicted RRC Connections IE received by the NG-RAN node1 in the previous DATA COLLECTION UPDATE message is cancelled.

#### 8.4.14.3	Unsuccessful Operation

Not applicable.

#### 8.4.14.4	Abnormal Conditions

Void.

------------------------------------------ NEXT CHANGE ------------------------------------------

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| >>Feedback Collection Duration | O |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements were collected. Unit: millisecond | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |
| Previous Prediction Cancel | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates a prediction. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

------------------------------------------ NEXT CHANGE ------------------------------------------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

(Skip Unchanged)

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-PreviousPredictionCancel,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

------------------------------------------ NEXT CHANGE ------------------------------------------

-- **************************************************************

--

#### -- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-NGRAN-Node2-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-CellMeasurementResultForDataCollection-List	CRITICALITY ignore	TYPE CellMeasurementResultForDataCollection-List		PRESENCE optional }|

{ ID id-UEAssociatedInfoResult-List					CRITICALITY ignore	TYPE UEAssociatedInfoResult-List							PRESENCE optional }|

{ ID id-NodeAssociatedInfoResult					CRITICALITY ignore	TYPE NodeAssociatedInfoResult								PRESENCE optional }|

{ ID id-PreviousPredictionCancel					CRITICALITY ignore	TYPE PreviousPredictionCancel								PRESENCE optional },

...

}

------------------------------------------ NEXT CHANGE ------------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

(Skip Unchanged)

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-FeedbackCollectionDuration,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

------------------------------------------ NEXT CHANGE ------------------------------------------

-- F

(Skip Unchanged)

FeedbackCollectionDuration ::= INTEGER (1..5000, ...)

------------------------------------------ NEXT CHANGE ------------------------------------------

-- P

(Skip Unchanged)

PreviousPredictionCancel ::= BIT STRING(SIZE(32))

------------------------------------------ NEXT CHANGE ------------------------------------------

-- U

(Skip Unchanged)

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						NG-RANnodeUEXnAPID,

uEPerformance								UEPerformance			OPTIONAL,

measuredUETrajectory						MeasuredUETrajectory	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-FeedbackCollectionDuration					CRITICALITY ignore	EXTENSION FeedbackCollectionDuration						PRESENCE optional },

...

}

------------------------------------------ NEXT CHANGE ------------------------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

(Skip Unchanged)

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

(Skip Unchanged)

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-PreviousPredictionCancel																			ProtocolIE-ID ::= xx1

id-FeedbackCollectionDuration																		ProtocolIE-ID ::= xx2

END

-- ASN1STOP