3GPP TSG-RAN WG3 Meeting #127	R3-250396

Athens, Greece, 17th – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0155 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on QoS Flow List and QoS Flow Identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, ZTE , China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The exisiting QoS Flow Mapping Indication IE is used in MC Bearer Context To Setup Response and BC Bearer Context To Setup Response message. However, this IE is only applied to DRB. Additionally, if applied to MRB, this IE is always to ”dl”. In addition, per TS23.501 and TS38.413, QoS Flow Identifier can be also applied to a MBS QoS flow within a MBS session. Therefore, the definition of QoS Flow List IE and QoS Flow Identifier IE need to be updated to support MRB and MBS QoS flow within a MBS session, respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To update the semantics description of QoS Flow List IE to support MRB in clause 9.3.1.12 - To update the definition of QoS Flow Identifier IE in clause 9.3.1.24 Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on MBS function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The exisiting QoS Flow List IE and QoS Flow Identifier IE not apply to MRB and MBS QoS flow within a MBS session, respectively. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.12, 9.3.1.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.473 CR 1539 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.3.1.12	QoS Flow List

This IE includes a list of QoS Flows that are identified by the QoS Flow Identifier.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow List |  | 1 |  |  | - |  |
| >QoS Flow Item |  | 1..<maxnoofQoSflows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.24 |  | - |  |
| >>QoS Flow Mapping Indication | O |  | 9.3.1.60 | Indicates that only the uplink or downlink QoS flow is mapped to the DRB. If applied to an MRB, the IE is always set to "dl". | YES | ignore |
| >>Data Forwarding Source IP Address | O |  | Transport Layer Address 9.3.2.4 | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | ENUMERATED (active, not active, …) | Indicates whether ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.3.1.24	QoS Flow Identifier

This IE identifies a QoS Flow within a PDU Session, a MBS QoS flow within a MBS session. Definition and use of the QoS Flow Identifier is specified in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0 ..63) |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////