3GPP TSG-RAN WG3 Meeting #127	R3-250403

Athens, Greece, 17 - 21 February 2025

Agenda Item:	15.2

Source:	Huawei

Title:	(TP for TS 36.300 and LS to SA2) Support of Full eNB as Regenerative Payload

Document for:	Discussion

# 1	Introduction

The last RAN plenary meeting approved the WID modification as follows [1]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

  - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In this paper, we further discuss the support of full eNB as regenerative payload based on the updated WID.

# 2	Discussion

## 2.1 eNB only as regenerative payload without CN functions

The last RAN3-126 meeting received the LS from SA2 on the Regenerative-based Satellite Access for E-UTRAN/EPC, where only the eNB is on-board, without any CN components. RAN3 discussed but without conclusion.

1. Overall Description: SA2 thanks for the LS on Support of Regenerative-based Satellite Access. SA2 identified that the LS from RAN3 referred only to “5GS” while in the last paragraph also referred to “AMF/MME”. SA2 also noticed that the RAN WI “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” (RP-242397) did not contain objectives for Regenerative-based Satellite Access for E-UTRAN/EPC. SA2 would like to inform RAN3 that the conclusions for FS_5GSAT_Ph3 for “KI#1: Support of Regenerative-based satellite access” without Store and Forward in clause 8.1 of TR 23.700-29 apply to both E-UTRAN/EPS and NG-RAN/5GS and therefore the related RAN3 impacts can affect both S1 and NG procedures. On the contrary, conclusions for FS_5GSAT_Ph3 for “KI#2: Support of Store and Forward Satellite operation” in clause 8.2 of TR 23.700-29 apply only to E-UTRAN/EPS. 2. Actions: To RAN3, RAN: ACTION: RAN3 and RAN is requested to take the above information into account.

As observed above from the approved WID [1] in the RAN plenary-106 meeting, it becomes clear that the eNB only regenerative payload architecture should be supported.

- Observation 1: The updated WID in RAN plenary meeting was agreed to support the eNB only as regenerative payload architecture.

Then at this meeting, it is proposed for RAN3 to reply to SA2 to indicate the support of the eNB only as regenerative payload, and the agreements made at this meeting could also be contained for SA2 to take into account.

- RAN3 to send reply LS on the eNB only as regenerative payload to SA2, as response to R3-247016/ S2-2410918, containing the potential agreements made at this meeting, the draft is in [5].

Below discuss the details for the eNB only as regenerative payload.

### 2.1.1 S1 removal and suspend procedure

For the regenerative payload agenda in NR NTN, due to the gNB on onboard movement, the NG interface would be discontinued when the gNB moves out of the service area of the AMF. Hence RAN3 agreed to introduce the NG removal procedure.

For eNB on-board regenerative architecture, it makes sense to support the S1 removal procedure taking the same principle as NR NTN. Whether to support NG suspend/resume is still under discussion. Hence the support of S1 suspend/resume can be pending.

- For eNB only as regenerative payload, RAN3 to introduce the S1 removal procedure. Whether S1 suspend/resume is needed is pending on NR NTN conclusion.

### 2.1.2 Feeder link switch

For NR NTN, RAN3#125 had the following agreement:

- RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked

This means the feeder link switch only changes the TNL of the gNB, and the AMF is not changed. This agreement should also apply for the eNB only as regenerative payload.

- For eNB only as regenerative payload, RAN3 assumes that during the feeder link switch, the MME is not changed for the UEs.

Currently the S1 interface supports the TNL recovery using the UE Information Retention IE, supported in the S1 SETUP REQUEST message. Specifically, the eNB can establish the new TNL association and the new S1 interface towards the same MME immediately after the feeder link switch, and include the UE Information Retention IE in the S1 SETUP REQUEST. In this way, the MME will not move the UE to IDLE after the old TNL is shut down, but retains the UE context. When receiving the UE Information Retention IE in the S1 SETUP REQUEST, the MME regards the TNL is recovered, and the UE context is transferred to the new S1 interface. Note that this solution is also documented in the second bullet of solution #35 in SA2 TR 23.700-29 [2]:

---Excerpt from TR 23.700-29--- Scenario 1 (no AMF/MME change): In case of deployment scenario 1, the TNL between gNB/eNB and AMF/MME would change while the gNB/eNB is serving a specific set of cells and Tracking Areas. To update the TNL association between gNB/eNB and AMF/MME different options are possible based on the existing standard: - Support for multiple TNL associations is available in 5GS (i.e. N2), as described in TS 23.501 [2] and TS 38.413 [12]. In this case the gNB can initiate a new TNL association via the new feederlink and remove the old TNL association that was active over the old feederlink. - Support for replacing an old N2/S1 interface with a new N2/S1 interface while preserving the active UE contexts is supported for both N2 and S1, as described in TS 38.413 [12] and TS 36.413 [14]. In this case the gNB/eNB establishes a new SCTP association via the new feederlink and sends a N2/S1 SETUP Request including the UE Retention Information IE. The AMF/MME accepts to retain the existing UE related contexts and signalling connections. The AMF/MME sends a N2/S1 SETUP Response and includes the UE Retention Information IE. - To update the N3/S1-U GTP-U tunnels with the new gNB/eNB TNL address, the gNB/eNB can provide the new tunnel endpoint information (F-TEIDs) to the core network by sending PDU SESSION RESOURCE MODIFY INDICATION / E-RAB MODIFICATION INDICATION. ---End of excerpt---

Hence, for eNB only as regenerative payload, the existing mechanism can support the feeder link switch.

- For eNB only as regenerative payload architecture, the existing mechanism can support the feeder link switch.

## 2.2 Store&Forward mode transition

Previous RAN2 meeting discussed the S&F operation mode transition with the following agreements.

RAN2-127bis agreements: 5. When present, the “S&F operation” indication has two possible settings: ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure). RAN2-128 agreements: - Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

For the transitioning from the S&F mode to normal mode, after the feeder link becomes available, the on-board eNB can access to the CN on the ground, and the UE accessing the on-board eNB after the mode transition works in the normal mode, and served by the CN on the ground. But for those served UEs accessing the on-board eNB before the mode transition, they have been already served by the on-board CN, and there is no need to force the CN changing to the ground for these UEs, since the interfaces between the on-board eNB and on-board CN are still maintained, and the on-board CN is still working for the UEs transitioned from the S&F mode to the normal mode.

Meanwhile SA2 is also the discussing the transition, hence the above should be revisited upon SA2 agreements.

- RAN3 assumes that when the mode transits from the S&F to the normal, no need to switch down the on-board CN. Those UEs accessing the on-board eNB after the mode transition are served by the CN on the ground, and those UEs accessing the on-board eNB before the mode transition could be continuously served by the CN on board. This can be further revisited upon SA2 agreements.

## 2.3 Stage 2 and stage 3 TP/CR

SA2 already agreed to support the Split MME architecture and Full CN on board architecture, where the agreed CR on the general descriptions could be found in [4].

At RAN side, RAN3 can try to agree the stage 3 TP to capture the above proposal, with the introduction of the S1 removal procedure. For stage 2, the basic S&F operation, the general S&F procedures can be captured. And for the eNB only as regenerative payload, the stage 2 can also take very similar change as NR NTN.

- RAN3 to agree the stage 2 TP and stage 3 CR.

# 3	Conclusion

This paper provides analysis for the eNB regenerative, the following observation and proposals are obtained:

- Observation 1: The updated WID in RAN plenary meeting was agreed to support the eNB only as regenerative payload architecture.

- RAN3 to send reply LS on the eNB only as regenerative payload to SA2, as response to R3-247016/ S2-2410918, containing the potential agreements made at this meeting, the draft is in [5].

- For eNB only as regenerative payload, RAN3 to introduce the S1 removal procedure. Whether S1 suspend/resume is needed is pending on NR NTN conclusion.

- For eNB only as regenerative payload, RAN3 assumes that during the feeder link switch, the MME is not changed for the UEs.

- For eNB only as regenerative payload architecture, the existing mechanism can support the feeder link switch.

- RAN3 assumes that when the mode transits from the S&F to the normal, no need to switch down the on-board CN. Those UEs accessing the on-board eNB after the mode transition are served by the CN on the ground, and those UEs accessing the on-board eNB before the mode transition could be continuously served by the CN on board. This can be further revisited upon SA2 agreements.

- RAN3 to agree the stage 2 TP and stage 3 CR.

# 4. Reference

- RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RANP#106.

- R3-247016 Reply LS on Support of Regenerative-based Satellite Access, SA2

- TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)

- S2-2412903 Introduction of Store and Forward feature in EPC, Nokia et al.

- R3-250404, [DRAFT] Reply LS on Support of Regenerative-based Satellite Access

# 5. TP for TS 36.300

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

NTN Gateway: an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link. An NTN Gateway is a TNL node.

NTN payload: a network node, embarked on board a satellite or high altitude platform station, providing connectivity functions, between the service link and the feeder link. In the current version of this specification, the NTN payload may be a TNL node (transparent payload) or a gNB (regenerative payload).

Editor’s Note: Pending RAN2 update, if any.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 4.12	Support of Non-Terrestrial Networks

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Support for non-terrestrial networks encompasses platforms that provide radio access through Geosynchronous orbits (GSO), Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO) or High Altitude Platform Systems (HAPS).

The Figure 4.12-1 and 4.12.1-X below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial access by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 4.12-1 the payload is transparent; in Figure 4.12.1-X the payload is regenerative and hosts a eNB.

Figure 4.12-1: Overall illustration of an NTN with transparent payload

Figure 4.12-1-X: Overall illustration of an NTN with regenerative payload hosting a eNB

NOTE:	Figure 4.12-1 and and 4.12.1-X illustrate an NTN; RAN4 aspects are out of scope.

The NTN transparent payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative payload terminates the Uu interface (via the service link) and the S1 interface (via the feeder link). The following connectivity is supported by the NTN payload:

Editor’s Note: The above section needs further checking.

-	An eNB may serve multiple transparent or regenerative NTN payloads;

-	An NTN payload may be served by multiple eNBs for transparent payload.

NOTE:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

-	A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads;

-	The feeder link is a transport link; it may transport the S1 interface between the MME and the eNB hosted by the regenerative payload;

-	The inter-satellite link is a transport link; it may transport the X2 interface between the eNBs hosted by the two regenerative payloads.

For NTN, the following applies in addition to Network entity related Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 23.21.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 23.21	Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks

### 23.21.1	General

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks (see clause 4.12) is only applicable to E-UTRA connected to EPC. UEs not supporting NTN are barred from accessing an NTN cell.

In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification.

To accommodate long propagation delays in NTN, increased timer values and window sizes, or delayed starting times are supported for the physical layer and for higher layers.

UL segmented transmission is supported for UL transmission with repetitions. The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for BL UEs or UEs in enhanced coverage and NPUSCH/NPRACH for NB-IoT UEs from one segment to the next segment.

In this release of the specification, NTN is only applicable to FDD system.

In NTN, the distance refers to Euclidean distance.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 23.21.5	Switchover

#### 23.21.5.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure.

Both hard and soft feeder link switchover are applicable to transparent or regenerative NTN payloads.

#### 23.21.5.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption may occur during the transition between the feeder links.

#### 23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover between two eNBs is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs' context between the two eNBs at feeder link switchover is performed by means of either S1 based or X2 based handover, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

### 23.21.6	Signalling

The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported in the following cases:

-	The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages;

-	The Cell Identity used for Paging Optimization in S1 interface;

-	The Cell Identity used for PWS.

For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the S1 and X2 handover messages, X2 Setup and X2 eNB Configuration Update procedures is expected to be Uu Cell ID.

The mapping between Mapped Cell ID(s) and geographical area(s) is configured in the RAN and Core Network.

NOTE 1:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate different geographical areas (e.g. overlapping and/or with different dimensions).

The eNB is responsible for constructing the Mapped Cell ID based on the UE location information, if available. The mapping may be pre-configured (e.g., depending on operator's policy) or up to implementation.

NOTE 2:	As described in TS 23.401 [17], the User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE's location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.

### 23.21.aa	S1 Removal

For the case of regenerative payload, when disconnecting from an MME is required due to e.g. the satellite movement in a LEO constellation, the eNB on board the satellite may request the removal of the S1 interface by initiating the S1 Removal procedure toward the MME.

### 23.21.bb	Store and Forward Satellite Operation

The eNB may support the store and forward satellite operation as specified in TS 23.401 [17]. When the eNB is working at the Store and Forward mode, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the ground link when the satellite can exchange data with the core network.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>