3GPP TSG-RAN WG3 Meeting #127	R3-250406

Athens, Greece, 17 - 21 February 2025

Agenda Item:	9.1

Source:	Huawei, Ericsson

Title:	Further discussion on UE Location Information for NB-IoT NTN

Document for:	Discussion

# 1	Introduction

In the previous meeting SA2 sent a LS on the UE Location Information for NB-IoT NTN, with the following contents in [1].

1. Overall Description: SA2 thanks RAN2 and RAN3 for the reply LS on UE Location Information for NB-IoT NTN. SA2 approved the attached CR which allows the MME, if requested from eNB, to send the coarse location information received from the UE to NB-IoT eNB. SA2 nevertheless observed that the CR approved by RAN3 (TS 36.413 CR1957) did not include the “Coarse UE Location Requested” IE in UL NAS Transport, Resume Request and the “Coarse UE Location IE” in Resume Response. Even though Coarse Location Information can be requested once in the Initial UE message also in these cases (and provided to the eNB in a downlink S1-AP message), this prevents the Coarse Location Information of the UE to be requested by eNB and provided from MME to eNB at a later stage during a RRC connection when user plane bearers are established. The relevant procedures in the SA2 specification are “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation” (see clause 5.3.4B.4 of TS 23.401), “Connection Resume procedure” (see clause 5.3.5A) and “MT-EDT procedure for User Plane CIoT EPS Optimisation” (see clause 5.3.5B). SA2 would like to understand if RAN2 and RAN3 has performed any analysis on whether Coarse Location Information is needed by the eNB in any of the above procedures. In case RAN2 and RAN3 conclude that Coarse Location Information is needed in these cases, SA2 would like to highlight that AS security is activated during these procedures which would also allow the UE to provide the Coarse Location Information to the eNB via the RRC protocol. 2. Actions: To RAN2, RAN3: ACTION: SA2 asks RAN2 and RAN3 to take the above information into account and indicate whether Coarse Location Information is needed by the eNB in the procedures defined in clause 5.3.4B.4, 5.3.5A and 5.3.5B.

This paper mainly provides our views on the SA2 LS.

# 2	Discussion

As observed from the LS above, SA2 invites RAN3 to provide feedback on whether the UE coarse location is also needed in other cases except for the case which was the agreed TS 36.413 CR1957. These new cases may include the “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation”, “Connection Resume procedure”, and “MT-EDT procedure for User Plane CIoT EPS Optimisation”.

SA2 is asking whether there is need for the eNB acquiring UE coarse location for these cases. If so, further discussion may be triggered whether the UE coarse location would be transferred first via NAS and then via S1, or directly via RRC since AS security is activated.

From RAN perspective, the main motivation for the eNB to acquire the UE coarse location is used for load balancing. The introduction of UE coase location during intial access was already a big step after pretty tough discussion during the past many meetings, considering the potential huge signalling overlead due to the transfer of the UE coarse location over the satellite link.

Bascially, we think we should stick to the old RAN3 agreements, and consider that current mechanism is sufficient for load balancing when the eNB knows the UE coarse location during the initial access, so that the eNB can decide whether to redirect the initial accessing UE if its location is near the cell border. For other cases, mainly related to CP or the UP CIOT optimizations, there is no further need for the eNB to acquire the coarse UE location.

First it is not really wise for the eNB to request the UE coarse location from the UE afterwards for those cases as listed in the SA2 LS. In this way, the eNB may need to request all its served UEs to report this coarse UE location, then decide which UE(s) should be redirected. This will bring huge signalling overhead over the radio interface.

Second, there is no further need for the CN to provide the coase location to the eNB for these cases. Otherwise, it would also lead to huge signalling overhead not only over the radio interface, but also over the network interface, when the CN gets coarse UE location from the UE via NAS, then provide to the eNB.

Hence, we have the following proposal.

- No need to introduce the UE coarse location in other cases other than the initial access (i.e., sticking to the old RAN3 agreements).

- The draft reply LS can be found in the Annex.

# 3	Conclusion

This paper provides analysis for the SA2 LS, and provides the following proposal and the draft LS.

- No need to introduce the UE coarse location in other cases other than the initial access (i.e., sticking to the old RAN3 agreements).

# 4. Reference

- S2-2412665, Reply LS on UE Location Information for NB-IoT NTN, SA2#166.

# Annex——Reply LS on UE Location Information for NB-IoT NTN

3GPP TSG-RAN3 Meeting #127	TDoc <TDoc#>

Athens, Greece, 17th Feb – 21st Feb, 2025

Title:	[DRAFT] Reply LS on UE Location Information for NB-IoT NTN

Response to:	Reply LS on UE Location Information for NB-IoT NTN (S2-2412665)

Release:	Rel-18

Work Item:		IoT_NTN_enh-Core

Source:	Huawei [will be RAN3]

To:	SA2

Cc:	RAN2, CT1

Contact person:	Philippe Reininger

philippe.reininger@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN3 thanks SA2 for the reply LS on UE Location Information for NB-IoT NTN.

RAN3 concluded that it is sufficient for the eNB to know the UE coarse location in case of the initial access. There is no need to further transfer the UE coarse location for those meintoned procedures wherein.

# 2	Actions

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into account.

# 3	Dates of next RAN3 meetings

RAN3#127bis           	07 – 11, April 2025	China

RAN3#128	19 – 23, May 2025		Malta