3GPP TSG-RAN WG3 Meeting #127	R3-250411

# Athens, Greece, 17th – 21st February, 2025

Title: 	On SON enhancements for Network Slicing

Source: 	Ericsson

Agenda item:	10.3.2

Document Type:	Discussion and decision

# 1	Introduction

The release 19 work item for Data Collection for SON/MDT in NR standalone and MR-DC focuses on enhancements for Rel-17/Rel-18 SON/MDT features such as MRO enhancement for LTM, CHO with candidate SCGs and subsequent CPAC, as well as on SON/MDT enhancements for Intra NTN mobility and Network Slicing [1].

During RAN3#125-bis meeting, RAN2 sent an LS to RAN3 (R3-245762 [2]) describing agreements reached due to workload issues in RAN2. As part of this LS RAN2 indicated that it has deprioritized discussions on SON/MDT for Slicing, and asked RAN3 to prioritize solutions that are RAN3-based.

In the context of discussions in RAN3, Ericsson’s paper to RAN3#125-bis [3] proposed SON enhancements, where Successful Handover Reports are enhanced with user plane interruption times, and a threshold placed on user plane interruption time to be used as an additional trigger for reporting SHR reports to the network. Proposals from other companies [4, 5, 6] also propose other SHR-related enhancements.

It was however agreed that such enhancements to SHR reports will not be pursued by RAN3 as part of Release 19 enhancements, both due to a lack of consensus in the group and due to the received LS from RAN2 which asked RAN3 to avoid any RAN2 impacts in the pursued solutions.

Subsequently to RAN3#126, Ericsson submitted a paper [7] with two cosigners, which proposed that RAN3 investigate network-based solutions to collecting per slice user-plane interruption times. However, the paper was not treated at the meeting. The current paper discusses further the network-based collection of user-plane interruption times.

In the second part of this discussion paper, we focus on the use case of slice coverage observability, and more specifically on the collection of per UE allowed and rejected slice access requests by the gNB. In light of the considerations previously presented for the use case of per-slice user plane interruption times, the solution presented in this paper for collecting rejected slice service requests is also a network-based solution with no UE impact.

# 2	Discussion

# 2.1 Slice user-plane interruption times

The Release 19 discussions on SON/MDT for Network Slicing have so far agreed to introduce Network Slicing related MDT enhancements (in the form of a Network Slice Area Scope) and other logged MDT discussions are underway. It is however important to note that no SON related enhancement has been agreed upon for further normative discussions in RAN3. The RAN2 decision to deprioritise SON/MDT for Slicing is another limiting factor in the solution space that RAN3 can explore to bring SON enhancements in the context of Network Slicing.

Given the interest from different companies in RAN3 to include SON related enhancements in this release, and the mandate from RAN plenary to discuss SON enhancements for Network Slicing, it is therefore proposed that RAN3 should discuss network-based solutions for SON enhancements.

Proposal 1: RAN3 to discuss network-based solutions for improving observability of per slice user-plane interruption times

In our contributions to earlier meetings, we had proposed, with the support of other companies, for the introduction of per slice user-plane interruption times during handovers as an insightful metric in the context of better network slice observability. The user-plane interruption time can be used to identify both situations where there is a slice discontinuity during a handover event, and in addition also obtain observability into the different slice-specific policies that may be active in neighbouring cells where certain slices may be prioritized over another. Therefore, Network-based observability of per slice user-plane interruption times can still provide valuable insights in the context of Network Slicing.

Proposal 2: RAN3 to further discuss network-based approaches to collect per slice user-plane interruption times during handovers.

Measuring the per-slice user-plane interruption time using a network-based approach inherently needs to involve both the source and the target nodes.

At the source node, this may correspond to:

- recording the time of the handover command, or the last transmission/reception time of packets on the user-plane in DL/UL corresponding to different slices.

At the target node, this corresponds to:

- recording an absolute or relative time since the UE arrived at the target node until when user-plane packets were sent or received in the UL/DL corresponding to the different slices.

The recorded time references at either of the nodes must however be exchanged to the other node (either from source to target or from target to source) to compute the user-plane interruption time. In release 18, RAN3 introduced the Data Collection Reporting Initiation and the Data Collection Reporting procedures for exchange of information between neighbor gNBs, including the exchange of UE performance and measured UE trajectory from the target to the source after a handover was executed. The Data Collection Reporting procedure, specifically the mechanisms already agreed for collection of UE performance and UE trajectory to use a framework where the source node can request the target node to collect certain metrics corresponding to a handover and report the collected information via the Data Collection Reporting Update message.

Below we describe how slice-based user-plane interruption times may be computed by extending the Data Collection Reporting procedure.

As a first step, the source node establishes a Data Collection Reporting procedure with the target node by sending a Data Collection Request message which includes the Registration Request for Data Collection IE set to "start". The IE Report Characteristics for Data Collection defines the objects that the target gNB is configured to report on. For the case of measuring slice-level user-plane interruption times, the tenth bit in this IE may be purposed to request the target node to report the time of transmission/reception of packets in DL/UL for a UE that is handed over. Note that this step is part of configuring the Data Collection Reporting procedure between the nodes.

At the time of handover, the source node may include in the handover request a reference to the above configured Data Collection Reporting procedure, e.g., the Data Collection ID, which is used to identify the configuration for collection of data after the UE is handed over. After successful handover completion, the target gNB may then record the timestamps of the first packet transmission/reception in DL/UL per slice and report to the source node as part of the next Data Collection Update message.

The source gNB after receiving the time references may compute the per-slice user-plane interruption times. In certain cases where the UE did not have access to certain slices at the target node, a certain large value may be used as an implicit indication to notify that the slice was unavailable at the target node.

Proposal 3: RAN3 to discuss and agree to extending the Data Collection Reporting Initiation and the Data Collection Reporting procedure to obtain slice-level user-plane interruption times.

A TP reflecting the above extensions to the Data Collection Reporting procedure is presented in the appendix.

# 2.2 Observability of slice access request at the gNB

In NR, network slicing may serve different purposes. On one hand, network slicing may be used to deliver geographically limited services, e.g., a shopping mall, stadium slice where special services may be available in a limited area, or different network-level policies may be in effect to handle the specialized traffic requirement. On the other hand, network slicing may also be used to deliver an enterprise slice, where the slice resources are available at the enterprise location, in addition to the enterprise users accessing the enterprise slice from their homes for example. While configuring a good overlap between the slice coverage and the office premises is rather easy, it may be comparatively more difficult to configure a good slice coverage overlapping with the remote working location of each employee.

Observation 1: Ensuring a good overlap between the slice coverage and the areas in which the authorized UEs should have access to the services provided by the slice requires a good observability of the UEs’ slice service requests at the RAN/OAM.

As a potential solution for the problem described above, the network may attempt to configure all cells in a geographic area to support a certain slice, however, these slices may have zero resources allocated to them on cells where the slice is not supposed to be available. UE requests to access services over the slice with no allocated resources is then known to the RAN. This information may then be logged and can then be used to perform slice re-planning, whereby resources may be allocated to the slice in certain cells in the registration area. The drawback of the above solution is that one or more registration areas may need to advertise support for slices which will likely not be deployed, which has both a configuration overhead and an overhead during execution, i.e., UEs may handover to the cell with an expectation that the slice is supported, but the lack of allocated resources to the slice is only known after the UE tries to access resources on the target cell after the handover. Due to the above drawbacks, logging UE’s service request in the network by advertising slice support with no allocated resources to the slice is not a viable solution.

An alternative solution to the problem is to log the service requests from the UE and the corresponding service response/rejections at the core network. The UE as part of the NAS Registration procedure requests for a list of slices to which the UE may eventually connect to. The core network, depending on UE subscription, current registration area, and other existing policies, may either allow or reject the use of certain slices by the UE. On one hand, the AMF and core network entities are unaware, by design, of cells in the RAN. However, on the other hand, the core network entities are aware of per-UE-level request to access services over a slice. This information is not available at the gNB at a per UE-level as NAS messages are transparent to the gNB.

Observation 2: The information available at the gNB and the AMF may be correlated in order to obtain per-cell per-UE-level slice access requests.

Proposal 4: RAN3 to discuss mechanisms to enable the gNB to obtain per-UE-level information from the AMF on allowed and rejected slice requests.

Once collected, the gNB may then signal information from the AMF, together with other information available at the gNB, e.g., cell-level information to the OAM for logging and as input to future slice re-planning procedures.

# Conclusion

In the previous sections we made the following observations:

Observation 1: Ensuring a good overlap between the slice coverage and the areas in which the authorized UEs should have access to the services provided by the slice requires a good observability of the UEs’ slice service requests at the RAN/OAM.

Observation 2: The information available at the gNB and the AMF may be correlated in order to obtain per-cell per-UE-level slice access requests.

Based on the discussion in the previous sections we propose the following:

Proposal 1: RAN3 to discuss network-based solutions for improving observability of per slice user-plane interruption times into network slicing

Proposal 2: RAN3 to further discuss network-based approaches to collect per slice user-plane interruption times during handovers.

Proposal 3: RAN3 to discuss and agree to extending the Data Collection Reporting Initiation and the Data Collection Reporting procedure to obtain slice-level user-plane interruption times.

Proposal 4: RAN3 to discuss mechanisms to enable the gNB to obtain from the AMF per-UE-level information on allowed and rejected slice requests.

# References

- RP-234038: WID: Data collection for SON/MDT in NR standalone and MR-DC

- LS to RAN3 on way forward on Rel-19 SONMDT

- R3-245396 Discussion on SON enhancements for network slicing (Ericsson)

- R3-245146 Discussion on SON enhancements for network slicing (China Telecom)

- R3-245312 [TP to BL CR to 38.300, MDT] MDT solution for slice support and slice-related mobility enhancements (Nokia)

- R3-245345 Further consideration on SON/MDT for Slicing (ZTE Corporation)

- R3-247497 On SON enhancements for Network Slicing

# TP to TS 38.423

--First change--

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the slice-specific information reporting is requested, the Slice To Report for Data Collection List IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Slice Available Capacity IE, if the tenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if

- the Slice To Report List for Data Collection IE is included for the cell, and

- the measurement object is admitted by NG-RAN node2.

- the per-slice User-plane Transmission Resumption Time IE, if the eleventh bit, “Per-slice User-plane Transmission Resumption Time” of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-- Unmodified changes omitted--

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost, Ninth Bit = Measured UE Trajectory, Tenth Bit = Predicted Slice Available Capacity, Eleventh Bit= Per-slice User-plane Transmission Resumption Time Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  |  | 1 |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Seventh Bit = Per-slice User-plane Transmission Resumption Time. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.1.3.28	DATA COLLECTION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally includes the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | - |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| >> Per-slice User-plane Transmission Resumption Time | O |  | 9.2.3.xxx | Contains information about the timestamps at which per-slice user-plane transmission resumed after the HO |  |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

-- Unmodified changes omitted--

#### 9.2.3.xxx	Per-slice User-plane Transmission Resumption Time

This IE indicates the recorded timestamps at which the first transmission of user-plane packets was observed in UL/DL directions for the different slices in use at the UE after being handed over to the target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Per-slice User-plane Transmission Resumption Time |  | 1..< maxnoofBPLMNs > |  | The recorded timestamps at which the first transmission of user-plane packets was observed in UL/DL directions for each s-NSSAI at the UE |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI UE User-plane Transmission Resumption Time List |  | 1 |  | Indicates the UE performance per network slice |
| >>S-NSSAI UE User-plane Transmission Resumption Time |  | 1 .. < maxnoofs-NSSAI > |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>User-plane transmission Resumption time UL | O |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. |
| >>>User-plane Transmission Resumption Time DL | O |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [37]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofs-NSSAI | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

--End of changes--