3GPP TSG-RAN WG3 Meeting #127	R3-250424

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.1

Source:	CATT

Title:	Discussion on UAV Regulation – Altitude Reporting

Document for:	Decision

- Introduction

In SA2 LS R3-250022[1], it has been confirmed the issue for reporting altitude and velocity for R18 UAV UEs. To address this, RAN3 needs further enhancement to support altitude reporting.

On Case 1, SA2 does not have any requirement dealing with Rel-18 UAV UEs, but SA2 believes current SA2 work for Rel-19 UAS_Ph3 on an unrelated use case requires AMF to get flight information such as altitude and velocity.

SA2 is specifying a new feature “Pre-flight Planning and In-flight Monitoring for UAVs” in clause 5.12, and USS changeover in clause 5.13 in TS 23.256. In case of in-flight monitoring for UAV, USS/UTM, may request for flight assistance information and that network notifies the USS if UAV deviates from the defined flight path including the UAV altitude and velocity.

SA2 agrees that the UAV altitude and velocity can be received from NG-RAN, assuming to reuse the existing feature like eventH1/H2 reporting together with additional trigger from the core network as specified in Rel-19.

Availability of such information is applicable to any UAV UE(s) that support the relevant procedures.

2. Discussion

2.1 SA2 Enhancement and Requirement

We will conduct the analysis based on the SA2 LS and TR 23.256[2]

2. Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point. The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events. NOTE 1: Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting. Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node).

Observation1: SA2 support 3 types of altitude reporting conditions:

- periodically,

- threshold-based(e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided)

- event-based (e.g. change of a tracking area/NG-RAN node)

- 3. Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for UE's altitude reporting to the aerial UE(s).

As indicate above, the CN will send the derived attitude threshold, and optionally, the indication about reporting periodicity to the NG-RAN for aerial altitude reporting. The combination of attitude threshold and reporting periodicity can result in four cases.

If the altitude thresholds, or, both the altitude thresholds and the reporting periodicity (i.e. threshold based) are received, the altitude reporting conditions are met when the aerial UE altitude becomes higher or lower than the thresholds.

If the altitude thresholds and the reporting periodicity (i.e. event based) are received, the altitude reporting conditions are met when the aerial UE altitude becomes higher or lower than the thresholds, and the event happens.

If the altitude thresholds and reporting periodicity (i.e. periodic based) are received, the altitude reporting conditions are met when the aerial UE altitude becomes higher or lower than the thresholds, and each time the periodic period is over..

Observation 2: The altitude reporting conditions are met based on the received thresholds and reporting periodicity: when the aerial UE altitude exceeds or falls below the thresholds, combined with either threshold -based, event-based, or periodic-based reporting criteria.

2.2 Altitude report from UE

In Rel-18, RAN2 introduced altitudeUE-r18 for reporting attitude of UAV. Ccurrently, the altitudeUE-r18 report can be configured as altitude-dependent, event-based measurement reporting (i.e., eventH1 and eventH2 as defined in TS 38.331 [3]) or both an altitude-dependent condition and an RSRP/RSRQ/SINR-based condition are met (i.e., eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1 and eventA5H2 in TS 38.331, commonly denoted as eventAxHy). And after the rigger event satisfied, UE can report attitude periodicity based on the configuration of reportInterval and reportAmount.

eventH1-r18                                SEQUENCE {

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

reportOnLeave-r18                           BOOLEAN,

timeToTrigger-r18                           TimeToTrigger,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventH2-r18                                SEQUENCE {

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

reportOnLeave-r18                           BOOLEAN,

timeToTrigger-r18                           TimeToTrigger,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

-------------------------------------------skipped-----------------------------------------------

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

Observation 3: Based on 38.331, RAN2 support Attitude report via altitudeUE-r18, and currently support altitude-dependent condition report (i.e., eventH1 and eventH2 ).

Observation 4: After the trigger event is satisfied, UE can report attitude periodicity based on the configuration of reportInterval and reportAmount.

2.4 Resolve Misalignment by RAN3

Based on the Observation1, 2 and Observation3, 4, it is evident that the requirements from SA2 and the reporting mechanisms implemented by RAN2 are not well-aligned. The primary discrepancy is that SA2 supports altitude reporting via SA2 defined event-based (e.g. change of a tracking area/NG-RAN node.)

Given the companies' desire to avoid any impact on UE capabilities following the completion of the RAN2 work item (WI) for UAV altitude reporting, it is preferable to address the misalignment between SA2 requirements and RAN2 mechanisms in RAN3. One potential solution is as follows: when the Core Network (CN) configures reporting periodicity as event-based to the NG-RAN node, the NG-RAN node can convert this into periodic reporting for the UE. Additionally, the NG-RAN node can filter the received altitude information and then report it to the CN.

If the altitude thresholds and reporting periodicity (i.e. event based) are received, the NG-RAN node shall configure the altitude thresholds, reportInterval and reportAmount to the UE. The UE shall initiate periodic reporting of altitudeUE-r18 based on the configured reportInterval. Upon receiving the altitudeUE-r18 reports from the UE, the NG-RAN node shall determine whether the SA2-defined trigger event (e.g., a change in tracking area or NG-RAN node) has occurred. The NG-RAN node shall only report the altitude to the CN if the trigger event is satisfied. In this case, potential impact including (1) reuse Area of Interest List IE in LOCATION REPORTIN CONTROL.(2) reuse “change of serving cell and UE presence in the area of interest” type or introduce a new “event based” type in Location Reporting Request Type IE. (3) Enhance UE location REPORT message to including UAV attitude IE.

Proposal 1: If the altitude thresholds and reporting periodicity (i.e. event based) are received from CN, NG-RAN node shall convert event based into reportInterval and reportAmount and configure to UE. The NG-RAN node shall only report the altitude to the CN if the trigger event is satisfied

If the altitude thresholds or the altitude thresholds and the reporting periodicity (i.e. threshold based) are received, NG-RAN node configured the altitude thresholds to UE, and report the received altitudeUE-r18 directly. In this case, potential impact including (1) add threshold IE in LOCATION REPORTIN CONTROL (2) introduce a new “threshold-based” type or reuse “direct” type in Location Reporting Request Type IE (3) Enhance UE location REPORT message to including UAV attitude IE.

Proposal 2: If the altitude thresholds or the altitude thresholds and the reporting periodicity (i.e. threshold based) are received, NG-RAN node shall configure the altitude thresholds to UE.

If the altitude thresholds and the reporting periodicity (i.e. periodic based) are received, the NG-RAN node shall configure the altitude thresholds and the periodic (Convert to the form of reportInterval and reportAmount) to UE. In this case, potential impact including (1) add threshold IE and periodic IE in LOCATION REPORTIN CONTROL (2) introduce a new “event-based” type or reuse “direct” type in Location Reporting Request Type IE (3) Enhance UE location REPORT message to including UAV attitude IE.

Proposal 3: If the altitude thresholds and the reporting periodicity (i.e. periodic based) are received, the NG-RAN node shall configure the altitude thresholds and the periodic (Convert to the form of reportInterval and reportAmount) to UE.

Proposal 4: Based on the above analyse, and try to minimize the impact in RAN3, the potential impact can be summary as below:

- Add threshold IE and periodic IE in LOCATION REPORTIN CONTROL message.

- Reuse “direct” for threshold-based and periodic altitude report, and reuse “change of serving cell and UE presence in the area of interest” for SA2 defined event-based report.

- Enhance User Location Information IE in UE location REPORT message to including UAV attitude IE

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation1: SA2 support 3 types of altitude reporting conditions:

- periodically,

- threshold-based(e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided)

- event-based (e.g. change of a tracking area/NG-RAN node)

Observation 2: The altitude reporting conditions are met based on the received thresholds and reporting periodicity: when the aerial UE altitude exceeds or falls below the thresholds, combined with either threshold -based, event-based, or periodic-based reporting criteria.

Observation 3: Based on 38.331, RAN2 support Attitude report via altitudeUE-r18, and currently support altitude-dependent condition report (i.e., eventH1 and eventH2 ).

Observation 4: After the trigger event is satisfied, UE can report attitude periodicity based on the configuration of reportInterval and reportAmount.

Proposal 1: If the altitude thresholds and reporting periodicity (i.e. event based) are received from CN, NG-RAN node shall convert event based into reportInterval and reportAmount and configure to UE. The NG-RAN node shall only report the altitude to the CN if the trigger event is satisfied

Proposal 2: If the altitude thresholds or the altitude thresholds and the reporting periodicity (i.e. threshold based) are received, NG-RAN node shall configure the altitude thresholds to UE.

Proposal 3: If the altitude thresholds and the reporting periodicity (i.e. periodic based) are received, the NG-RAN node shall configure the altitude thresholds and the periodic (Convert to the form of reportInterval and reportAmount) to UE.

Proposal 4: Based on the above analyse, and try to minimize the impact in RAN3, the potential impact can be summary as below:

- Add threshold IE and periodic IE in LOCATION REPORTIN CONTROL message.

- Reuse “direct” for threshold-based and periodic altitude report, and reuse “change of serving cell and UE presence in the area of interest” for SA2 defined event-based report.

- Enhance User Location Information IE in UE location REPORT message to including UAV attitude IE

4. Reference

[1]	R3-250022, Reply LS on UAV regulation

[2]	TS 23.256, "Unmanned Aircraft System (UAS) Connectivity, Identification and Tracking,"

[3]	TS 38.331, NR; Radio Resource Control (RRC) protocol specification