3GPP TSG-RAN WG3 Meeting #127	R3-250445

Athens, Greece, 17th – 21st February 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for BL CR 38.413] NGAP protocol elemets for A-IoT

Document for:	Discussions & Approval

# 1	Introduction

This document discusses stage-3 related specification consequences of the tasks given to RAN3 in the WID approved in RP-243326:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

(with the location reporting handled in a separate paper for Agenda Item 16.3)

# 2	Discussion

Building on the architecture principles developed in R3-250444, we need to analyse whether the main NG interface management function, the NG Setup procedure, can be made applicable for all connectivity options.

Assuming an NG-RAN node, i.e. an A-IoT enabled gNB solely deployed for support of A-IoT, there are of course many items in the respective messages that cannot be applied for A-IoT. Nevertheless, the main IEs, node identification and name, and the supported TA List can be made applicable for A-IoT in a way that routing of A-IoT signalling traffic based on Tracking Areas throughout the A-IoT enabled 5GS can be utilised as it is utilised for legacy 5GS functions, especially for the indirect connectivity option: A TAI configured to to signify that A-IoT enabled gNB can indeed be used for the indirect connectivity option, with the understanding that only part of the information configured in NG-RAN and 5GC and exchanged over NG-C (if at all) is used to be communicated to A-IoT devices.

Proposal 1:	Agree to re-use the existing NG Setup procedure for establishing the NG-C interface between an A-IoT enabled gNB and an A-IoT enabled AMF or AIOTF with a TAI configured for the sole purpose of signifying an A-IoT enabled gNB. Applicablity of existing IEs need to be further discussed and inform SA2 about this approach (see draft LS in R3-250446).

Indicating the support of A-IoT within the NG Setup Request and Response messages would need to be introduced – it appears to be appropriate to do that within the Supported TA Item in the Request message and on main message level in the Response message. Non-support of A-IoT should lead to failing the setup procedure. Whether definition of support indications should allow introduction of several (potentially non-backwards compatible) versions of A-IoT (e.g. “short-range” as in Rel-19 for passive and simple devices and “active device support” for long-range/outdoor devices in later releases) can be further debated and would impact, from a protocol point of view the code-point name and semantics for the A-IoT support indications.

Proposal 2:	Introduce in the NG Setup Request message a new RAT Information code-point for A-IoT, in the Response message an A-IoT support indication and further discuss whether Rel-19 should foresee the extension of these indications for future, potentially non-backwards compatible versions of A-IoT.

Indicating the support of A-IoT within the NG Setup Request and Response messages would need to be introduced – it appears to be appropriate to do that within the Supported TA Item in the Request message and on main message level in the Response message. Non-support of A-IoT should lead to failing the setup procedure. Whether definition of support indications should allow introduction of several (potentially non-backwards compatible) versions of A-IoT (e.g. “short-range” as in Rel-19 for passive and simple devices and “active device support” for long-range/outdoor devices in later releases) can be further debated and would impact, from a protocol point of view the code-point name and semantics for the A-IoT support indications.

Proposal 3:	Introduce in the NG Setup Request message a new RAT Information code-point for A-IoT, in the Response message an A-IoT support indication and further discuss

Based on the common view on NG-C signalling, it can be observed that the Inventory and Command procedure consist each of

-	a class-1 procedure of preparing the A-IoT transaction, where the successful outcome of this preparation indicates that the A-IoT enabled gNB has sufficient resources to perform the transaction

-	a class-2 proceudure to report the outcome of the A-IoT transaction after having contacted the A-IoT device.

we propose the following:

Proposal 4:	Introduce a class-1 A-IoT Inventory Preparation and a class-1 A-IoT Command Preparation procedure, as well as class-2 procedures for A-IoT Inventory Report and A-IoT Command Report.

In order to specify a complete set of necessary procedures for A-IoT transaction management RAN and CN initiated transaction release procedures would need to be introduced, similar to the e.g. UE Context management procedures. It could be of course also discussed whether the report procedures include a kind of “end-of-transaction” flag, but having an explicit protocol solution at hand to avoid defining an empty report PDU seems to be a much cleaner approach.

Proposal 5:	Introduce A-IoT transaction release procedures, a CN triggered class-1 A-IoT Release procedure and a RAN triggered class 2 A-IoT Release Required procedure.

As shortly discussed in the paper on architectural aspects in R3-250444, both connectivity options between the A-IoT enabled gNB and the AIOTF needs to be supported. With the understanding, the for A-IoT transaction management procedures the connectivity options are not recognisable from an NG-RAN point of view, we propose to adopt the same approach as for SMF related information in case of PDU Session Resource management procedures, i.e. to define A-IoT transaction specific containers within NGAP, which are transparent to the AMF in case of the indirect connectivity option.

Proposal 6:	Introduce A-IoT transaction specific containers in NGAP, which are transparent to the AMF in case the indirect connectivity option is deployed. Which A-IoT specific message needs such container and which information needs to be processed by the AMF is FFS and inform SA2 about this decision (see draft LS in R3-250446).

In order to allow A-IoT transaction stemming from different AIOTFs and to allow an AIOTF to repeat A-IOT transactions concerning a range/group of A-IoT devices (e.g. in case insufficient results were received by the AIOTF), it has been discussed to introduce an A-IoT Transaction ID, which is defined appropriately to allow A-IoT devices distinguishing requests from different AIOTFs as well as recognising repeated transaction requests from the same AIOTF, which would require the A-IoT Transaction ID to be allocated accordingly. We propose to agree on such transaction identifier and liaise SA2 and RAN2 to further comment and further elaborate on that approach.

Proposal 7:	Introduce an A-IoT Transaction ID that enables A-IoT enabled gNBs and A-IoT devices distinguishing requests from different AIOTFs as well as recognising repeated transaction requests from the same AIOTF. Liaise SA2 and RAN2 to comment on that approach. (see draft LS in R3-250446).

We have discussed in R3-250442 the possibility to define the A-IoT Transaction Area by means of A-IoT Service Areas. Further possibilities would be to either not indicate any area at all (i.e. the transaction scope is the whole A-IoT enabled gNB), a list of A-IoT Reader IDs (in case the AIOTF has collected sufficient evidence to reduce the transaction scope to a couple of Reader IDs), and some additional possibilities which may be added in later Releases. Whether the A-IoT Transaction Area should allow the combination of these (and future) possibilities or only one possibility could be further discussed. A further aspect is whether the same A-IoT Transaction Area definition should be used on a per transaction level and on a per device/device-group level within the Inventory/Command Request message.

Proposal 8:	Introduce the A-IoT Transaction Area containing the possibility to indicate A-IoT Service Areas as discussed in R3-250442. Further discuss whether an A-IoT Transaction Area should contain further options, whether a combination of these possibilities should be allowed and whether an A-IoT Transaction Area should be indicated in transaction (i.e. message) level only or also on device/device-group level.

As a final item, information concerning the A-IoT device identification and A-IoT Assistance information is left (with respect to the text proposal in the next section).

One item to be discussed would be whether the A-IoT enabled gNB would need to distinguish between A-IoT device identifiers concerning a single device and identifiers concerning a group of devices. Discussions on potential A-IoT Assistance information concerned indicating potentially the number of responses to be expected in case the A-IoT device ID is associated with a group of devices, which could serve as an indication for a group ID (a specific code-point could indicate that such information is not available, but still multiple responses are possible).

Further A-IoT assistance information may concern the expected Application Layer PDU for which respective radio resources have to be prepared and other items.

As these discussions are also related to items handled in other groups, we suggest to only include the respective protocol items in the text proposal but leave further details to progress in other groups and come back to them in the next meeting.

Proposal 8:	Introduce placeholders for A-IoT device identification and A-IoT assistance information and review progress in other groups at the next meeting.

Final proposal:	Agree on the text proposals for TS 38.410 and TS 38.413 in the next chapter.

# 3	Text Proposals

## 3.1	Text Proposal for TS 38.410

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 4.4	NG interface capabilities

The NG interface supports:

-	procedures to establish, maintain and release NG-RAN part of PDU sessions;

-	procedures to perform intra-RAT handover and inter-RAT handover;

-	the separation of each UE on the protocol level for user specific signalling management;

-	the transfer of NAS signalling messages between UE and AMF;

-	mechanisms for resource reservation for packet data streams;

-	procedures to establish, maintain and release NG-RAN part of MBS sessions;

-	procedures to support A-IoT for Inventory and Command.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 5.x1	A-IoT Function

The A-IoT functions supports communication between NG-RAN and 5GC for performing Inventory and Command.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 6.x2	A-IoT Procedures

The following list of A-IoT procedures are supported:

-	A-IoT Inventory Preparation;

-	A-IoT Inventory Report;

-	A-IoT Command Preparation;

-	A-IoT Command Report;

-	A-IoT Transaction Release;

-	A-IoT Transaction Release Required.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

## 3.2	Text Proposal for TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| A-IoT Inventory Preparation | AIOT INVENTORY REQUEST | AIOT INVENTORY RESPONSE | AIOT INVENTORY REJECT |
| A-IoT Command Preparation | AIOT COMMAND REQUEST | AIOT COMMAND RESPONSE | AIOT COMMAND REJECT |
| A-IoT Release | AIOT RELEASE COMMAND | AIOT RELEASE COMPLETE |  |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| RAN Paging Request | RAN PAGING REQUEST |
| A-IoT Inventory Report | AIOT INVENTORY REPORT |
| A-IoT Command Report | AIOT COMMAND REPORT |
| A-IoT Transaction Release Required | AIOT TRANSACTION RELEASE REQUIRED |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

If the NG-SETUP REQUEST message contains the RAT Information IE set to "A-IoT-short-range", the AMF shall, if supported, include the A-IoT Supported IE set to "A-IoT-short-range" in the NG SETUP RESPONSE message.

Editor’s Note:	code-point-names FFS

#### 8.7.1.3	Unsuccessful Operation

Figure 8.7.1.3-1: NG setup: unsuccessful operation

If the AMF cannot accept the setup, it should respond with an NG SETUP FAILURE message and appropriate cause value.

If the NG SETUP FAILURE message includes the Time to Wait IE, the NG-RAN node shall wait at least for the indicated time before reinitiating the NG Setup procedure towards the same AMF.

#### 8.7.1.4	Abnormal Conditions

If the AMF does not identify any of the PLMNs/SNPNs indicated in the NG SETUP REQUEST message, it shall reject the NG Setup procedure with an appropriate cause value.

If none of the RATs indicated by the NG-RAN node in the NG SETUP REQUEST message is supported by the AMF, then the AMF shall fail the NG Setup procedure with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.6.2	NG SETUP RESPONSE

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Served GUAMI List |  | 1 |  |  | YES | reject |
| >Served GUAMI Item |  | 1..<maxnoofServedGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |
| >>Backup AMF Name | O |  | AMF Name 9.3.3.21 |  | - |  |
| >>GUAMI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Relative AMF Capacity | M |  | 9.3.1.32 |  | YES | ignore |
| PLMN Support List |  | 1 |  |  | YES | reject |
| >PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Slice Support List | M |  | 9.3.1.17 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support | O |  | 9.3.3.44 | If NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>Extended Slice Support List | O |  | 9.3.1.191 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >>Onboarding Support | O |  | ENUMERATED (true, ...) | Indication of onboarding support. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| Mobile IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for mobile IAB. | YES | ignore |
| A-IoT Support Information | O |  | 9.3.1.w1 | Indication of support for A-IoT. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedGUAMIs | Maximum no. of GUAMIs served by an AMF. Value is 256. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.125	RAT Information

This IE provides RAT related information associated with a TAC, used as described in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAT Information | M |  | ENUMERATED (unlicensed, nb-IoT, ..., nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT, A-IoT-short-range) | FFS on code-point name. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.w1	A-IoT Support Information

This IE provides support information for A-IoT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Support Information | M |  | ENUMERATED (A-IoT-short-range, ...) | FFS on coding and code-point name. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.x1	A-IoT Transaction Management Procedures

### 8.x1.1	A-IoT Inventory Preparation

#### 8.x1.1.1	General

The purpose of the A-IoT Inventory Preparation procedure is to request the NG-RAN node to prepare inventory of one or several A-IoT devices. The procedure uses non-UE associated signalling.

#### 8.x1.1.2	Successful Operation

Figure 8.x1.1.2-1: A-IoT Inventory Preparation, successful operation.

The AMF initiates the procedure by sending a AIOT INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the AIOT INVENTORY RESPONSE message.

If the A-IoT Command Follow On IE set to "true" is included in the AIoT Inventory Request Transfer IE contained in the AIOT INVENTORY REQUEST message, the NG-RAN node shall act as specified in TS 38.300 [8].

If the A-IoT Assistance Information IE is included in the AIoT Inventory Request Transfer IE contained in the AIOT INVENTORY REQUEST message, the NG-RAN node may use it as specified in TS 38.300 [8].

If the NG-RAN node is able to perform the requested A-IoT inventory it shall send the AIOT INVENTORY RESPONSE message to the AMF.

#### 8.x1.1.3	Unsuccessful Operation

Figure 8.x1.1.3-1: A-IoT Inventory Preparation, unsuccessful operation.

If the NG-RAN node is not able to perform the requested A-IoT inventory it shall send the AIOT INVENTORY REJECT message to the AMF.

#### 8.x1.1.4		Abnormal Conditions

Void.

### 8.x1.2	A-IoT Inventory Report

#### 8.x1.2.1	General

The purpose of the A-IoT Inventory Report procedure is to report to the AMF inventory results. The procedure uses non-UE associated signalling.

#### 8.x1.2.2	Successful Operation

Figure 8.x1.2.2-1: Broadcast Session Release Required, successful operation.

The NG-RAN node initiates the procedure by sending a AIOT INVENTORY REPORT message to the AMF.

#### 8.x1.2.3	Abnormal Conditions

Void.

### 8.x1.3	A-IoT Command Preparation

#### 8.x1.3.1	General

The purpose of the A-IoT Command Preparation procedure is to request the NG-RAN node to prepare command towards one or several A-IoT devices. The procedure uses non-UE associated signalling.

#### 8.x1.3.2	Successful Operation

Figure 8.x1.3.2-1: A-IoT Command Preparation, successful operation.

The AMF initiates the procedure by sending a AIOT COMMAND REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the AIOT COMMAND RESPONSE message.

If the A-IoT Assistance Information IE is included in the AIoT Command Request Transfer IE contained in the AIOT COMMAND REQUEST message, the NG-RAN node may use it as specified in TS 38.300 [8].

If the NG-RAN node is able to perform the requested A-IoT command it shall send the AIOT COMMAND RESPONSE message to the AMF.

#### 8.x1.3.3	Unsuccessful Operation

Figure 8.x1.3.3-1: A-IoT Inventora Preparation, unsuccessful operation.

If the NG-RAN node is not able to perform the requested A-IoT command it shall send the AIOT COMMAND REJECT message to the AMF.

#### 8.x1.3.4		Abnormal Conditions

Void.

### 8.x1.4	A-IoT Command Report

#### 8.x1.4.1	General

The purpose of the A-IoT Command Report procedure is to report to the AMF command results. The procedure uses non-UE associated signalling.

#### 8.x1.4.2	Successful Operation

Figure 8.x1.4.2-1: A-IoT Command Report, successful operation.

The NG-RAN node initiates the procedure by sending a AIOT COMMAND REPORT message to the AMF.

#### 8.x1.4.3	Abnormal Conditions

Void.

### 8.x1.5	A-IoT Transaction Release

#### 8.x1.5.1	General

The purpose of the A-IoT Transaction Release procedure is to request the NG-RAN node to release an ongoing A-IoT transaction. The procedure uses non-UE associated signalling.

#### 8.x1.5.2	Successful Operation

Figure 8.x1.5.2-1: A-IoT Transaction Release, successful operation.

The AMF initiates the procedure by sending a AIOT TRANSACTION RELEASE REQUEST message to the NG-RAN node.

Upon reception of the AIOT TRANSACTION RELEASE REQUEST message, the NG-RAN node shall stop performing any action related to the indicated A-IoT transaction, release all related resources and respond with the AIOT TRANSACTION RELEASE RESPONSE message.

#### 8.x1.5.3	Unsuccessful Operation

Not applicable.

#### 8.x1.5.4	Abnormal Conditions

Void.

### 8.x1.6		A-IoT Transaction Release Required

#### 8.x1.6.1	General

The purpose of the A-IoT Transaction Release Required procedure is to trigger the AMF to release the indicated ongoing A-IoT transaction. The procedure uses non-UE associated signalling.

#### 8.x1.6.2	Successful Operation

Figure 8.x1.6.2-1: A-IoT Transaction Release Required, successful operation.

The NG-RAN node initiates the procedure by sending an AIOT TRANSACTION RELEASE REQUIRED message to the AMF.

Upon reception of the AIOT TRANSACTION RELEASE REQUIRED message, the AMF shall realize that the NG-RAN node has stopped performing any further action related to the indicated A-IoT transaction and initiate the release of the A-IoT transaction.

#### 8.x1.6.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.x2		A-IoT Transaction Management Messages

#### 9.2.x2.1	AIOT INVENTORY REQUEST

This message is sent by the AMF to trigger the preparation of an A-IoT inventory.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| A-IoT Transaction Area | M |  | 9.3.1.y2 |  | YES | reject |
| A-IoT Inventory Request Transfer | M |  | OCTET STRING | Containing the A-IoT Inventory Request Transfer IE specified in subclause 9.3.x3.1 | YES | reject |

#### 9.2.x2.2	AIOT INVENTORY RESPONSE

This message is sent by the NG-RAN node to report the successful outcome of the request from the AIOT INVENTORY REQUEST message..

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| A-IoT Inventory Response Transfer (FFS) | M |  | OCTET STRING | Containing the A-IoT Inventory Response Transfer IE specified in subclause 9.3.x3.2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x2.3	AIOT INVENTORY REJECT

This message is sent by the NG-RAN node to report the unsuccessful outcome of the request from the AIOT INVENTORY REQUEST message..

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| A-IoT Inventory Reject Transfer (FFS) | M |  | OCTET STRING | Containing the A-IoT Inventory Reject Transfer IE specified in subclause 9.3.x3.3 | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x2.4	AIOT INVENTORY REPORT

This message is sent by the NG-RAN node to report the inventory results.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| A-IoT Inventory Report Transfer | M |  | OCTET STRING | Containing the A-IoT Inventory Report Transfer IE specified in subclause 9.3.x3.4 | YES | reject |

#### 9.2.x2.5	AIOT COMMAND REQUEST

This message is sent by the AMF to trigger the preparation of an A-IoT command.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| A-IoT Transaction Area | M |  | 9.3.1.y2 |  | YES | reject |
| A-IoT Command Request Transfer | M |  | OCTET STRING | Containing the A-IoT Command Request Transfer IE specified in subclause 9.3.x3.5 | YES | reject |

#### 9.2.x2.6	AIOT COMMAND RESPONSE

This message is sent by the NG-RAN node to report the successful outcome of the request from the AIOT COMMAND REQUEST message..

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| A-IoT Command Response Transfer (FFS) | M |  | OCTET STRING | Containing the A-IoT Command Response Transfer IE specified in subclause 9.3.x3.6 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x2.7	AIOT COMMAND REJECT

This message is sent by the NG-RAN node to report the unsuccessful outcome of the request from the AIOT COMMAND REQUEST message..

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| A-IoT Command Reject Transfer (FFS) | M |  | OCTET STRING | Containing the A-IoT Command Reject Transfer IE specified in subclause 9.3.x3.7 | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x2.8	AIOT COMMAND REPORT

This message is sent by the NG-RAN node to report the command results.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| A-IoT Command Report Transfer | M |  | OCTET STRING | Containing the A-IoT Command Report Transfer IE specified in subclause 9.3.x3.8 | YES | reject |

#### 9.2.x2.9	AIOT TRANSACTION RELEASE REQUEST

This message is sent by the AMF to release an ongoing A-IoT transaction.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.x2.10	AIOT TRANSACTION RELEASE RESPONSE

This message is sent by the NG-RAN node to acknowledge the AIOT TRANSACTION RELEASE REQUEST message.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x2.11	AIOT TRANSACTION RELEASE REQUIRED

This message is sent by the NG-RAN node to trigger the AMF to initiate the release of an onging A-IoT transaction.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Transaction ID | M |  | 9.3.1.y1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.y1	A-IoT Transaction ID

This IE is used to identify an A-IoT transaction.

Editor’s Note:	Further details on A-IoT Transaction ID are FFS (e.g. scope, where to specify etc.).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Transaction ID | M |  | FFS |  |

#### 9.3.1.y2	A-IoT Transaction Area

This IE is used to identify the are within which an A-IoT transaction shall be performed.

Editor’s Note:	Further details on A-IoT Transaction Area FFS, also whether this IE can be associated to a whole A-IoT transaction as well as on a per A-IoT Device (group) level.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Transaction Area | M |  | FFS |  |

#### 9.3.1.y3	A-IoT Device Identifier

This IE is used to identify an A-IoT device or a group of A-IoT devices.

Editor’s Note:	Further details on A-IoT Device Identifier FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Device Identifier | M |  | FFS |  |

#### 9.3.1.y4	A-IoT Assistance Information

This IE contains assistance information to support the NG-RAN node performing the A-IoT transaction, as specified in TS 38.300 [8].

Editor’s Note:	Further details on A-IoT Assistance Information FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Assistance Information | M |  | FFS |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.x3.1	A-IoT Inventory Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Device ID Inventory Request List |  | 1 |  |  | YES | reject |
| >A-IoT Device ID Inventory Request Item |  | 1..<maxnoofAIoTDeviceIDs> |  |  | EACH | reject |
| >>A-IoT Device Identifier | M |  | 9.3.1.y3 |  | - |  |
| >>A-IoT Transaction Area | M |  | 9.3.1.y2 |  | - |  |
| >>A-IoT Assistance Information | O |  | 9.3.1.y4 |  | - |  |
| A-IoT Command Follow On | O |  | ENUMERATED (true, ...) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTDeviceIDs | Maximum no. of A_IoT device identifiers. Value is 256. (FFS) |

#### 9.3.x3.2	A-IoT Inventory Response Transfer

Void.

Editor’s Note:	Any information necessary to be included in this container is FFS.

#### 9.3.x3.3	A-IoT Inventory Reject Transfer

Void.

Editor’s Note:	Any information necessary to be included in this container is FFS.

#### 9.3.x3.4	A-IoT Inventory Report Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Device ID Inventory Report List |  | 1 |  |  | YES | reject |
| >A-IoT Device ID Inventory Report Item |  | 1..<maxnoofAIoTDeviceIDs> |  |  | EACH | reject |
| >>A-IoT Device Identifier | M |  | 9.3.1.y3 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTDeviceIDs | Maximum no. of A_IoT device identifiers. Value is 256. (FFS) |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.x3.5	A-IoT Command Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Device ID Command Request List |  | 1 |  |  | YES | reject |
| >A-IoT Device ID Command Request Item |  | 1..<maxnoofAIoTDeviceIDs> |  |  | EACH | reject |
| >>A-IoT Device Identifier | M |  | 9.3.1.y3 |  | - |  |
| >>A-IoT Transaction Area | M |  | 9.3.1.y2 |  | - |  |
| >>A-IoT Command PDU | M |  | OCTET STRING |  | - |  |
| >>A-IoT Assistance Information | O |  | 9.3.1.y4 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTDeviceIDs | Maximum no. of A_IoT device identifiers. Value is 256. (FFS) |

#### 9.3.x3.6	A-IoT Command Response Transfer

Void.

Editor’s Note:	Any information necessary to be included in this container is FFS.

#### 9.3.x3.7	A-IoT Command Reject Transfer

Void.

Editor’s Note:	Any information necessary to be included in this container is FFS.

#### 9.3.x3.8	A-IoT Command Report Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Device ID Command Report List |  | 1 |  |  | YES | reject |
| >A-IoT Device ID Command Report Item |  | 1..<maxnoofAIoTDeviceIDs> |  |  | EACH | reject |
| >>A-IoT Device Identifier | M |  | 9.3.1.y3 |  | - |  |
| >>A-IoT Command PDU | M |  | OCTET STRING |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTDeviceIDs | Maximum no. of A_IoT device identifiers. Value is 256. (FFS) |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>