3GPP TSG-RAN WG3 Meeting #127	R3-250449

# Athens, Greece, 17th – 21st February, 2025

Title: 	Discussion on multiple Xn Setup procedures

Source: 	Ericsson

Agenda item:	9.2

Document Type:	Discussion and decision

# 1. Introduction

During RAN3-126, a discussion on how to handle duplicated Xn Setup procedures was taken.

During such discussions, the following understanding was brought forward:

In case an additional Xn setup procedure is triggered for an already established Xn-C interface instance, it shall not impact the already established Xn-C interface instance regardless if the additional Xn setup procedure is successful or failed

However, the group could not converge on any specific solution for the problem identified.

This paper continues such discussions and proposes a way forward.

# 2. Discussion

It is common understanding that an Xn interface is considered operational after an Xn Setup procedure is successfully completed, and that a single Xn Setup Procedure shall be carried out to trigger the establishment of an Xn interface.

This is to a certain extent supported by the following note in TS38.423:

NOTE 1: If Xn-C signalling transport is shared among multiple Xn-C interface instances, one Xn Setup procedure is issued per Xn-C interface instance to be setup, i.e. several Xn Setup procedures may be issued via the same TNL association after that TNL association has become operational.

In some deployment scenarios it has been seen that, after an Xn Setup procedure has been successfully carried out between a source and a target gNB, a second Xn Setup procedure is started by the target gNB. 
As acknowledged by the note above, the latter should not occur. However, the standard does not specify how a node receiving such second Xn Setup Request should behave.

It should be noted that, after the first Xn Setup procedure terminates successfully, an Xn signalling connection is in place. Any solution to resolve the issue of duplicated Xn setup procedures should ensure that an Xn signalling connection can be reliably established and not be removed at the occurrence of duplicated Xn setup procedures, as that will imply failure to reach a stable Xn signalling connection. At the same time, any solution for this problem should ensure that duplicated Xn Setup procedures would not occur any longer, as that would imply wasting of signalling and processing power, which risks to generate error cases.

One option discussed to address this issue has been to ignore a second Xn Setup Request message received after an Xn interface instance is successfully established. However, this approach could lead to further issues, such as retransmission by the node originating the second Xn Setup Request. Such retransmissions, if constantly ignored by the receiving node, could be repeated indefinitely, causing resource wastage over transport and at processing level. Such case could also derive in error cases at the node initiating the duplicate Xn Setup procedure, as such node might assume that the receiving node is subject to failure (as it is not responding).

Observation 1: Allowing the standard to acknowledge that an additional Xn Setup Request message, triggered after an Xn-C interface instance is already established, shall be ignored by the receiver implies unnecessary increase of signalling and processing load and it might result in failure cases.

Another approach presented to solve the problem could be to fail the second Xn Setup procedure.

Indeed, this solution would work if failing of the second (duplicate) Xn Setup procedure would occur without triggering any impact on the already established Xn interface instance, namely without triggering a removal of such interface. For this reason, the common understanding on which a number of companies converged during RAN3-126 is as follows:

In case an additional Xn setup procedure is triggered for an already established Xn-C interface instance, it shall not impact the already established Xn-C interface instance regardless if the additional Xn setup procedure is successful or failed

The objection raised for this approach during the last RAN3 meeting was that there might be implementations that trigger an Xn interface removal when an Xn Setup procedure failure is triggered. This may result in removal of the already established interface as well as a loophole situation where the Xn would be constantly setup after first Xn Setup procedure and torn down after duplicate Xn Setup procedures.

Observation 2: Allowing the standard to acknowledge that an additional Xn Setup procedure, triggered after an Xn-C interface instance is already established, shall be failed might result in Xn removals and in a perpetual failure case situation.

A third option to handle the issue is to accept the second (duplicate) Xn Setup Request message and to let the receiving node respond with a successful Xn Setup Response message. Some of the issues raised with this approach were that:

- If the second Xn Setup Request implies a different Xn configuration, with respect to the first established Xn signalling connection, there would be a problem of which Xn configuration to adopt (the first or the second)

- Legacy procedures may not be able to reply successfully to the second Xn Setup Request, as they might consider reception of such second Xn Setup Request as a logical error (the receiver´s state is not compatible with the procedure)

Observation 3: Allowing the standard to acknowledge that an additional Xn Setup procedure, triggered after an Xn-C interface instance is already established, may be successfully completed might result in an out of synch between the source and target NG-RAN nodes Xn configuration. Some implementations may not be able to successfully complete a duplicate Xn Setup procedure and trigger a failure procedure instead.

Obviously, the main issue here is that since Rel15 the Xn specifications have left the NG-RAN node behaviour for the interoperability case discussed in this paper, unspecified. That implied that each vendor has taken its own implementation decisions on how to handle such error case. This situation makes it difficult, if not impossible, to resolve this problem in a way that is backwards compatible with all the NG-RAN node implementations already deployed in commercial networks.

For this reason, it is acknowledged that the problem exists, but it is also acknowledged that a standard level solution cannot be achieved. 
Solutions will have to be derived at network integration level, namely based on implementation changes agreed between vendors and operators.

Conclusion 1: Since Rel15 the Xn specifications have left the NG-RAN node behaviour for the case of duplicated Xn Setup procedures, unspecified. For that, any standardised solution may not be backwards compatible with the NG-RAN node implementations deployed in commercial mobile networks

Conclusion 2: RAN3 to conclude that solutions for the problem of duplicated Xn Setup procedures are left to implementation and network integration

# 3. Conclusion

# In this paper the issue of duplicated Xn Setup procedures has been analysed on the basis of the discussions carried out at RAN3-126.

# The following Observations and conclusions were derived:

# Observation 1: Allowing the standard to acknowledge that an additional Xn Setup Request message, triggered after an Xn-C interface instance is already established, shall be ignored by the receiver implies unnecessary increase of signalling and processing load and it might result in failure cases.

# Observation 2: Allowing the standard to acknowledge that an additional Xn Setup procedure, triggered after an Xn-C interface instance is already established, shall be failed might result in Xn removals and in a perpetual failure case situation.

# Observation 3: Allowing the standard to acknowledge that an additional Xn Setup procedure, triggered after an Xn-C interface instance is already established, may be successfully completed might result in an out of synch between the source and target NG-RAN nodes Xn configuration. Some implementations may not be able to successfully complete a duplicate Xn Setup procedure and trigger a failure procedure instead.

# Conclusion 1: Since Rel15 the Xn specifications have left the NG-RAN node behaviour for the case of duplicated Xn Setup procedures, unspecified. For that, any standardised solution may not be backwards compatible with the NG-RAN node implementations deployed in commercial mobile networks

# Conclusion 2: RAN3 to conclude that solutions for the problem of duplicated Xn Setup procedures are left to implementation and network integration