3GPP TSG-RAN WG3 Meeting #127	R3-250451

# Athens, Greece, 17th – 21st February, 2025

Title: 	Discussion on Data Collection Reporting procedure’s interaction with Handover Request procedure

Source: 	Ericsson, JIO Platforms (JPL), InterDigital, FiberCop

Agenda item:	9.2

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we discuss details on the procedure text for the Data Collection Reporting procedure and whether the specifications so far agreed reflect the common understanding gained in RAN3 during the course of Release 18.

The paper highlights inconsistencies in the procedure text and a number of possible ways to resolve them.

# 2. Discussion

The Rel.18 work item on AI/ML for NG-RAN specified data collection enhancements and signalling support for AI/ML-based use cases. Among the specified enhancements were two new procedures: the Data Collection Reporting Initiation (class 1) and the Data Collection Reporting (class 2) procedures.

The Data Collection Reporting Initiation procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. After successful execution of the Data Collection Reporting Initiation procedure, the requested metrics are reported to the requesting NG-RAN node over the Data Collection Reporting procedure.

For the Measured UE Trajectory and UE performance after handover, the measurement ID of the Data Collection Reporting Initiation procedure is included in the Handover Request message as an implicit indication for reporting of metrics as requested by the corresponding Data Collection Reporting Initiation procedure to the requesting node. The measurement of the requested metrics commences after a successful handover, and it ends after at least one of the specified exit conditions is satisfied. The collected measurements are signalled to the requesting node over the following transmission of the Data Collection Update message, after at least one of the exit conditions has been satisfied, depending on the periodicity configured by the original Data Collection Reporting Initiation procedure.

The text describing the above in TS 38.423 is as follows:

If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node.

The statement “… the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover, via the Data Collection Reporting procedure …” may be interpreted as the target NG-RAN node reporting the measurements to the source NG-RAN node immediately after a successful handover via the Data Collection Reporting procedure. While the intention is to specify that the reporting happens via the Data Collection Reporting procedure after a successful handover and after the exit conditions have been triggered and at the specified reporting periodicity of the Data Collection Reporting procedure.

For this reason, the following correction to remove any ambiguity in the text is suggested:

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. The procedure uses UE-associated signalling. #### 8.2.1.2 Successful Operation --- Unmodified text omitted --- Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures: If the Data Collection ID IE is contained in the HANDOVER REQUEST message and after at least one of the exit conditions specified in the corresponding configuration is triggered, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node,.

Conclusion 1: in the Handover Preparation procedure text, it shall be clarified that the requested information configured via the Data Collection Reporting Initiation procedure are reported after successful handover and if at least one of the exit conditions is fulfilled

The above-described conditions for terminating the feedback collection and signalling the collected feedback, i.e., after the exit conditions are triggered and at the next transmission of the Data Collection Update message, are also described in Clause 8.4.13.2 in TS 38.423. Relevant text is reported below:

### 8.4.13 Data Collection Reporting Initiation #### 8.4.13.1 General This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. The procedure uses non UE-associated signalling. #### 8.4.13.2 Successful Operation --- Unmodified text omitted --- If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell. The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

It can be noted that the text above states that “NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: […]”

Namely, the text implies that collection of measurements starts when the handover is successfully executed and is terminated when one of the exit conditions is fulfilled.

Conclusion 2: For UE Performance reporting the target gNB starts collection of UE performance metrics after successful execution of a UE handover and it stops collection of UE performance metrics when at least one of the specified exit conditions for the UE performance measurement collection is fulfilled

The text continues with saying that “The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.”

Namely, the text implies that the result of UE performance collection, which, as stated above, terminates when at least one exit condition is fulfilled, is reported at the next available Data Collection Update message after data collection is concluded.

If the above understanding has to be maintained, then the procedure text should be similar to that for UE Trajectory reporting, which is reported below:

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE; - the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2. The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

We therefore identify the following option to clarify the text and to remove any ambiguity with it:

Option 1: Clarify that the UE Performance is reported only once after the UE performance collection is terminated.

For this option the following correction is proposed:

### 8.4.13 Data Collection Reporting Initiation #### 8.4.13.1 General This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. The procedure uses non UE-associated signalling. #### 8.4.13.2 Successful Operation --- Unmodified text omitted --- If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall report the UE performance only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

However, we note that during the course of Rel-18, and specifically at RAN3-119bis, the following was agreed:

- UE performance feedback can be reported through one-time reporting or periodic reporting.

The above agreement is not in line with what the current specification text states. The above agreement implies that UE Performance can be reported periodically during the UE performance collection period as well as via the Data Collection Update message signalled soon after the UE performance collection period.

If the agreements taken in RAN3 had to be reflected in the procedure text for the Data Collection Reporting Initiation procedure, the following option would have to be supported.

Option 2: Correct the Data Collection Reporting Initiation procedure to state that the result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message after collection is terminated and it may be reported via DATA COLLECTION UPDATE messages during the Collection Time Duration for UE Performance.

For this option the following correction is proposed:

### 8.4.13 Data Collection Reporting Initiation #### 8.4.13.1 General This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. The procedure uses non UE-associated signalling. #### 8.4.13.2 Successful Operation --- Unmodified text omitted --- If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell. The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message after collection is terminated and it may be reported via DATA COLLECTION UPDATE messages during the Collection Time Duration for UE Performance.

Our preference is to acknowledge the agreements taken during Rel18 and to propose the corrections in Option 2.

Proposal 1: Correct the Data Collection Reporting Initiation procedure to state that the result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message after collection is terminated and it may be reported via DATA COLLECTION UPDATE messages during the Collection Time Duration for UE Performance.

# 3. Conclusion

# In this contribution carry out an analysis of the procedure text for the Handover Preparation procedure and for the Data Collection Reporting Initiation procedure. We derive the following Conclusions and proposal:

# Conclusion 1: In the Handover Preparation procedure text it shall be clarified that the requested information configured via the Data Collection Reporting Initiation procedure are reported after successful handover and if at least one of the exit conditions is fulfilled

# Conclusion 2: For UE Performance reporting the target gNB starts collection of UE performance metrics after successful execution of a UE handover and it stops collection of UE performance metrics when at least one of the specified exit conditions for the UE performance measurement collection is fulfilled

# Proposal 1: Correct the Data Collection Reporting Initiation procedure to state that the result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message after collection is terminated and it may be reported via DATA COLLECTION UPDATE messages during the Collection Time Duration for UE Performance.

A CR reflecting the main proposals in this paper is presented in R3-250452