3GPP TSG-RAN WG3 Meeting #127	R3-250459

Athens, Greece, 17–21 February 2025

Title:	AI/ML support for NR-DC

Source:	Ericsson, JIO Platforms (JPL), InterDigital, FiberCop

Agenda item:	11.4

Document Type:	Discussion and agreement

# Introduction

At the RAN3#126 meeting, the following agreements and “to-be-continued” were captured for AI/ML assisted mobility optimization for NR-DC in the meeting minutes:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

Other information, e.g., predicted SCG bearer?

In this contribution, we discuss the consequences of the above agreements and other aspects related to the enhancements needed in NG-RAN interfaces to support this use case.

# Discussion

## Enhancements to support mobility optimization for NR-DC

Given the conclusions captured in the TR 38.743, we would like to focus the discussion on two scenarios:

- a UE is in single connectivity, and the serving gNB infers the decision to initiate SN Addition

- a UE is in NR-DC, and the MN infers the decision to initiate SN Change

The following types of enhancements are needed to support the scenarios above:

- Enhancements to assist inference at the MN (future MN or current MN, depending on the scenario)

- Enhancements to provide feedback to the MN

- Additional data collection for NR-DC

To assist inference at the MN (both if it is a “future MN” or a “current MN”), it is possible to reuse to a large extent the Rel-18 signaling design established to support AI/ML assisted use cases. As already agreed in the RAN3#126 meeting, we can reuse the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to allow an MN (future or current) to request and obtain from a neighbor gNB (acting as SN) the reporting of UE performance. This is achieved by including the Data Collection ID IE in the XnAP S-NODE ADDITION REQUEST message, which signals to the (new or target) SN that it should start collecting the UE performance previously requested by the MN

Another part of the Rel-18 solution for collection of UE performance after a handover concerns the conditions that the target gNB needs to monitor to stop the collection of UE performance requested by the source gNB. For NR-DC, similar conditions can be defined, so that the SN takes them into account to terminate the collection of UE performance requested by the MN. In our view, at least the following conditions should be considered:

- Expiration of a timer (e.g. reporting time interval), after successful SN Addition;

- the SN is removed from NR-DC (MN initiated or SN initiated).

- The PSCell is changed

Proposal 1:  For AI/ML assisted mobility optimization for NR-DC, define the following conditions for the SN to stop collection of UE performance requested by the MN: 1) expiration of a timer, after successful SN Addition; 2) SN removal (MN initiated or SN initiated); 3) Change of the PSCell.

Furthermore, in the Rel-18 solution, a condition is specified according to which the target gNB should stop collecting UE performance when the “UE is handed over to another cell”. UE performance is currently reported from target gNB to source gNB without an explicit indication of the cell they refer to. This is good enough for the Handover case, since the target cell is explicitly signalled in the HANDOVER REQUEST and the target gNB knows exactly for which cell it should report the UE performance. Also, since the collection should stop when UE is handed over to another cell, there is only one cell the UE performance can refer to. With some enhancements, the existing signaling design can be reused for NR-DC as we discuss below.

First, the reported UE performance should be extended to include the PSCell used by the UE, so that it is clear to the MN as to which cell the UE performance refers. The MN can use this information, e.g., to understand whether its assumptions to select the SN were optimal or not.

Proposal 2:  For AI/ML assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

It is reasonable to think that the MN has inferred the action of SN Addition (or SN Change) assuming that adding (or changing) the SN would bring some benefits to the UE. We think that it is good if the MN informs the SN about some of the assumptions made, so that the SN can try to provide the best possible service to the UE.

For example, when an MN infers the action to add or change SN, it can assume that a specific PSCell will be selected for the UE. This is obvious because the UE performance feedback received in the past for each UE is specific to a PSCell (described in Proposal 2). Hence, if UEs were served well by an SN, on one or more PSCells, the MN is more likely to predict that the SN and one or more specific PSCells, are the best options for a DC configuration at the UE.

Observation 1:  An MN infers the action to add or change SN based on an assumption that a specific PSCell will be selected for the UE.

Another example is that when the MN carries out an AI/ML generated SN Addition, it may request to the SN the establishment of SN-terminated bearers. However, the inferred decision can be based on the assumption that specific QoS flows will be served by the SN (as SCG). This is not in line with what could happen today, where an SN may decide that all the QoS flows for which the MN requests SN-terminated bearers, shall be served by MCGs. Therefore, when the MN derives an AI/ML based request to the SN for some resources for SN terminated bearers, it should tell the SN that this request was made assuming that SN could serve some or all of the QoS flows as SCGs. After receiving this notification, the SN is left free to decide whether to serve the QoS flows as MCG or SCG, but it can use the information and try to accommodate the MN assumptions, if possible.

Observation 2:  An MN infers the action to add or change SN based on an assumption that a certain amount of UE traffic will be served by the new SN as SCG bearers.

Therefore, we propose that, at SN Addition, the MN could tell the SN that the MN assumes that a specific cell, e.g. CellA, is selected as PSCell. The SN is free to decide independently, but it can try to select CellA as PSCell if possible. It is also proposed that the MN indicates to the SN which are the assumptions in terms of the QoS flows the SN will have to serve as SCG. We note that both these two pieces of information are important together, i.e., which cell will serve the UE (as PSCell) and the amount of traffic (in terms of the QoS flows in the SCG).

Proposal 3:  Enhance dual connectivity procedures (e.g., SN addition, MN-initiated SN change) to indicate MN assumptions to be considered by the SN: cell the MN assumed as PSCell, QoS flows the MN assumed to be served as SCGs.

# Conclusions

Based on the discussion in this paper, we derived the following proposals:

Proposal 1:  For AI/ML assisted mobility optimization for NR-DC, define the following conditions for the SN to stop collection of UE performance requested by the MN: 1) expiration of a timer, after successful SN Addition; 2) SN removal (MN initiated or SN initiated); 3) Change of the PSCell.

Proposal 2:  For AI/ML assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

Observation 1:  An MN infers the action to add or change SN based on an assumption that a specific PSCell will be selected for the UE.

Observation 2:  An MN infers the action to add or change SN based on an assumption that a certain amount of UE traffic will be served by the new SN as SCG bearers.

Proposal 3:  Enhance dual connectivity procedures (e.g., SN addition, MN-initiated SN change) to indicate MN assumptions to be considered by the SN: cell the MN assumed as PSCell, QoS flows the MN assumed to be served as SCGs.

A TP mirroring the above proposals is presented in R3-250460.