3GPP TSG-RAN WG3#127	R3-250464

Athens, GR, 17-21 February 2025

Title:	[Draft] LS on Continuous MDT

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	RAN3

To:	SA5

Cc:

Contact Person:

#### Name:                Angelo Centonza

E-mail Address:   angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

In the S5-244658 LS sent to RAN3, SA5 mentioned the following:

“SA5 agree that there is a benefit to have a (temporary) identifier in the management-based MDT measurement report. This identity shall be unique within the lifetime of the measurement session. So, some data analytics function (e.g., AIML related function) can have a better understanding that the collected data sets are measured from the same source. Therefore, it would be good to provide some kind of identifier to indicate the measurement was done in the same or different UE.”

RAN3 is working on the following objective as part of the AI/ML for NG-RAN WID (RP-242385):

  - Continuous MDT collection targeting the same UE across RRC states

The objective covers the possibility of using Management Based MDT for collection of MDT measurements from the same UE while it moves between different NG-RAN nodes and while it changes RRC states. The latter shares similarities with what stated by SA5 in S5-244658.

To fulfil the objective above RAN3 has identified two problem statements that need to be resolved:

-	Measurement Continuity Problem: how to ensure that the same UE is selected for collection of MDT measurements during sessions in the same RRC state, across different RRC states and across RRC_Connected mobility.

-	Trace Correlation Problem: how to ensure that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same data source (UE).

To solve the Trace Correlation Problem, RAN3 agreed that it would be beneficial to derive a (temporary) identifier, to be reported together with the management-based MDT measurement report, from parameters contained in the Trace Record header, or in general from parameters already signalled today from the RAN to the TCE as part of the MDT measurement reports. This ensures the following advantages:

- Parameters contained in the MDT measurement report signalled from RAN to TCE are already specified. Reusing these parameters minimises the impact on the current MDT design and implementation

- Reusing parameters contained in the MDT measurement report ensures avoiding impacts on the UE

- Reusing parameters contained in the MDT measurement report maintains the current functional distribution for MDT at the nodes already supporting MDT today

For the reasons above, RAN3 would like to suggest to SA5 and get feedback on whether a “correlation” identifier could be derived from one or more of the following parameters:

- Trace Record

- Trace Recording Session Reference

- Time Stamp

- CGI of the cell where the measurements were collected

- RAN UE ID

Furthermore, to solve the measurement continuity problem, RAN3 agreed that

- The OAM provides to the RAN means to identify that the Management Based MDT process is for Continuous MDT, preferably reusing existing Trace Record information

- The OAM provides to the RAN means to identify the overall duration of the continuous MDT collection process

- Source gNB signals to Target gNB, as part of the Handover Signalling, an indication that the UE has been selected for Continuous MDT. Target gNB uses this information to re-select the UE for Continuous MDT.

RAN3 would like to get feedback from SA5 on whether the above agreements are feasible to solve the measurement continuity problem stated above.

2. Actions to SA5:

RAN3 kindly asks SA5 to take the above into account and to provide feedback on the solution steps described above.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN WG3 Meeting RAN3#127bis			7 - 11 April 2025		China, CN

TSG-RAN WG3 Meeting RAN3#128			19 - 23 April 2025		Malta, MT