3GPP TSG RAN WG3 Meeting #127	R3-250474

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	8.1

Source:	Ericsson

Title :	Discussion on RAN2 LS on XR UL bit rate control

Document for:	Discussions & Approval

# Introduction

In this contribution, we discuss the LS received from RAN2 [1], in which RAN2 informs RAN3 of its agreement on REL-19 XR UL bit rate control and requests feedback from RAN3. The LS text is copied below.

1. Overall Description: RAN2 has made the following agreement regarding a solution on UL bit rate control Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. 2. Actions: To SA4, RAN3: ACTION: RAN2 kindly asks SA4 and RAN3 to take above into consideration and provide feedback on the agreement provided, if any.

# Background and Discussion

At first notice, the RAN2 LS lacks context; there is no clear background information nor benefits described. A tracing exercise reveals that this agreement is related to a series of agreements that are documented in RAN2#128 chairman minutes and that we list below to help understand the context of this agreement [2]:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

As can be seen from the above list, RAN2 is discussing whether RAN can support Rate indication from gNB to UE. Two options are considered by RAN2: by QoS flow via MAC CE, or QoS flow via RRC while MAC CE is per DRB.

Rate indication by QoS flow via MAC CE:

From RAN3’s perspective, this solution can be translated as gNB-DU indicating the QoS Flows to throttle by sending a MAC CE to UE. However, the scheduler located at gNB-DU does not perform rate control on a per QoS flow level, but on a per DRB basis, as QoS flow-to-DRB mapping happens at the gNB-CU together with admission control. There can be different number of QoS flows mapped to a DRB and the gNB-DU is not required to monitor the QFIs, but only the DRB ID and DRB QoS indicated in the list of the DRB to Be Setup List IE present in the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message.

Furthermore, the notification control at the gNB-DU is performed on a per DRB ID basis. This is captured in the procedural text below from TS 38.473 and associated tabular:

### 8.3.7 Notify #### 8.3.7.1 General The purpose of the Notify procedure is to enable the gNB-DU to inform the gNB-CU that the QoS of an already established GBR DRB cannot be fulfilled any longer or that it can be fulfilled again. The procedure uses UE-associated signalling. #### 8.3.7.2 Successful Operation Figure 8.3.7.2-1: Notify procedure. Successful operation. The gNB-DU initiates the procedure by sending a NOTIFY message. The NOTIFY message shall contain the list of the GBR DRBs associated with notification control for which the QoS is not fulfilled anymore or for which the QoS is fulfilled again by the gNB-DU. The gNB-DU may also indicate an alternative QoS parameters set which it can currently fulfil in the Current QoS Parameters Set Index IE.The gNB-DU may also include the TSC feedback information in the TSC Traffic Characteristics Feedback IE. Upon reception of the NOTIFY message, the gNB-CU may identify which are the affected PDU sessions and QoS flows. The gNB-CU may inform the 5GC that the QoS for these PDU sessions or QoS flows is not fulfilled any longer or it is fulfilled again.

#### 9.2.2.13 NOTIFY This message is sent by the gNB-DU to notify the gNB-CU that the QoS for already established DRBs associated with notification control is not fulfilled any longer or it is fulfilled again. Direction: gNB-DU ® gNB-CU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. gNB-CU UE F1AP ID; M; 9.3.1.4; YES; reject. gNB-DU UE F1AP ID; M; 9.3.1.5; YES; reject. DRB Notify List; 1; YES; reject. >DRB Notify Item IEs; <1 .. maxnoofDRBs>; EACH; reject. >>DRB ID; M; 9.3.1.8; -. >>Notification Cause; M; ENUMERATED(Fulfilled, Not-Fulfilled, ...); -. >>Current QoS Parameters Set Index; O; Alternative QoS Parameters set Notify Index 9.3.1.124; Index to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameter set.; YES; ignore. >>TSC Traffic Characteristics Feedback; O; 9.3.1.302; YES; ignore. Range bound; Explanation. maxnoofDRBs; Maximum no. of DRB allowed towards one UE, the maximum value is 64..

Hence, adding a new functionality for the gNB-DU to perform rate control on a per QoS flow indication changes the implementation of legacy scheduler behaviour and adds a new complexity level to monitor the QoS flow Parameters of a QFI instead of DRB QoS.

Observation1: The gNB-DU where scheduler resides performs rate monitoring on a per DRB level. Changing the behaviour to perform rate control indication per QoS flow will create additional RAN3 impacts and modifies legacy scheduler behaviour.

Another aspect which is unclear from the RAN2 LS, is whether the option of performing bit rate control indication per QoS Flow at the gNB-DU should be for GBR or non GRB QoS Flows. As we know, notification control in F1AP is for GBR DRB, as the RAN is committed to uphold a ‘Guaranteed Bit Rate’ which is signaled from CN for the GBR QoS flows mapping to the DRBs. If the scheduler needs now to perform rate indication for non-GRB QoS Flows, it does not have any clear target for monitoring and the indication can fluctuate randomly. It is questionable therefore if any rate recommendation makes sense to UE from gNB-DU in such a logical construct. Many features already like L4S and QoS monitory per packet delay work on a per DRB basis. Hence, this new functionality is a big twist and risks at making the specification overly complex.

Observation 2: It is unclear from the RAN2 LS whether the option of performing bit rate control indication per QoS Flow at the gNB-DU should be for GBR or non GRB QoS Flows. If for non GBR QoS Flows, it is questionable if any rate recommendation can make sense with the risk of making the specification overly complex.

We note also that in general the RAN2 LS failed in describing the overall usefulness of the functionality of per QoS flow MAC CE indication, and thus the benefits, if any, are obscure from RAN3’s perspective.

Observation 3: The RAN2 LS failed in describing the usefulness of the per QoS flow MAC CE indication functionality, and thus the benefits, if any, are unclear from RAN3 perspective.

Rate indication by QoS flow via RRC and DRB via MAC-CE:

This second option does not appear to have any impact on RAN3 specifications, since the gNB-CU hosting the RRC functionality can report QoS flow information to the UE via RRC signaling, which would be up to RAN2 to define. The gNB-DU, as mentioned above, knows the associated DRB ID, so it can indicate the DRB ID to the UE via MAC-CE. The combination of indicating QFIs via RRC and the DRB ID via MAC-CE would be a less impactful and damaging alternative to QFIs via MAC-CE from RAN3 specification point of view. However, in terms of final usefulness and feasibility, we can only speculate as RAN3 does not have a clear understanding of the purpose nor benefits of rate indication mechanisms.

Observation 4: The option of indication QoS Flow via RRC and DRB via MAC-CE does not appear to have any RAN3 impact and is in-line with F1AP structure and the scheduler’s monitoring capability.

Based on the above observations, RAN3 should reply to RAN2 that the option of “Rate indication by QoS flow via MAC CE” is largely impacting RAN3 specification, incoherent with scheduler monitoring on per DRB basis and overly complicated if non-GBR need to also be considered, not holding any clear benefits. Hence RAN3 does not recommend pursuing this option. While the second option “indication QoS Flow via RRC and DRB via MAC-CE” does not appear to have any RAN3 impact and is in-line with F1AP structure and the scheduler’s monitoring capability.

Proposal 1: RAN3 to reply to RAN2 that the option of “Rate indication by QoS flow via MAC CE” is largely impacting RAN3 specification, incoherent with scheduler monitoring on per DRB basis and overly complicated if non-GBR need to also be considered, not holding any clear benefits. While the second option “indication QoS Flow via RRC and DRB via MAC-CE” does not appear to have any RAN3 impact and is in-line with F1AP structure and the scheduler’s monitoring capability.

A reply LS is available in [3].

# Conclusion

In this contribution we discuss the questions to RAN3 from the RAN LS [1].

Observations and proposals are listed below:

Observation1: The gNB-DU where scheduler resides performs rate monitoring on a per DRB level. Changing the behaviour to perform rate control indication per QoS flow will create additional RAN3 impacts and modifies legacy scheduler behaviour.

Observation 2: It is unclear from the RAN2 LS whether the option of performing bit rate control indication per QoS Flow at the gNB-DU should be for GBR or non GRB QoS Flows. If for non GBR QoS Flows It is questionable if any rate recommendation can make sense with the risk of making the specification overly complex.

Observation 3: The RAN2 LS failed in describing the usefulness of the per QoS flow MAC CE indication functionality, and thus the benefits, if any, are unclear from RAN3 perspective.

Observation 4: The option of indication QoS Flow via RRC and DRB via MAC-CE does not appear to have any RAN3 impact and is in-line with F1AP structure and the scheduler’s monitoring capability.

Proposal 1: RAN3 to reply to RAN2 that the option of “Rate indication by QoS flow via MAC CE” is largely impacting RAN3 specification, incoherent with scheduler monitoring on per DRB basis and overly complicated if non-GBR need to also be considered, not holding any clear benefits. Hence RAN3 does not recommend pursuing this option. While the second option “indication QoS Flow via RRC and DRB via MAC-CE” does not appear to have any RAN3 impact and is in-line with F1AP structure and the scheduler’s monitoring capability.

# References

[1] R3-250012, LS on XR UL bit rate control, RAN2

[2] RP-242517, SR for XR WI, Rapporteur (Nokia), section 2.2.1.2

[3] R3-250475, Draft LS Reply on XR UL bit rate control, Ericsson