3GPP TSG-RAN WG3 Meeting #127	R3-250490

Athens, Greece, 17 - 21 February, 2025

Agenda Item:	20.2

Source:	Ericsson

Title:	Discussion on NRPPa signalling design for AI/ML for PHY

Document for:	Discussion, Approval

- Introduction

## RAN1 agreements

For the sake of tracking and clarity, we list below the full up-to-date list of RAN1 agreements, up to RAN1#119, related to Cases 3a and 3b which are of relevance to RAN3. This might result in a long list of agreements, but it is important for RAN3 to track what RAN1 has decided on this feature, as RAN1 is the main driver for this WI.

The latest relevant RAN1 agreements are listed further below.

RAN1 agreements: - Measurements for model input - General Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. - Measurement types - Sample vs path Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. - Time domain info Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. - Phase information Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc. - Measurement reporting - Case 3a Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. - Case 2b/3b Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. - Performance monitoring Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. - Training and Data collection - General Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Conclusion - It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information - Training data collection for case 3a and 3b Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption (replaces the one above) For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. For training data collection of AI/ML based positioning, the collected data sample can include the following components: - Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement - Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

RAN1##119 agreements Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

## RAN3 agreements

Similarly, for the sake of tracking our internal RAN3 progress, we list the agreements and FFS taken in last meetings:

RAN3#125bis agreements - The existing LCS framework is re-used for AI/ML positioning. - Case 3a agreements: - The AI/ML model (inference and training functions) is at the gNB. - Consider aggregated gNB case first, then split architecture. - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. - FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). - The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - Case 3b agreements: - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. - The details of the measurements are still pending and subject to further discussion in RAN1.

RAN3#126 agreements - A list of solutions was captured for training data collection in use case 3a: - Solution 1a: gNB-triggered Data Collection Request via a new NGAP message - Solution 1b: gNB-triggered Data Collection Request via a new NRPPa message - Solution1c: gNB-triggered Data Collection Request via the existing NRPPa message - Solution 2: “Xn-like Data Collection”over NRPPa - The following open points were captured about the solutions above: - FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). - FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. - The following agreement was taken on the procedure solution for label retrieval in case 3a: - WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. - The following agreement was captured concerning inference output measurements generated at the RAN: - Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. - Concerning the IEs in the existing measurement request procedure, the following was captured reflecting the common understanding: - Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. - One open point on the procedures for the LMF to request positioning data for case 3a is the following: - FFS if LMF should be aware that the reported measurements are AI/ML generated. - The following agreement was taken: - Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

In this contribution, we continue discussing the corresponding RAN3 impacts to support data collection for Case 3a and Case 3b.

We also discuss the LS received from SA2 in [2].

- Discussion

## 2.1	Case 3a (gNB-sided model) data collection discussion

Data Collection:

The problem of data collection for training and/or monitoring when the AI/ML model is hosted at the NG-RAN node to infer positioning measurements is still unresolved in 3GPP.

Some examples were proposed in last meetings that are captured in chair notes. E.g., there are signalling solutions via NRPPa, NGAP, using new or existing procedures. We list them below:

  - Solution 1a: gNB-triggered Data Collection Request via a new NGAP message

  - Solution 1b: gNB-triggered Data Collection Request via a new NRPPa message

  - Solution1c: gNB-triggered Data Collection Request via the existing NRPPa message

  - Solution 2: “Xn-like Data Collection”over NRPPa

Solution 1b should be ruled out as the 3GPP specifications do not support the case where the NG-RAN initiates an NRPPa transaction on its own. This was also confirmed in the agreed WA, as follows:

	WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

Furthermore, the RAN does not know which LMF can handle the data collection request.

Solution 1c affects and misuses existing procedures in which a response message from the NG-RAN is used as a training data request message to LMF, and another request message from LMF is used to convey the labels. This leads to a dependency between different class 1 procedures because request messages are sent in Class 1 response messages, and response messages are sent in Class 1 request messages. Therefore, this solution is to be ruled out as well.

Observation 1: Solutions 1b and 1c should be ruled out.

Practically, a data collection request should be performed per UE, that is currently being served by the NG-RAN node to avoid generating a lot of traffic and incurring heavy signalling loads on the air and RAN-CN interfaces when initiating positioning for several UEs. Hence, a crucial question is how to identify the UE for which the NG-RAN will trigger a data collection request.

Observation 2: A crucial question is how to identify the UE for which the NG-RAN will trigger a data collection request for.

If the NG-RAN triggers the request for data collection request using a UE associated NGAP procedure, that simplifies the process of UE identification by the means of the existing UE AP IDs. This also ensures that the NG-RAN knows for which UE currently served it needs to request receiving the labels or ground truths, so that it can combine such labels with measurements taken for the same UE (e.g., UL SRS measurements) to form a set of training data.

In terms of stage 2 design, when the AMF receives a NGAP Data Collection request message from the NG-RAN including the UE AP IDs, it can provide the label directly if such information is readily available at the AMF as part of the UE positioning context. Otherwise, the AMF triggers existing TS 23.273 procedures, to retrieve the positioning information from the LMF. This process can re-use the existing Determine Location procedures defined between AMF and LMF with no added specification impacts. Perhaps only some stage 2 updates may be needed.

Below a signalling flow detailing the signalling solution using solution 1a:

Fig.1 : gNB-triggered Data Collection Request via a new NGAP message

- The gNB is currently serving a UE for which part A information (i.e., UL SRS measurements) is available and wishes to receive the part B information, consisting of labels to form a data set for training. Based on gNB implementation, the gNB decides when/if to trigger a DATA COLLECTION REQUEST message to the serving AMF of the served UE. - The DATA COLLECTION REQUEST message is an NGAP UE associated message containing the NGAP UE IDs, and type of requested labels (UE position, time stamp). - The AMF receives the request message from the NG-RAN node. If the labels are already available at AMF as part of UE positioning context, the AMF responds directly with the message in step 7. Otherwise, it initiates positioning for the UE for data collection after selecting an appropriate LMF. - The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE. - The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 of TS 23.273. - The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF with the current location of the UE. - The AMF returns the requested label to the NG-RAN node in a DATA COLLECTION RESPONSE message. If for some reason (e.g., no UE consent, positioning failed, etc.), the AMF returns instead a failure message with appropriate cause value. - The gNB combines the received label with the TRP measurements to form the data training sets.

The above signalling is based on solution 1a that introduces a new NGAP procedure for Data Collection Exchange between NG-RAN and AMF. This simplifies the UE identification and does not rely on additional NRPPa signalling where LMF first advertises of the available UEs that can be requested for data collection by gNB [1].

Observation 3: Solution 1a introduces new NGAP UE associated procedure for Data Collection Exchange between NG-RAN and AMF. This simplifies the UE identification and reduces signalling.

This solution also gives flexibility to NG-RAN to decide when it should acquire data sets for which models were not sufficiently trained or for which model performance was not sufficiently monitored, which is up to gNB implementation. One aspect to consider however, is if Part A information is already available at gNB at this stage. The part A information consists of sample measurements with time stamp. RAN3 can assume that gNB has this information readily available since the UE is already being served and configured for UL transmission, before deciding to trigger the data collection request.

Observation 4: It is up to gNB implementation when to trigger the Data Collection Request message. It is assumed that gNB already have part A (UL SRS measurements) available for the served UE.

Proposal 1: RAN3 to discuss NGAP based solution for data collection to support case 3a with introduction of new NGAP UE associated procedure for Data Collection Exchange between NG-RAN and AMF.

Another important aspect to agree on is that the AI/ML model is hosted at gNB-CU. This saves on signalling as the TRP does not need to trigger an F1AP procedure to request and receive ground truth that needs to be relayed to LMF via gNB-CU. The other advantage of this approach is that the gNB-CU is connected to many more TRPs than the gNB-DU, hence having access to a much larger amount of measurements. This enables collection of a much wider set of data with which it is possible to perform more accurate and extensive training.
We note that this is the preference of most companies from past meetings and therefore should be anchored as an agreement to establish the correct context to Case 3a discussions.

Proposal 2: The AI/ML model for Case 3a is located at gNB-CU.

## 2.2	Case 3a (gNB-sided model) measurement inference discussion

Measurements for case 3a include UL RTOA and LOS/NLOS indication. According to RAN1 agreement the LOS/NLOS indicator can’t be reported independently from other measurements. Therefore, the current LOS/NLOS indication remains as per legacy semantics, i.e. an indication on the input channel measurements for the reported measurement.

Observation 5: The current LOS/NLOS indication remains as per legacy semantics, i.e. an indication on the input channel measurements for the reported measurement.

The open issue is whether a measurement generated by AI/ML model at NG-RAN should be flagged to the LMF, compared to legacy. In our view the LMF shall understand that the measurement corresponds to Rel-19 measurement generated using AI/ML methods. One reason is that the related attributes (e.g., quality, LoS-NLoS) may differ if the measurement was not inferred (e.g., the measurement may be in NLoS in case of non-inferred). Another reason is that the LMF may adopt different behaviours if the measurements provided by the RAN are AI/ML inferred. As an example, the LMF may decide to request measurements from more RAN nodes if the measurement received from the RAN are AI/ML inferred. This is to ensure a higher amount of measurements and to be able to filter out measurements with high deviation, while still being able to position the UE.

Observation 6: AI/ML positioning measurements are different from legacy positioning measurements; the attributes may also differ. The LMF may also adopt different behaviours if the measurements provided by the RAN are AI/ML inferred.

One option to “flag” is to add in the TRP measurement report in NRPPa an IE indicating that the requested measurement is inferred, i.e., has been generated via AI/ML.

### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Measured Result Item; 1 .. <maxnoPosMeas>; -. >CHOICE Measured Results Value; M; -. >>UL Angle of Arrival; 9.2.38. >>UL SRS-RSRP; INTEGER (0..126). >>UL RTOA Measurement; 9.2.39. >>gNB Rx-Tx Time Difference; 9.2.40. >>Z-AoA; 9.2.67; YES; reject. >>Multiple UL-AoA; 9.2.71; YES; reject. >>UL SRS-RSRPP; 9.2.72; YES; reject. >>UL RSCP; 9.2.92; YES; reject. >Time Stamp; M; 9.2.42; -. >Measurement Quality; O; 9.2.43; -. >Measurement Beam Information; O; 9.2.57; -. >SRS Resource type; O; 9.2.73; YES; ignore. >ARP ID; O; 9.2.75; YES; ignore. >LoS/NLoS Information; O; 9.2.77; YES; ignore. >Inferred measurement; O; ENUMERATED (true,…); This IE is only valid for UL-RTOA and gNB Rx-Tx Time Difference measurements; YES; ignore.

This option avoids introducing new IEs in both the request and response messages, which can be especially useful if later other positioning measurements (e.g., AoA) are introduced as part of model output. This also has small spec impacts. For Rel-19, this IE should be limited to UL-RTOA and gNB Rx-Tx Time Difference measurements. Such limitation can be specified via semantics descriptions.

Proposal 3: The AI/ML positioning measurements must be distinguishable from the legacy measurements in the TRP Measurement report.

Proposal 4: Add a flag in the NRPPa TRP report to indicate that the associated measurement is inferred, i.e., AI/ML generated.

## 2.3	Case 3b (LMF-sided model) discussion

RAN1 has agreed that measurements to support case 3b inference at LMF, can be either UL RTOA measurements (legacy) or so-called “sample” measurements defined as per the agreement below:

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

As was noted by some companies in last meeting, the LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. The encoding is not fully clear as RAN1 still has some FFS. RAN1 may inform us via their usual LS with excel sheet on PHY layer parameters.

However, as can be seen from the highlighted text above, if the RAN used different values from what LMF requested for sample measurement reporting,  the gNB may indicate to LMF the values it actually used.

Observation 7: the LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa to request for sample measurement reporting. FFS on final encoding.

Observation 8: the gNB may use parameter values Nt, Nt', k different from those requested by LMF and it should indicate which ones it has used together with the sample measurement report.

To progress, it is proposed to add the parameters Nt, Nt', k in the MEASUREMENT REQUEST message from LMF to NG-RAN and TRP Measurement Report IE within the MEASUREMENT RESPONSE message from NG-RAN to LMF.

Proposal 5: add the parameters Nt, Nt', k in the MEASUREMENT REQUEST message from LMF to NG-RAN and TRP Measurement Report IE within the MEASUREMENT RESPONSE message from NG-RAN to LMF.

This has also impacts on F1 to retrieve the sample data from TRPs located at the gNB-DU. Therefore, it is proposed to add the parameters in the corresponding positioning message and IE in F1AP.

Proposal 5: add the parameters Nt, Nt', k in the corresponding F1AP positioning message and IE.

It is proposed to liaise RAN1 and RAN2 and inform them of our agreements, an LS is available in Annex 1.

A draft CR set to NRPPa/NGAP and F1AP is available in [3].

## 2.4	SA2 LS discussion

RAN3 has received an LS from SA2 in [2] asking to provide feedback on the editor’s notes below in TS 23.273 v.19.1.0:

Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS.

The work in SA2 is related to LMF-sided model, i.e., Case 3b in RAN WID. Regarding the two first editor’s notes, RAN3 can mention to SA2 the following agreement related to LMF obtaining training data from NG-RAN by re-using existing NRPPa Measurement procedure from TS 38.455.

- Case 3b agreements: - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. Namely, in Case 3b, where measurement reporting from RAN to LMF exists already, we can reuse NRPPA - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The part A information from TRP to LMF is below form RAN1 agreement. Eventually with F1AP impact in case of split architecture.:

Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information.

The change to TS 38.455 however will appear when Rel-19 specs are out, so SA2 will have to wait for a while until June 2025.

Proposal 6: RAN3 to reply SA2 with the RAN3 agreement on re-using existing NRPPa Measurement procedures in TS 38.455 to collect and report input data to LMF to support the case of LMF-sided AI/ML model for positioning. RAN3 should state that the changes will appear in the Rel-19 version of TS 38.455 (June 2025).

Regarding the third editor´s note, there is no explicit information from RAN to LMF on the load status of the RAN. According to TS38.455, the RAN responds with a MEASUREMENT FAILURE message when the measurements cannot be reported to LMF.

TS 38.455 section 8.5.1.2

“If the NG-RAN node cannot configure any of the requested measurements for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, it shall respond with a MEASUREMENT FAILURE message with an appropriate cause value.”

The RAN simply notifies the LMF that a measurement cannot be provided according to the following cause values. Cause values group together different failure reasons, therefore load reasons are covered by the existing cause values.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related |
| Requested Item not Supported | The NG-RAN node does not support the requested measurement object, or cannot provide the requested information item. |
| Requested Item Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object or information item. |
| Serving NG-RAN node changed | The UE has moved to another serving NG-RAN node. |
| Requested Item not Supported on Time | The NG-RAN node is unable to provide the measurement results on time. |

Observation 9: There is no load indication from RAN to LMF. Existing NRPPa cause values can implicitly indicate that failure can be due to load status.

Proposal 7: RAN3 reply to SA2 that LMF does not receive any explicit indication from RAN on load status. Current cause values in NRPPa implicitly indicate that failure can be due to load status.

indicate that e.g., a requested item is not supported or temporally not available in the NRPPa measurement failure report.

A draft reply LS is provided in [3].

- Conclusion

In this contribution, we have made the following observations and propose the following:

Observation 1: Solutions 1b and 1c should be ruled out.

Observation 2: A crucial question is how to identify the UE for which the NG-RAN will trigger a data collection request for.

Observation 3: Solution 1a introduces new NGAP UE associated procedure for Data Collection Exchange between NG-RAN and AMF. This simplifies the UE identification and reduces signalling.

Observation 4: It is up to gNB implementation when to trigger the Data Collection Request message. It is assumed that gNB already have part A (UL SRS measurements) available for the served UE.

Proposal 1: RAN3 to discuss NGAP based solution for data collection to support case 3a with introduction of new NGAP UE associated procedure for Data Collection Exchange between NG-RAN and AMF.

Proposal 2: The AI/ML model for Case 3a is located at gNB-CU.

Proposal 3: The AI/ML positioning measurements must be distinguishable from the legacy measurements in the TRP Measurement report.

Proposal 4: Add a flag in the NRPPa TRP report to indicate that the associated measurement is inferred, i.e., AI/ML generated.

Proposal 3: The AI/ML positioning measurements must be distinguishable from the legacy measurements in the TRP Measurement report.

Proposal 4: Add a flag in the NRPPa TRP report to indicate that the associated measurement is inferred, i.e., AI/ML generated.

Observation 7: The LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa to request for sample measurement reporting. FFS on final encoding.

Observation 8: The gNB may use parameter values Nt, Nt', k different from those requested by LMF and it should indicate which ones it has used together with the sample measurement report.

Proposal 5: Add the parameters Nt, Nt', k in the MEASUREMENT REQUEST message from LMF to NG-RAN and TRP Measurement Report IE within the MEASUREMENT RESPONSE message from NG-RAN to LMF.

Proposal 5: Add the parameters Nt, Nt', k in the corresponding F1AP positioning message and IE.

Proposal 6: RAN3 to reply SA2 with the RAN3 agreement on re-using existing NRPPa Measurement procedures in TS 38.455 to collect and report input data to LMF to support the case of LMF-sided AI/ML model for positioning. RAN3 should state that the changes will appear in the Rel-19 version of TS 38.455 (June 2025).

Observation 9: There is no load indication from RAN to LMF. Existing NRPPa cause values can implicitly indicate that failure can be due to load status.

Proposal 7: RAN3 reply to SA2 that LMF does not receive any explicit indication from RAN on load status. Current cause values in NRPPa implicitly indicate that failure can be due to load status.

indicate that e.g., a requested item is not supported or temporally not available in the NRPPa measurement failure report.

- References

- R3-247578 Discussion on RAN3 impacts to support AI/ML Positioning accuracy enhancements, Ericsson, RAN3#126.

- R3-250074, LS on LMF-based AI/ML Positioning for case 3b, SA2(Ericsson)

- R3-250491, CRs set to NRPPa, F1AP and NGAP BL CRs for AI/ML for PHY

- R3-250489, Draft Reply LS on LMF-based AI/ML Positioning for case 3b, Ericsson

- Annex 1 – LS to RAN1

3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, Greece, 17 - 21 February, 2025

# Title:	LS reply on RAN3 agreements on AI/ML positioning

# Response to:

# Release:	Release 19

# Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN1, RAN2

Cc:

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 has started discussions on support of Case3a and Case 3b for AI/ML positioning.

RAN3 has made the following agreements:

Case 3a, assisted AI/ML positioning, NG-RAN hosted model: - NG-RAN requests ground truth label and related data (i.e., part B information as called by RAN1) from the AMF on a per UE basis. AMF triggers positioning (if UE position information not available already) and provide the information to NG-RAN. - It is up to gNB implementation when/If NG-RAN triggers the data collection request. It is considered that part A information is available at the RAN when triggering the request for part B - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). - RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - RAN3 agrees to re-use the Measurement procedures defined in NRPPa to support requesting and reporting the inferred measurements to LMF for Case 3a. FFS how to flag AI/ML measurements from legacy ones. - The AI/ML model is located at gNB-CU. Case 3b, direct AI/ML positioning, LMF-sided model: - RAN3 agrees to re-use the Measurement procedures defined in NRPPa and F1AP to support requesting and reporting the related measurements t(UL-RTOA and sample based measurements) to LMF for Case 3b.

2. Actions:

To RAN1, RAN2.

ACTION: 	RAN3 asks RAN1 and RAN2 to take the above into account.

3. Date of Next TSG RAN WG3 Meetings:

TSG-RAN3 Meeting #127bis	7 - 11 April 2025				Wuhan, CN

TSG-RAN3 Meeting #128	May 19-23, 2025				Malta, MT