3GPP TSG RAN WG3 Meeting #127	R3-250494

Athens, Greece, 17 - 21 February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1543 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | E-CID measurement enhancement [E-CID-enh1] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT Docomo, Nokia, Polaris Wireless, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current E-CID framework is designed on the concept of “one cell level measurement”, however one cell can comprise several TRPs which can report different measurements. To allow for multiple reporting of measurements, it is proposed to add a request for multiple measurement quantities the E-CID Measurement procedures, so that more measurements can be collected from the gNB-DU side. Based on the available transmitting TRPs in the gNB-DU, the gNB-DU reports the list of measurement result to the gNB-CU. This will improve the accuracy of E-CID positioning method. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce a E-CID TRP Measurement Request Info IE to allow gNB-CU to request a list of measurements. - Introduce an E-CID TRP Measurement Result IE in the E-CID MEASUREMENT INITIATION RESPONSE and E-CID MEASUREMENT INITIATION REPORT messages, containing a list of TRPs with a list of results per TRP indicating the measurement (currently only UL-AoA), Time stamp and measurement quality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to exploit the existence of several angles per TRP when MIMO transmission is active, which could be reported in the E-CID measurement reports per TRP, each with a measurement quality indicator. This would improve positioning accuracy with UL-AoA measurements for calculating the UE position using E-CID method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.13.12.2, 8.13.14.2, 9.2.12.20, 9.2.12.21, 9.2.12.24, 9.3.1.x1(new), 9.3.1.x2(new), 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.455 CR 0186 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 8.13.12	E-CID Measurement Initiation

#### 8.13.12.1	General

The purpose of E-CID Measurement Initiation procedure is to allow the gNB-CU to request the gNB-DU to report E-CID measurements used by LMF to compute the location of the UE. The procedure uses UE-associated signalling.

#### 8.13.12.2	Successful Operation

Figure 8.13.12.2-1: E-CID Measurement Initiation procedure, successful operation

The gNB-CU initiates the procedure by sending an E-CID MEASUREMENT INITIATION REQUEST message. If the gNB-DU is able to initiate the requested E-CID measurements, it shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message.

If the E-CID Report Characteristics IE is set to "OnDemand", the gNB-DU shall return the result of the measurements in the E-CID MEASUREMENT INITIATION RESPONSE message including, if available, the Geographical Coordinates IE in the E-CID Measurement Result IE and the Cell Portion ID IE, and the gNB-CU shall consider that the E-CID measurements for the UE have been terminated by the gNB-DU. The Measured Results List IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT INITIATION RESPONSE message when measurement quantities other than "Default" have been requested.

If the E-CID Report Characteristics IE is set to "OnDemand" and the E-CID TRP Measurement Request Info IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the gNB-DU shall, if supported, provide the corresponding TRP measurements, if available in the gNB-DU, in the E-CID TRP Measurement Result IE in the E-CID MEASUREMENT INITIATION RESPONSE message.

Interaction with the E-CID Measurement Report procedure:

If the E-CID Report Characteristics IE is set to "Periodic", the gNB-DU shall initiate the requested measurements and shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message without including the E-CID Measurement Result IE, Cell Portion ID IE, or E-CID TRP Measurement Result IE in this message. The gNB-DU shall then periodically initiate the E-CID Measurement Report procedure for the measurements, with the requested reporting periodicity.

//Next change//

### 8.13.14	E-CID Measurement Report

#### 8.13.14.1	General

The purpose of E-CID Measurement Report procedure is for the gNB-DU to provide the E-CID measurements for the UE to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.13.14.2	Successful Operation

Figure 8.13.14.2-1: E-CID Measurement Report procedure, successful operation

The gNB-DU initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results List IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement quantities other than "Default" have been requested.

If available, the gNB-DU shall include the Geographical Coordinates IE in the E-CID Measurement Result IE in the E-CID MEASUREMENT REPORT message.

If available, the gNB-DU shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message.

If available, the gNB-DU shall include the Mobile Access Point Location Information IE in the E-CID MEASUREMENT REPORT message.

If the E-CID TRP Measurement Result IE is included in the E-CID MEASUREMENT REPORT message, the gNB-CU shall, if supported, consider it as the measurement results from the indicated TRPs.

//Next change//

#### 9.2.12.20	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by gNB-CU to initiate E-CID measurements.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| E-CID Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| E-CID Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms, extended) | The codepoint “extended” is not applicable. This IE is not applicable to NR Angle of Arrival. | YES | reject |
| E-CID Measurement Quantities |  | 1 .. <maxnoofMeasE-CID> |  |  | EACH | reject |
| >E-CID Measurement Quantities Item | M |  | ENUMERATED (Default, NR Angle of Arrival, …, NR Timing Advance) | If “Default” is the only requested measurement quantity, it indicates that the Measured Results List IE need not be included in response or reporting messages. | - |  |
| Measurement Periodicity NR-AoA | C-ifReportCharacteristicsPeriodicAndMeasQuantityItemAoA |  | ENUMERATED (160ms, 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 40960ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| E-CID TRP Measurement Request Info | O |  | 9.3.1.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasE-CID | Maximum no. of E-CID measured quantities that can be configured and reported with one message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the E-CID Report Characteristics IE is set to the value "Periodic". |
| ifReportCharacteristicsPeriodicAndMeasQuantityItemAoA | This IE shall be present if the E-CID Report Characteristics IE is set to the value "Periodic" and the E-CID Measurement Quantities Item IE is set to the value "NR Angle of Arrival". |

#### 9.2.12.21	E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by gNB-DU to indicate that the requested E-CID measurement is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.3.1.199 |  | YES | ignore |
| Cell Portion ID | O |  | 9.3.1.200 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| E-CID TRP Measurement Result | O |  | 9.3.1.x2 |  | YES | ignore |

//Next change//

#### 9.2.12.24	E-CID MEASUREMENT REPORT

This message is sent by gNB-DU to report the results of the requested E-CID measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| E-CID Measurement Result | M |  | 9.3.1.199 |  | YES | ignore |
| Cell Portion ID | O |  | 9.3.1.200 |  | YES | ignore |
| E-CID TRP Measurement Result | O |  | 9.3.1.x2 |  | YES | ignore |

//Next change//

### 9.3.1.x1 	E-CID TRP Measurement Request Info

This information element contains the requested E-CID TRP measurement information.

| IE/Group Name |  | Presence | Range |  |  | IE Type and Reference |  | Semantics Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-CID TRP Measurement Quantities |  | 1 |  |  |
| >E-CID TRP Measurement Quantities Item |  | 0 .. < maxnoMeasE-CID> |  |  |
| >>E-CID Measurement Quantities Value | M |  | ENUMERATED (NR Angle of Arrival, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasE-CID | Maximum no. of E-CID measured quantities that can be configured and reported with one message. Value is 64. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one measurement message. Value is 64. |

### 9.3.1.x2	E-CID TRP Measurement Result

This information element contains the E-CID TRP measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-CID TRP Measurement Response List |  | 1 |  |  |
| >E-CID TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID | M |  | 9.3.1.197 |  |
| >>E-CID Measured Result Item | M | 1 ..< maxnoMeasE-CID> |  |  |
| >>>CHOICE E-CID Measured Results Value | M |  |  |  |
| >>>>UL Angle of Arrival |  |  | 9.3.1.167 |  |
| >>>Time Stamp | M |  | 9.3.1.171 | Time Stamp of the corresponding measured result. |
| >>>Measurement Quality | O |  | 9.3.1.172 | Measurement Quality of the corresponding measured result. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one measurement message. Value is 64. |
| maxnoMeasE-CID | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

//Next change//

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

Omitted text unchanged

SRSReservationType,

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMResetInformation,

E-CID-TRP-Measurement-Request-Info,

E-CID-TRP-Measurement-Result

Omitted text unchanged

id-SRSReservationType,

id-RequestedSRSPreconfigurationCharacteristics-List,

id-SRSPreconfiguration-List,

id-SRSInformation,

id-TAInformation-List,

id-NonIntegerDRXCycle,

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-E-CID-TRP-Measurement-Request-Info,

id-E-CID-TRP-Measurement-Result,

Omitted text unchanged

-- **************************************************************

--

##### -- E-CID Measurement Initiation Request

--

-- **************************************************************

E-CIDMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationRequest-IEs}},

...

}

E-CIDMeasurementInitiationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID			CRITICALITY reject	TYPE LMF-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID			CRITICALITY reject	TYPE RAN-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-E-CID-ReportCharacteristics		CRITICALITY reject	TYPE E-CID-ReportCharacteristics		PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementPeriodicity	CRITICALITY reject	TYPE MeasurementPeriodicity		PRESENCE conditional	}|

-- The above IE shall be present if the E-CID-ReportCharacteristics IE is set to “periodic” –-

{ ID id-E-CID-MeasurementQuantities		CRITICALITY reject	TYPE E-CID-MeasurementQuantities	PRESENCE mandatory}|

{ ID id-PosMeasurementPeriodicityNR-AoA	CRITICALITY reject	TYPE PosMeasurementPeriodicityNR-AoA		PRESENCE conditional}|

-- The IE shall be present if the E-CID-ReportCharacteristics IE is set to “periodic” and the E-CID-MeasurementQuantities-Item IE in the E-CID-MeasurementQuantities IE is set to the value "angleOfArrivalNR"--

{ ID id-E-CID-TRP-Measurement-Request-Info			CRITICALITY ignore	TYPE E-CID-TRP-Measurement-Request-Info		PRESENCE optional},

...

}

-- **************************************************************

--

##### -- E-CID Measurement Initiation Response

--

-- **************************************************************

E-CIDMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationResponse-IEs}},

...

}

E-CIDMeasurementInitiationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE optional}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-E-CID-TRP-Measurement-Result	CRITICALITY ignore	TYPE E-CID-TRP-Measurement-Result		PRESENCE optional},

...

}

Omitted text unchanged

-- **************************************************************

--

#### -- E-CID MEASUREMENT REPORT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Report

--

-- **************************************************************

E-CIDMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementReport-IEs}},

...

}

E-CIDMeasurementReport-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE mandatory }|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional}|

{ ID id-E-CID-TRP-Measurement-Result	CRITICALITY ignore	TYPE E-CID-TRP-Measurement-Result		PRESENCE optional},

...

}

//Next change//

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

Omitted text unchanged

E-CID-MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxnoofMeasE-CID)) OF ProtocolIE-SingleContainer { {E-CID-MeasurementQuantities-ItemIEs} }

E-CID-MeasurementQuantities-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-E-CID-MeasurementQuantities-Item	CRITICALITY reject	TYPE E-CID-MeasurementQuantities-Item		PRESENCE mandatory}

}

E-CID-MeasurementQuantities-Item ::= SEQUENCE {

e-CIDmeasurementQuantitiesValue				E-CID-MeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { E-CID-MeasurementQuantitiesValue-ExtIEs} } OPTIONAL

}

E-CID-MeasurementQuantitiesValue-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasurementQuantitiesValue ::= ENUMERATED {

default,

angleOfArrivalNR,

... ,

timingAdvanceNR

}

E-CID-MeasurementResult ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates 	OPTIONAL,

measuredResults-List		E-CID-MeasuredResults-List 	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL

}

E-CID-MeasurementResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MobileAccessPointLocation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE optional }|

{ ID id-E-CID-MeasuredResultsAssociatedInfoList	CRITICALITY ignore	EXTENSION E-CID-MeasuredResultsAssociatedInfoList		PRESENCE optional},

...

}

E-CID-MeasuredResults-List ::= SEQUENCE (SIZE(1..maxnoofMeasE-CID)) OF E-CID-MeasuredResults-Item

E-CID-MeasuredResults-Item ::= SEQUENCE {

e-CID-MeasuredResults-Value 	E-CID-MeasuredResults-Value,

iE-Extensions			ProtocolExtensionContainer {{ E-CID-MeasuredResults-Item-ExtIEs }}	 OPTIONAL

}

E-CID-MeasuredResults-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasuredResults-Value ::= CHOICE {

valueAngleofArrivalNR	UL-AoA,

choice-extension		ProtocolIE-SingleContainer { { E-CID-MeasuredResults-Value-ExtIEs} }

}

E-CID-MeasuredResults-Value-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-NR-TADV			CRITICALITY ignore	TYPE NR-TADV	PRESENCE mandatory	},

...

}

E-CID-TRP-Measurement-Request-Info ::= SEQUENCE {

eCID-TRPMeasQuantity			E-CID-TRP-Measurement-Quantities,

iE-extensions					ProtocolExtensionContainer { { E-CID-TRP-Measurement-Request-Info-ExtIEs } } OPTIONAL,

...

}

E-CID-TRP-Measurement-Request-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-TRP-Measurement-Quantities ::= SEQUENCE (SIZE (1.. maxnoofMeasE-CID)) OF E-CID-TRP-Measurement-Quantities-Item

E-CID-TRP-Measurement-Quantities-Item ::= SEQUENCE {

eCID-MeasurementQuantityValue	E-CID-MeasurementQuantityValue,

iE-extensions					ProtocolExtensionContainer { { E-CID-TRP-Measurement-Quantities-Item-ExtIEs } } OPTIONAL,

...

}

E-CID-TRP-Measurement-Quantities-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasurementQuantityValue ::= ENUMERATED {

angleOfArrivalNR,

...

}

E-CID-TRP-Measurement-Result ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF E-CID-TRP-MeasurementResponseItem

E-CID-TRP-MeasurementResponseItem ::= SEQUENCE {

tRP-ID							TRPID,

e-CID-measurementResult			E-CID-TrpMeasurementResult,

iE-extensions					ProtocolExtensionContainer { { E-CID-TRP-MeasurementResponseItem-ExtIEs } } OPTIONAL,

...

}

E-CID-TRP-MeasurementResponseItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoofMeasE-CID)) OF E-CID-TrpMeasurementResultItem

E-CID-TrpMeasurementResultItem ::= SEQUENCE {

e-CID-measuredResultsValue			E-CID-TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer {{ E-CID-TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

E-CID-TrpMeasurementResultItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-TrpMeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

choice-extension	ProtocolIE-Single-Container { { E-CID-TrpMeasuredResultsValue-ExtIEs } }

}

E-CID-TrpMeasuredResultsValue-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

//Next change//

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

Omitted text unchanged

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-E-CID-TRP-Measurement-Request-Info				ProtocolIE-ID ::= xxx

id-E-CID-TRP-Measurement-Result						ProtocolIE-ID ::= xxx

END

-- ASN1STOP