3GPP TSG-RAN WG3 Meeting #127	R3-250508

Athens, Greece, February 17th – 21st 2025

Agenda item:	8.3

Source: 	Ericsson

Title: 	MBS Communication Service Type for QMC

Document for:	Agreement

# Discussion

RAN3 received an LS from SA4 in R3-250026, where SA4 informs RAN3 that it included an indication of Communication Service Type in the QMC configuration container sent to the UE. SA4 has done based on the RAN3 agreement from RAN3#125 meeting and the subsequent LS in R3-244789. The indication agreed by SA4 enables the UE to conduct QMC for the correct MBS communication service type.

As an attachment to the LS, SA4 has also disclosed the corresponding agreed CR of TS 26.247 in S4-242152. The agreed indication corresponds to the communicationServiceType attribute, shown below:

| Element or Attribute Name |  | Use | Description |
| --- | --- | --- | --- |
|  | (Skip) |  |  |
|  | @communicationServiceType | OD default= ”all” | When present, this attribute indicates in which communication service type the QoE collection is requested: - The value mbsMulticast refers to the MBS Multicast communication service per clause 21.1 of TS 38.300 [71]. - The value mbsBroadcast refers to the MBS Broadcast communication service per clause 21.1 of TS 38.300 [71]. - The value all refers to all communication service types. When absent, quality metrics collection is requested for all MBS modes. |

We note that the communicationServiceType attribute has three possible values, to indicate the communication service type in which the QoE collection is requested, namely: mbsMulticast, mbsBroadcast, and all. The all codepoint instructs the UE to perform the QoE measurements for any MBS communication service type.

Nevertheless, the corresponding NGAP MBS Communication Service Type IE, has only two codepoints: broadcast and multicast, as shown below. This means that there is a misalignment between the TS 38.413 and TS 26.247.

| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast, ...) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Observation 1: The value “all” of the communicationServiceType attribute in TS 26.247 newly introduced by SA4 has no corresponding codepoint in the MBS Communication Service Type IE in TS 38.413.

If the OAM wants to collect QoE measurements for any MBS communication service type, it will set the value of communicationServiceType attribute to all, but, as per current version of TS 38.413, it will not be able to indicate the same to the NG-RAN node.

Observation 2:  An NG-RAN node cannot configure a UE to collect QoE measurements for all MBS communication service types.

The above misalignment between the TS 26.247 and TS 38.413, needs to be fixed. Hence, to ensure that the QoE configuration information sent to the gNB allows to collect QoE measurements for all MBS communication service type, we propose to add a third codepoint to the encoding of the MBS Communication Service Type IE, with the same meaning as the value “all” of the communicationServiceType attribute.

In addition, RAN3 also introduced the MBS Communication Service Type IE in XnAP, so this IE also needs to be aligned with the communicationServiceType attribute in TS 26.247.

Proposal 1:  Add a codepoint with value “all” to the MBS Communication Service Type IE in TS 38.413 and TS 38.423 and agree the corresponding CRs for TS 38.413 and TS 38.423 in R3-250509 and R3-250510, respectively.

In addition, even if OAM requests QoE measurements for “all” MBS communication service types, the MCE receiving the QoE measurement reports should be able to determine which MBS communication service type a certain QoE report refers to. The easiest way to address this issue may be to add into the QoE reports an indication of whether the reported QoE measurements refer to Multicast, or Broadcast.

Proposal 2:  Send an LS asking SA4 to add to QoE reports an indication of whether the reported QoE measurements refer to MBS Multicast, or MBS Broadcast.

# Conclusions

In this contribution we observe and propose the following:

Observation 1: The value “all” of the communicationServiceType attribute in TS 26.247 newly introduced by SA4 has no corresponding codepoint in the MBS Communication Service Type IE in TS 38.413.

Observation 2:  An NG-RAN node cannot configure a UE to collect QoE measurements for all MBS communication service types.

Proposal 1:  Add a codepoint with value “all” to the MBS Communication Service Type IE in TS 38.413 and TS 38.423 and agree the corresponding CRs for TS 38.413 and TS 38.423 in R3-250509 and R3-250510, respectively.

Proposal 2:  Send an LS asking SA4 to add to QoE reports an indication of whether the reported QoE measurements refer to MBS Multicast, or MBS Broadcast.